PART 145--REPAIR STATIONS

Special Federal Aviation Regulations

SFAR No. 36 [Note]

Subpart A--General

Sec.

145.1  Applicability.

145.2  Performance of maintenance, preventive maintenance, alterations and

required inspections for an air carrier or commercial operator under the

continuous airworthiness requirements of Parts 121 and 127, and for

airplanes under the inspection program required by Part 125.

145.3  Certificate required.

145.11  Application and issue.

145.13  Certification of foreign repair stations: Special requirements.

145.15  Change or renewal of certificates.

145.17  Duration of certificates.

145.19  Display of certificate.

145.21  Change of location or facilities.

145.23  Inspection.

145.25  Advertising.

Subpart B--Domestic Repair Stations

145.31  Ratings.

145.33  Limited ratings.

145.35  Housing and facility requirements.

145.37  Special housing and facility requirements.

145.39  Personnel requirements.

145.41  Recommendation of persons for certification as repairmen.

145.43  Records of supervisory and inspection personnel.

145.45  Inspection systems.

145.47  Equipment and materials: Ratings other than limited ratings.

145.49  Equipment and materials: Limited rating.

145.51  Privileges of certificates.

145.53  Limitations of certificates.

145.55  Maintenance of personnel, facilities equipment, and materials.

145.57  Performance standards.

145.59  Inspection of work performed.

145.61  Performance records and reports.

145.63  Reports of defects or unairworthy conditions.

Subpart C--Foreign Repair Stations

145.71  General requirements.

145.73  Scope of work authorized.

145.75  Personnel.

145.77  General operating rules.

145.79  Records and reports.

Subpart D--Limited Ratings for Manufacturers

145.101  Application and issue.

145.103  Privileges of certificates.

145.105  Performance standards.

Appendix A to Part 145

Authority: Secs. 313, 314, 601, and 607, 72 Stat. 752; 49 U.S.C. 1354(a),

1355, 1421, and 1427, unless otherwise noted.

Source: Docket No. 1157, 27 FR 6662, July 13, 1962, unless otherwise noted.

Editorial Note: For miscellaneous technical amendments in this Part 145,

see Amdt. 145-3, 31 FR 3336, Mar. 3, 1966 and Amdt. 145-6, 31 FR 9211, July

6, 1966.

Special Federal Aviation Regulation

SFAR No. 36

Editorial Note: For the text of SFAR No. 36, see Part 121 of this chapter.

*****************************************************************************

59 FR 3936, No. 18, Jan. 27, 1994

SFAR Amdt. No. 36-6

SUMMARY: This final rule amends and extends Special Federal Aviation

Regulation (SFAR) No. 36, which provides that authorized repair station and

aircraft operating certificate holders may approve aircraft products or

articles for return to service after accomplishing major repairs using self-

developed repair data that have not been approved by the Federal Aviation

Administration (FAA). Amendments include clarification of the scope of the

SFAR authorization. Extension of the regulation continues to provide, for

those that qualify, an alternative from the requirement to obtain FAA

approval of major repair data on a case-by-case basis, and allows additional

time for the FAA to incorporate the SFAR provisions into the regulations.

DATES: Effective January 23, 1994 and terminates January 23, 1999.

*****************************************************************************

Subpart A--General

Sec. 145.1   Applicability.

(a) This part prescribes the requirements for issuing repair station

certificates and associated ratings to facilities for the maintenance and

alteration of airframes, powerplants, propellers, or appliances, and

prescribes the general operating rules for the holders of those certificates

and ratings.

(b) A certificated repair station located in the United States is called a

"domestic repair station". A repair station located outside of the United

States is called a "foreign repair station".

(c) A manufacturer of aircraft, aircraft engines, propellers, appliances,

or parts thereof, may be issued a Repair Station Certificate with a limited

rating under Subpart D of this part. Sections 145.11 through 145.79 do not

apply to applicants for, or holders of, certificates issued under Subpart D

of this part. Any facility where the holder of a certificate issued under

Subpart D of this part exercises his privileges under that certificate may be

referred to as a "manufacturer's maintenance facility."

[Doc. No. 1157, 27 FR 6662, July 13, 1962, as amended by Amdt. 145-4, 31 FR

5249, Apr. 1, 1966]

Sec. 145.2   Performance of maintenance, preventive maintenance, alterations

and required inspections for an air carrier or commercial operator under

the continuous airworthiness requirements of Parts 121 and 127, and for

airplanes under the inspection program required by Part 125.

(a) Each repair station that performs any maintenance, preventive

maintenance, alterations, or required inspections for an air carrier or

commercial operator having a continuous airworthiness program under Part 121

or Part 127 of this chapter shall comply with Subpart L of Part 121 (except

Secs. 121.363, 121.369, 121.373, and 121.379) or Subpart I of Part 127

(except Secs. 127.131, 127.134, 127.136, and 127.140) of this chapter, as

applicable. In addition, such repair station shall perform that work in

accordance with the air carrier's or commercial operator's manual.

(b) Each repair station that performs inspections on airplanes governed by

Part 125 of this chapter shall do that work in accordance with the inspection

program approved for the operator of the airplane.

[Amdt. 145-7, 31 FR 10614, Aug. 9, 1966, as amended by Amdt. 145-17, 45 FR

67235, Oct. 9, 1980]

Sec. 145.3   Certificate required.

No person may operate as a certificated repair station without, or in

violation of, a repair station certificate. In addition, an applicant for a

certificate may not advertise as a certificated repair station until the

certificate has been issued to him.

Sec. 145.11   Application and issue.

(a) An application for a repair station certificate and rating, or for an

additional rating, is made on a form and in a manner prescribed by the

Administrator, and submitted with duplicate copies of--

(1) [Reserved]

(2) Its inspection procedures manual;

(3) A list of the maintenance functions to be performed for it, under

contract, buy another agency under Sec. 145.49 or Appendix A; and

(4) In the case of an applicant for a propeller rating (class 2) or any

accessory rating (class 1, 2, or 3), a list, by type or make, as applicable,

of the propeller or accessory for which he seeks approval.

(b) An applicant who meets the requirements of this part is entitled to a

repair station certificate with appropriate ratings prescribing such

operations specifications and limitations as are necessary in the interests

of safety.

[Doc. No. 1157, 27 FR 6662, July 13, 1962, as amended by Amdt. 145-5, 31 FR

8585, June 21, 1966]

Sec. 145.13   Certification of foreign repair stations: Special requirements.

Before applying under Sec. 145.11, an applicant for a foreign repair

station certificate must notify the FAA office having jurisdiction over the

area in which the applicant is located of his intention to so apply and send

that office a statement of his reasons for wanting a repair station at his

place of business. In addition to the information required by Sec. 145.11,

the applicant must furnish two copies of a suitably bound brochure, including

a physical description of his facilities (with photographs), a description of

his inspection system, and organizational chart, the names and titles of

managing and supervisory personnel, and a list of services obtained under

contract, if any, with the names of the contractors and the types of services

they perform. In addition, the applicant must furnish evidence that the fee

prescribed by Appendix A of Part 187 of this chapter has been paid.

[Doc. No. 1157, 27 FR 6662, July 13, 1962, as amended by Amdt. 145-20, 47 FR

35694, Aug. 16, 1982]

Sec. 145.15   Change or renewal of certificates.

(a) Each of the following requires the certificate holder to apply for a

change in a repair station certificate, on a form and in the manner

prescribed by the Administrator:

(1) A change in the location or housing and facilities of the station.

(2) A request to revise or amend a rating.

(b) If the holder of a repair station certificate sells or transfers its

assets, the new owner must apply for an amended certificate, in the manner

prescribed in Sec. 145.11 and, if applicable, Sec. 145.13.

(c) A person requesting renewal of a foreign repair station certificate

shall, within 30 days before his current certificate expires, send the

request to the FAA office having jurisdiction over the station. If he does

not make the request within that period, he must follow the procedure

prescribed in Sec. 145.13 for applying for a new certificate, but without

copies of the brochure.

[Doc. No. 1157, 27 FR 6662, July 13, 1962, as amended by Amdt. 145-8, 32 FR

15670, Nov. 14, 1967]

Sec. 145.17   Duration of certificates.

(a) A domestic repair station certificate or rating is effective until it

is surrendered, suspended, or revoked.

(b) A foreign repair station certificate or rating expires at the end of 12

months after the date on which it was issued, unless it is sooner

surrendered, suspended, or revoked. However, if the station continues to

comply with Sec. 145.71 and applies for renewal before expiration of such

certificate or rating, its certificate or rating may be renewed for 24

months.

(c) The holder of a certificate that expires or is surrendered, suspended,

or revoked, shall return it to the Administrator.

[Doc. No. 1157, 27 FR 6662, July 13, 1962, as amended by Amdt. 145-16, 43 FR

22643, May 25, 1978]

Sec. 145.19   Display of certificate.

Each holder of a repair station certificate shall display the certificate

and ratings at a place in the repair station that is normally accessible to

the public and is not obscured. The certificate must be available for

inspection by the Administrator.

Sec. 145.21   Change of location or facilities.

(a) The holder of a repair station certificate may not make any change in

its location or in its housing and facilities that are required by Sec.

145.35, unless the change is approved in writing in advance.

(b) The Administrator may prescribe the conditions under which a repair

station may operate while it is changing its location or housing facilities.

Sec. 145.23   Inspection.

Each certificated repair station shall allow the Administrator to inspect

it, at any time, to determine its compliance with this part. The inspections

cover the adequacy of the repair stations inspection system, records, and its

general ability to comply with this part. After such an inspection is made,

the repair station is notified, in writing, of any defects found during the

inspection.

[Amdt. 145-5, 31 FR 8585, June 21, 1966]

Sec. 145.25   Advertising.

(a) Whenever the advertising of a certificated repair station indicates

that it is certificated, it must clearly state its certificate number.

(b) Paragraph (a) of this section applies to advertising in--

(1) Business letterheads;

(2) Billheads and statements;

(3) Customer estimates and inspection forms;

(4) Hangar or shop signs;

(5) Magazines, periodicals, or trade journals; or

(6) Any form of promotional media.

Subpart B--Domestic Repair Stations

Sec. 145.31   Ratings.

The following ratings are issued under this subpart:

(a) Airframe ratings. (1) Class 1: Composite construction of small

aircraft.

(2) Class 2: Composite construction of large aircraft.

(3) Class 3: All-metal construction of small aircraft.

(4) Class 4: All-metal construction of large aircraft.

(b) Powerplant ratings. (1) Class 1: Reciprocating engines of 400

horsepower or less.

(2) Class 2: Reciprocating engines of more than 400 horsepower.

(3) Class 3: Turbine engines.

(c) Propeller ratings. (1) Class 1: All fixed pitch and ground adjustable

propellers of wood, metal, or composite construction.

(2) Class 2: All other propellers, by make.

(d) Radio ratings. (1) Class 1: Communication equipment: Any radio

transmitting equipment or receiving equipment, or both, used in aircraft to

send or receive communications in flight, regardless of carrier frequency or

type of modulation used; including auxiliary and related aircraft interphone

systems, amplifier systems, electrical or electronic inter-crew signaling

devices, and similar equipment; but not including equipment used for

navigation of the aircraft or as an aid to navigation, equipment for

measuring altitude or terrain clearance, other measuring equipment operated

on radio or radar principles, or mechanical, electrical, gyroscopic, or

electronic instruments that are a part of communications radio equipment.

(2) Class 2: Navigational equipment: Any radio system used in aircraft for

en route or approach navigation, except equipment operated on radar or pulsed

radio frequency principles, but not including equipment for measuring

altitude or terrain clearance or other distance equipment operated on radar

or pulsed radio frequency principles.

(3) Class 3: Radar equipment: Any aircraft electronic system operated on

radar or pulsed radio frequency principles.

(e) Instrument ratings. (1) Class 1: Mechanical: Any diaphragm, bourdon

tube, aneroid, optical, or mechanically driven centrifugal instrument that is

used on aircraft or to operate aircraft, including tachometers, airspeed

indicators, pressure gauges drift sights, magnetic compasses, altimeters, or

similar mechanical instruments.

(2) Class 2: Electrical: Any self-synchronous and electrical indicating

instruments and systems, including remote indicating instruments, cylinder

head temperature gauges, or similar electrical instruments.

(3) Class 3: Gyroscopic: Any instrument or system using gyroscopic

principles and motivated by air pressure or electrical energy, including

automatic pilot control units, turn and bank indicators, directional gyros,

and their parts, and flux gate and gyrosyn compasses.

(4) Class 4: Electronic: Any instruments whose operation depends on

electron tubes, transistors, or similar devices including capacitance type

quantity gauges, system amplifiers, and engine analyzers.

(f) Accessory ratings. (1) Class 1: Mechanical accessories that depend on

friction, hydraulics, mechanical linkage, or pneumatic pressure for

operation, including aircraft wheel brakes, mechanically driven pumps,

carburetors, aircraft wheel assemblies, shock absorber struts and hydraulic

servo units.

(2) Class 2: Electrical accessories that depend on electrical energy for

their operation, and generators, including starters, voltage regulators,

electric motors, electrically driven fuel pumps magnetos, or similar

electrical accessories.

(3) Class 3: electronic accessories that depend on the use of an electron

tube transistor, or similar device, including supercharger, temperature, air

conditioning controls, or similar electronic controls.

Sec. 145.33   Limited ratings.

(a) Whenever the Administrator finds it appropriate, he may issue a limited

rating to a domestic repair station that maintains or alters only a

particular type of airframe, powerplant, propeller, radio, instrument, or

accessory, or parts thereof, or performs only specialized maintenance

requiring equipment and skills not ordinarily found in regular repair

stations. Such a rating may be limited to a specific model aircraft, engine,

or constituent part, or to any number of parts made by a particular

manufacturer.

(b) Limited ratings are issued for--

(1) Airframes of a particular make and model;

(2) Engines of a particular make and model;

(3) Propellers of a particular make and model;

(4) Instruments of a particular make and model;

(5) Radio equipment of a particular make and model;

(6) Accessories of a particular make and model;

(7) Landing gear components;

(8) Floats, by make;

(9) Nondestructive inspection, testing, and processing;

(10) Emergency equipment;

(11) Rotor blades, by make and model;

(12) Aircraft fabric work; and

(13) Any other purpose for which the Administrator finds the applicant's

request is appropriate.

(c) For a limited rating for specialized services, the operations

specifications of the station shall contain the specification used in

performing that specialized service. The specification may either be a civil

or military one that is currently used by industry and approved by the

Administrator or one developed by the applicant and approved by the

Administrator.

Sec. 145.35   Housing and facility requirements.

(a) An applicant for a domestic repair station certificate and rating, or

for an additional rating, must comply with paragraphs (b) to (h) of this

section and provide suitable--

(1) Housing for its necessary equipment and material;

(2) Space for the work for which it seeks a rating;

(3) Facilities for properly storing, segregating, and protecting materials,

parts, and supplies; and

(4) Facilities for properly protecting parts and subassemblies during

disassembly, cleaning, inspection, repair, alteration, and assembly;

so that work being done is protected from weather elements, dust, and heat;

workers are protected so that the work will not be impaired by their physical

efficiency; and maintenance operations have efficient and proper facilities.

(b) The applicant must provide suitable shop space where machine tools and

equipment are kept and where the largest amount of bench work is done. The

shop space need not be partitioned but machines and equipment must be

segregated whenever--

(1) Machine or woodwork is done so near an assembly area that chips or

material might inadvertently fall into assembled or partially assembled work;

(2) Unpartitioned parts cleaning units are near other operations;

(3) Fabric work is done in an area where there are oils and greases;

(4) Painting or spraying is done in an area so arranged that paint or paint

dust can fall on assembled or partially assembled work;

(5) Paint spraying, cleaning, or machining operations are done so near

testing operations that the precision of test equipment might be affected;

and

(6) In any other case the Administrator determines it is necessary.

(c) The applicant must provide suitable assembly space in an enclosed

structure where the largest amount of assembly work is done. The assembly

space must be large enough for the largest item to be worked on under the

rating he seeks and must meet the requirements of paragraph (a) of this

section.

(d) The applicant must provide suitable storage facilities used exclusively

for storing standard parts, spare parts, and raw materials, and separated

from shop and working space. He must organize the storage facilities so that

only acceptable parts and supplies will be issued for any job, and must

follow standard good practices for properly protecting stored materials.

(e) The applicant must store and protect parts being assembled or

disassembled, or awaiting assembly or disassembly, to eliminate the

possibility of damage to them.

(f) The applicant must provide suitable ventilation for his shop, assembly,

and storage areas so that the physical efficiency of his workers is not

impaired.

(g) The applicant must provide adequate lighting for all work being done so

that the quality of the work is not impaired.

(h) The applicant must control the temperature of the shop and assembly

area so that the quality of the work is not impaired. Whenever special

maintenance operations are being performed, such as fabric work or painting,

the temperature and humidity control must be adequate to insure the

airworthiness of the article being maintained.

Sec. 145.37   Special housing and facility requirements.

(a) In addition to the housing and facility requirements in Sec. 145.35, an

applicant for a domestic repair station certificate and rating, or for an

additional rating, for airframes, powerplants, propellers, instruments,

accessories, or radios must meet the requirements of paragraphs (b) to (f) of

this section.

(b) An applicant for an airframe rating must provide suitable permanent

housing for at least one of the heaviest aircraft within the weight class of

the rating he seeks. If the location of the station is such that climatic

conditions allow work to be done outside, permanent work docks may be used if

they meet the requirements of Sec. 145.35(a).

(c) An applicant for either a powerplant or accessory rating must provide

suitable trays, racks, or stands for segregating complete engine or accessory

assemblies from each other during assembly and disassembly. He must provide

covers to protect parts awaiting assembly or during assembly to prevent dust

or other foreign objects from entering into or falling on those parts.

(d) An applicant for a propeller rating must provide suitable stands,

racks, or other fixtures for the proper storage of propellers after being

worked on.

(e) An applicant for a radio rating must provide suitable storage

facilities to assure the protection of parts and units that might deteriorate

from dampness or moisture.

(f) An applicant for an instrument rating must provide a reasonably dust

free shop if the shop allocated to final assembly is not air conditioned.

Shop and assembly areas must be kept clean at all times to reduce the

possibility of dust or other foreign objects getting into instrument

assemblies.

Sec. 145.39   Personnel requirements.

(a) An applicant for a domestic repair station certificate and rating, or

for an additional rating, must provide adequate personnel who can perform,

supervise, and inspect the work for which the station is to be rated. The

officials of the station must carefully consider the justifications and

abilities of their employees and shall determine the abilities of its

uncertificated employees performing maintenance operations on the basis of

practical tests or employment records. The repair station is primarily

responsible for the satisfactory work of its employees.

(b) The number of repair station employees may vary according to the type

and volume of its work. However, the applicant must have enough properly

qualified employees to keep up with the volume of work in process, and may

not reduce the number of its employees below that necessary to efficiently

produce airworthy work.

(c) Each repair station shall determine the abilities of its supervisors

and shall provide enough of them for all phases of its activities. However,

the Administrator may determine the ability of any supervisor by inspecting

his employment and experience records or by a personal test. Each supervisor

must have direct supervision over working groups but need not have over-all

supervision at management level. Whenever apprentices or students are used in

working groups on assemblies or other operations that might be critical to

the aircraft, the repair station shall provide at least one supervisor for

each 10 apprentices or students, unless the apprentices or students are

integrated into groups of experienced workers.

(d) Each person who is directly in charge of the maintenance functions of a

repair station must be appropriately certificated as a mechanic or repairman

under Part 65 of this chapter and must have had at least 18 months of

practical experience in the procedures, practices, inspection methods,

materials, tools, machine tools, and equipment generally used in the work for

which the station is rated. Experience as an apprentice or student mechanic

may not be counted in computing the 18 months of experience. In addition, at

least one of the persons so in charge of maintenance functions for a station

with an airframe rating must have had experience in the methods and

procedures prescribed by the Administrator for returning aircraft to service

after 100-hour, annual, and progressive inspections.

(e) Each limited repair station shall have employees with detailed

knowledge of the particular maintenance function or technique for which it is

rated, based on attending a factory school or long experience with the

product or technique involved.

Sec. 145.41  Recommendation of persons for certification as repairmen.

(a) When a person applies for a domestic repair station certificate and

rating(s) or additional rating(s) that require a repairman, that person

must--

(1) Recommend at least one person for certification as a repairman;

(2) Certify to the Administrator that the person recommended meets the

requirements of Sec. 65.101 of this chapter; and

(3) Certify that the person recommended is able to perform and supervise

the assigned work.

(b) Each person recommended per paragraph (a)(1) of this section must be at

or above the level of shop foreman or department head or be responsible for

supervising the work performed by the repair station. A qualified person so

recommended may be certificated as a repairman.

(Secs. 313, 314, and 601 through 610, of the Federal Aviation Act of 1958, as

amended (49 U.S.C. 1354, 1355, 1421 through 1430); sec. 6(c), Dept. of

Transportation Act (49 U.S.C. 1655(c)))

[Doc. No. 21269, 47 FR 33390, Aug. 2, 1982]

Sec. 145.43   Records of supervisory and inspection personnel.

(a) Each applicant for a domestic repair station certificate and rating, or

for an additional rating, must have, and each certificated domestic repair

station shall maintain, a roster of--

(1) Its supervisory personnel, including the names of the officials of the

station that are responsible for its management and the names of its

technical supervisors, such as foreman and crew chiefs; and

(2) Its inspection personnel, including the names of the chief inspector

and those inspectors who make final airworthiness determinations before

releasing an article to service.

(b) The station shall also provide a summary of the employment of each

person whose name is on the roster. The summary must contain enough

information as to each person on the roster to show compliance with the

experience requirements of this subpart, including--

(1) His present title (e.g., chief inspector, metal shop foreman, etc.);

(2) His total years of experience in the type of work he is doing;

(3) His past employment record, with names of places and term of employment

by month, and year;

(4) The scope of his present employment (e.g., airframe overhaul, airframe

final assembly, engine inspection, department, etc.); and

(5) The type and number of the mechanic or repairman certificate that he

holds, and the ratings on that certificate.

(c) The station shall change the roster, as necessary, to reflect--

(1) Terminating the employment of any person whose name is on the roster;

(2) Assigning any person to duties that require his name to be carried on

the roster; or

(3) Any appreciable change in the duties and scope of assignment of any

person whose name is on the roster.

(d) The station shall keep the roster and employment summaries required by

this section, subject to inspection by the Administrator upon his request.

(e) A domestic repair station may not use the services of a person directly

in charge of maintenance or alteration unless it keeps current records on him

as required by this section.

[Doc. No. 1157, 27 FR 6662, June 13, 1962, as amended by Amdt. 145-5, 31 FR

8585, June 21, 1966; Amdt. 145-15, 41 FR 47230, Oct. 28, 1976]

Sec. 145.45   Inspection systems.

(a) An applicant for a repair station certificate, and rating or for an

additional rating, must have an inspection system that will produce

satisfactory quality control and conform to paragraphs (b) to (f) of this

section.

(b) The applicant's inspection personnel must be thoroughly familiar with

all inspection methods, techniques, and equipment used in their specialty to

determine the quality or airworthiness of an article being maintained or

altered. In addition, they must--

(1) Maintain proficiency in using various inspection aids intended for that

purpose;

(2) Have available and understand current specifications involving

inspection tolerances, limitations, and procedures established by the

manufacturer of the product being inspected and with other forms of

inspection information such as FAA airworthiness directives and bulletins;

and

(3) In cases where magnetic, fluorescent, or other forms of mechanical

inspection devices are to be used, be skilled in operating that equipment and

be able to properly interpret defects indicated by it.

(c) The applicant must provide a satisfactory method of inspecting incoming

material to insure that, before it is placed in stock for use in an aircraft

or part thereof, it is in a good state of preservation and is free from

apparent defects or malfunctions.

(d) The applicant must provide a system of preliminary inspection of all

articles he maintains to determine the state of preservation or defects. He

shall enter the results of each inspection on an appropriate form supplied by

it and keep the form with the article until it is released to service.

(e) The applicant must provide a system so that before working on any

airframe, powerplant, or part thereof that has been involved in an accident,

it will be inspected thoroughly for hidden damage, including the areas next

to the obviously damaged parts. He shall enter the results of this inspection

on the inspection form required by paragraph (d) of this section.

(f) At the time he applies for a repair station certificate, the applicant

must provide a manual containing inspection procedures, and thereafter

maintain it in current condition at all times. The manual must explain the

internal inspection system of the repair station in a manner easily

understood by any employee of the station. It must state in detail the

inspection requirements in paragraphs (a) to (e) of this section, and the

repair station's inspection system including the continuity of inspection

responsibility, samples of inspection forms, and the method of executing

them. The manual must refer whenever necessary to the manufacturer's

inspection standards for the maintenance of the particular article. The

repair station must give a copy of the manual to each of its supervisory and

inspection personnel and make it available to its other personnel. The repair

station is responsible for seeing that all supervisory and inspection

personnel thoroughly understand the manual.

[Doc. No. 1157, 27 FR 6662, June 13, 1962, as amended by Amdt. 145-15, 41 FR

47230, Oct. 28, 1976]

Sec. 145.47   Equipment and materials: Ratings other than limited ratings.

(a) An applicant for a domestic repair station certificate and rating, or

for an additional rating, must have the equipment and materials necessary to

efficiently perform the functions appropriate to the ratings he seeks. An

applicant for an airframe, propeller, powerplant, radio, instrument, or

accessory rating must be equipped to perform the functions listed in Appendix

A to this part that are appropriate for the rating he seeks.

(b) The equipment and materials required by this part must be of such type

that the work for which they are being used can be done competently and

efficiently. The station shall ensure that all inspection and test equipment

is tested at regular intervals to ensure correct calibration to a standard

derived from the National Bureau of Standards or to a standard provided by

the equipment manufacturer. In the case of foreign equipment, the standard of

the country of manufacture may be used if approved by the Administrator. The

equipment and materials required for the various ratings must be located on

the premises, and under the full control of the station, unless they are used

for a function that the repair station is authorized to obtain by contract.

If it obtains them by contract, the repair station shall determine the

airworthiness of the article involved, unless the contractor is an

appropriately rated repair station.

(c) A certificated domestic or foreign repair station may contract

maintenance and alteration of components of a type certificated product to a

noncertificated source identified in the repair station's inspection

procedures manual provided:

(1) The repair station is the manufacturer who originally manufactured the

product for which it holds a U.S. type certificate;

(2) The contracted component is included as part of the type certificated

product;

(3) The component maintenance is done by the original component

manufacturer or its manufacturing licensee; and

(4) Before such a component is returned to service, the repair station

ensures that it is being returned to service in accordance with the repair

station's quality control system as approved by the Administrator and set

forth in the repair station's operations specifications and inspection

procedures manual.

(d) The applicant shall choose suitable tools and equipment for the

functions named in Appendix A to this part, as appropriate to each of his

ratings, using those the manufacturer of the article involved recommends for

maintaining or altering that article, or their equivalent.

(Secs. 313, 314, and 601 through 610, of the Federal Aviation Act of 1958, as

amended (49 U.S.C. 1354, 1355, 1421 through 1430); sec. 6(c), Dept. of

Transportation Act (49 U.S.C. 1655(c)))

[Doc. No. 1157, 27 FR 6662, July 13, 1962, as amended by Amdt. 145-19, 47 FR

33391, Aug. 2, 1982; Amdt. 145-21, 53 FR 47375, Nov. 22, 1988]

Sec. 145.49   Equipment and materials: Limited rating.

(a) An applicant for a limited rating (other than specialized services)

under Sec. 145.33, must have the equipment and materials to perform any job

function appropriate to the rating and class specified in Sec. 145.47 for the

rating he seeks. However, he need not be equipped for a function that does

not apply to the particular make or model article for which he seeks a

rating, if he shows that it is not necessary under the recommendations of the

manufacturer of the article.

(b) An applicant for a rating for specialized services or techniques under

Sec. 145.33 must--

(1) For magnetic and penetrant inspection, have the equipment and materials

for wet and dry magnetic inspection techniques, residual and continuous

methods, and portable equipment for the inspection of welds both on and off

the aircraft;

(2) For emergency equipment maintenance, have the equipment and materials

to perform inspections, repairs, and tests of all kinds of inflated

equipment, the re-packing, re-marking, re-sealing, and re-stocking of life

rafts, and the weighing, refilling, and testing of carbon dioxide fire

extinguishers and oxygen containers;

(3) For rotor blade maintenance, have the equipment, materials, and

technical data recommended by the manufacturer; and

(4) For aircraft fabric work, have the equipment and materials to apply

protective coatings to structures, machine stitch fabric panels, perform

covering, sewing, and rib stitching operations, apply dope and paint using

temperature and humidity control equipment, install patches, grommets, tapes,

hooks, and similar equipment, and refinish entire aircraft and aircraft

parts.

Sec. 145.51   Privileges of certificates.

A certificated domestic repair station may--

(a) Maintain or alter any airframe, powerplant, propeller, instrument,

radio, or accessory, or part thereof, for which it is rated;

(b) Approve for return to service any article for which it is rated after

it has been maintained or altered;

(c) In the case of a station with an airframe rating, perform 100-hour,

annual or progressive inspections, and return the aircraft to service; and

(d) Maintain or alter any article for which it is rated at a place other

than the repair station, if--

(1) The function would be performed in the same manner as when performed at

the repair station and in accordance with Secs. 145.57 to 145.61;

(2) All necessary personnel, equipment, material, and technical data is

available at the place where the work is to be done; and

(3) The inspection procedures manual of the station sets forth approved

procedures governing work to be performed at a place other than the repair

station.

However, a certificated repair station may not approve for return to service

any aircraft, airframe, aircraft engine, propeller, or appliance after major

repair or major alteration unless the work was done in accordance with

technical data approved by the Administrator.

[Doc. No. 1157, 27 FR 6662, July 13, 1962, as amended by Amdt. 145-2, 29 FR

5451, Apr. 23, 1964]

Sec. 145.53   Limitations of certificates.

A certificated domestic repair station may not maintain or alter any

airframe, powerplant, propeller, instrument, radio, or accessory for which it

is not rated, and may not maintain or alter any article for which it is rated

if it requires special technical data, equipment, or facilities that are not

available to it.

Sec. 145.55   Maintenance of personnel, facilities, equipment, and materials.

Each certificated domestic repair station shall provide personnel,

facilities equipment, and materials at least equal in quality and quantity to

the standards currently required for the issue of the certificate and rating

that it holds.

Sec. 145.57   Performance standards.

(a) Except as provided in Sec. 145.2, each certificated domestic repair

station shall perform its maintenance and alteration operations in accordance

with the standards in Part 43 of this chapter. It shall maintain, in current

condition, all manufacturers' service manuals, instructions, and service

bulletins that relate to the articles that it maintains or alters.

(b) In addition, each certificated domestic repair station with a radio

rating shall comply with those sections of Part 43 of this chapter that apply

to electric systems, and shall use materials that conform to approved

specifications for equipment appropriate to its rating. It shall use test

apparatus, shop equipment, performance standards, test methods, alterations,

and calibrations that conform to the manufacturers' specifications or

instructions, approved specification, and, if not otherwise specified, to

accept good practices of the aircraft radio industry.

[Doc. No. 1157, 27 FR 6662, July 13, 1962, as amended by Amdt. 145-5, 31 FR

8585, June 21, 1966; Amdt. 145-7, 31 FR 10614, Aug. 9, 1966]

Sec. 145.59   Inspection of work performed.

(a) Each certificated domestic repair station shall, before approving an

airframe, powerplant, propeller, instrument, radio, or accessory for return

to service after maintaining or altering it, have that article inspected by a

qualified inspector. After performing a maintenance or alteration operation,

the station shall certify on the maintenance or alteration record of the

article that it is airworthy with respect to the work performed.

(b) For the purposes of paragraph (a) of this section, the qualified

inspector must be a person employed by the station, who has shown by

experience as a journeyman that he understands the inspection methods,

techniques, and equipment used in determining the air-worthiness of the

article concerned. He must also be proficient in using various types of

mechanical and visual inspection aids appropriate for the article being

inspected.

[Doc. No. 1157, 27 FR 6662, July 13, 1962, as amended by Amdt. 145-16, 43 FR

22643, May 25, 1978]

Sec. 145.61   Performance records and reports.

Each certificated domestic repair station shall maintain adequate records

of all work that it does, naming the certificated mechanic or repairman who

performed or supervised the work, and the inspector of that work. The station

shall keep each record for at least two years after the work it applies to is

done.

Sec. 145.63   Reports of defects or unairworthy conditions.

(a) Each certificated domestic repair station shall report to the

Administrator within 72 hours after it discovers any serious defect in, or

other recurring unairworthy condition of, an aircraft, powerplant, or

propeller, or any component of any of them. The report shall be made on a

form and in a manner prescribed by the Administrator, describing the defect

or malfunction completely without withholding any pertinent information.

(b) In any case where the filing of a report under paragraph (a) of this

section might prejudice the repair station, it shall refer the matter to the

Administrator for a determination as to whether it must be reported. If the

defect or malfunction could result in an imminent hazard to flight, the

repair station shall use the most expeditious method it can to inform the

Administrator.

(c) The holder of a domestic repair station certificate that is also the

holder of a part 121, 127, or 135 certificate, a Type Certificate (including

a Supplemental Type Certificate), a Parts Manufacturer Approval (PMA), or a

TSO authorization, or that is the licensee of a Type Certificate, need not

report a failure, malfunction, or defect under this section if the failure,

malfunction, or defect has been reported by it, under Sec. 21.3, Sec. 37.17,

Sec. 121.703, Sec. 127.313, or Sec. 135.57 of this chapter.

[Doc. No. 1157, 27 FR 6662, July 13, 1962, as amended by Amdt. 145-9, 35 FR

3155, Feb. 19, 1970; Amdt. 145-13, 35 FR 18189, Nov. 28, 1970]

Subpart C--Foreign Repair Stations

Sec. 145.71   General requirements.

A repair station certificate with appropriate ratings may be issued for a

foreign repair station if the Administrator determines that it will be

necessary for maintaining or altering United States registered aircraft and

aircraft engines, propellers, appliances, and component parts thereof for use

on United States registered aircraft. A foreign repair station must meet the

requirements for a domestic repair station certificate, except those in Secs.

145.39 through 145.43.

[Doc. No. 25454, 53 FR 47376, Nov. 22, 1988]

Sec. 145.73   Scope of work authorized.

(a) A certificated foreign repair station may, with respect to United

States registered aircraft, maintain or alter aircraft, airframes,

powerplants, propellers, or component parts thereof. The Administrator may

prescribe operations specifications containing limitations that the

Administrator determines necessary to comply with the airworthiness

requirements of this chapter.

(b) A certificated foreign repair station may perform only the specific

services and functions within the ratings and classes that are stated in its

operations specifications.

[Doc. No. 25454, 53 FR 47376, Nov. 22, 1988]

Sec. 145.75   Personnel.

(a) Each applicant for a foreign repair station certificate and rating, or

for an additional rating, must provide enough personnel who are able to

perform, supervise, and inspect the work for which he seeks a rating, with

regard being given to its volume of work.

(b) The supervisors and inspectors of each certificated foreign repair

station must understand the regulations in this chapter, FAA airworthiness

directives, and the maintenance and service instructions of the manufacturers

of the articles to be worked on. However, they do not need airman

certificates issued under this chapter and, along with the persons performing

the work of the station, are not considered to be airmen within the meaning

of section 101(7) of the Federal Aviation Act of 1958 (49 U.S.C. 1301) with

respect to work performed in connection with their employment by the foreign

repair station.

(c) In cases where the persons engaged in supervision or final inspection

are not certificated under this chapter or by the country in which the

station is located, their qualifications are determined by the Administrator,

based on their ability to meet the requirements of paragraph (a) of this

section as shown by oral or practical test or any other method the

Administrator elects.

(d) No person may be responsible for the supervision or final inspection of

work on an aircraft of United States registry at a foreign repair station

unless he can read, write, and understand English.

Sec. 145.77   General operating rules.

Each certificated foreign repair station shall comply with the operating

rules prescribed in Subpart B of this part, except for Secs. 145.61 and

145.63, and has the privileges of a domestic repair station as provided in

Sec. 145.51.

Sec. 145.79   Records and reports.

(a) Each certificated foreign repair station shall maintain such records,

and make such reports, with respect to United States registered aircraft, as

the Administrator finds necessary, including those prescribed in paragraphs

(b) and (c) of this section.

(b) Each certificated foreign repair station shall keep a record of the

maintenance and alteration it performs on United States registered aircraft,

in enough detail to show the make, model, identification number, and serial

number of the aircraft involved, and a description of the work. In a case of

major repairs or major alterations, or both, it shall report on a form and in

a manner prescribed by the Administrator, giving the original copy to the

aircraft owner and sending a copy to the Administrator through the FAA office

having jurisdiction over the station. However, if a major repair or

alteration is made on a United States scheduled flag air carrier aircraft,

the report may be made in the log or other record provided by the carrier for

that purpose. Upon request, the station shall make all of its maintenance and

alteration records available to the Administrator.

(c) Each certificated foreign repair station shall, within 72 hours after

it discovers any serious defect in, or other recurring unairworthy condition

of, any aircraft, powerplant, propeller, or any component of any of them,

that it works on under this part, report that defect or unairworthy condition

to the Administrator.

(d) The holder of a foreign repair station certificate that is also the

holder of a Type Certificate (including a Supplemental Type Certificate), a

Parts Manufacturer Approval (PMA), or a TSO authorization or that is the

licensee of a Type Certificate need not report a failure, malfunction, or

defect under this section if the failure, malfunction, or defect has been

reported by it, under Sec. 21.3 of this chapter or Sec. 37.17 of this

chapter.

[Doc. No. 1157, 27 FR 6662, July 13, 1962, as amended by Amdt. 145-9, 35 FR

3155, Feb. 19, 1970; Amdt. 145-13, 35 FR 18189, Nov. 28, 1970]

Subpart D--Limited Ratings for Manufacturers

Authority: Secs. 313(a), 601, 602, 605, and 607, 72 Stat. 752; 49 U.S.C.

1354(a), 1421, 1422, 1425, and 1427.

Source: Docket No. 1221, 31 FR 5249, Apr. 1, 1966, unless otherwise noted.

Sec. 145.101   Application and issue.

(a) Upon application in a form and manner prescribed by the Administrator,

a repair station certificate with a limited rating for a manufacturer may be

issued without further showing to--

(1) The holder or licensee of a Type Certificate who has an approved

production inspection system;

(2) The holder of a Production Certificate;

(3) Any person who meets the requirements of Sec. 21.303 of this chapter,

and has the prescribed Fabrication Inspection System; and

(4) The holder of a Technical Standard Order (TSO) authorization.

(b) Limited ratings are issued under paragraph (a) of this section for--

(1) Aircraft manufactured by the holder of the rating under a Type

Certificate or a Production Certificate;

(2) Aircraft engines manufactured by the holder of the rating under a Type

Certificate or a Production Certificate;

(3) Propellers manufactured by the holder of the rating under a Type

Certificate or a Production Certificate;

(4) Appliances manufactured by the holder of the rating (i) under a Type

Certificate, (ii) under a Production Certificate, (iii) under a TSO

authorization, or (iv) in accordance with Sec. 21.303 of this chapter; and

(5) Parts manufactured by the holder of the rating under a TSO

authorization or in accordance with Sec. 21.303 of this chapter.

Sec. 145.103   Privileges of certificates.

(a) The holder of a repair station certificate issued under this subpart

may maintain and approve for return to service any article for which it is

rated, and perform preventive maintenance on that article, if certificated

mechanics or repairmen are employed directly in charge of the maintenance and

preventive maintenance.

(b) The privileges granted under this section apply to any location or

facility unless the certificate limits the holder to specific locations or

facilities.

Sec. 145.105   Performance standards.

Except as provided in Sec. 145.2, each holder of a certificate issued under

this subpart shall perform its maintenance and preventive maintenance

operations in accordance with Part 43 of this chapter.

[Amdt. 145-7, 31 FR 10614, Aug. 9, 1966]

Appendix A to Part 145

Note: When an asterisk (*) is shown after any job function listed in this

appendix it indicates that the applicant need not have the equipment and

material on his premises for performing this job function provided he

contracts that particular type work to an outside agency having such

equipment and material.

(a) An applicant for a Class 1, 2, 3, or 4 airframe rating must provide

equipment and material necessary for efficiently performing the following job

functions:

(1) Steel structural components:

Repair or replace steel tubes and fittings using the proper welding

techniques when appropriate.

Anticorrosion treatment of the interior and exterior of steel parts,

Metal plating or anodizing*,

Simple machine operations such as making bushings, bolts, etc.,

Complex machine operations involving the use of planers, shapers, milling

machines, etc.*,

Fabricate steel fittings,

Abrasive air blasting and chemical cleaning operations*,

Heat treatment*,

Magnetic inspection*,

Repair or rebuilt metal tanks*.

(2) Wood structure:

Splice wood spars,

Repair ribs and spars (wood),

Fabricate wood spars*,

Repair or replace metal ribs,

Interior alignment of wings,

Repair or replace plywood skin,

Treatment against wood decay.

(3) Alloy skin and structural components:

Repair and replace metal skin, using power tools and equipment,

Repair and replace alloy members and components such as tubes, channels,

cowlings, fittings, attach angles, etc.,

Alignment of components using jigs or fixtures as in the case of joining

fuselage sections or other similar operations,

Make up wooden forming blocks or dies,

Fluorescent inspection of alloy components*,

Fabricate alloy members and components such as tubes, channels, cowlings,

fittings, attach angles, etc.*

(4) Fabric covering:

Repairs to fabric surfaces,

Recovering and refinishing of components and entire aircraft*.

(5) Control systems:

Renewing control cables, using swaging and splicing techniques,

Rigging complete control system,

Renewing or repairing all control system hinge point components such as

pins, bushings, etc.,

Install control system units and components.

(6) Landing gear systems:

Renew or repair all landing gear hinge point components and attachments

such as bolts, bushings, fittings, etc.,

Overhaul and repair elastic shock absorber units,

Overhaul and repair hydraulic-pneumatic shock absorber units*,

Overhaul and repair brake system components*,

Conduct retraction cycle tests,

Overhaul and repair electrical circuits,

Overhaul and repair hydraulic system components*,

Repair or fabricate hydraulic lines.

(7) Electric wiring systems:

Diagnose malfunctions,

Repair or replace wiring,

Installation of electrical equipment,

Bench check electrical components (this check is not to be confused with

the more complex functional test after overhaul).

(8) Assembly operations:

Assembly of airframe component parts such as landing gear, wings, controls,

etc.,

Rigging and alignment of airframe components, including the complete

aircraft and control system,

Installation of powerplants,

Installation of instruments and accessories,

Assembly and fitting of cowling, fairings, etc.,

Repair and assembly of plastic components such as windshields, windows,

etc.,

Jack or hoist complete aircraft.

Conduct aircraft weight and balance operations (this function will be

conducted in draft-free area)*,

Balance control surfaces.

(b) An applicant for any class of power-plant rating must provide equipment

and material necessary for efficiently performing the following job functions

appropriate to the class of rating applied for:

(1) Classes 1 and 2. (i) Maintain and alter powerplants, including

replacement of parts:

Chemical and mechanical cleaning,

Disassembly operations,

Replacement of valve guides and seats*,

Replacement of bushings, bearings, pins, inserts, etc.,

Plating operations (copper, silver, cadmium, etc.)*,

Heating operations (involving the use of recommended techniques requiring

controlled heating facilities),

Chilling or shrinking operations,

Removal and replacement of studs,

Inscribing or affixing identification information,

Painting of powerplants and components,

Anticorrosion treatment for parts,

Replacement and repair of powerplant alloy sheet metal and steel components

such as baffles, fittings, etc.*

(ii) Inspect all parts, using appropriate inspection aids:

Magnetic, fluorescent and other acceptable inspection aids*,

Precise determination of clearances and tolerances of all parts,

Inspection for alignment of connecting rods, crankshafts, impeller shafts,

etc.,

Balancing of parts, including crankshafts, impellers, etc.*,

Inspection of valve springs.

(iii) Accomplish routine machine work:

Precision grinding, honing and lapping operations (includes crankshaft,

cylinder barrels, etc.)*,

Precision drilling, tapping, boring, milling and cutting operations*,

Reaming of inserts, bushings, bearings and other similar components,

Refacing of valves.

(iv) Perform assembly operations:

Valve and ignition timing operations,

Fabricate and test ignition harnesses,

Fabricate and test rigid and flexible fluid lines,

Prepare engines for long- or short-term storage,

Functional check powerplant accessories (this check is not to be confused

with the more complex performance test of overhaul)*,

Hoist engines by mechanical means,

Install engines in aircraft*,

Align and adjust engine controls*,

Installation of engines in aircraft and alignment and adjustment of engine

controls, when completed, must be inspected by either an appropriately rated

certificated mechanic or certificated repairman. Persons supervising or

inspecting these functions must thoroughly understand the pertinent

installation details involved.

(v) Test overhauled powerplants in compliance with manufacturers'

recommendations: The test equipment will be the same as recommended by the

manufacturers of the particular engines undergoing test or equivalent

equipment that will accomplish the same purpose. The testing function may be

performed by the repair station itself, or may be contracted to an outside

agency. In either case the repair station will be responsible for the final

acceptance of the tested engine.

(2) Class 3. Functional and equipment requirements for turbine engines will

be governed entirely by the recommendations of the manufacturer, including

techniques, inspection methods, and test.

(c) An applicant for any class of propeller rating must provide equipment

and material necessary for efficiently performing the following job functions

appropriate to the class of rating applied for:

(1) Class 1. (i) Maintain and alter propellers, including installation and

replacement of parts:

Replace blade tipping,

Refinish wood propellers,

Make wood inlays,

Refinish plastic blades,

Straighten bent blades within repairable tolerances,

Modify blade diameter and profile,

Polish and buff,

Painting operations,

Remove from and reinstall on powerplants.

(ii) Inspect components, using appropriate inspection aids:

Inspect propellers for conformity with manufacturer's drawings and

specifications,

Inspect hubs and blades for failures and defects, using magnetic or

fluorescent inspection devices*,

Inspect hubs and blades for failures and defects, using all visual aids,

including the etching of parts,

Inspect hubs for wear of splines or keyways or any other defect.

(iii) Repair or replace components: (Not applicable to this class).

(iv) Balance propellers:

Test for proper track on aircraft,

Test for horizontal and vertical unbalance (this test will be accomplished

with the use of precision equipment).

(v) Test propeller pitch-changing mechanisms: (Not applicable to this

class).

(2) Class 2. (i) Maintain and alter propellers, including installation and

the replacement of parts:

All functions listed under paragraph (c)(1)(i) of this appendix when

applicable to the make and model propeller for which a rating is sought,

Properly lubricate moving parts,

Assemble complete propeller and subassemblies, using special tools when

required.

(ii) Inspect components, using appropriate inspection aids: All functions

listed under paragraph (c)(1)(ii) of this appendix when applicable to the

make and model propeller for which a rating is sought.

(iii) Repair or replace component parts:

Replace blades, hubs, or any of their components,

Repair or replace anti-icing devices,

Remove nicks or scratches from metal blades,

Repair or replace electrical propeller components.

(iv) Balance propellers: All functions listed under paragraph (c)(1)(iv) of

this appendix when applicable to the make and model propeller for which a

rating is sought.

(v) Test propeller pitch-changing mechanism:

Test hydraulically, propellers and components,

Test electrically operated propellers and components,

Test of constant speed devices*.

(d) An applicant for a radio rating must provide equipment and materials as

follows:

(1) For a Class 1 (Communications) radio rating, the equipment and

materials necessary for efficiently performing the job functions listed in

paragraph (4) and the following job functions:

The testing and repair of headsets, speakers, and microphones.

The measuring of radio transmitter power output.

(2) For a Class 2 (Navigation) radio rating, the equipment and materials

necessary for efficiently performing the job functions listed in paragraph

(4) and the following job functions:

The testing and repair of headsets.

The testing of speakers.

The repair of speakers.*

The measuring of loop antenna sensitivity by appropriate methods.

The determination and compensation for quadrantal error in aircraft

direction finder radio equipment.

The calibration of any radio navigational equipment, enroute and approach

aids, or similar equipment, appropriate to this rating to approved

performance standards.

(3) For Class 3 (Radar) radio rating, the equipment and materials necessary

for efficiently performing the job functions listed in paragraph (4) and the

following job functions:

The measuring of radio transmitter power output.

The metal plating of transmission lines, wave guides, and similar equipment

in accordance with appropriate specifications.*

The pressurization of appropriate radar equipment with dry air, nitrogen,

or other specified gases.

(4) For all classes of radio ratings, the equipment and materials necessary

for efficiently performing the following job functions:

Perform physical inspection of radio systems and components by visual and

mechanical methods.

Perform electrical inspection of radio systems and components by means of

appropriate electrical and/or electronic test instruments.

Check aircraft wiring, antennas, connectors, relays, and other associated

radio components to detect installation faults.

Check engine ignition systems and aircraft accessories to determine sources

of electrical interference.

Check aircraft power supplies for adequacy and proper functioning.

Test radio instruments.*

Overhaul, test, and check dynamotors, inverters, and other radio electrical

apparatus.*

Paint and refinish equipment containers.*

Accomplish appropriate methods of marking calibrations, or other

information on radio control panels and other components, as required.*

Make and reproduce drawings, wiring diagrams, and other similar material

required to record alterations and/or modifications to radio (photographs may

be used in lieu of drawings when they will serve as an equivalent or better

means of recording).*

Fabricate tuning shaft assemblies, brackets, cable assemblies, and other

similar components used in radios or aircraft radio installations.*

Align tuned circuits (RF and IF).

Install and repair aircraft antennas.

Install complete radio systems in aircraft and prepare weight and balance

reports* (That phase of radio installation requiring alterations to the

aircraft structure must be performed, supervised, and inspected by qualified

personnel).

Measure modulation values, noise, and distortion in radios.

Measure audio and radio frequencies to appropriate tolerances and perform

calibration necessary for the proper operation of radios.

Measure radio component values (inductance, capacitance, resistance, etc.).

Measure radiofrequency transmission line attenuation.

Determine wave forms and phase in radios when applicable.

Determine proper aircraft radio antenna, lead-in and transmission line

characteristics and locations for type of radio equipment to which connected.

Determine operational condition of radio equipment installed in aircraft by

using appropriate portable test apparatus.

Determine proper location for radio antennas on aircraft.

Test all types of electronic tubes, transistors, or similar devices in

equipment appropriate to the rating.

(e) An applicant for any class of instrument rating must provide equipment

and material necessary for efficiently performing the following job

functions, in accordance with pertinent specifications and manufacturers'

recommendations, appropriate to the class of rating applied for:

(1) Class 1. (i) Diagnose instrument malfunctions: Diagnose malfunctioning

of the following instruments:

Rate of climb indicators,

Altimeters,

Air speed indicators,

Vacuum indicators,

Oil pressure gauges,

Fuel pressure gauges,

Hydraulic pressure gauges,

Deicing pressure gauges,

Pitot-static tube,

Direct indicating compasses,

Accelerometer,

Direct indicating tachometers,

Direct reading fuel quantity gauges,

Optical (sextants, drift sights, etc.)*.

(ii) Maintain and alter instruments, including installation and replacement

of parts:

Perform these functions on instruments listed under paragraph (e)(1)(i) of

this appendix.

The function of installation includes fabrication of instrument panels and

other installation structural components. The repair station should be

equipped to perform this function. However, it may be contracted to a

competent outside agency equipped to perform the function.

(iii) Inspect, test and calibrate instruments: Perform these functions on

instruments listed under paragraph (e)(1)(i) of this appendix, on and off the

aircraft, when appropriate.

(2) Class 2. (i) Diagnose instrument malfunctions: Diagnose malfunctioning

of the following instruments:

Tachometers,

Synchroscope,

Electric temperature indicators,

Electric resistance type indicators,

Moving magnet type indicators,

Resistance type fuel indicators,

Warning units (oil-fuel),

Selsyn systems and indicators,

Self-synchronous systems and indicators,

Remote indicating compasses,

Fuel quantity indicators,

Oil quantity indicators,

Radio indicators,

Ammeters,

Voltmeters.

(ii) Maintain and alter instruments, including installation and the

replacement of parts:

Perform these functions on instruments listed under paragraph (e)(2)(i) of

this appendix.

The function of installation includes fabrication of instrument panels and

other installation structural components. The repair station should be

equipped to perform this function. However, it may be contracted to a

competent outside agency equipped to perform the function.

(iii) Inspect, test and calibrate instruments: Perform these functions on

instruments listed under paragraph (e)(2)(i) of this appendix, on and off the

aircraft, when appropriate.

(3) Class 3. (i) Diagnose instrument malfunctions: Diagnose malfunctioning

of the following instruments:

Turn and bank indicators,

Directional gyros,

Horizon gyros,

Auto pilot control units and components*,

Remote reading direction indicators*.

(ii) Maintain and alter instruments, including installation and replacement

of parts:

Perform these functions on instruments listed under paragraph (e)(3)(i) of

this appendix.

The function of installation includes fabrication of instrument panels and

other installation structural components. The repair station should be

equipped to perform this function. However, it may be contracted to a

competent outside agency equipped to perform the function.

(iii) Inspect, test and calibrate instruments: Perform these functions on

instruments listed under paragraph (e)(3)(i) of this appendix, on and off the

aircraft, when appropriate.

(4) Class 4. (i) Diagnose instrument malfunctions: Diagnose malfunctioning

of the following instruments:

Capacitance type quantity gauge,

Other electronic instruments,

Engine analyzers.

(ii) Maintain and alter instruments, including installation and replacement

of parts:

Perform these functions on instruments listed under paragraph (e)(4)(i) of

this appendix.

The function of installation includes fabrication of instrument panels and

other installation structural components. The repair station should be

equipped to perform this function. However, it may be contracted to a

competent outside agency equipped to perform the function.

(iii) Inspect, test and calibrate instruments: Perform these functions on

instruments listed under paragraph (e)(4)(i) of this appendix, on and off the

aircraft, when appropriate.

(f) An applicant for a Class 1, 2, or 3 accessory rating must provide

equipment and material necessary for efficiently performing the following job

functions, in accordance with pertinent specifications and the manufacturers'

recommendations:

(1) Diagnose accessory malfunctions.

(2) Maintain and alter accessories, including installation and the

replacement of the parts.

(3) Inspect, test, and, where necessary, calibrate accessories.

(Secs. 313, 314, and 601 through 610, of the Federal Aviation Act of 1958, as

amended (49 U.S.C. 1354, 1355, 1421 through 1430); sec. 6(c), Dept. of

Transportation Act (49 U.S.C. 1655(c)))

[Doc. No. 1157, 27 FR 11693, Nov. 28, 1962, as amended by Amdt. 145-14, 35 FR

19349, Dec. 22, 1970; Amdt. 145-19, 47 FR 33391, Aug. 2, 1982]