PART 45--IDENTIFICATION AND REGISTRATION MARKING

Subpart A--General

Sec.

45.1  Applicability.

Subpart B--Identification of Aircraft and Related Products

45.11  General.

45.13  Identification data.

45.14  Identification of critical components.

45.15  Replacement and modification parts.

Subpart C--Nationality and Registration Marks

45.21  General.

45.22  Exhibition, antique, and other aircraft: Special rules.

45.23  Display of marks; general.

45.25  Location of marks on fixed-wing aircraft.

45.27  Location of marks; nonfixed-wing aircraft.

45.29  Size of marks.

45.31  Marking of export aircraft.

45.33  Sale of aircraft; removal of marks.

Authority: 49 U.S.C. app. 1348, 1354, 1401, 1402, 1421, 1423, 1522,

1655(c).

Source: Docket No. 2047, 29 FR 3223, Mar. 11, 1964, unless otherwise noted.

Special Federal Aviation Regulation No. 27  [Removed.  55 FR 32861, Aug. 10,

1990]

EDITORIAL NOTE: For the convenience of the user, the removed text is set

out below.

Special Federal Aviation Regulation No. 27--Fuel Venting and Exhaust Emission

Requirements for Turbine Engine Powered Airplanes

EDITORIAL NOTE: For the text of SFAR No. 27, see Part 11 of this chapter.

*****************************************************************************

55 FR 32856, No. 155, Aug. 10, 1990

SUMMARY: This final rule codifies as new part 34 all of the applicable

aircraft engine fuel venting and exhaust emission requirements of Special

Federal Aviation Regulation (SFAR) 27-5, and the test procedures specified

under the regulations implementing the Clean Air Act. This rule consolidates

all of the requirements and test procedures into this part, and inserts into

other affected parts the requirements to comply with new part 34. New part 34

does not alter any of the requirements specified under SFAR 27-5 or the

regulations implementing the Clean Air Act.

EFFECTIVE DATE: September 10, 1990.

*****************************************************************************

Subpart A--General

Sec. 45.1  Applicability.

This part prescribes the requirements for--

(a) Identification of aircraft, and identification of aircraft engines and

propellers that are manufactured under the terms of a type or production

certificate:

(b) Identification of certain replacement and modified parts produced for

installation on type certificated products; and

(c) Nationality and registration marking of U.S. registered aircraft.

[Doc. No. 2047, 29 FR 3223, Mar. 11, 1964, as amended by Amdt. 45-3, 32 FR

188, Jan. 10, 1967]

Subpart B--Identification of Aircraft and Related Products

Sec. 45.11   General.

(a) Aircraft and aircraft engines. Aircraft covered under Sec. 21.182 of

this chapter must be identified, and each person who manufacturers an

aircraft engine under a type or production certificate shall identify that

engine, by means of a fireproof plate that has the information specified in

Sec. 45.13 of this part marked on it by etching, stamping, engraving, or

other approved method of fireproof marking. The identification plate for

aircraft must be secured in such a manner that it will not likely be defaced

or removed during normal service, or lost or destroyed in an accident. Except

as provided in paragraphs (c) and (d) of this section, the aircraft

identification plate must be secured to the aircraft fuselage exterior so

that it is legible to a person on the ground, and must be either adjacent to

and aft of the rear-most entrance door or on the fuselage surface near the

tail surfaces. For aircraft engines, the identification plate must be affixed

to the engine at an accessible location in such a manner that it will not

likely be defaced or removed during normal service, or lost or destroyed in

an accident.

(b) Propellers and propeller blades and hubs. Each person who manufactures

a propeller, propeller blade, or propeller hub under the terms of a type or

production certificate shall identify his product by means of a plate,

stamping, engraving, etching, or other approved method of fireproof

identification that is placed on it on a noncritical surface, contains the

information specified in Sec. 45.13, and will not be likely to be defaced or

removed during normal service or lost or destroyed in an accident.

(c) For manned free balloons, the identification plate prescribed in

paragraph (a) of this section must be secured to the balloon envelope and

must be located, if practicable, where it is legible to the operator when the

balloon is inflated. In addition, the basket and heater assembly must be

permanently and legibly marked with the manufacturer's name, part number (or

equivalent) and serial number (or equivalent).

(d) On aircraft manufactured before March 7, 1988, the identification plate

required by paragraph (a) of this section may be secured at an accessible

exterior or interior location near an entrance, if the model designation and

builder's serial number are also displayed on the aircraft fuselage exterior.

The model designation and builder's serial number must be legible to a person

on the ground and must be located either adjacent to and aft of the rear-most

entrance door or on the fuselage near the tail surfaces. The model

designation and builder's serial number must be displayed in such a manner

that they are not likely to be defaced or removed during normal service.

[Amdt. 45-3, 32 FR 188, Jan. 10, 1967 as amended by Amdt. 45-7, 33 FR 14402,

Sept. 25, 1968; Amdt. 45-12, 45 FR 60183, Sept. 11, 1980; 45 FR 85597, Dec.

29, 1980; Amdt. 45-17, 52 FR 34101, Sept. 9, 1987; 52 FR 36566, Sept. 30,

1987]

Sec. 45.13  Identification data.

(a) The identification required by Sec. 45.11 (a) and (b) shall include the

following information:

(1) Builder's name.

(2) Model designation.

(3) Builder's serial number.

(4) Type certificate number, if any.

(5) Production certificate number, if any.

(6) For aircraft engines, the established rating.

(7) On or after January 1, 1984, for aircraft engines specified in part 34

of this chapter, the date of manufacture as defined in Sec. 34.1 of that

part, and a designation, approved by the Administrator of the FAA, that

indicates compliance with the applicable exhaust emission provisions of part

34 and 40 CFR part 87. Approved designations include COMPLY, EXEMPT, and NON-

US as appropriate.

(i) The designation COMPLY indicates that the engine is in compliance with

all of the applicable exhaust emissions provisions of part 34. For any engine

with a rated thrust in excess of 26.7 kilonewtons (6000 pounds) which is not

used or intended for use in commercial operations and which is in compliance

with the applicable provisions of part 34, but does not comply with the

hydrocarbon emissions standard of Sec. 34.21(d), the statement "May not be

used as a commercial aircraft engine" must be noted in the permanent

powerplant record that accompanies the engine at the time of manufacture of

the engine.

(ii) The designation EXEMPT indicates that the engine has been granted an

exemption pursuant to the applicable provision of Sec. 34.7 (a)(1), (a)(4),

(b), (c), or (d), and an indication of the type of exemption and the reason

for the grant must be noted in the permanent powerplant record that

accompanies the engine from the time of manufacture of the engine.

(iii) The designation NON-US indicates that the engine has been granted an

exemption pursuant to Sec. 34.7(a)(1), and the notation "This aircraft may

not be operated within the United States", or an equivalent notation approved

by the Administrator of the FAA, must be inserted in the aircraft logbook, or

alternate equivalent document, at the time of installation of the engine.

(8) Any other information the Administrator finds appropriate.

(b) Except as provided in paragraph (d)(1) of this section, no person may

remove, change, or place identification information required by paragraph (a)

of this section, on any aircraft, aircraft engine, propeller, propeller

blade, or propeller hub, without the approval of the Administrator.

(c) Except as provided in paragraph (d)(2) of this section, no person may

remove or install any identification plate required by Sec. 45.11 of this

part, without the approval of the Administrator.

(d) Persons performing work under the provisions of Part 43 of this chapter

may, in accordance with methods, techniques, and practices acceptable to the

Administrator--

(1) Remove, change, or place the identification information required by

paragraph (a) of this section on any aircraft, aircraft engine, propeller,

propeller blade, or propeller hub; or

(2) Remove an identification plate required by Sec. 45.11 when necessary

during maintenance operations.

(e) No person may install an identification plate removed in accordance

with paragraph (d)(2) of this section on any aircraft, aircraft engine,

propeller, propeller blade, or propeller hub other than the one from which it

was removed.

[Amdt. 45-3, 32 FR 188, Jan. 10, 1967, as amended by Amdt. 45-10, 44 FR

45379, Aug. 2, 1979; Amdt. 45-12, 45 FR 60183, Sept. 11, 1980; Amdt. 45-20,

55 FR 32861, Aug. 10, 1990; 55 FR 37287, Sept. 10, 1990]

*****************************************************************************

55 FR 32856, No. 155, Aug. 10, 1990

SUMMARY: This final rule codifies as new part 34 all of the applicable

aircraft engine fuel venting and exhaust emission requirements of Special

Federal Aviation Regulation (SFAR) 27-5, and the test procedures specified

under the regulations implementing the Clean Air Act. This rule consolidates

all of the requirements and test procedures into this part, and inserts into

other affected parts the requirements to comply with new part 34. New part 34

does not alter any of the requirements specified under SFAR 27-5 or the

regulations implementing the Clean Air Act.

EFFECTIVE DATE: September 10, 1990.

*****************************************************************************

Sec. 45.14  Identification of critical components.

Each person who produces a part for which a replacement time, inspection

interval, or related procedure is specified in the Airworthiness Limitations

section of a manufacturer's maintenance manual or Instructions for Continued

Airworthiness shall permanently and legibly mark that component with a part

number (or equivalent) and a serial number (or equivalent).

[Amdt. 45-16, 51 FR 40703, Nov. 7, 1986]

Sec. 45.15  Replacement and modification parts.

(a) Except as provided in paragraph (b) of this section, each person who

produces a replacement or modification part under a Parts Manufacturer

Approval issued under Sec. 21.303 of this chapter shall permanently and

legibly mark the part with--

(1) The letters "FAA-PMA";

(2) The name, trademark, or symbol of the holder of the Parts Manufacturer

Approval;

(3) The part number; and

(4) The name and model designation of each type certificated product on

which the part is eligible for installation.

(b) If the Administrator finds that a part is too small or that it is

otherwise impractical to mark a part with any of the information required by

paragraph (a) of this section, a tag attached to the part or its container

must include the information that could not be marked on the part. If the

marking required by paragraph (a)(4) of this section is so extensive that to

mark it on a tag is impractical, the tag attached to the part or the

container may refer to a specific readily available manual or catalog for

part eligibility information.

[Amdt. 45-8, 37 FR 10660, May 26, 1972, as amended by Amdt. 45-14, 47 FR

13315, Mar. 29, 1982]

Subpart C--Nationality and Registration Marks

Sec. 45.21  General.

(a) Except as provided in Sec. 45.22, no person may operate a U.S.-

registered aircraft unless that aircraft displays nationality and

registration marks in accordance with the requirements of this section and

Secs. 45.23 through 45.33.

(b) Unless otherwise authorized by the Administrator, no person may place

on any aircraft a design, mark, or symbol that modifies or confuses the

nationality and registration marks.

(c) Aircraft nationality and registration marks must--

(1) Except as provided in paragraph (d) of this section, be painted on the

aircraft or affixed by any other means insuring a similar degree of

permanence;

(2) Have no ornamentation;

(3) Contrast in color with the background; and

(4) Be legible.

(d) The aircraft nationality and registration marks may be affixed to an

aircraft with readily removable material if--

(1) It is intended for immediate delivery to a foreign purchaser;

(2) It is bearing a temporary registration number; or

(3) It is marked temporarily to meet the requirements of Sec. 45.22(c)(1)

or Sec. 45.29(h) of this part, or both.

[Doc. No. 8093, Amdt. 45-5, 33 FR 450, Jan 12, 1968, as amended by Amdt. 45-

17, 52 FR 34102, Sept. 9, 1987]

Sec. 45.22  Exhibition, antique, and other aircraft: Special rules.

(a) When display of aircraft nationality and registration marks in

accordance with Secs. 45.21 and 45.23 through 45.33 would be inconsistent

with exhibition of that aircraft, a U.S.-registered aircraft may be operated

without displaying those marks anywhere on the aircraft if:

(1) It is operated for the purpose of exhibition, including a motion

picture or television production, or an airshow;

(2) Except for practice and test fights necessary for exhibition purposes,

it is operated only at the location of the exhibition, between the exhibition

locations, and between those locations and the base of operations of the

aircraft; and

(3) For each flight in the United States:

(i) It is operated with the prior approval of the Flight Standards District

Office, in the case of a flight within the designated airport control zone of

the takeoff airport, or within 5 miles of that airport if it has no

designated control zone; or

(ii) It is operated under a flight plan filed under either Sec. 91.153 or

Sec. 91.169 of this chapter describing the marks it displays, in the case of

any other flight.

(b) A small U.S.-registered aircraft built at least 30 years ago or a U.S.-

registered aircraft for which an experimental certificate has been issued

under Sec. 21.191(d) or 21.191(g) for operation as an exhibition aircraft or

as an amateur-built aircraft and which has the same external configuration as

an aircraft built at least 30 years ago may be operated without displaying

marks in accordance with Secs. 45.21 and 45.23 through 45.33 if:

(1) It displays in accordance with Sec. 45.21(c) marks at least 2 inches

high on each side of the fuselage or vertical tail surface consisting of the

Roman capital letter "N" followed by:

(i) The U.S. registration number of the aircraft; or

(ii) The symbol appropriate to the airworthiness certificate of the

aircraft ("C", standard; "R", restricted; "L", limited; or "X", experimental)

followed by the U.S. registration number of the aircraft; and

(2) It displays no other mark that begins with the letter "N" anywhere on

the aircraft, unless it is the same mark that is displayed under paragraph

(b)(1) of this section.

(c) No person may operate an aircraft under paragraph (a) or (b) of this

section--

(1) In an ADIZ or DEWIZ described in Part 99 of this chapter unless it

temporarily bears marks in accordance with Secs. 45.21 and 45.23 through

45.33;

(2) In a foreign country unless that country consents to that operation; or

(3) In any operation conducted under Part 121, 127, 133, 135, or 137 of

this chapter.

(d) If, due to the configuration of an aircraft, it is impossible for a

person to mark it in accordance with Secs. 45.21 and 45.23 through 45.33, he

may apply to the Administrator for a different marking procedure.

[Doc. No. 8093, Amdt. 45-5, 33 FR 450, Jan. 12, 1968, as amended by Amdt. 45-

13, 46 FR 48603, Oct. 1, 1981; Amdt. 45-19, 54 FR 39291, Sept. 25, 1989;

Amdt. 45-18, 54 FR 34330, Aug. 18, 1989]

Effective Date Note: Amdt. No. 45-21, 56 FR 65653, Dec. 17, 1991, revised

paragraph (a)(3)(i) effective September 16, 1993 to read as follows:

(a) * * *

(3) * * *

(i) It is operated with the prior approval of the Flight Standards District

Office, in the case of a flight within the lateral boundaries of the surface

areas of Class B, Class C, Class D, or Class E airspace designated for the

takeoff airport, or within 4.4 nautical miles of that airport if it is within

Class G airspace; or

*****************************************************************************

56 FR 65638, No. 242, Dec. 17, 1991

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) to

adopt certain recommendations of the National Airspace Review (NAR)

concerning changes to regulations and procedures in regard to airspace

classifications. These changes are intended to :(1) Simplify airspace

designations; (2) achieve international commonality of airspace designations;

(3) increase standardization of equipment requirements for operations in

various classifications of airspace; (4) describe appropriate pilot

certificate requirements, visual flight rules (VFR) visibility and distance

from cloud rules, and air traffic services offered in each class of airspace;

and (5) satisfy the responsibilities of the United States as a member of the

International Civil Aviation Organization (ICAO). The final rule also amends

the requirement for minimum distance from clouds in certain airspace areas

and the requirements for communications with air traffic control (ATC) in

certain airspace areas; eliminates airport radar service areas (ARSAs),

control zones, and terminal control areas (TCAs) as airspace classifications;

and eliminates the term "airport traffic area." The FAA believes simplified

airspace classifications will reduce existing airspace complexity and thereby

enhance safety.

EFFECTIVE DATE: These regulations become effective September 16, 1993,

except that Secs. 11.61(c), 91.215(d), 71.601, 71.603, 71.605, 71.607, and

71.609 and Part 75 become effective December 12, 1991, and except that

amendatory instruction number 20, Sec. 71.1 is effective as of September 15,

1993, and that Secs. 71.11 and 71.19 become effective October 15, 1992. The

incorporation by reference of FAA Order 7400.7 in Sec. 71.1 (amendatory

instruction number 20) is approved by the Director of the Federal Register

as of December 17, 1991, through September 15, 1993. The incorporation by

reference of FAA Order 7400.9 in Sec. 71.1 (amendatory instruction number 24)

is approved by the Director of the Federal Register as of September 16,

1993 through September 15, 1994.

*****************************************************************************

Sec. 45.23  Display of marks; general.

(a) Each operator of an aircraft shall display on that aircraft marks

consisting of the Roman capital letter "N" (denoting United States

registration) followed by the registration number of the aircraft. Each

suffix letter used in the marks displayed must also be a Roman capital

letter.

(b) When marks that include only the Roman capital letter "N" and the

registration number are displayed on limited or restricted category aircraft

or experimental or provisionally certificated aircraft, the operator shall

also display on that aircraft near each entrance to the cabin or cockpit, in

letters not less than 2 inches nor more than 6 inches in height, the words

"limited," "restricted," "experimental," or "provisional airworthiness," as

the case may be.

[Doc. No. 8093, Amdt. 45-5, 33 FR 450, Jan. 12, 1968, as amended by Amdt. 45-

9, 42 FR 41102, Aug. 15, 1977]

Sec. 45.25  Location of marks on fixed-wing aircraft.

(a) The operator of a fixed-wing aircraft shall display the required marks

on either the vertical tail surfaces or the sides of the fuselage, except as

provided in Sec. 45.29(f).

(b) The marks required by paragraph (a) of this section shall be displayed

as follows:

(1) If displayed on the vertical tail surfaces, horizontally on both

surfaces, horizontally on both surfaces of a single vertical tail or on the

outer surfaces of a multivertical tail. However, on aircraft on which marks

at least 3 inches high may be displayed in accordance with Sec. 45.29(b)(1),

the marks may be displayed vertically on the vertical tail surfaces.

(2) If displayed on the fuselage surfaces, horizontally on both sides of

the fuselage between the trailing edge of the wing and the leading edge of

the horizontal stabilizer. However, if engine pods or other appurtenances are

located in this area and are an integral part of the fuselage side surfaces,

the operator may place the marks on those pods or appurtenances.

[Amdt. 45-9, 42 FR 41102, Aug. 15, 1977]

Sec. 45.27  Location of marks; nonfixed-wing aircraft.

(a) Rotorcraft. Each operator of a rotorcraft shall display on that

rotorcraft horizontally on both surfaces of the cabin, fuselage, boom, or

tail the marks required by Sec. 45.23.

(b) Airships. Each operator of an airship shall display on that airship the

marks required by Sec. 45.23, horizontally on--

(1) The upper surface of the right horizontal stabilizer and on the under

surface of the left horizontal stabilizer with the top of the marks toward

the leading edge of each stabilizer; and

(2) Each side of the bottom half of the vertical stabilizer.

(c) Spherical balloons. Each operator of a spherical balloon shall display

the marks required by Sec. 45.23 in two places diametrically opposite and

near the maximum horizontal circumference of that balloon.

(d) Nonspherical balloons. Each operator of a nonspherical balloon shall

display the marks required by Sec. 45.23 on each side of the balloon near its

maximum cross section and immediately above either the rigging band or the

points of attachment of the basket or cabin suspension cables.

[Doc. No. 2047, 29 FR 3223, Mar. 11, 1964, as amended by Amdt. 45-15, 48 FR

11392, Mar. 17, 1983]

Sec. 45.29  Size of marks.

(a) Except as provided in paragraph (f) of this section, each operator of

an aircraft shall display marks on the aircraft meeting the size requirements

of this section.

(b) Height. Except as provided in paragraph (h) of this part, the

nationality and registration marks must be of equal height and on--

(1) Fixed-wing aircraft, must be at least 12 inches high, except that:

(i) An aircraft displaying marks at least 2 inches high before November 1,

1981 and an aircraft manufactured after November 2, 1981, but before January

1, 1983, may display those marks until the aircraft is repainted or the marks

are repainted, restored, or changed;

(ii) Marks at least 3 inches high may be displayed on a glider;

(iii) Marks at least 3 inches high may be displayed on an aircraft for

which an experimental certificate has been issued under Sec. 21.191(d) or

21.191(g) for operating as an exhibition aircraft or as an amateur-built

aircraft when the maximum cruising speed of the aircraft does not exceed 180

knots CAS; and

(iv) Marks may be displayed on an exhibition, antique, or other aircraft in

accordance with Sec. 45.22.

(2) Airships, spherical balloons, and nonspherical balloons, must be at

least 3 inches high; and

(3) Rotorcraft, must be at least 12 inches high, except that rotorcraft

displaying before April 18, 1983, marks required by Sec. 45.29(b)(3) in

effect on April 17, 1983, and rotorcraft manufactured on or after April 18,

1983, but before December 31, 1983, may display those marks until the

aircraft is repainted or the marks are repainted, restored, or changed.

(c) Width. Characters must be two-thirds as wide as they are high, except

the number "1", which must be one-sixth as wide as it is high, and the

letters "M" and "W" which may be as wide as they are high.

(d) Thickness. Characters must be formed by solid lines one-sixth as thick

as the character is high.

(e) Spacing. The space between each character may not be less than one-

fourth of the character width.

(f) If either one of the surfaces authorized for displaying required marks

under Sec. 45.25 is large enough for display of marks meeting the size

requirements of this section and the other is not, full-size marks shall be

placed on the larger surface. If neither surface is large enough for full-

size marks, marks as large as practicable shall be displayed on the larger of

the two surfaces. If any surface authorized to be marked by Sec. 45.27 is not

large enough for full-size marks, marks as large as practicable shall be

placed on the largest of the authorized surfaces.

(g) Uniformity. The marks required by this part for fixed-wing aircraft

must have the same height, width, thickness, and spacing on both sides of the

aircraft.

(h) After March 7, 1988, each operator of an aircraft penetrating an ADIZ

or DEWIZ shall display on that aircraft temporary or permanent nationality

and registration marks at least 12 inches high.

[Doc. No. 2047, 29 FR 3223, Mar. 11, 1964, as amended by Amdt. 45-2, 31 FR

9863, July 21, 1966; Amdt. 45-9, 42 FR 41102, Aug. 15, 1977; Amdt. 45-13, 46

FR 48604, Oct. 1, 1981; Amdt. 45-15, 48 FR 11392, Mar. 17, 1983; Amdt. 45-17,

52 FR 34102, Sept. 9, 1987; 52 FR 36566, Sept. 30, 1987]

Sec. 45.31  Marking of export aircraft.

A person who manufactures an aircraft in the United States for delivery

outside thereof may display on that aircraft any marks required by the State

of registry of the aircraft. However, no person may operate an aircraft so

marked within the United States, except for test and demonstration flights

for a limited period of time, or while in necessary transit to the purchaser.

Sec. 45.33   Sale of aircraft; removal of marks.

When an aircraft that is registered in the United States is sold, the

holder of the Certificate of Aircraft Registration shall remove, before its

delivery to the purchaser, all United States marks from the aircraft, unless

the purchaser is--

(a) A citizen of the United States;

(b) An individual citizen of a foreign country who is lawfully admitted for

permanent residence in the United States; or

(c) When the aircraft is to be based and primarily used in the United

States, a corporation (other than a corporation which is a citizen of the

United States) lawfully organized and doing business under the laws of the

United States or any State thereof.

[Amdt. 45-11, 44 FR 61938, Oct. 29, 1979]