PART 171--NON-FEDERAL NAVIGATION FACILITIES

Subpart A--VOR Facilities

Sec.

171.1  Scope.

171.3  Requests for IFR procedure.

171.5  Minimum requirements for approval.

171.7  Performance requirements.

171.9  Installation requirements.

171.11  Maintenance and operations requirements.

171.13  Reports.

Subpart B--Nondirectional Radio Beacon Facilities

171.21  Scope.

171.23  Requests for IFR procedure.

171.25  Minimum requirements for approval.

171.27  Performance requirements.

171.29  Installation requirements.

171.31  Maintenance and operations requirements.

171.33  Reports.

Subpart C--Instrument Landing System (ILS) Facilities

171.41  Scope.

171.43  Requests for IFR procedure.

171.45  Minimum requirements for approval.

171.47  Performance requirements.

171.49  Installation requirements.

171.51  Maintenance and operations requirements.

171.53  Reports.

Subpart D--True Lights

171.61  Air navigation certificate: Revocation and termination.

Subpart E--General

171.71  Materials incorporated by reference.

171.73  Alternative forms of reports.

171.75  Submission of requests.

Subpart F--Simplified Directional Facility (SDF)

171.101  Scope.

171.103  Requests for IFR procedure.

171.105  Minimum requirements for approval.

171.107  Definition.

171.109  Performance requirements.

171.111  Ground standards and tolerances.

171.113  Installation requirements.

171.115  Maintenance and operations requirements.

171.117  Reports.

Subpart G--Distance Measuring Equipment (DME)

171.151  Scope.

171.153  Requests for IFR procedure.

171.155  Minimum requirements for approval.

171.157  Performance requirements.

171.159  Installation requirements.

171.161  Maintenance and operations requirements.

171.163  Reports.

Subpart H--VHF Marker Beacons

171.201  Scope.

171.203  Requests for IFR procedure.

171.205  Minimum requirements for approval.

171.207  Performance requirements.

171.209  Installation requirements.

171.211  Maintenance and operations requirements.

171.213  Reports.

Subpart I--Interim Standard Microwave Landing System (ISMLS)

171.251  Scope.

171.253  Definitions.

171.255  Requests for IFR procedures.

171.257  Minimum requirements for approval.

171.259  Performance requirements: General.

171.261  Localizer performance requirements.

171.263  Localizer automatic monitor system.

171.265  Glide path performance requirements.

171.267  Glide path automatic monitor system.

171.269  Marker beacon performance requirements.

171.271  Installation requirements.

171.273  Maintenance and operations requirements.

171.275  Reports.

Subpart J--Microwave Landing System (MLS)

171.301  Scope.

171.303  Definitions.

171.305  Requests for IFR procedure.

171.307  Minimum requirements for approval.

171.309  General requirements.

171.311  Signal format requirements.

171.313  Azimuth performance requirements.

171.315  Azimuth monitor system requirements.

171.317  Approach elevation performance requirements.

171.319  Approach elevation monitor system requirements.

171.321  DME and marker beacon performance requirements.

171.323  Fabrication and installation requirements.

171.325  Maintenance and operations requirements.

171.327  Operational records.

Authority: 49 U.S.C. app. 1343, 1346, 1348, 1354(a), 1355, 1401, 1421-1430,

1472(c), 1502, and 1522; 49 U.S.C. 106(g).

Source: Docket No. 5034, 29 FR 11337, Aug. 6, 1964, unless otherwise noted.

Subpart A--VOR Facilities

Sec. 171.1   Scope.

This subpart sets forth minimum requirements for the approval and operation

on non-Federal VOR facilities that are to be involved in the approval of

instrument flight rules and air traffic control procedures related to those

facilities.

[Doc. No. 5034, 29 FR 11337, Aug. 6, 1964, as amended by Amdt. 171-2, 31 FR

5408, Apr. 6, 1966; Amdt. 171-7, 35 FR 12711, Aug. 11, 1970]

Sec. 171.3   Requests for IFR procedure.

(a) Each person who requests an IFR procedure based on a VOR facility that

he owns must submit the following information with that request:

(1) A description of the facility and evidence that the equipment meets the

performance requirements of Sec. 171.7 and is installed in accordance with

Sec. 171.9.

(2) A proposed procedure for operating the facility.

(3) A proposed maintenance organization and maintenance manual that meets

the requirements of Sec. 171.11.

(4) A statement of intention to meet the requirements of this subpart.

(5) A showing that the facility has an acceptable level of operational

reliability and an acceptable standard of performance. Previous equivalent

operational experience with a facility with identical design and operational

characteristics will be considered in showing compliance with this paragraph.

(b) After the FAA inspects and evaluates the facility, it advises the owner

of the results and of any required changes in the facility or the maintenance

manual or maintenance organization. The owner must then correct the

deficiencies, if any, and operate the facility for an in-service evaluation

by the FAA.

[Doc. No. 5034, 29 FR 11337, Aug. 6, 1964, as amended by Amdt. 171-7, 35 FR

12711, Aug. 11, 1970]

Sec. 171.5   Minimum requirements for approval.

(a) The following are the minimum requirements that must be met before the

FAA will approve an IFR procedure for a non-Federal VOR:

(1) The facility's performance, as determined by air and ground inspection,

must meet the requirements of Sec. 171.7.

(2) The installation of the equipment must meet the requirements of Sec.

171.9.

(3) The owner must agree to operate and maintain the facility in accordance

with Sec. 171.11.

(4) The owner must agree to furnish periodic reports, as set forth in Sec.

171.13, and must agree to allow the FAA to inspect the facility and its

operation whenever necessary.

(5) The owner must assure the FAA that he will not withdraw the facility

from service without the permission of the FAA.

(6) The owner must bear all costs of meeting the requirements of this

section and of any flight or ground inspections made before the facility is

commissioned, except that the Federal Aviation Administration may bear

certain of these costs subject to budgetary limitations and policy

established by the Administrator.

(b) If the applicant for approval meets the requirements of paragraph (a)

of this section, the FAA commissions the facility as a prerequisite to its

approval for use in an IFR procedure. The approval is withdrawn at any time

the facility does not continue to meet those requirements.

[Doc. No. 5034, 29 FR 11337, Aug. 6, 1964, as amended by Amdt. 171-6, 35 FR

10288, June 24, 1970]

Sec. 171.7   Performance requirements.

(a) The VOR must perform in accordance with the "International Standards

and Recommended Practices, Aeronautical Telecommunications," Part I,

paragraph 3.3 (Annex 10 to the Convention on International Civil Aviation),

except that part of paragraph 3.3.2.1 specifying a radio frequency tolerance

of 0.005 percent, and that part of paragraph 3.3.7 requiring removal of only

the bearing information. In place thereof, the frequency tolerance of the

radio frequency carrier must not exceed plus or minus 0.002 percent, and all

radiation must be removed during the specified deviations from established

conditions and during periods of monitor failure.

(b) Ground inspection consists of an examination of the design features of

the equipment to determine that there will not be conditions that will allow

unsafe operations because of component failure or deterioration.

(c) The monitor is checked periodically, during the in-service test

evaluation period, for calibration and stability The tests are made with a

standard "Reference and variable phase signal generator" and associated test

equipment, including an oscilloscope and portable field detector. In general,

the ground check is conducted in accordance with section 8.4 of FAA Handbook

AF P 6790.9 "Maintenance Instruction for VHF Omniranges", adapted for the

facility concerned.

(d) Flight tests to determine the facility's adequacy for operational

requirements and compliance with applicable "Standards and Recommended

Practices" are conducted in accordance with the "U.S. Standard Flight

Inspection Manual", particularly section 201.

(e) After January 1, 1975, the owner of the VOR shall modify the facility

to perform in accordance with paragraph 3.3.5.7 of Annex 10 to the Convention

on International Civil Aviation within 180 days after receipt of notice from

the Administrator that 50 kHz channel spacing is to be implemented in the

area and that a requirement exists for suppression of 9960 Hz subcarrier

harmonics.

[Doc. No. 5034, 29 FR 11337, Aug. 6, 1964, as amended by Amdt. 171-7, 35 FR

12711, Aug. 11, 1970; Amdt. 171-9, 38 FR 28557, Oct. 15, 1973]

Sec. 171.9   Installation requirements.

(a) The facility must be installed according to accepted good engineering

practices, applicable electric and safety codes, and the installation must

meet at least the Federal Communication Commission's licensing requirements.

(b) The facility must have a reliable source of suitable primary power,

either from a power distribution system or locally generated, with a

supplemental standby system, if needed.

(c) Dual transmitting equipment with automatic changeover is preferred and

may be required to support certain IFR procedures.

(d) There must be a means for determining, from the ground, the performance

of the equipment, including the antenna, initially and periodically.

(e) A facility intended for use as an instrument approach aid for an

airport must have or be supplemented by (depending on circumstances) the

following ground-air or landline communications services:

(1) At facilities outside of and not immediately adjacent to air traffic

control areas, there must be ground-air communications from the airport

served by the facility. Separate communications channels are acceptable.

(2) At facilities within or immediately adjacent to air traffic control

areas, there must be the ground-air communications required by paragraph

(e)(1) of this section and reliable communications (at least a landline

telephone) from the airport to the nearest FAA air traffic control or

communication facility.

Paragraphs (e) (1) and (2) of this section are not mandatory at airports

where an adjacent FAA facility can communicate with aircraft on the ground at

the airport and during the entire proposed instrument approach procedure. In

addition at low traffic density airports within or immediately adjacent to

air traffic control areas, and where extensive delays are not a factor, the

requirements of paragraphs (e) (1) and (2) of this section may be reduced to

reliable communications (at least a landline telephone) from the airport to

the nearest FAA air traffic control or communication facility, if an adjacent

FAA facility can communicate with aircraft during the proposed instrument

approach procedure, at least down to the minimum en route altitude of the

controlled area.

[Doc. No. 5034, 29 FR 11337, Aug. 6, 1964, as amended by Amdt. 171-7, 35 FR

12711, Aug. 11, 1970]

EFFECTIVE DATE NOTE: Amdt. 171-16, 56 FR 65664, Dec. 17, 1991, revised

paragraphs (e)(1) and (e)(2) of Sec. 171.9 effective September 16, 1993. For

the convenience of the user, the revised text is set forth as follows:

Sec. 171.9   Installation requirements.

* * * * *

(e) * * *

(1) At facilities outside of and not immediately adjacent to controlled

airspace, there must be ground-air communications from the airport served by

the facility. Separate communications channels are acceptable.

(2) At facilities within or immediately adjacent to controlled airspace,

there must be the ground-air communications required by paragraph (e)(1) of

this section and reliable communications (at least a landline telephone) from

the airport to the nearest FAA air traffic control or communication facility.

Paragraphs (e) (1) and (2) of this section are not mandatory at airports

where an adjacent FAA facility can communicate with aircraft on the ground at

the airport and during the entire proposed instrument approach procedure. In

addition, at low traffic density airports within or immediately adjacent to

controlled airspace and where extensive delays are not a factor, the

requirements of paragraphs (e) (1) and (2) of this section may be reduced to

reliable communications (at least a landline telephone) from the airport to

the nearest FAA air traffic control or communication facility, if an adjacent

FAA facility can communicate with aircraft during the proposed instrument

approach procedure, at least down to the minimum en route altitude for the

controlled airspace area.

Sec. 171.11   Maintenance and operations requirements.

(a) The owner of the facility must establish an adequate maintenance system

and provide qualified maintenance personnel to maintain the facility at the

level attained at the time it was commissioned. Each person who maintains a

facility must meet at least the Federal Communications Commission's licensing

requirements and show that he has the special knowledge and skills needed to

maintain the facility including proficiency in maintenance procedures and the

use of specialized test equipment.

(b) The owner must prepare, and obtain FAA approval of, an operations and

maintenance manual that sets forth mandatory procedures for operations,

preventive maintenance, and emergency maintenance, including instructions on

each of the following:

(1) Physical security of the facility.

(2) Maintenance and operations by authorized persons only.

(3) FCC licensing requirements for operating and maintenance personnel.

(4) Posting of licenses and signs.

(5) Relations between the facility and FAA air traffic control facilities,

with a description of the boundaries of controlled airspace over or near the

facility, instructions for relaying air traffic control instructions and

information (if applicable), and instructions for the operation of an air

traffic advisory service if the VOR is located outside of controlled

airspace.

(6) Notice to the Administrator of any suspension of service.

(7) Detailed and specific maintenance procedures and servicing guides

stating the frequency of servicing.

(8) Air-ground communications, if provided, expressly written or

incorporating appropriate sections of FAA manuals by reference.

(9) Keeping of station logs and other technical reports, and the submission

of reports required by Sec. 171.13.

(10) Monitoring of the facility.

(11) Inspections by United States personnel.

(12) Names, addresses, and telephone numbers of persons to be notified in

an emergency.

(13) Shutdowns for routine maintenance and issue of "Notices to Airmen" for

routine or emergency shutdowns (private use facilities may omit the "Notices

to Airmen").

(14) An explanation of the kinds of activity (such as construction or

grading) in the vicinity of the facility that may require shutdown or

recertification of the facility by FAA flight check.

(15) Procedures for conducting a ground check of course accuracy.

(16) Commissioning of the facility.

(17) An acceptable procedure for amending or revising the manual.

(18) The following information concerning the facility:

(i) Location by latitude and longitude to the nearest second, and its

position with respect to airport layouts.

(ii) The type, make, and model of the basic radio equipment that will

provide the service.

(iii) The station power emission and frequency.

(iv) The hours of operation.

(v) Station identification call letters and method of station

identification, whether by Morse code or recorded voice announcement, and the

time spacing of the identification.

(vi) A description of the critical parts that may not be changed, adjusted,

or repaired without an FAA flight check to confirm published operations.

(c) The owner shall make a ground check of course accuracy each month in

accordance with procedures approved by the FAA at the time of commissioning,

and shall report the results of the checks as provided in Sec. 171.13.

(d) If the owner desires to modify the facility, he must submit the

proposal to the FAA and may not allow any modifications to be made without

specific approval.

(e) The owner's maintenance personnel must participate in initial

inspections made by the FAA. In the case of subsequent inspections, the owner

or his representative shall participate.

(f) Whenever it is required by the FAA, the owner shall incorporate

improvements in VOR maintenance brought about by progress in the state of the

art. In addition, he shall provide a stock of spare parts, including vacuum

tubes, of such a quantity to make possible the prompt replacement of

components that fail or deteriorate in service.

(g) The owner shall provide all approved test instruments needed for

maintenance of the facility.

(h) The owner shall close the facility upon receiving two successive pilot

reports of its malfunctioning.

[Doc. No. 5034, 29 FR 11337, Aug. 6, 1964, as amended by Amdt. 171-2, 31 FR

5408, Apr. 6, 1966]

Sec. 171.13   Reports.

The owner of each facility to which this subpart applies shall make the

following reports on forms furnished by the FAA, at the times indicated, to

the FAA Regional office for the area in which the facility is located:

(a) Record of meter readings and adjustments (Form FAA-198). To be filled

out by the owner with the equipment adjustments and meter readings as of the

time of commissioning, with one copy to be kept in the permanent records of

the facility and two copies to the appropriate Regional office of the FAA.

The owner shall revise the form after any major repair, modernization, or

returning, to reflect an accurate record of facility operation and

adjustment.

(b) Facility maintenance log (FAA Form 6003-1). This form is a permanent

record of all equipment malfunctioning met in maintaining the facility,

including information on the kind of work and adjustments made, equipment

failures, causes (if determined), and corrective action taken. The owner

shall keep the original of each report at the facility and send a copy to the

appropriate Regional office of the FAA at the end of the month in which it is

prepared.

(c) Radio equipment operation record (Form FAA-418). To contain a complete

record of meter readings, recorded on each scheduled visit to the facility.

The owner shall keep the original of each month's record at the facility and

send a copy of it to the appropriate Regional office of the FAA.

(d) [Reserved]

(e) VOR ground check error data (Forms FAA-2396 and 2397). To contain

results of the monthly course accuracy ground check in accordance with FAA

Handbook AF P 6790.9 "Maintenance Instructions for VHF Omniranges". The owner

shall keep the originals in the facility and send a copy of each form to the

appropriate Regional office of the FAA on a monthly basis.

(49 U.S.C. 1348)

[Doc. No. 5034, 29 FR 11337, Aug. 6, 1964, as amended by Amdt. 171-5, 34 FR

15245, Sept. 30, 1969; Amdt. 171-10, 40 FR 36110, Aug. 19, 1975]

Subpart B--Nondirectional Radio Beacon Facilities

Sec. 171.21   Scope.

(a) This subpart sets forth minimum requirements for the approval and

operation of non-Federal, nondirectional radio beacon facilities that are to

be involved in the approval of instrument flight rules and air traffic

control procedures related to those facilities.

(b) A nondirectional radio beacon ("H" facilities domestically--NDB

facilities internationally) radiates a continuous carrier of approximately

equal intensity at all azimuths. The carrier is modulated at 1020 cycles per

second for station identification purposes.

[Doc. No. 5034, 29 FR 11337, Aug. 6, 1964, as amended by Amdt. 171-2, 31 FR

5408, Apr. 6, 1966; Amdt. 171-7, 35 FR 12711, Aug. 11, 1970]

Sec. 171.23   Requests for IFR procedure.

(a) Each person who requests an IFR procedure based on a nondirectional

radio beacon facility that he owns must submit the following information with

that request:

(1) A description of the facility and evidence that the equipment meets the

performance requirements of Sec. 171.27 and is installed in accordance with

Sec. 171.29.

(2) A proposed procedure for operating the facility.

(3) A proposed maintenance arrangement and a maintenance manual that meets

the requirements of Sec. 171.31.

(4) A statement of intention to meet the requirements of this subpart.

(5) A showing that the facility has an acceptable level of operational

reliability and an acceptable standard of performance. Previous equivalent

operational experience with a facility with identical design and operational

characteristics will be considered in showing compliance with this

subparagraph.

(b) After the FAA inspects and evaluates the facility, it advises the owner

of the results and of any required changes in the facility or the maintenance

manual or maintenance organization. The owner must then correct the

deficiencies, if any, and operate the facility for an in-service evaluation

by the FAA.

[Doc. No. 5034, 29 FR 11337, Aug. 6, 1964, as amended by Amdt. 171-7, 35 FR

12711, Aug. 11, 1970]

Sec. 171.25   Minimum requirements for approval.

(a) The following are the minimum requirements that must be met before the

FAA will approve an IFR procedure for a non-Federal, nondirectional radio

beacon facility under this subpart:

(1) The facility's performances, as determined by air and ground

inspection, must meet the requirements of Sec. 171.27.

(2) The installation of the equipment must meet the requirements of Sec.

171.29.

(3) The owner must agree to operate and maintain the facility in accordance

with Sec. 171.31.

(4) The owner must agree to furnish periodic reports, as set forth in Sec.

171.33, and agree to allow the FAA to inspect the facility and its operation

whenever necessary.

(5) The owner must assure the FAA that he will not withdraw the facility

from service without the permission of the FAA.

(6) The owner must bear all costs of meeting the requirements of this

section and of any flight or ground inspections made before the facility is

commissioned, except that the Federal Aviation Administration may bear

certain of these costs subject to budgetary limitations and policy

established by the Administrator.

(b) If the applicant for approval meets the requirements of paragraph (a)

of this section, the FAA commissions the facility as a prerequisite to its

approval for use in an IFR procedure. The approval is withdrawn at any time

the facility does not continue to meet those requirements. In addition, the

facility may be de-commissioned whenever the frequency channel is needed for

higher priority common system service.

[Doc. No. 5034, 29 FR 11337, Aug. 6, 1964, as amended by Amdt. 171-6, 35 FR

10288, June 24, 1970]

Sec. 171.27   Performance requirements.

(a) The facility must meet the performance requirements set forth in the

"International Standards and Recommended Practices, Aeronautical

Telecommunications, Part I, paragraph 3.4" (Annex 10 to the Convention on

International Civil Aviation), except that identification by on-off keying of

a second carrier frequency, separated from the main carrier by 1020 Hz plus

or minus 50 Hz, is also acceptable.

(b) The facility must perform in accordance with recognized and accepted

good electronic engineering practices for the desired service.

(c) Ground inspection consists of an examination of the design features of

the equipment to determine (based on recognized and accepted good engineering

practices) that there will not be conditions that will allow unsafe

operations because of component failure or deterioration.

(d) Flight tests to determine the facility's adequacy for operational

requirements and compliance with applicable "Standards and Recommended

Practices" are conducted in accordance with the "U.S. Standard Flight

Inspection Manual", particularly section 207. The original test is made by

the FAA and later tests shall be made under arrangements, satisfactory to the

FAA, that are made by the owner.

[Doc. No. 5034, 29 FR 11337, Aug. 6, 1964, as amended by Amdt. 171-7, 35 FR

12711, Aug. 11, 1970]

Sec. 171.29   Installation requirements.

(a) The facility must be installed according to accepted good engineering

practices, applicable electric and safety codes, and FCC licensing

requirements.

(b) The facility must have a reliable source of suitable primary power.

(c) Dual transmitting equipment may be required to support some IFR

procedures.

(d) A facility intended for use as an instrument approach aid for an

airport must have or be supplemented by (depending on the circumstances) the

following ground-air or landline communications services:

(1) At facilities outside of and not immediately adjacent to air traffic

control areas, there must be ground-air communications from the airport

served by the facility. Voice on the aid controlled from the airport is

acceptable.

(2) At facilities within or immediately adjacent to air traffic control

areas, there must be the ground-air communications required by paragraph

(d)(1) of this section and reliable communications (at least a landline

telephone) from the airport to the nearest FAA air traffic control or

communication facility. Paragraphs (d) (1) and (2) of this section are not

mandatory at airports where an adjacent FAA facility can communicate with

aircraft on the ground at the airport and during the entire proposed

instrument approach procedure. In addition, at low traffic density airports

within or immediately adjacent to air traffic control areas, and where

extensive delays are not a factor, the requirements of paragraphs (d) (1) and

(2) of this section may be reduced to reliable communications (at least a

landline telephone) from the airport to the nearest FAA air traffic control

or communications facility, if an adjacent FAA facility can communicate with

aircraft during the proposed instrument approach procedure, at least down to

the minimum en route altitude of the controlled area.

EFFECTIVE DATE NOTE: Amdt. 171-16, 56 FR 65664, Dec. 17, 1991, revised

paragraphs (d)(1) and (d)(2) of Sec. 171.29 effective September 16, 1993. For

the convenience of the user, the revised text is set forth as follows:

Sec. 171.29   Installation requirements.

* * * * *

(d) * * *

(1) At facilities outside of and not immediately adjacent to controlled

airspace, there must be ground-air communications from the airport served by

the facility. Voice on the aid controlled from the airport is acceptable.

(2) At facilities within or immediately adjacent to controlled airspace,

there must be the ground-air communications required by paragraph (d)(1) of

this section and reliable communications (at least a landline telephone) from

the airport to the nearest FAA air traffic control or communication facility.

Paragraphs (d) (1) and (2) of this section are not mandatory at airports

where an adjacent FAA facility can communicate with aircraft on the ground at

the airport and during the entire proposed instrument approach procedure. In

addition, at low traffic density airports within or immediately adjacent to

controlled airspace, and where extensive delays are not a factor, the

requirements of paragraphs (d) (1) and (2) of this section may be reduced to

reliable communications (at least a landline telephone) from the airport to

the nearest FAA air traffic control or communications facility, if an

adjacent FAA facility can communicate with aircraft during the proposed

instrument approach procedure, at least down to the minimum en route altitude

for the controlled airspace area.

Sec. 171.31   Maintenance and operations requirements.

(a) The owner of the facility must establish an adequate maintenance system

and provide qualified maintenance personnel to maintain the facility at the

level attained at the time it was commissioned. Each person who maintains a

facility must meet at least the Federal Communications Commission's licensing

requirements and show that he has the special knowledge and skills needed to

maintain the facility including proficiency in maintenance procedures and the

use of specialized test equipment.

(b) The owner must prepare, and obtain approval of, an operations and

maintenance manual that sets forth mandatory procedures for operations,

preventive maintenance, and emergency maintenance, including instructions on

each of the following:

(1) Physical security of the facility.

(2) Maintenance and operations by authorized persons only.

(3) FCC licensing requirements for operating and maintenance personnel.

(4) Posting of licenses and signs.

(5) Relations between the facility and FAA air traffic control facilities,

with a description of the boundaries of controlled airspace over or near the

facility, instructions for relaying air traffic control instructions and

information (if applicable), and instructions for the operation of an air

traffic advisory service if the facility is located outside of controlled

airspace.

(6) Notice to the Administrator of any suspension of service.

(7) Detailed arrangements for maintenance flight inspection and servicing

stating the frequency of servicing.

(8) Air-ground communications, if provided, expressly written or

incorporating appropriate sections of FAA manuals by reference.

(9) Keeping of station logs and other technical reports, and the submission

of reports required by Sec. 171.33.

(10) Monitoring of the facility, at least once each half hour, to assure

continuous operation.

(11) Inspections by United States personnel.

(12) Names, addresses, and telephone numbers of persons to be notified in

an emergency.

(13) Shutdowns for routine maintenance and issue of "Notices to Airmen" for

routine or emergency shutdowns (private use facilities may omit the "Notices

to Airmen").

(14) Commissioning of the facility.

(15) An acceptable procedure for amending or revising the manual.

(16) The following information concerning the facility:

(i) Location by latitude and longitude to the nearest second, and its

position with respect to airport layouts.

(ii) The type, make, and model of the basic radio equipment that will

provide the service.

(iii) The station power emission and frequency.

(iv) The hours of operation.

(v) Station identification call letters and method of station

identification, whether by Morse code or recorded voice announcement, and the

time spacing of the identification.

(c) If the owner desires to modify the facility, he must submit the

proposal to the FAA and meet applicable requirements of the FCC.

(d) The owner's maintenance personnel must participate in initial

inspections made by the FAA. In the case of subsequent inspections, the owner

or his representative shall participate.

(e) The owner shall provide a stock of spare parts, including vacuum tubes,

of such a quantity to make possible the prompt replacement of components that

fail or deteriorate in service.

(f) The owner shall close the facility upon receiving two successive pilot

reports of its malfunctioning.

[Doc. No. 5034, 29 FR 11337, Aug. 6, 1964, as amended by Amdt. 171-2, 31 FR

5408, Apr. 6, 1966]

Sec. 171.33   Reports.

The owner of each facility to which this subpart applies shall make the

following reports, at the times indicated, to the FAA Regional office for the

area in which the facility is located:

(a) Record of meter readings and adjustments (Form FAA-198). To be filled

out by the owner or his maintenance representative with the equipment

adjustments and meter readings as of the time of commissioning, with one copy

to be kept in the permanent records of the facility and two copies to the

appropriate Regional Office of the FAA. The owner shall revise the form after

any major repair, modernization, or returning, to reflect an accurate record

of facility operation and adjustment.

(b) Facility maintenance log (FAA Form 6030-1). This form is a permanent

record of all equipment malfunctioning met in maintaining the facility,

including information on the kind of work and adjustments made, equipment

failures, causes (if determined), and corrective action taken. The owner

shall keep the original of each report at the facility and send a copy to the

appropriate Regional Office of the FAA at the end of the month in which it is

prepared.

(c) Radio equipment operation record (Form FAA-418). To contain a complete

record of meter readings, recorded on each scheduled visit to the facility.

The owner shall keep the original of each month's record at the facility and

send a copy of it to the appropriate Regional Office of the FAA.

[Doc. No. 5034, 29 FR 11337, Aug. 6, 1964, as amended by Amdt. 171-10, 40 FR

36110, Aug. 19, 1975]

Subpart C--Instrument Landing System (ILS) Facilities

Sec. 171.41   Scope.

This subpart sets forth minimum requirements for the approval and operation

of non-Federal Instrument Landing System (ILS) Facilities that are to be

involved in the approval of instrument flight rules and air traffic control

procedures related to those facilities.

[Doc. No. 5034, 29 FR 11337, Aug. 6, 1964, as amended by Amdt. 171-2, 31 FR

5408, Apr. 6, 1966; Amdt. 171-7, 35 FR 12711, Aug. 11, 1970]

Sec. 171.43   Requests for IFR procedure.

(a) Each person who requests an IFR procedure based on an ILS facility that

he owns must submit the following information with that request:

(1) A description of the facility and evidence that the equipment meets the

performance requirements of Sec. 171.47 and is installed in accordance with

Sec. 171.49.

(2) A proposed procedure for operating the facility.

(3) A proposed maintenance organization and a maintenance manual that meets

the requirements of Sec. 171.51.

(4) A statement of intent to meet the requirements of this subpart.

(5) A showing that the facility has an acceptable level of operational

reliability and an acceptable standard of performance. Previous equivalent

operational experience with a facility with identical design and operational

characteristics will be considered in showing compliance with this

subparagraph.

(b) After the FAA inspects and evaluates the facility, it advises the owner

of the results and of any required changes in the facility or the maintenance

manual or maintenance organization. The owner must then correct the

deficiencies, if any, and operate the facility for an in-service evaluation

by the FAA.

[Doc. No. 5034, 29 FR 11337, Aug. 6, 1964, as amended by Amdt. 171-7, 35 FR

12711, Aug. 11, 1970]

Sec. 171.45   Minimum requirements for approval.

(a) The following are the minimum requirements that must be met before the

FAA will approve an IFR procedure for a non-Federal Instrument Landing

System:

(1) The facility's performance, as determined by air and ground inspection,

must meet the requirements of Sec. 171.47.

(2) The installation of the equipment must meet the requirements of Sec.

171.49.

(3) The owner must agree to operate and maintain the facility in accordance

with Sec. 171.51.

(4) The owner must agree to furnish periodic reports, as set forth in Sec.

171.53 and agree to allow the FAA to inspect the facility and its operation

whenever necessary.

(5) The owner must assure the FAA that he will not withdraw the facility

from service without the permission of the FAA.

(6) The owner must bear all costs of meeting the requirements of this

section and of any flight or ground inspections made before the facility is

commissioned, except that the Federal Aviation Administration may bear

certain of these costs subject to budgetary limitations and policy

established by the Administrator.

(b) If the applicant for approval meets the requirements of paragraph (a)

of this section, the FAA commissions the facility as a prerequisite to its

approval for use in an IFR procedure. The approval is withdrawn at any time

the facility does not continue to meet those requirements. In addition, the

facility may be de-commissioned whenever the frequency channel is needed for

higher priority common system service.

[Doc. No. 5034, 29 FR 11337, Aug. 6, 1964, as amended by Amdt. 171-6, 35 FR

10288, June 24, 1970]

Sec. 171.47   Performance requirements.

(a) The Instrument Landing System must perform in accordance with the

"International Standards and Recommended Practices, Aeronautical

Telecommunications, Part I, Paragraph 3.1" (Annex 10 to the Convention on

International Civil Aviation) except as follows:

(1) The first part of paragraph 3.1.3, relating to suppression of radiation

wholly or in part in any or all directions outside the 20-degree sector

centered on the course line to reduce localizer does not apply.

(2) Radiation patterns must conform to limits specified in 3.1.3.3 and

3.1.3.4, but this does not mean that suppression of radiation to the rear of

the antenna array to satisfy difficult siting positions (as per 3.1.3.1.4) is

not allowed. For example, if a reflector screen for the antenna array is

required to overcome a siting problem, the area to the rear of the localizer

may be made unusable and should be so advertised.

(3) A third marker beacon (inner marker) is not required.

(4) The frequency tolerance of the radio frequency carrier must not exceed

plus or minus 0.002 percent.

(b) Ground inspection consists of an examination of the design features of

the equipment to determine that there will not be conditions that will allow

unsafe operations because of component failure or deterioration.

(c) The monitor is checked periodically, during the in-service test

evaluation period, for calibration and stability. These tests, and ground

checks of glide slope and localizer radiation characteristics, are conducted

in accordance with FAA Handbooks AF P 6750.1 and AF P 6750.2 "Maintenance

Instructions for ILS Localizer Equipment" and "Maintenance Instructions for

ILS Glide Slope Equipment".

(d) Flight tests to determine the facility's adequacy for operational

requirements and compliance with applicable "Standards and Recommended

Practices" are conducted in accordance with the "U.S. Standard Flight

Inspection Manual", particularly section 217.

[Doc. No. 5034, 29 FR 11337, Aug. 6, 1974, as amended by Amdt. 171-9, 38 FR

28557, Oct. 15, 1973]

Sec. 171.49   Installation requirements.

(a) The facility must be of a permanent nature, located, constructed, and

installed according to ICAO Standards (Annex 10), accepted good engineering

practices, applicable electric and safety codes, and FCC licensing

requirements.

(b) The facility must have a reliable source of suitable primary power,

either from a power distribution system or locally generated. A determination

by the Administrator as to whether a facility will be required to have stand-

by power for the localizer, glide slope and monitor accessories to supplement

the primary power, will be made for each airport based upon operational

minimums and density of air traffic.

(c) A determination by the Administrator as to whether a facility will be

required to have dual transmitting equipment with automatic changeover for

localizer and glide slope components, will be made for each airport based

upon operational minimums and density of air traffic.

(d) There must be a means for determining, from the ground, the performance

of the equipment (including antennae), initially and periodically.

(e) The facility must have, or be supplemented by (depending on the

circumstances) the following ground-air or landline communications services:

(1) At facilities outside of and not immediately adjacent to air traffic

control zones or areas, there must be ground-air communications from the

airport served by the facility. The utilization of voice on the ILS frequency

should be determined by the facility operator on an individual basis.

(2) At facilities within or immediately adjacent to air traffic control

zones or areas, there must be the ground-air communications required by

paragraph (e)(1) of this section and reliable communications (at least a

landline telephone) from the airport to the nearest FAA air traffic control

or communication facility.

Paragraphs (e) (1) and (2) of this section are not mandatory at airports

where an adjacent FAA facility can communicate with aircraft on the ground at

the airport and during the entire proposed instrument approach procedure. In

addition, at low traffic density airports within or immediately adjacent to

air traffic control zones or areas, and where extensive delays are not a

factor, the requirements of paragraphs (e) (1) and (2) of this section may be

reduced to reliable communications (at least a landline telephone) from the

airport to the nearest FAA air traffic control or communications facility, if

an adjacent FAA facility can communicate with aircraft during the proposed

instrument approach procedure down to the airport surface or at least to the

minimum approach altitude.

[Doc. No. 5034, 29 FR 11337, Aug. 6, 1964, as amended by Amdt. 171-6, 35 FR

10288, June 24, 1970]

EFFECTIVE DATE NOTE: Amdt. 171-16, 56 FR 65664, Dec. 17, 1991, revised

paragraph (e) of Sec. 171.49 effective September 16, 1993. For the

convenience of the user, the revised text is set forth as follows:

Sec. 171.49   Installation requirements.

* * * * *

(e) The facility must have, or be supplemented by (depending on the

circumstances) the following ground-air or landline communications services:

(1) At facilities outside of and not immediately adjacent to controlled

airspace, there must be ground-air communications from the airport served by

the facility. The utilization of voice on the ILS frequency should be

determined by the facility operator on an individual basis.

(2) At facilities within or immediately adjacent to controlled airspace,

there must be the ground-air communications required by paragraph (e)(1) of

this section and reliable communications (at least a landline telephone) from

the airport to the nearest FAA air traffic control or communications

facility.

Paragraphs (e)(1) and (e)(2) of this section are not mandatory at airports

where an adjacent FAA facility can communicate with aircraft on the ground at

the airport and during the entire proposed instrument approach procedure. In

addition, at low traffic density airports within or immediately adjacent to

controlled airspace, and where extensive delays are not a factor, the

requirements of paragraphs (e)(1) and (e)(2) of this section may be reduced

to reliable communications (at least a landline telephone) from the airport

to the nearest FAA air traffic control or communications facility, if an

adjacent FAA facility can communicate with aircraft during the proposed

instrument approach procedure down to the airport surface or at least to the

minimum approach altitude.

Sec. 171.51   Maintenance and operations requirements.

(a) The owner of the facility must establish an adequate maintenance system

and provide qualified maintenance personnel to maintain the facility at the

level attained at the time it was commissioned. Each person who maintains a

facility must meet at least the Federal Communications Commission's licensing

requirements and show that he has the special knowledge and skills needed to

maintain the facility including proficiency in maintenance procedures and the

use of specialized test equipment.

(b) The owner must prepare, and obtain approval of, an operations and

maintenance manual that sets forth mandatory procedures for operations,

preventive maintenance, and emergency maintenance, including instructions on

each of the following:

(1) Physical security of the facility.

(2) Maintenance and operations by authorized persons only.

(3) FCC licensing requirements for operating and maintenance personnel.

(4) Posting of licenses and signs.

(5) Relation between the facility and FAA air traffic control facilities,

with a description of the boundaries of controlled airspace over or near the

facility, instructions for relaying air traffic control instructions and

information (if applicable), and instructions for the operations of an air

traffic advisory service if the facility is located outside of controlled

airspace.

(6) Notice to the Administrator of any suspension of service.

(7) Detailed and specific maintenance procedures and servicing guides

stating the frequency of servicing.

(8) Air-ground communications, if provided, expressly written or

incorporating appropriate sections of FAA manuals by reference.

(9) Keeping of station logs and other technical reports, and the submission

of reports required by Sec. 171.53.

(10) Monitoring of the facility.

(11) Inspections by United States personnel.

(12) Names, addresses, and telephone numbers of persons to be notified in

an emergency.

(13) Shutdowns for routine maintenance and issue of "Notices to Airmen" for

routine or emergency shutdowns (private use facilities may omit the "Notices

to Airmen").

(14) Commissioning of the facility.

(15) An acceptable procedure for amending or revising the manual.

(16) An explanation of the kinds of activities (such as construction or

grading) in the vicinity of the facility that may require shutdown or

recertification of the facility by FAA flight check.

(17) Procedures for conducting a ground check or localizer course alignment

width, and clearance, and glide slope elevation angle and width.

(18) The following information concerning the facility:

(i) Facility component locations with respect to airport layout, instrument

runway, and similar areas.

(ii) The type, make, and model of the basic radio equipment that will

provide the service.

(iii) The station power emission and frequencies of the localizer, glide

slope, markers, and associated compass locators, if any.

(iv) The hours of operation.

(v) Station identification call letters and method of station

identification and the time spacing of the identification.

(vi) A description of the critical parts that may not be changed, adjusted,

or repaired without an FAA flight check to confirm published operations.

(c) The owner shall make a ground check of the facility each month in

accordance with procedures approved by the FAA at the time of commissioning,

and shall report the results of the checks as provided in Sec. 171.53.

(d) If the owner desires to modify the facility, he must submit the

proposal to the FAA and may not allow any modifications to be made without

specific approval.

(e) "The owner's maintenance personnel must participate in initial

inspections made by the FAA. In the case of subsequent inspections, the owner

or his representative shall participate."

(f) Whenever it is required by the FAA, the owner shall incorporate

improvements in ILS maintenance brought about by progress in the state of the

art. In addition, he shall provide a stock of spare parts, including vacuum

tubes, of such a quantity to make possible the prompt replacement of

components that fail or deteriorate in service.

(g) The owner shall provide FAA approved test instruments needed for

maintenance of the facility.

(h) The owner shall close the facility upon receiving two successive pilot

reports of its malfunctioning.

[Doc. No. 5034, 29 FR 11337, Aug. 6, 1964, as amended by Amdt. 171-2, 31 FR

5408, Apr. 6, 1966]

Sec. 171.53   Reports.

The owner of each facility to which this subpart applies shall make the

following reports, at the times indicated, to the FAA Regional Office for the

area in which the facility is located:

(a) Record of meter readings and adjustments (Form FAA-198). To be filled

out by the owner or his maintenance representative with the equipment

adjustments and meter readings as of the time of commissioning, with one copy

to be kept in the permanent records of the facility and two copies to the

appropriate Regional Office of the FAA. The owner shall revise the form after

any major repair, modernization, or retuning, to reflect an accurate record

of facility operation and adjustment.

(b) Facility maintenance log (FormFAA 6030-1). This form is a permanent

record of all equipment malfunctioning met in maintaining the facility,

including information on the kind of work and adjustments made, equipment

failures, causes (if determined), and corrective action taken. The owner

shall keep the original of each report at the facility and send a copy to the

appropriate Regional Office of the FAA at the end of each month in which it

is prepared.

(c) Radio equipment operation record (Form FAA-418). To contain a complete

record of meter readings, recorded on each scheduled visit to the facility.

The owner shall keep the original of each month's record at the facility and

send a copy of it to the appropriate Regional Office of the FAA.

[Doc. No. 5034, 29 FR 11337, Aug. 6, 1964, as amended by Amdt. 171-5, 34 FR

15245, Sept. 30, 1969; Amdt. 171-10, 40 FR 36110, Aug. 19, 1975]

Subpart D--True Lights

Sec. 171.61   Air navigation certificate: Revocation and termination.

(a) Except as provided in paragraph (b) of this section, each air

navigation certificate of "Lawful Authority to Operate a True Light" is

hereby revoked, and each application therefor is hereby terminated.

(b) Paragraph (a) of this section does not apply to--

(1) A certificate issued to a Federal-Aid Airport Program sponsor who was

required to apply for that certificate under regulations then in effect, and

who has not surrendered that certificate under Sec. 151.86(e) of this

chapter; or

(2) An application made by a Federal-Aid Airport Program sponsor who was

required to make that application under regulations then in effect, and who

has not terminated that application under Sec. 151.86(e) of this chapter.

(49 U.S.C. 1101-1120; sec. 307, 72 Stat. 749, 49 U.S.C. 1348)

[Amdt. 171-4, 33 FR 12545, Sept. 5, 1968]

Subpart E--General

Sec. 171.71   Materials incorporated by reference.

Copies of standards, recommended practices and documents incorporated by

reference in this part are available for the use of interested persons at any

FAA Regional Office and FAA Headquarters. An historical file of these

materials is maintained at Headquarters, Federal Aviation Administration, 800

Independence Avenue SW., Washington, DC 20590.

[Amdt. 171-8, 36 FR 5584, Mar. 25, 1971]

Sec. 171.73   Alternative forms of reports.

On a case-by-case basis, a Regional Administrator may accept any report in

a format other than the FAA form required by this part if he is satisfied

that the report contains all the information required on the FAA form and can

be processed by FAA as conveniently as the FAA form.

(49 U.S.C. 1348)

[Amdt. 171-5, 34 FR 15245, Sept. 30, 1969, as amended by Amdt. 171-15, 54 FR

39296, Sept. 25, 1989]

Sec. 171.75   Submission of requests.

(a) Requests for approval of facilities not having design and operational

characteristics identical to those of facilities currently approved under

this part, including requests for deviations from this part for such

facilities, must be submitted to the Director, Advanced Systems Design

Service.

(b) The following requests must be submitted to the Regional Administrator

of the region in which the facility is located:

(1) Requests for approval of facilities that have design and operational

characteristics identical to those of facilities currently approved under

this part, including requests for deviations from this part for such

facilities.

(2) Requests for deviations from this part for facilities currently

approved under this part.

(3) Requests for modification of facilities currently approved under this

part.

[Amdt. 171-7, 35 FR 12711, Aug. 11, 1970, as amended by Amdt. 171-15, 54 FR

39296, Sept. 25, 1989]

Subpart F--Simplified Directional Facility (SDF)

Source: Docket No. 10116, 35 FR 12711, Aug. 11, 1970, unless otherwise

noted.

Sec. 171.101   Scope.

This subpart sets forth minimum requirements for the approval and operation

of non-Federal Simplified Directional Facilities (SDF) that are to be

involved in the approval of instrument flight rules and air traffic control

procedures related to those facilities.

Sec. 171.103   Requests for IFR procedure.

(a) Each person who requests an IFR procedure based on an SDF that he owns

must submit the following information with that request:

(1) A description of the facility and evidence that the equipment meets the

performance requirements of Sec. 171.109 and the standards and tolerances of

Sec. 171.111, and is installed in accordance with Sec. 171.113.

(2) A proposed procedure for operating the facility.

(3) A proposed maintenance organization and a maintenance manual that meets

the requirements of Sec. 171.115.

(4) A statement of intent to meet the requirements of this subpart.

(5) A showing that the facility has an acceptable level of operational

reliability as prescribed in Sec. 171.111(k), and an acceptable standard of

performance. Previous equivalent operational experience with a facility with

identical design and operational characteristics will be considered in

showing compliance with this paragraph.

(b) After the Federal Aviation Administration inspects and evaluates the

facility, it advises the owner of the results and of any required changes in

the facility or the maintenance manual or maintenance organization. The owner

must then correct the deficiencies, if any, and operate the facility for an

in-service evaluation by the Federal Aviation Administration.

Sec. 171.105   Minimum requirements for approval.

(a) The following are the minimum requirements that must be met before the

Federal Aviation Administration will approve an IFR procedure for a non-

Federal Simplified Directional Facility:

(1) A suitable frequency channel must be available.

(2) The facility's performance, as determined by air and ground inspection,

must meet the requirements of Secs. 171.109 and 171.111.

(3) The installation of the equipment must meet the requirements of Sec.

171.113.

(4) The owner must agree to operate and maintain the facility in accordance

with Sec. 171.115.

(5) The owner must agree to furnish periodic reports as set forth in Sec.

171.117, and agree to allow the FAA to inspect the facility and its operation

whenever necessary.

(6) The owner must assure the FAA that he will not withdraw the facility

from service without the permission of the FAA.

(7) The owner must bear all costs of meeting the requirements of this

section and of any flight or ground inspections made before the facility is

commissioned, except that the FAA may bear certain of these costs subject to

budgetary limitations and policy established by the Administrator.

(b) If the applicant for approval meets the requirements of paragraph (a)

of this section, the FAA commissions the facility as a prerequisite to its

approval for use in an IFR procedure. The approval is withdrawn at any time

the facility does not continue to meet those requirements. In addition, the

facility is licensed by the Federal Communications Commission. The Federal

Aviation Administration recommends cancellation or nonrenewal of the Federal

Communications Commission license whenever the frequency channel is needed

for higher priority common system service.

Sec. 171.107   Definition.

As used in this subpart:

"SDF" (simplified directional facility) means a directional aid facility

providing only lateral guidance (front or back course) for approach from a

final approach fix.

"DDM" (difference in depth of modulation) means the percentage modulation

depth of the larger signal minus the percentage modulation depth of the

smaller signal, divided by 100.

"Angular displacement sensitivity" means the ratio of measured DDM to the

corresponding angular displacement from the appropriate reference line.

"Back course sector" means the course sector on the opposite end of the

runway from the front course sector.

"Course line" means the locus of points along the final approach course at

which the DDM is zero.

"Course sector" means a sector in a horizontal plane containing the course

line and limited by the loci of points nearest to the course line at which

the DDM is 0.155.

"Displacement sensitivity" means the ratio of measured DDM to the

corresponding lateral displacement from the appropriate reference line.

"Front course sector" means the course sector centered on the course line

in the direction from the runway in which a normal final approach is made.

"Half course sector" means the sector in a horizontal plane containing the

course line and limited by the loci of points nearest to the course line, at

which the DDM is 0.0775.

"Point A" means a point on the front course in the approach direction a

distance of 4 nautical miles from the threshold.

"Point A1" means a point on the front course in the approach direction a

distance of 1 statute mile from the threshold.

"Point A2" means a point on the front course at the threshold.

"Reference datum" means a point at a specified height located vertically

above the intersection of the course and the threshold.

"Missed approach point" means the point on the final approach course, not

farther from the final approach fix than Point "A2", at which the approach

must be abandoned, if the approach and subsequent landing cannot be safely

completed by visual reference, whether or not the aircraft has descended to

the minimum descent altitude.

Sec. 171.109   Performance requirements.

(a) The Simplified Directional Facility must perform in accordance with the

following standards and practices:

(1) The radiation from the SDF antenna system must produce a composite

field pattern which is amplitude modulated by a 90 Hz and a 150 Hz tone. The

radiation field pattern must produce a course sector with the 90 Hz tone

predominating on one side of the course and with the 150 Hz tone

predominating on the opposite side.

(2) When an observer faces the SDF from the approach end of runway, the

depth of modulation of the radiofrequency carrier due to the 150 Hz tone must

predominate on his right hand and that due to the 90 Hz tone must predominate

on his left hand.

(3) All horizontal angles employed in specifying the SDF field patterns

must originate from the center of the antenna system which provides the

signals used in the front course sector.

(4) The SDF must operate on odd tenths or odd tenths plus a twentieth MHz

within the frequency band 108.1 MHz to 111.95 MHz. The frequency tolerance of

the radio frequency carrier must not exceed plus or minus 0.002 percent.

(5) The radiated emission from the SDF must be horizontally polarized. The

vertically polarized component of the radiation on the course line must not

exceed that which corresponds to an error one-twentieth of the course sector

width when an aircraft is positioned on the course line and is in a roll

attitude of 20 deg. from the horizontal.

(6) The SDF must provide signals sufficient to allow satisfactory operation

of a typical aircraft installation within the sector which extends from the

center of the SDF antenna system to distances of 18 nautical miles within a

plus or minus 10 deg. sector and 10 nautical miles within the remainder of

the coverage when alternative navigational facilities provide satisfactory

coverage within the intermediate approach area. SDF signals must be

receivable at the distances specified at and above a height of 1,000 feet

above the elevation of the threshold, or the lowest altitude authorized for

transition, whichever is higher. Such signals must be receivable, to the

distances specified, up to a surface extending outward from the SDF antenna

and inclined at 7 deg. above the horizontal.

(7) The modulation tones must be phase-locked so that within the half

course sector, the demodulated 90 Hz and 150 Hz wave forms pass through zero

in the same direction within 20 deg. of phase relative to the 150 Hz

component, every half cycle of the combined 90 Hz and 150 Hz wave form.

However, the phase need not be measured within the half course sector.

(8) The angle of convergence of the final approach course and the extended

runway centerline must not exceed 30 deg.. The final approach course must be

aligned to intersect the extended runway centerline between points A1 and the

runway threshold. When an operational advantage can be achieved, a final

approach course that does not intersect the runway or that intersects it at a

distance greater than point A1 from the threshold, may be established, if

that course lies within 500 feet laterally of the extended runway centerline

at a point 3,000 feet outward from the runway threshold. The mean course line

must be maintained within +/-10 percent of the course sector width.

(9) The nominal displacement sensitivity within the half course sector must

be 50 microamperes/degree. The nominal course sector width must be 6 deg..

When an operational advantage can be achieved, a nominal displacement

sensitivity of 25 microamperes/degree may be established, with a nominal

course sector width of 12 deg. with proportional displacement sensitivity.

The lateral displacement sensitivity must be adjusted and maintained within

the limits of plus or minus 17 percent of the nominal value.

(10) The off-course (clearance) signal must increase at a substantially

linear rate with respect to the angular displacement from the course line up

to an angle on either side of the course line where 175 microamperes of

deflection is obtained. From that angle to +/-10 deg., the off-course

deflection must not be less than 175 microamperes. From +/-10 deg. to +/-35

deg. the off-course deflection must not be less than 150 microamperes. With

the course adjusted to cause any of several monitor alarm conditions, the

aforementioned values of 175 microamperes in the sector 10 deg. each side of

course and 150 microamperes in the sector +/-10 deg. to +/-35 deg. may be

reduced to 160 microamperes and 135 microamperes, respectively. These

conditions must be met at a distance of 18 nautical miles from the SDF

antenna within the sector 10 deg. each side of course line and 10 nautical

miles from the SDF antenna within the sector +/-10 deg. to +/-35 deg. each

side of course line.

(11) The SDF may provide a ground-to-air radiotelephone communication

channel to be operated simultaneously with the navigation and identification

signals, if that operation does not interfere with the basic function. If a

channel is provided, it must conform with the following standards:

(i) The channel must be on the same radiofrequency carrier or carriers as

used for the SDF function, and the radiation must be horizontally polarized.

Where two carriers are modulated with speech, the relative phases of the

modulations on the two carriers must avoid the occurence of nulls within the

coverage of the SDF.

(ii) On centerline, the peak modulation depth of the carrier or carriers

due to the radiotelephone communications must not exceed 50 percent but must

be adjusted so that the ratio of peak modulation depth due to the

radiotelephone communications to that due to the identification signal is

approximately 9:1.

(iii) The audiofrequency characteristics of the radiotelephone channel must

be flat to within 3 db relative to the level at 1,000 Hz over the range from

300 Hz to 3,000 Hz.

(12)(i) The SDF must provide for the simultaneous transmission of an

identification signal, specific to the runway and approach direction, on the

same radiofrequency carrier or carriers as used for the SDF function. The

transmission of the identification signal must not interfere in any way with

the basic SDF function.

(ii) The identification signal must be produced by Class A2 modulation of

the radiofrequency carrier or carriers using a modulation tone of 1020 Hz

within +/-50 Hz. The depth of modulation must be between the limits of 5 and

15 percent except that, where a radiotelephone communication channel is

provided, the depth of modulation must be adjusted so that the ratio of peak

modulation depth due to radiotelephone communications to that due to the

identification signal modulation is approximately 9:1. The emissions carrying

the identification signal must be horizontally polarized.

(iii) The identification signal must employ the International Morse Code

and consist of three letters.

(iv) The identification signal must be transmitted at a speed corresponding

to approximately seven words per minute, and must be repeated at

approximately equal intervals, not less than six times per minute. When SDF

transmission is not available for operational use, including periods of

removal of navigational components or during maintenance or test

transmissions, the identification signal must be suppressed.

(b) It must be shown during ground inspection of the design features of the

equipment that there will not be conditions that will allow unsafe operations

because of component failure or deterioration.

(c) The monitor must be checked periodically during the in-service test

evaluation period for calibration and stability. These tests, and ground

checks of SDF radiation characteristics must be conducted in accordance with

the maintenance manual required by Sec. 171.115(c) and must meet the

standards and tolerances contained in Sec. 171.111(j).

(d) The monitor system must provide a warning to the designated control

point(s) when any of the conditions of Sec. 171.111(j) occur, within the time

periods specified in that paragraph.

(e) Flight inspection to determine the adequacy of the facility's

operational performance and compliance with applicable performance

requirements must be conducted in accordance with the "U.S. Standard Flight

Inspection Manual." Tolerances contained in the U.S. Standard Flight

Inspection Manual, section 217, must be complied with except as stated in

paragraph (f) of this section.

(f) Flight inspection tolerances specified in section 217 of the "U.S.

Standard Flight Inspection Manual" must be complied with except as follows:

(1) Course sector width. The nominal course sector width must be 6 deg..

When an operational advantage can be achieved, a nominal course sector width

of 12 deg. may be established. Course sector width must be adjusted and

maintained within the limits of +/-17 percent of the nominal value.

(2) Course alignment. The mean course line must be adjusted and maintained

within the limits of +/-10 percent of the nominal course sector width.

(3) Course structure. Course deviations due to roughness, scalloping, or

bends must be within the following limitations:

(i) Front course. (a) Course structure from 18 miles from runway threshold

to Point A must not exceed +/-40 microamperes;

(b) Point A to Point A-1--linear decrease from not more than +/-40

microamperes at Point A to not more than +/-20 microamperes at Point A-1;

(c) Point A-1 to Missed Approach Point--not more than +/-20 microamperes;

(d) Monitor tolerances: width +/-17 percent of nominal; alignment--+/-10

percent of nominal course sector width.

(ii) Back course. (a) Course structure 18 miles from runway threshold to 4

miles from runway threshold must not exceed +/-40 microamperes. Four miles to

1 mile from R/W must not exceed +/-40 microamperes decreasing to not more

than +/-20 microamperes, at a linear rate.

(b) Monitor tolerances: width--+/-17 percent of nominal; alignment--+/-10

percent of nominal course sector width.

[Doc. No. 10116, 35 FR 12711, Aug. 11, 1970, as amended by Amdt. 171-9, 38 FR

28557, Oct. 15, 1973]

Sec. 171.111   Ground standards and tolerances.

Compliance with this section must be shown as a condition to approval and

must be maintained during operation of the SDF.

(a) Frequency. (1) The SDF must operate on odd tenths or odd tenths plus a

twentieth MHz within the frequency band 108.1 MHz to 111.95 MHz. The

frequency tolerance of the radio frequency carrier must not exceed plus or

minus 0.002 percent.

(2) The modulating tones must be 90 Hz and 150 Hz within +/-2.5 percent.

(3) The identification signal must be 1020 Hz within +/-50 Hz.

(4) The total harmonic content of the 90 Hz tone must not exceed 10

percent.

(5) The total harmonic content of the 150 Hz tone must not exceed 10

percent.

(b) Power output. The normal carrier power output must be of a value which

will provide coverage requirements of Sec. 171.109(a)(6) when reduced by 3 dB

to the monitor RF power reduction alarm point specified in Sec.

171.111(j)(3).

(c) VSWR. (1) The VSWR of carrier and sideband feedlines must be a nominal

value of 1/1 and must not exceed 1.2/1.

(2) The sponsor will also provide additional manufacturer's ground

standards and tolerances for all VSWR parameters peculiar to the equipment

which can effect performance of the facility in meeting the requirements

specified in Secs. 171.109 and 171.111.

(d) Insulation resistance. The insulation resistance of all coaxial

feedlines must be greater than 20 megohms.

(e) Depth of modulation. (1) The depth of modulation of the radio frequency

carrier due to each of the 90 Hz and 150 Hz tones must be 20 percent +/-2

percent along the course line.

(2) The depth of modulation of the radiofrequency carrier due to the 1020

Hz identification signal must be within 5 percent to 15 percent.

(f) Course sector width. The standard course sector width must be 6 deg. or

12 deg.. The course sector must be maintained with +/-17 percent of the

standard.

(g) Course alignment. Course alignment must be as specified in Sec.

171.109(a)(8).

(h) Back course alignment and width. If a back course is provided,

standards and tolerances for back course sector width and alignment must be

the same as course sector width and course alignment specified in paragraphs

(f) and (g) of this section.

(i) Clearance. Clearance must be as specified in Sec. 171.109(a)(10).

(j) Monitor standards and tolerances. (1) The monitor system must provide a

warning to the designated control point(s) when any of the conditions

described in this paragraph occur, within the time periods specified in

paragraph (j)(6) of this section.

(2) Course shift alarm: The monitor must alarm and cause radiation to

cease, or identification and navigation signals must be removed, if the

course alignment deviates from standard alignment by 10 percent or more of

the standard course sector width.

(3) RF power reduction alarm: The monitor must alarm and cause radiation to

cease, or identification and navigation signals must be removed, if the

output power is reduced by 3 db or more from normal.

(4) Modulation level alarm: The monitor must alarm and cause radiation to

cease, or identification and navigation signals must be removed, if the 90 Hz

and 150 Hz modulation levels decrease by 17 percent or more.

(5) Course sector width alarm: The monitor must alarm and cause radiation

to cease, or identification and navigation signals must be removed, for a

change in course sector width to a value differing by +/-17 percent or more

from the standard.

(6) Monitor delay before shutdown: Radiation must cease, or identification

and navigation signals must be removed, within 10 seconds after a fault is

detected by the monitor, and no attempt must be made to resume radiation for

a period of at least 20 seconds. If an automatic recycle device is used, not

more than three successive recycles may be permitted before a complete SDF

shutdown occurs.

(k) Mean time between failures. The mean time between failures must not be

less than 800 hours. This measure is applied only to equipment failures

(monitor or transmitting equipment, including out of tolerance conditions)

which result in facility shutdown. It does not relate to the responsiveness

of the maintenance organization.

(l) Course alignment stability. Drift of the course alignment must not

exceed one-half the monitor limit in a 1-week period.

[Doc. No. 10116, 35 FR 12711, Aug. 11, 1970, as amended by Amdt. 171-9, 38 FR

28558, Oct. 15, 1973]

Sec. 171.113   Installation requirements.

(a) The facility must be installed according to accepted good engineering

practices, applicable electric and safety codes, and FCC requirements.

(b) The SDF facility must have the following basic components:

(1) VHF SDF equipment and associated monitor system;

(2) Remote control, and indicator equipment (remote monitor) when required

by the FAA;

(3) A final approach fix; and

(4) Compass locator (COMLO) or marker if suitable fixes and initial

approach routes are not available from existing facilities.

(c) The facility must have a reliable source of suitable primary power,

either from a power distribution system or locally generated. Also, adequate

power capacity must be provided for operation of test and working equipment

at the SDF. A determination by the Federal Aviation Administration as to

whether a facility will be required to have standby power for the SDF and

monitor accessories to supplement the primary power will be made for each

airport based upon operational minimums and density of air traffic.

(d) A determination by the Federal Aviation Administration as to whether a

facility will be required to have dual transmitting equipment with automatic

changeover for the SDF will be made for each airport based upon operational

minimums and density of air traffic.

(e) There must be a means for determining, from the ground, the performance

of the equipment (including antennae), initially and periodically.

(f) The facility must have the following ground/air or landline

communication services:

(1) At facilities outside of and not immediately adjacent to air traffic

control zones or areas, there must be ground/air communications from the

airport served by the facility. The utilization of voice on the SDF should be

determined by the facility operator on an individual basis.

(2) At facilities within or immediately adjacent to air traffic control

zones or areas, there must be the ground/air communications required by

paragraph (b)(1) of this section and reliable communications (at least a

landline telephone) from the airport to the nearest Federal Aviation

Administration air traffic control or communications facility.

Compliance with paragraphs (f) (1) and (2) of this section need not be shown

at airports where an adjacent Federal Aviation Administration facility can

communicate with aircraft on the ground at the airport and during the entire

proposed instrument approach procedure. In addition, at low traffic density

airports within or immediately adjacent to air traffic control zones or

areas, and where extensive delays are not a factor, the requirements of

paragraphs (f) (1) and (2) of this section may be reduced to reliable

communications (at least a landline telephone) from the airport to the

nearest Federal Aviation Administration air traffic control or communication

facility, if an adjacent Federal Aviation Administration facility can

communicate with aircraft during the proposed instrument approach procedure

down to the airport surface or at least to the minimum approach altitude.

(g) At those locations where two separate SDF facilities serve opposite

ends of a single runway, an interlock must insure that only the facility

serving the approach direction in use can radiate, except where no

operationally harmful interference results.

(h) At those locations where, in order to alleviate frequency congestion,

the SDF facilities serving opposite ends of one runway employ identical

frequencies, an interlock must insure that the facility not in operational

use cannot radiate.

(i) Provisions for maintenance and operations by authorized persons only.

(j) Where an operational advantage exists, the installation may omit a back

course.

EFFECTIVE DATE NOTE: Amdt. 171-16, 56 FR 65664, Dec. 17, 1991, revised

paragraph (f) of Sec. 171.113 effective September 16, 1993. For the

convenience of the user, the revised text is set forth as follows:

Sec. 171.113   Installation requirements.

* * * * *

(f) The facility must have the following ground-air or landline

communication services:

(1) At facilities outside of and not immediately adjacent to controlled

airspace, there must be ground-air communications from the airport served by

the facility. The utilization of voice on the SDF should be determined by the

facility operator on an individual basis.

(2) At facilities within or immediately adjacent to controlled airspace,

there must be ground/air communications required by paragraph (b)(1) of this

section and reliable communications (at least a landline telephone) from the

airport to the nearest Federal Aviation Administration air traffic control or

communications facility.

Compliance with paragraphs (f) (1) and (2) of this section need not be shown

at airports where an adjacent Federal Aviation Administration facility can

communicate with aircraft on the ground at the airport and during the entire

proposed instrument approach procedure. In addition, at low traffic density

airports within or immediately adjacent to controlled airspace, and where

extensive delays are not a factor, the requirements of paragraphs (f) (1) and

(2) of this section may be reduced to reliable communications (at least a

landline telephone) from the airport to the nearest Federal Aviation

Administration air traffic control or communications facility, if an adjacent

Federal Aviation Administration facility can communicate with aircraft during

the proposed instrument approach procedure down to the airport surface or at

least down to the minimum approach altitude.

Sec. 171.115   Maintenance and operations requirements.

(a) The owner of the facility shall establish an adequate maintenance

system and provide qualified maintenance personnel to maintain the facility

at the level attained at the time it was commissioned. Each person who

maintains a facility shall meet at a minimum the Federal Communications

Commission's licensing requirements and show that he has the special

knowledge and skills needed to maintain the facility, including proficiency

in maintenance procedures and the use of specialized test equipment.

(b) The SDF must be designed and maintained so that the probability of

operation within the performance requirements specified is high enough to

insure an adequate level of safety. In the event out-of-tolerance conditions

develop, the facility shall be removed from operation, and the designated

control point notified.

(c) The owner must prepare, and obtain approval of, and each person

operating or maintaining the facility shall comply with, an operations and

maintenance manual that sets forth procedures for operations, preventive

maintenance, and emergency maintenance, including instructions on each of the

following:

(1) Physical security of the facility. This includes provisions for

designating critical areas relative to the facility and preventing or

controlling movements within the facility that may adversely affect SDF

operations.

(2) Maintenance and operations by authorized persons only.

(3) Federal Communications Commission requirements for operating personnel

and maintenance personnel.

(4) Posting of licenses and signs.

(5) Relation between the facility and Federal Aviation Administration air

traffic control facilities, with a description of the boundaries of

controlled airspace over or near the facility, instructions for relaying air

traffic control instructions and information (if applicable), and

instructions for the operation of an air traffic advisory service if the

facility is located outside of controlled airspace.

(6) Notice to the Administrator of any suspension of service.

(7) Detailed and specific maintenance procedures and servicing guides

stating the frequency of servicing.

(8) Air-ground communications, if provided, expressly written or

incorporating appropriate sections of Federal Aviation Administration manuals

by reference.

(9) Keeping of station logs and other technical reports, and the submission

of reports required by Sec. 171.117.

(10) Monitoring of the facility.

(11) Names, addresses, and telephone numbers of persons to be notified in

an emergency.

(12) Inspection by U.S. personnel.

(13) Shutdowns for routine maintenance and issue of "Notices to Airmen" for

routine or emergency shutdowns, except that private use facilities may omit

"Notices to Airmen."

(14) Commissioning of the facility.

(15) An acceptable procedure for amending or revising the manual.

(16) An explanation of the kinds of activities (such as construction or

grading) in the vicinity of the facility that may require shutdown or

certification of the facility by Federal Aviation Administration flight

check.

(17) Procedure for conducting a ground check of SDF course alignment, width

and clearance.

(18) The following information concerning the facility:

(i) Facility component locations with respect to airport layout, instrument

runway, and similar areas;

(ii) The type, make, and model of the basic radio equipment that will

provide the service;

(iii) The station power emission and frequencies of the SDF, markers and

associated COMLOs, if any;

(iv) The hours of operation;

(v) Station identification call letters and method of station

identification and the time spacing of the identification;

(vi) A description of the critical parts that may not be changed, adjusted,

or repaired without a Federal Aviation Administration flight check to confirm

published operations.

(d) The owner shall make a ground check of the facility each month in

accordance with procedures approved by the Federal Aviation Administration at

the time of commissioning, and shall report the results of the checks as

provided in Sec. 171.117.

(e) If the owner desires to modify the facility, he shall submit the

proposal to the Federal Aviation Administration and may not allow any

modifications to be made without specific approval.

(f) The owner's maintenance personnel shall participate in initial

inspections made by the Federal Aviation Administration. In the case of

subsequent inspections, the owner or his representatives shall participate.

(g) Whenever it is required by the Federal Aviation Administration, the

owner shall incorporate improvements in SDF maintenance. In addition, he

shall provide a stock of spare parts, of such a quantity, to make possible

the prompt replacement of components that fail or deteriorate in service.

(h) The owner shall provide Federal Aviation Administration approved test

instruments needed for maintenance of the facility.

(i) The owner shall close the facility by ceasing radiation and shall issue

a "Notice to Airmen" that the facility is out of service (except that private

use facilities may omit "Notices to Airmen"), upon receiving two successive

pilot reports of its malfunctioning.

Sec. 171.117   Reports.

The owner of each facility to which this subpart applies shall make the

following reports, at the time indicated, to the Federal Aviation

Administration Regional Office for the area in which the facility is located:

(a) Record of meter readings and adjustments (Form FAA-198). To be filled

out by the owner or his maintenance representative with the equipment

adjustments and meter readings as of the time of commissioning, with one copy

to be kept in the permanent records of the facility and two copies to the

appropriate Regional Office of the Federal Aviation Administration. The owner

shall revise the form after any major repair, modification, or retuning, to

reflect an accurate record of facility operation and adjustment.

(b) Facility maintenance log (FAA Form 6030-1) This form is a permanent

record of all equipment malfunctioning met in maintaining the facility,

including information on the kind of work and adjustments made, equipment

failures, causes (if determined), and corrective action taken. The owner

shall keep the original of each report at the facility and send a copy to the

appropriate Regional Office of the Federal Aviation Administration at the end

of each month in which it is prepared.

(c) Radio equipment operation record (Form FAA-418), containing a complete

record of meter readings, recorded on each scheduled visit to the facility.

The owner shall keep the original of each month's record at the facility and

send a copy of it to the appropriate Regional Office of the Federal Aviation

Administration.

[Doc. No. 10116, 35 FR 12711, Aug. 11, 1970, as amended by Amdt. 171-10, 40

FR 36110, Aug. 19, 1975]

Subpart G--Distance Measuring Equipment (DME)

Source: Docket No. 10116, 35 FR 12715, Aug. 11, 1970, unless otherwise

noted.

Sec. 171.151   Scope.

This subpart sets forth minimum requirements for the approval and operation

of non-Federal DME facilities that are to be involved in the approval of

instrument flight rules and air traffic control procedures related to those

facilities.

Sec. 171.153   Requests for IFR procedure.

(a) Each person who requests an IFR procedure based on a DME facility that

he owns must submit the following information with that request:

(1) A description of the facility and evidence that the equipment meets the

performance requirements of Sec. 171.157 and is installed in accordance with

Sec. 171.159.

(2) A proposed procedure for operating the facility.

(3) A proposed maintenance organization and maintenance manual that meets

the requirement of Sec. 171.161.

(4) A statement of intention to meet the requirements of this subpart.

(5) A showing that the facility has an acceptable level of operational

reliability and an acceptable standard of performance. Previous equivalent

operational experience with a facility with identical design and operational

characteristics will be considered in showing compliance with this paragraph.

(b) After the Federal Aviation Administration inspects and evaluates the

facility, it advises the owner of the results and of any required changes in

the facility or the maintenance manual or maintenance organization. The owner

must then correct the deficiencies, if any, and operate the facility for an

in-service evaluation by the Federal Aviation Administration.

Sec. 171.155   Minimum requirements for approval.

(a) The following are the minimum requirements that must be met before the

Federal Aviation Administration will approve an IFR procedure for a non-

Federal DME:

(1) A suitable frequency channel must be available.

(2) The facility's performance, as determined by air and ground inspection,

must meet the requirements of Sec. 171.157.

(3) The installation of the equipment must meet the requirements of Sec.

171.159.

(4) The owner must agree to operate and maintain the facility in accordance

with Sec. 171.161.

(5) The owner must agree to furnish periodic reports, as set forth in Sec.

171.163, and must agree to allow the Federal Aviation Administration to

inspect the facility and its operation whenever necessary.

(6) The owner must assure the Federal Aviation Administration that he will

not withdraw the facility from service without the permission of the Federal

Aviation Administration.

(7) The owner must bear all costs of meeting the requirements of this

section and of any flight or ground inspections made before the facility is

commissioned, except that the Federal Aviation Administration may bear

certain of these costs subject to budgetary limitations and policy

established by the Administrator.

(b) If the applicant for approval meets the requirements of paragraph (a)

of this section, the Federal Aviation Administration commissions the facility

as a prerequisite to its approval for use in an IFR procedure. The approval

is withdrawn at any time the facility does not continue to meet those

requirements.

Sec. 171.157   Performance requirements.

(a) The DME must meet the performance requirements set forth in the

"International Standards and Recommended Practices. Aeronautical

Telecommunications, Part I, Paragraph 3.5" (Annex 10 to the Convention of

International Civil Aviation).

(b) It must be shown during ground inspection of the design features of the

equipment that there will not be conditions that will allow unsafe operations

because of component failure or deterioration.

(c) The monitor must be checked periodically, during the in-service test

evaluation period, for calibration and stability. These tests and ground

tests of the functional and performance characteristics of the DME

transponder must be conducted in accordance with the maintenance manual

required by Sec. 171.161(b).

(d) Flight inspection to determine the adequacy of the facility's

operational performance and compliance with applicable "Standards and

Recommended Practices" must be accomplished in accordance with the "U.S.

Standard Flight Inspection Manual."

[Docket No. 10116, 35 FR 12715, Aug. 11,1970, as amended by Amdt. 171-13, 50

FR 4875, Nov. 27, 1985]

Sec. 171.159   Installation requirements.

(a) The facility must be installed according to accepted good engineering

practices, applicable electric and safety codes, and Federal Communications

Commission requirements.

(b) The facility must have a reliable source of suitable primary power,

either from a power distribution system or locally generated, with a

supplemental standby system, if needed.

(c) Dual transmitting equipment with automatic changeover is preferred and

may be required to support certain IFR procedures.

(d) There must be a means for determining from the ground, the performance

of the equipment, initially and periodically.

(e) A facility intended for use as an instrument approach aid for an

airport must have or be supplemented by the following ground air or landline

communications services:

(1) At facilities outside of and not immediately adjacent to air traffic

control areas, there must be ground-air communications from the airport

served by the facility. Separate communications channels are acceptable.

(2) At facilities within or immediately adjacent to air traffic control

areas, there must be the ground-air communications required by paragraph

(e)(1) of this section and reliable communications (at least a landline

telephone) from the airport to the nearest Federal Aviation Administration

air traffic control or communication facility. Separate communications

channels are acceptable.

Compliance with paragraphs (e) (1) and (2) of this section need not be shown

at airports where an adjacent Federal Aviation Administration facility can

communicate with aircraft on the ground at the airport and during the entire

proposed instrument approach procedure. In addition, at low traffic density

airports within or immediately adjacent to air traffic control zones or

areas, and where extensive delays are not a factor, the requirements of

paragraphs (e) (1) and (2) of this section may be reduced to reliable

communications (at least a landline telephone) from the airport to the

nearest Federal Aviation Administration air traffic control or communications

facility, if an adjacent Federal Aviation Administration facility can

communicate with aircraft during the proposed instrument approach procedure,

at least down to the minimum en route altitude for the controlled area.

EFFECTIVE DATE NOTE: Amdt. 171-16, 56 FR 65665, Dec. 17, 1991, revised

paragraphs (e)(1) and (e)(2) of Sec. 171.159 effective September 16, 1993.

For the convenience of the user, the revised text is set forth as follows:

Sec. 171.159   Installation requirements.

* * * * *

(e) * * *

(1) At facilities outside of and not immediately adjacent to controlled

airspace, there must be ground-air communications from the airport served by

the facility. Separate communications channels are acceptable.

(2) At facilities within or immediately adjacent to controlled airspace,

there must be the ground-air communications required by paragraph (e)(1) of

this section and reliable communications (at least a landline telephone) from

the airport to the nearest Federal Aviation Administration air traffic

control or communications facility. Separate communications channels are

acceptable.

Compliance with paragraphs (e) (1) and (2) of this section need not be shown

at airports where an adjacent Federal Aviation Administration facility can

communicate with aircraft on the ground at the airport and during the entire

proposed instrument approach procedure. In addition, at low traffic density

airports within or immediately adjacent to controlled airspace, and where

extensive delays are not a factor, the requirements of paragraphs (e) (1) and

(2) of this section may be reduced to reliable communications (at least a

landline telephone) from the airport to the nearest Federal Aviation

Administration air traffic control or communications facility, if an adjacent

Federal Aviation Administration facility can communicate with aircraft during

the proposed instrument approach procedure, at least down to the minimum en

route altitude for the controlled airspace area.

Sec. 171.161   Maintenance and operations requirements.

(a) The owner of the facility shall establish an adequate maintenance

system and provide qualified maintenance personnel to maintain the facility

at the level attained at the time it was commissioned. Each person who

maintains a facility shall meet at a minimum the Federal Communications

Commission's licensing requirements and show that he has the special

knowledge and skills needed to maintain the facility, including proficiency

in maintenance procedures and the use of specialized test equipment.

(b) The owner must prepare and obtain Federal Aviation Administration

approval of, and each person operating or maintaining the facility shall

comply with, an operations and maintenance manual that sets forth procedures

for operations, preventive maintenance, and emergency maintenance, including

instructions on each of the following:

(1) Physical security of the facility.

(2) Maintenance and operations by authorized persons only.

(3) Federal Communications Commission's requirements and maintenance

personnel.

(4) Posting of licenses and signs.

(5) Relations between the facility and Federal Aviation Administration air

traffic control facilities, with a description of the boundaries of

controlled airspace over or near the facility, instructions for relaying air

traffic control instructions and information (if applicable), and

instructions for the operation of an air traffic advisory service if the DME

is located outside of controlled airspace.

(6) Notice to the Administrator of any suspension of service.

(7) Detailed and specific maintenance procedures and servicing guides

stating the frequency of servicing.

(8) Air-ground communications, if provided, expressly written or

incorporating appropriate sections of Federal Aviation Administration manuals

by reference.

(9) Keeping of station logs and other technical reports, and the submission

of reports required by Sec. 171.163.

(10) Monitoring of the facility.

(11) Inspections by U.S. personnel.

(12) Names, addresses, and telephone numbers of persons to be notified in

an emergency.

(13) Shutdowns for routine maintenance and issue of "Notices to Airmen" for

routine or emergency shutdowns, except that private use facilities may omit

the "Notices to Airmen."

(14) An explanation of the kinds of activity (such as construction or

grading) in the vicinity of the facility that may require shutdown or

reapproval of the facility by Federal Aviation Administration flight check.

(15) Commissioning of the facility.

(16) An acceptable procedure for amending or revising the manual.

(17) The following information concerning the facility:

(i) Location by latitude and longitude to the nearest second, and its

position with respect to airport layouts.

(ii) The type, make, and model of the basic radio equipment that will

provide the service.

(iii) The station power emission and frequency.

(iv) The hours of operation.

(v) Station identification call letters and methods of station

identification, whether by Morse code or recorded voice announcement, and the

time spacing of the identification.

(vi) A description of the critical parts that may not be changed, adjusted,

or repaired without an FAA flight check to confirm published operations.

(c) The owner shall make a monthly ground operational check in accordance

with procedures approved by the FAA at the time of commissioning, and shall

report the results of the checks as provided in Sec. 171.163.

(d) If the owner desires to modify the facility, he shall submit the

proposal to the FAA and may not allow any modifications to be made without

specific approval.

(e) The owner's maintenance personnel shall participate in initial

inspections made by the FAA. In the case of subsequent inspections, the owner

or his representative shall participate.

(f) Whenever it is required by the FAA, the owner shall incorporate

improvements in DME maintenance.

(g) The owner shall provide a stock of spare parts of such a quantity to

make possible the prompt replacement of components that fail or deteriorate

in service.

(h) The owner shall provide FAA-approved test instruments needed for

maintenance of the facility.

(i) The owner shall shut down the facility (i.e., cease radiation and issue

a NOTAM that the facility is out-of-service) upon receiving two successive

pilot reports of its malfunctioning.

Sec. 171.163   Reports.

The owner of each facility to which this subpart applies shall make the

following reports on forms furnished by the FAA, at the time indicated, to

the FAA Regional office for the area in which the facility is located:

(a) Record of meter readings and adjustments (Form FAA-198). To be filled

out by the owner with the equipment adjustments and meter readings as of the

time of commissioning, with one copy to be kept in the permanent records of

the facility and two copies to the appropriate Regional office of the FAA.

The owner shall revise the form after any major repair, modification, or

returning, to reflect an accurate record of facility operation and

adjustment.

(b) Facility maintenance log (FAA Form 6030-1). This form is a permanent

record of all equipment malfunctioning met in maintaining the facility,

including information on the kind of work and adjustments made, equipment

failures, causes (if determined), and corrective action taken. The owner

shall keep the original of each report at the facility and send a copy to the

appropriate Regional Office of the Federal Aviation Administration at the end

of the month in which it is prepared.

(c) Radio equipment operation record (Form FAA-418), containing a complete

record of meter readings, recorded on each scheduled visit to the facility.

The owner shall keep the original of each month's record at the facility and

send a copy of it to the appropriate Regional Office of the Federal Aviation

Administration.

[Doc. No. 10116, 35 FR 12715, Aug. 11, 1970, as amended by Amdt. 171-10, 40

FR 36110, Aug. 19, 1975]

Subpart H--VHF Marker Beacons

Source: Docket No. 10116, 35 FR 12716, Aug. 11, 1970, unless otherwise

noted.

Sec. 171.201   Scope.

(a) This subpart sets forth minimum requirements for the approval and

operation of non-Federal VHF marker beacon facilities that are to be involved

in the approval of instrument flight rules and air traffic control procedures

related to those facilities.

Sec. 171.203   Requests for IFR procedure.

(a) Each person who requests an IFR procedure which will incorporate the

use of a VHF marker beacon facility that he owns must submit the following

information with that request:

(1) A description of the facility and evidence that the equipment meets the

performance requirements of Sec. 171.207 and is installed in accordance with

Sec. 171.209.

(2) A proposed procedure for operating the facility.

(3) A proposed maintenance organization and a maintenance manual that meets

the requirements of Sec. 171.211.

(4) A statement of intent to meet the requirement of this subpart.

(5) A showing that the facility has an acceptable level of operational

reliability, and an acceptable standard of performance. Previous equivalent

operational experience may be shown to comply with this subparagraph.

(b) After the Federal Aviation Administration inspects and evaluates the

facility, it advises the owner of the results and of any required changes in

the facility or the maintenance manual or maintenance organization. The owner

shall then correct the deficiencies, if any, and operate the facility for an

in-service evaluation by the Federal Aviation Administration.

Sec. 171.205   Minimum requirements for approval.

(a) The following are the minimum requirements that must be met before the

Federal Aviation Administration will approve an IFR procedure which

incorporates the use of a non-Federal VHF marker beacon facility under this

subpart:

(1) The facility's performances, as determined by air and ground

inspection, must meet the requirements of Sec. 171.207.

(2) The installation of the equipment must meet the requirements of Sec.

171.209.

(3) The owner must agree to operate and maintain the facility in accordance

with Sec. 171.211.

(4) The owner must agree to furnish periodic reports, as set forth in Sec.

171.213, and agree to allow the Federal Aviation Administration to inspect

the facility and its operation whenever necessary.

(5) The owner must assure the Federal Aviation Administration that he will

not withdraw the facility from service without the permission of the Federal

Aviation Administration.

(6) The owner must bear all costs of meeting the requirements of this

section and of any flight or ground inspections made before the facility is

commissioned, except that the Federal Aviation Administration may bear

certain of these costs subject to budgetary limitations and policy

established by the Administrator.

(b) If the applicant for approval meets the requirements of paragraph (a)

of this section, the Federal Aviation Administration commissions the facility

as a prerequisite to its approval for use in an IFR procedure. The approval

is withdrawn at any time the facility does not continue to meet those

requirements.

Sec. 171.207   Performance requirements.

(a) VHF Marker Beacons must meet the performance requirements set forth in

the "International Standards and Recommended Practices, Aeronautical

Telecommunications, Part I, paragraphs 3.1.6 and 3.6." (Annex 10 to the

Convention on International Civil Aviation) except those portions that

pertain to identification. Identification of a marker beacon (75 MHz) must be

in accordance with "U.S. Standard Flight Inspection Manual," Sec. 219.

(b) The facility must perform in accordance with recognized and accepted

good electronic engineering practices for the desired service. The facility

must be checked periodically during the in-service test evaluation period for

calibration and stability. These tests and ground tests of the marker

radiation characteristics must be conducted in accordance with the

maintenance manual required by Sec. 171.211(b).

(c) It must be shown during ground inspection of the design features of the

equipment that there will not be conditions that will allow unsafe operations

because of component failure or deterioration.

(d) Flight inspection to determine the adequacy of the facility's

operational performance and compliance with applicable "Standards and

Recommended Practices" are conducted in accordance with the "U.S. Standard

Flight Inspection Manual." The original test is made by the Federal Aviation

Administration and later tests must be made under arrangements, satisfactory

to the Federal Aviation Administration, that are made by the owner.

Sec. 171.209   Installation requirements.

(a) The facility must be installed according to accepted good engineering

practices, applicable electric and safety codes, and Federal Communications

Commission requirements.

(b) The facility must have a reliable source of suitable primary power.

(c) Dual transmitting equipment may be required, if applicable, to support

certain IFR procedures.

(d) At facilities within or immediately adjacent to air traffic control

areas, and that are intended for use as instrument approach aids for an

airport, there must be ground-air communications or reliable communications

(at least a landline telephone) from the airport to the nearest Federal

Aviation Administration air traffic control or communication facility.

Compliance with this paragraph need not be shown at airports where an

adjacent Federal Aviation Administration facility can communicate with

aircraft on the ground at the airport and during the entire proposed

instrument approach procedure. In addition, at low traffic density airports

within or immediately adjacent to air traffic control zones or areas, and

where extensive delays are not a factor, the requirements of this paragraph

may be reduced to reliable communications (at least a landline telephone)

from the airport to the nearest Federal Aviation Administration air traffic

control or communications facility, if an adjacent Federal Aviation

Administration facility can communicate with aircraft during the proposed

instrument approach procedure, at least down to the minimum en route altitude

for the controlled area.

EFFECTIVE DATE NOTE: Amdt. 171-16, 56 FR 65665, Dec. 17, 1991, revised

paragraph (d) of Sec. 171.209 effective September 16, 1993. For the

convenience of the user, the revised text is set forth as follows:

Sec. 171.209   Installation requirements.

* * * * *

(d) At facilities within or immediately adjacent to controlled airspace and

that are intended for use as instrument approach aids for an airport, there

must be ground-air communications or reliable communications (at least a

landline telephone) from the airport to the nearest Federal Aviation

Administration air traffic control or communication facility. Compliance with

this paragraph need not be shown at airports where an adjacent Federal

Aviation Administration facility can communicate with aircraft on the ground

at the airport and during the entire proposed instrument approach procedure.

In addition, at low traffic density airports within or immediately adjacent

to controlled airspace, and where extensive delays are not a factor, the

requirements of this paragraph may be reduced to reliable communications (at

least a landline telephone) from the airport to the nearest Federal Aviation

Administration air traffic control or communications facility, if an adjacent

Federal Aviation Administration facility can communicate with aircraft during

the proposed instrument approach procedure, at least down to the minimum en

route altitude for the controlled airspace area.

Sec. 171.211   Maintenance and operations requirements.

(a) The owner of the facility shall establish an adequate maintenance

system and provide qualified maintenance personnel to maintain the facility

at the level attained at the time it was commissioned. Each person who

maintains a facility shall meet at a minimum the Federal Communications

Commission's licensing requirements and show that he has the special

knowledge and skills needed to maintain the facility, including proficiency

in maintenance procedures and the use of specialized test equipment.

(b) The owner must prepare, and obtain approval of, and each person who

operates or maintains the facility shall comply with, an operations and

maintenance manual that sets forth procedures for operations, preventive

maintenance, and emergency maintenance, including instructions on each of the

following:

(1) Physical security of the facility.

(2) Maintenance and operations by authorized persons only.

(3) Federal Communications Commission's requirements for operating and

maintenance personnel.

(4) Posting of licenses and signs.

(5) Relations between the facility and Federal Aviation Administration air

traffic control facilities, with a description of the boundaries of

controlled airspace over or near the facility, instructions for relaying air

traffic control instructions and information (if applicable).

(6) Notice to the Administrator of any suspension of service.

(7) Detailed arrangements for maintenance, flight inspection, and

servicing, stating the frequency of servicing.

(8) Keeping of station logs and other technical reports, and the submission

of reports required by Sec. 171.213.

(9) Monitoring of the facility, at least once each half hour, to assure

continuous operation.

(10) Inspections by U.S. personnel.

(11) Names, addresses, and telephone numbers of persons to be notified in

an emergency.

(12) Shutdowns for routine maintenance and issue of "Notices to Airmen" for

routine or emergency shutdowns (private use facilities may omit the "Notice

to Airmen").

(13) Commissioning of the facility.

(14) An acceptable procedure for amending or revising the manual.

(15) The following information concerning the facility:

(i) Location by latitude and longitude to the nearest second, and its

position with respect to airport layouts.

(ii) The type, make, and model of the basic radio equipment that will

provide the service.

(iii) The station power emission and frequency.

(iv) The hours of operation.

(v) Station identification call letters and methods of station

identification, whether by Morse Code or recorded voice announcement, and the

time spacing of the identification.

(c) If the owner desires to modify the facility, he shall submit the

proposal to the Federal Aviation Administration and meet applicable

requirements of the Federal Communications Commission, and must not allow any

modification to be made without specific approval by the Federal Aviation

Administration.

(d) The owner's maintenance personnel shall participate in initial

inspections made by the Federal Aviation Administration. In the case of

subsequent inspections, the owner or his representative shall participate.

(e) The owner shall provide a stock of spare parts, of such a quantity to

make possible the prompt replacement of components that fail or deteriorate

in service.

(f) The owner shall shut down the facility by ceasing radiation, and shall

issue a "Notice to Airmen" that the facility is out of service (except that

private use facilities may omit "Notices to Airmen") upon receiving two

successive pilot reports of its malfunctioning.

Sec. 171.213   Reports.

The owner of each facility to which this subpart applies shall make the

following reports, at the times indicated, to the Federal Aviation

Administration Regional Office for the area in which the facility is located:

(a) Record of meter readings and adjustments (Form FAA-198). To be filled

out by the owner or his maintenance representative with the equipment

adjustments and meter readings as of the time of commissioning, with one copy

to be kept in the permanent records of the facility and two copies to the

appropriate Regional Office of the Federal Aviation Administration. The owner

must revise the form after any major repair, modification, or retuning, to

reflect an accurate record of facility operation and adjustment.

(b) Facility maintenance log (FAA Form 6030-1). This form is a permanent

record of all equipment malfunctioning met in maintaining the facility,

including information on the kind of work and adjustments made, equipment

failures, causes (if determined), and corrective action taken. The owner

shall keep the original of each report at the facility and send a copy to the

appropriate Regional Office of the Federal Aviation Administration at the end

of the month in which it is prepared.

(c) Radio equipment operation record (Form FAA-418), containing a complete

record of meter readings, recorded on each scheduled visit to the facility.

The owner shall keep the original of each month's record at the facility and

send a copy of it to the appropriate Regional Office of the Federal Aviation

Administration.

[Doc. No. 10116, 35 FR 12716, Aug. 11, 1970, as amended by Amdt. 171-10, 40

FR 36110, Aug. 19, 1975]

Subpart I--Interim Standard Microwave Landing System (ISMLS)

Source: Docket No. 14120, 40 FR 36110, Aug. 19, 1975, unless otherwise

noted.

Sec. 171.251   Scope.

This subpart sets forth minimum requirements for the approval and operation

of non-Federal Interim Standard Microwave Landing System (ISMLS) facilities

that are to be involved in the approval of instrument flight rules and air

traffic control procedures related to those facilities.

Sec. 171.253   Definitions.

As used in this subpart:

"Angular displacement sensitivity" (Glide Slope) means the ratio of

measured DDM to the corresponding angular displacement from the appropriate

reference line.

"Collocated ground station" means the type of ground station which

transmits two or more guidance signals simultaneously from a common location.

"Course line" means the locus of points nearest to the runway centerline in

any horizontal plane at which the DDM is zero.

"Course sector (full)" means a sector in a horizontal plane containing the

course line and limited by the loci of points nearest to the course line at

which the DDM is 0.155.

"Course sector (half)" means the sector in a horizontal plane containing

the course line and limited by the loci of points nearest to the course line

at which DDM is 0.0775.

"DDM" means difference in depth of modulation. The percentage modulation

depth of the larger signal minus the percentage modulation depth of the

smaller signal, divided by 100.

"Displacement sensitivity" (Localizer) means the ratio of measured DDM to

the corresponding lateral displacement from the appropriate reference line.

"Facility Performance Category I--ISMLS" means an ISMLS which provides

guidance information from the coverage limit of the ISMLS to the point at

which the localizer course line intersects the ISMLS glide path at a height

of 200 feet or less above the horizontal plane containing the threshold.

"Glide path" means that locus of points in the vertical plane containing

the runway center line at which the DDM is zero, which, of all such loci, is

the closest to the horizontal plane.

"Glide path angle" (<theta>) means the angle between a straight line which

represents the mean of the ISMLS glide path and the horizontal.

"Glide path sector (full)" means the sector in the vertical plane

containing the ISMLS glide path and limited by the loci of points nearest to

the glide path at which the DDM is 0.175. The ISMLS glide path sector is

located in the vertical plane containing the runway centerline, and is

divided by the radiated glide path in two parts called upper sector and lower

sector, referring respectively to the sectors above and below the glide path.

"Glide path sector (half)" means the sector in the vertical plane

containing the ISMLS glide path and limited by the loci of points nearest to

the glide path at which the DDM is 0.0875.

"ISMLS Point 'A' " means an imaginary point on the glide path/localizer

course measured along the runway centerline extended, in the approach

direction, four nautical miles from the runway threshold.

"ISMLS Point 'B' " means an imaginary point on the glide path/localizer

course measured along the runway centerline extended, in the approach

direction, 3500 feet from the runway threshold.

"ISMLS Point 'C' " means a point through which the downward extended

straight portion of the glide path (at the commissioned angle) passes at a

height of 100 feet above the horizontal plane containing the runway

threshold.

"Interim standard microwave landing system" (ISMLS) means a ground station

which transmits azimuth and elevation angle information which, when decoded

and processed by the airborne unit, provides signal performance capable of

supporting approach minima for V/STOL and CTOL operations and operates with

the signal format and tolerances specified in Secs. 171.259, 171.261,

171.263, 171.265, and 171.267.

"Integrity" means that quality which relates to the trust which can be

placed in the correctness of the information supplied by the facility.

"Mean corrective time" means the average time required to correct an

equipment failure over a given period, after a service man reaches the

facility.

"Mean time between failures" means the average time between equipment

failure over a given period.

"Reference datum" means a point at a specified height located vertically

above the intersection of the runway centerline and the threshold and through

which the downward extended straight portion of the ISMLS glide path passes.

"Split type ground station" means the type of ground station in which the

electronic components for the azimuth and elevation guidance are contained in

separate housings or shelters at different locations, with the azimuth

portion of the ground station located at the stop end of the runway, and the

elevation guidance near the approach end of the runway.

Sec. 171.255   Requests for IFR procedures.

(a) Each person who requests an IFR procedure based on an ISMLS facility

that he owns must submit the following information with that request:

(1) A description of the facility and evidence that the equipment meets the

performance requirements of Secs. 171.259, 171.261, 171.263, 171.265,

171.267, and 171.269, and is installed in accordance with Sec. 171.271.

(2) A proposed procedure for operating the facility.

(3) A proposed maintenance organization and a maintenance manual that meets

the requirements of Sec. 171.273.

(4) A statement of intent to meet the requirements of this subpart.

(5) A showing that the ISMLS facility has an acceptable level of

operational reliability, maintainability and acceptable standard of

performance. Previous equivalent operational experience with a facility with

identical design and operational characteristics will be considered in

showing compliance with this paragraph.

(b) After the FAA inspects and evaluates the ISMLS facility, it advises the

owner of the results and of any required changes in the ISMLS facility or in

the maintenance manual or maintenance organization. The owner must then

correct the deficiencies, if any, and operate the ISMLS facility for an

inservice evaluation by the FAA.

Sec. 171.257   Minimum requirements for approval.

(a) The following are the minimum requirements that must be met before the

FAA approves an IFR procedure for a non-Federal ISMLS facility:

(1) The performance of the ISMLS facility, as determined by flight and

ground inspection conducted by the FAA, must meet the requirements of Secs.

171.259, 171.261, 171.263, 171.265, 171.267, and 171.269.

(2) The installation of the equipment must meet the requirements of Sec.

171.271.

(3) The owner must agree to operate and maintain the ISMLS facility in

accordance with Sec. 171.273.

(4) The owner must agree to furnish periodic reports as set forth in Sec.

171.275 and agree to allow the FAA to inspect the facility and its operation

whenever necessary.

(5) The owner must assure the FAA that he will not withdraw the ISMLS

facility from service without the permission of the FAA.

(6) The owner must bear all costs of meeting the requirements of this

section and of any flight or ground inspection made before the ISMLS facility

is commissioned, except that the FAA may bear certain costs subject to

budgetary limitations and policy established by the Administrator.

(b) If the applicant for approval meets the requirements of paragraph (a)

of this section, the FAA approves the ISMLS facility for use in an IFR

procedure. The approval is withdrawn at any time that the ISMLS facility does

not continue to meet those requirements. In addition, the ISMLS facility may

be de-commissioned whenever the frequency channel is needed for higher

priority common system service.

Sec. 171.259   Performance requirements: General.

(a) The ISMLS consists of the following basic components:

(1) C-Band (5000 MHz-5030 MHz) localizer equipment, associated monitor

system, and remote indicator equipment;

(2) C-Band (5220 MHz-5250 MHz) glide path equipment, associated monitor

system, and remote indicator equipment;

(3) VHF marker beacons (75 MHz), associated monitor systems, and remote

indicator equipment.

(4) An ISMLS airborne receiver or a VHF/UHF ILS receiver modified to be

capable of receiving the ISMLS signals. This modification requires the

addition of a C-Band antenna, a converter unit, a microwave/ILS mode control,

and a VHF/UHF receiver modification kit.

(b) The electronic ground equipments in paragraph (a)(1), (2), and (3) of

this section, must be designed to operate on a nominal 120/240 volt, 60 Hz,

3-wire single phase AC power source.

(c) ISMLS ground equipment must meet the following service conditions:

(1) AC line parameters, DC voltage, elevation, and duty:

120 V nominal value, 102 V to 138 V (+/-1 V).*

208 V nominal value, 177 V to 239 V (+/-2 V).*

240 V nominal value, 204 V to 276 V (+/-0.2 V).*

AC line frequency (60 Hz), 57 Hz to 63 Hz (+/-0.2 Hz).*

DC voltage (48 V), 44 V to 52 V (+/-0.5 V).*

*Note: Where discrete values of the above frequency or voltages are

specified for testing purposes, the tolerances given in parentheses indicated

by an asterisk apply to the test instruments used to measure these

parameters.

Elevation, 0 to 10,000 ft. above sea level.

Duty, continuous, unattended.

(2) Ambient conditions for localizer and glide path equipment:

Temperature, -10 deg. C to +50 deg. C.

Relative humidity, 5% to 90%.

(3) Ambient conditions for marker beacon facilities and all other equipment

installed outdoors (for example, antennae, field detectors, and shelters):

Temperature, -50 deg. C. to +70 deg. C.

Relative humidity, 5% to 100%.

(4) All equipment installed outdoors must operate satisfactorily under the

following conditions:

Wind velocity, 0-100 MPH (not including gusts).

Hail stones, 1/2 '' diameter.

Rain, provide coverage through a distance of 5 nautical miles with rain

falling at a rate of 50 millimeters per hour, and with rain falling at the

rate of 25 millimeters per hour for the additional design performance range

of the system.

Ice loading, encased in 1/2 '' radial thickness of clear ice.

(d) The ISMLS must perform in accordance with the following standards and

practices for Facility Performance Category I operation:

(1) The ISMLS must be constructed and adjusted so that, at a specified

distance from the threshhold, similar instrumental indications in the

aircraft represent similar displacements from the course line or ISMLS glide

path, as appropriate, regardless of the particular ground installation in

use.

(2) The localizer and glide path components listed in paragraphs (a)(1) and

(a)(2) of this section which form part of an ISMLS, must comply at least with

the standard performance requirements specified herein. The marker beacon

components listed in paragraph (a)(3) of this section which form part of an

ISMLS, must comply at least with the standard performance requirements

specified in Subpart H of this part.

(3) The ISMLS must be so designed and maintained that the probability of

operation is within the performance requirements specified in Sec.

171.273(k).

(e) The signal format and pairing of the runway localizer and glide path

transmitter frequencies of an ISMLS must be in accordance with the frequency

plan approved by the FAA, and must meet the following signal format

requirements:

(1) The localizer and glide slope stations must transmit angular guidance

information on a C-band microwave carrier on narrow, scanned antenna beams

that are encoded to produce a modulation in space which, after averaging over

several beam scans, is equivalent to the modulation used for conventional ILS

as specified in Subpart C of this part, except that the frequency tolerance

may not exceed +/-0.0001 percent.

(2) Guidance modulation must be impressed on the microwave carrier of the

radiated signal in the form of a summation of 90 Hz and 150 Hz sinusoidal

modulation corresponding to the pointing direction of the particular beam

which radiates the signal.

(3) Each of the effective beam positions must be illuminated in a

particular sequence for a short time interval. The modulation impressed on

each beam must be a sample of the combined 90 Hz and 150 Hz waveform

appropriate for that particular beam direction and time slot, and must be

accomplished by appropriately varying the length of time the carrier is

radiated during each beam illumination interval.

(4) For those cases where the scanning beam fills the coverage space in

steps, the incremental step must not exceed 0.6 times the beam width where

the beam is in the proportional guidance sector. In the clearance region, the

step may not exceed 0.8 times the beam width.

(5) At least one pulse duration modulation (pdm) sample pulse per beam

width of scan must be provided.

(6) The minimum pulse duration must be 40 microseconds.

(7) The minimum beam scan cycle must be 600 Hz.

(8) The minimum duty ratio detectable by a receiver located anywhere in the

coverage areas defined by this specification may not be less than 0.1.

Detected duty ratio means the ratio of the average energy per scan detected

at a point in space to the average energy per scan transmitted in all

directions through the transmitting antenna.

(9) The localizer must produce a C-band unmodulated reference frequency

signal of sufficient strength to allow satisfactory operation of an aircraft

receiver within the specified localizer and glide path coverage sectors.

Pairing of this reference frequency with the localizer and glide slope

frequencies must be in accordance with a frequency plan approved by the FAA.

Sec. 171.261   Localizer performance requirements.

This section prescribes the performance requirements for localizer

equipment components of the ISMLS.

(a) The localizer antenna system must:

(1) Be located on the extension of the centerline of the runway at the stop

end;

(2) Be adjusted so that the course line be on a vertical plane containing

the centerline of the runway served;

(3) Have the minimum height necessary to comply with the coverage

requirements prescribed in paragraph (j) of this section;

(4) Be located at a distance from the stop end of the runway that is

consistent with safe obstruction clearance practices;

(5) Not obscure any light of the approach landing system; and

(6) Be installed on frangible mounts or beyond the 1000' light bar.

(b) On runways where limited terrain prevents the localizer antennae from

being positioned on the runway centerline extended, and the cost of the land

fill or a tall tower antenna support is prohibitive, the localizer antenna

array may be offset, including a collocated ground station, so that the

course intercepts the centerline at a point determined by the amount of the

angular offset and the glide path angle. If other than a runway centerline

localizer is used, the criteria in Subpart C of Part 97 of this chapter is

applicable.

(c) At locations where two separate ISMLS facilities serve opposite ends of

of a single runway, an interlock must ensure that only the facility serving

the approach direction being used will radiate.

(d) The radiation from the localizer antenna system must produce a

composite field pattern which is pulse duration modulated, the time average

equivalent to amplitude modulation by a 90 Hz and 150 Hz tone. The localizer

station must transmit angular guidance information over a C-band microwave

carrier on narrow, scanned antenna beams that are encoded to produce a

modulation in space which, after averaging over several beam scans, is

equivalent to the modulation used for conventional ILS as specified in

Subpart C of this part. The radiation field pattern must produce a course

sector with one tone predominating on one side of the course and with the

other tone predominating on the opposite side. When an observer faces the

localizer from the approach end of the runway, the depth of modulation of the

radio frequency carrier due to the 150 Hz tone must predominate on his right

hand and that due to the 90 Hz tone must predominate on his left hand.

(e) All horizontal angles employed in specifying the localizer field

patterns must originate from the center of the localizer antenna system which

provides the signals used in the front course sector.

(f) The ISMLS course sector angle must be adjustable between 3 degrees and

9 degrees. The applicable course sector angle will be established and

approved on an individual basis.

(g) The ISMLS localizer must operate in the band 5000 MHz to 5030 MHz. The

frequency tolerance may not exceed +/-0.0001 percent.

(h) The emission from the localizer must be vertically polarized. The

horizontally polarized component of the radiation of the course line may not

exceed that which corresponds to a DDM error of 0.016 when an aircraft is

positioned on the course line and is in a roll attitude of 20 degrees from

the horizontal.

(i) The localizer must provide signals sufficient to allow satisfactory

operation of a typical aircraft installation within the localizer and glide

path coverage sectors. The localizer coverage sector must extend from the

center of the localizer antenna system to distances of 18 nautical miles

minimum within +/-10 degrees from the front course line, and 10 nautical

miles minimum between +/-10 degrees and +/-35 degrees from the front course

line. The ISMLS localizer signals must be receivable at the distances

specified up from a surface extending outward from the localizer antenna and

within a sector in the elevation plane from 0.300 to 1.750 of the established

glide path angle (<theta>).

(j) Except as provided in paragraph (k) of this section, in all parts of

the coverage volume specified in paragraph (i) of this section, the peak

field strength may not be less than -87 dBW/m 2 , and must permit

satisfactory operational usage of ISMLS localizer facilities.

(k) The minimum peak field strength on the ISMLS glide path and within the

localizer course sector from a distance of 10 nautical miles to a height of

100 feet (30 meters) above the horizontal plane containing the threshold, may

not be less than +87 dBW/m 2 .

(l) Above 16 degrees, the ISMLS localizer signals must be reduced to as low

a value as practicable.

(m) Bends in the course line may not have amplitudes which exceed the

following:

Zone                   Amplitude (DDM) (95 pct. probability)

Outer limit of coverage to:

ISMLS point "A"                     0.031.

ISMLS point "A" to ISMLS point "B"  0.031 at ISMLS point "A" decreasing at

linear rate to 0.015 at ISMLS point

"B".

ISMLS point "B" to ISMLS point "C"  0.015.

(n) The amplitudes referred to in paragraph (m) of this section are the

DDMs due to bends as realized on the mean course line, when correctly

adjusted.

(o) The radio frequency carrier must meet the following requirements:

(1) The nominal depth of modulation of the radio frequency carrier due to

each of the 90 Hz and 150 Hz tones must be 20 percent along the course line.

(2) The depth of modulation of the radio frequency carrier due to each of

the 90 Hz and 150 Hz tones must be between 18 and 22 percent.

(3) The frequency tolerance of the 90 Hz and 150 Hz modulated tones must be

within +/-25 percent.

(4) Total harmonic content of the 90 Hz tone may not exceed 10 percent.

(5) Total harmonic content of the 150 Hz tone may not exceed 10 percent.

However, a 300 Hz tone may be transmitted for identification purposes.

(6) At every half cycle of the combined 90 Hz and 150 Hz wave form, the

modulation tones must be phase-locked so that within the half course sector,

the demodulated 90 Hz and 150 Hz wave forms pass through zero in the same

direction within 20 degrees with phase relative to the 150 Hz component.

However, the phase need not be measured within the half course sector.

(p) The mean course line must be adjusted and maintained within +/-.015DDM

from the runway centerline at the ISMLS reference datum.

(q) The nominal displacement sensitivity within the half course sector at

the ISMLS reference datum, must be 0.00145 DDM/meter (0.00044DDM/foot).

However, where the specified nominal displacement sensitivity cannot be met,

the displacement sensitivity must be adjusted as near as possible to that

value.

(r) The lateral displacement sensitivity must be adjusted and maintained

within 17 percent of the nominal value. Nominal sector width at the ISMLS

reference datum is 210 meters (700 feet).

(s) The increase of DDM must be substantially linear with respect to

angular displacement from the front course line where DDM is zero, up to

angle on either side of the front course line where the DDM is 0.180. From

that angle to +/-10 degrees, the DDM may not be less than 0.180. From +/-10

degrees to +/-35 degrees, the DDM may not be less than 0.155.

(t) The localizer must provide for the simultaneous transmission of an

identification signal which meets the following:

(1) It must be specific to the runway and approach direction, on the same

radio frequency carrier, as used for the localizer function.

(2) Transmission of the identification signal may not interfere in any way

with the basic localizer function.

(3) The signal must be produced by pulse duration modulation of the radio

frequency carrier resulting in a detected audio tone in the airborne VHF

receiver of 1020 Hz +/-50Hz.

(4) The depth of modulation must be between the limits of 10 and 12

percent.

(5) The emissions carrying the identification signal must be vertically

polarized.

(6) The identification signal must employ the International Morse Code and

consist of three letters. It must be preceded by the International Morse Code

signal of the letter "M" followed by a short pause where it is necessary to

distinguish the ISMLS facility from other navigational facilities in the

immediate area. At airports where both an ISMLS and an ILS are in operation,

each facility must have a different identification call sign.

(7) The signal must be transmitted at a speed corresponding to

approximately seven words per minute, and must be repeated at approximately

equal intervals, not less than six times per minute, during which time the

localizer is available for operational use. When the localizer is not

available for transmission, the identification signal must be suppressed.

Sec. 171.263   Localizer automatic monitor system.

(a) The ISMLS localizer equipment must provide an automatic monitor system

that transmits a warning to designated local and remote control points when

any of the following occurs:

(1) A shift of the mean course line of the localizer from the runway

centerline equivalent to more than 0.015 DDM at the ISMLS reference datum.

(2) For localizers in which the basic functions are provided by the use of

a single-frequency system, a reduction of power output to less than 50

percent of normal or a loss of ground station identification transmissions.

(3) Changes of displacement sensitivity to a value differing by more than

17 percent from nominal value for the localizer.

(4) Failure of any part of the monitor itself. Such failure must

automatically produce the same results as the malfunctioning of the element

being monitored.

(b) Within 10 seconds of the occurrence of any of the conditions prescribed

in paragraph (a) of this section, including periods of zero radiation,

localizer signal radiation must cease or the navigation and identification

components must be removed.

Sec. 171.265   Glide path performance requirements.

This section prescribes the performance requirements for glide path

equipment components of the ISMLS. These requirements are based on the

assumption that the aircraft is heading directly toward the facility.

(a) The glide slope antenna system must be located near the approach end of

the runway, and the equipment must be adjusted so that the vertical path line

will be in a sloping horizontal plane containing the centerline of the runway

being served, and satisfy the coverage requirements prescribed in paragraph

(g) of this section. For the purpose of obstacle clearance, location of the

glide slope antenna system must be in accordance with the criteria specified

in Subpart C of Part 97 of this chapter.

(b) The radiation from the glide path antenna system must produce a

composite field pattern which is pulse duration modulated by a 90 Hz and a

150 Hz tone, which is the time average equivalent to amplitude modulation.

The pattern must be arranged to provide a straight line descent path in the

vertical plane containing the centerline of the runway, with the 150 Hz tone

predominating below the path and the 90 Hz tone predominating above the path

to at least an angle equal to 1.75<theta>. As used in this section theta

(<theta>), denotes the nominal glide path angle. The glide path angle must be

adjusted and maintained within 0.075<theta>.

(c) The glide path equipment must be capable of producing a radiated glide

path from 3 to 9 degrees with respect to the horizontal. However, ISMLS glide

path angles in excess of 3 degrees may be used to satisfy instrument approach

procedures or to overcome an obstruction clearance problem, only in

accordance with the criteria specified in Subpart C of Part 97 of this

chapter.

(d) The downward extended straight portion of the ISMLS glide path must

pass through the ISMLS reference datum at a height ensuring safe guidance

over obstructions and safe and efficient use of the runway served. The height

of the ISMLS reference datum must be in accordance with Subpart C of Part 97

of this chapter.

(e) The glide path equipment must operate in the band 5220 MHz to 5250 MHz.

The frequency tolerance may not exceed +/-0.0001 percent.

(f) The emission from the glide path equipment must be vertically

polarized.

(g) The glide path equipment must provide signals sufficient to allow

satisfactory operation of a typical aircraft installation insectors of 8

degrees on each side of the centerline of the ISMLS glide path, to a distance

of at least 10 nautical miles up to 1.75<theta> and down to 0.45<theta> above

the horizontal or to such lower angle at which 0.22 DDM is realized.

(h) To provide the coverage for glide path performance specified in

paragraph (g) of this section, the minimum peak field strength within this

coverage sector must be -82 dBW/m 2 . The peak field strength must be

provided on the glide path down to a height of 30 meters (100 feet) above the

horizontal plane containing the threshold.

(i) Bends in the glide path may not have amplitudes which exceed the

following:

Amplitude

(DDM) (95

pct.

Zone                      probability)

Outer limit of coverage to ISMLS point "C."        0.035.

The amplitude referred to is the DDM due to bends as realized on the mean

ISMLS glide path correctly adjusted. In regions of the approach where ISMLS

glide path curvature is significant, bend amplitude is calculated from the

mean curved path, and not the downward extended straight line.

(j) Guidance modulation must be impressed on the microwave carrier of the

radiated glide slope signal in the form of a unique summation of 90 Hz and

150 Hz sinusoidal modulation corresponding to the point direction of the

particular beam which radiates the signal. Each of the effective beam

positions must be illuminated in sequence for a short time interval. The scan

rate must be synchronous with the 90 and 150 Hz tone base. The modulation

impressed on each beam must be a sample of the combined 90 Hz and 150 Hz

waveform appropriate for that particular beam direction and time slot. The

actual modulation must be accomplished by appropriately varying the length of

time the carrier is radiated during each beam illumination interval.

(k) The nominal depth of modulation of the radio frequency carrier due to

each of the 90 Hz and 150 Hz tones must be 40 percent along the ISMLS glide

path. The depth of modulation may not deviate outside the limits of 37.5

percent to 42.5 percent.

(l) The following tolerances apply to the frequencies of the modulating

tones:

(1) The modulating tones must be 90 Hz and 150 Hz within 2.5 percent.

(2) The total harmonic content of the 90 Hz tone may not exceed 10 percent.

(3) The total harmonic content of the 150 Hz tone may not exceed 10

percent.

(m) At every half cycle of the combined 90 Hz and 150 Hz wave form, the

modulation must be phase-locked so that, within the ISMLS half glide path

sector, the demodulated 90 Hz and 150 Hz wave forms pass through zero in the

same direction within 20 degrees of phase relative to the 150 Hz component.

However, the phase need not be measured within the ISMLS half glide path

sector.

(n) The nominal angular displacement sensitivity must correspond to a DDM

of 0.0875 at an angular displacement above and below the glide path of

0.12<theta>. The glide path angular displacement sensitivity must be adjusted

and maintained within +/-25 percent of the nominal value selected. The upper

and lower sectors must be as symmetrical as practicable within the limits

prescribed in this paragraph.

(o) The DDM below the ISMLS glide path must increase smoothly for

decreasing angle until a value of 0.22 DDM is reached. This value must be

achieved at an angle not less than 0.30<theta> above the horizontal. However,

if it is achieved at an angle above 0.45<theta>, the DDM value may not be

less than 0.22 at least down to an angle of 0.45<theta>.

[Doc. No. 14120, 40 FR 36110, Aug. 19, 1975; 40 FR 41093, Sept. 5, 1975; 40

FR 43719, Sept. 23, 1975]

Sec. 171.267   Glide path automatic monitor system.

(a) The ISMLS glide path equipment must provide an automatic monitor system

that transmits a warning to designated local and remote control points when

any of the following occurs:

(1) A shift of the mean ISMLS glide path angle equivalent to more than

0.075<theta>.

(2) For glide paths in which the basic functions are provided by the use of

a single frequency system, a reduction of power output to less than 50

percent.

(3) A change of the angle between the glide path and the line below the

glide path (150 Hz predominating), at which a DDM of 0.0875 is realized by

more than +/-0.0375<theta>.

(4) Lowering of the line beneath the ISMLS glide path at which a DDM of

0.0875 is realized to less than 0.75<theta> from the horizontal.

(5) Failure of any part of the monitor itself. Such failure must

automatically produce the same results as the malfunctioning of the element

being monitored.

(b) At glide path facilities where the selected nominal angular

displacement sensitivity corresponds to an angle below the ISMLS glide path,

which is close to or at the maximum limits specified, an adjustment to the

monitor operating limits may be made to protect against sector deviations

below 0.75<theta> from the horizontal.

(c) Within 10 seconds of the occurrence of any of the conditions prescribed

in paragraph (a) of this section, including periods of zero radiation, glide

path signal radiation must cease.

Sec. 171.269   Marker beacon performance requirements.

ISMLS marker beacon equipment must meet the performance requirements

prescribed in Subpart H of this part.

Sec. 171.271   Installation requirements.

(a) The ISMLS facility must be permanent in nature, located, constructed,

and installed according to accepted good engineering practices, applicable

electric and safety codes, FCC licensing requirements, and paragraphs (a) and

(c) of Sec. 171.261.

(b) The ISMLS facility must have a reliable source of suitable primary

power, either from a power distribution system or locally generated. Adequate

power capacity must be provided for the operation of test and working

equipment of the ISMLS.

(c) The ISMLS facility must have a continuously engaged or floating battery

power source for the ground station for continued normal operation if the

primary power fails. A trickle charge must be supplied to recharge the

batteries during the period of available primary power. Upon loss and

subsequent restoration of power, the batteries must be restored to full

charge within 24 hours. When primary power is applied, the state of the

battery charge may not affect the operation of the ISMLS ground station. The

battery must permit continuation of normal operation for at least two hours

under the normal operating conditions. The equipment must meet all

specification requirements with or without batteries installed.

(d) There must be a means for determining, from the ground, the performance

of the equipment including antennae, both initially and periodically.

(e) The facility must have, or be supplemented by, ground-air or landline

communications services. At facilities within or immediately adjacent to air

traffic control areas, and that are intended for use as instrument approach

aids for an airport, there must be ground-air communications or reliable

communications (at least a landline telephone) from the airport to the

nearest Federal Aviation Administration air traffic control or communication

facility. Compliance with this paragraph need not be shown at airports where

an adjacent Federal Aviation Administration facility can communicate with

aircraft on the ground at the airport and during the entire proposed

instrument approach procedure. In addition, at low traffic density airports

within or immediately adjacent to air traffic control zones or areas, and

where extensive delays are not a factor, the requirements of this paragraph

may be reduced to reliable communications (at least a landline telephone)

from the airport to the nearest Federal Aviation Administration air traffic

control or communications facility, if an adjacent Federal Aviation

Administration facility can communicate with aircraft during the proposed

instrument approach procedure, at least down to the minimum en route altitude

for the controlled area.

(f) Except where no operationally harmful interference will result, at

locations where two separate ISMLS facilities serve opposite ends of a single

runway, an interlock must ensure that only the facility serving the approach

direction in use can radiate.

EFFECTIVE DATE NOTE: Amdt. 171-16, 56 FR 65665, Dec. 17, 1991, revised

paragraph (e) of Sec. 171.271 effective September 16, 1993. For the

convenience of the user, the revised text is set forth as follows:

Sec. 171.271   Installation requirements.

(e) The facility must have, or be supplemented by, ground-air or landline

communications services. At facilities within or immediately adjacent to

controlled airspace and that are intended for use as instrument approach aids

for an airport, there must be ground-air communications or reliable

communications (at least a landline telephone) from the airport to the

nearest Federal Aviation Administration air traffic control or communication

facility. Compliance with this paragraph need not be shown at airports where

an adjacent Federal Aviation Administration facility can communicate with

aircraft on the ground at the airport and during the entire proposed

instrument approach procedure. In addition, at low traffic density airports

within or immediately adjacent to controlled airspace, and where extensive

delays are not a factor, the requirements of this paragraph may be reduced to

reliable communications (at least a landline telephone) from the airport to

the nearest Federal Aviation Administration air traffic control or

communications facility, if an adjacent Federal Aviation Administration

facility can communicate with aircraft during the proposed instrument

approach procedure, at least down to the minimum en route altitude for the

controlled area.

Sec. 171.273   Maintenance and operations requirements.

(a) The owner of the facility must establish an adequate maintenance system

and provide qualified maintenance personnel to maintain the facility at the

level attained at the time it was commissioned. Each person who maintains a

facility must meet at least the Federal Communications Commission's licensing

requirements and show that he has the special knowledge and skills needed to

maintain the facility, including proficiency in maintenance procedures and

the use of specialized test equipment.

(b) In the event of out-of-tolerance conditions or malfunctions, as

evidenced by receiving two successive pilot reports, the owner must close the

facility be ceasing radiation, and issue a "Notice to Airman" (NOTAM) that

the facility is out of service.

(c) The owner must prepare, and obtain approval of, an operations and

maintenance manual that sets forth mandatory procedures for operations,

periodic maintenance, and emergency maintenance, including instructions on

each of the following:

(1) Physical security of the facility.

(2) Maintenance and operations by authorized persons.

(3) FCC licensing requirements for operations and maintenance personnel.

(4) Posting of licenses and signs.

(5) Relation between the facility and FAA air traffic control facilities,

with a description of the boundaries of controlled airspace over or near the

facility, instructions for relaying air traffic control instructions and

information, if applicable, and instructions for the operation of an air

traffic advisory service if the facility is located outside of controlled

airspace.

(6) Notice to the Administrator of any suspension of service.

(7) Detailed and specific maintenance procedures and servicing guides

stating the frequency of servicing.

(8) Air-ground communications, if provided, expressly written or

incorporating appropriate sections of FAA manuals by reference.

(9) Keeping of station logs and other technical reports, and the submission

of reports required by Sec. 171.275.

(10) Monitoring of the ISMLS facility.

(11) Inspections by United States personnel.

(12) Names, addresses, and telephone numbers of persons to be notified in

an emergency.

(13) Shutdowns for periodic maintenance and issue of "Notices to Airmen"

for routine or emergency shutdowns.

(14) Commissioning of the ISMLS facility.

(15) An acceptable procedure for amending or revising the manual.

(16) An explanation of the kinds of activities (such as construction or

grading) in the vicinity of the ISMLS facility that may require shutdown or

recertification of the ISMLS facility by FAA flight check.

(17) Procedures for conducting a ground check of the localizer course

alignment, width, and clearance, glide path elevation angle and course width,

and marker beacon power, and modulation.

(18) The following information concerning the ISMLS facility:

(i) Facility component locations with respect to airport layout, instrument

runways, and similar areas.

(ii) The type, make, and model of the basic radio equipment that provides

the service.

(iii) The station power emission and frequencies of the ISMLS localizer,

glide path, beacon markers, and associated compass locators, if any.

(iv) The hours of operation.

(v) Station identification call letters and method of station

identification and the time spacing of the identification.

(vi) A description of the critical parts that may not be changed, adjusted,

or repaired without an FAA flight check to confirm published operations.

(d) The owner or his maintenance representative must make a ground check of

the ISMLS facility periodically in accordance with procedures approved by the

FAA at the time of commissioning, and must report the results of the checks

as provided in Sec. 171.275.

(e) Modifications to an ISMLS facility may be made only after approval by

the FAA of the proposed modification submitted by the owner.

(f) The owner or the owner's maintenance representative must participate in

inspections made by the FAA.

(g) Whenever it is required by the FAA, the owner must incorporate

improvements in ISMLS maintenance.

(h) The owner or his maintenance representative must provide a sufficient

stock of spare parts, including solid state components, or modules to make

possible the prompt replacement of components or modules that fail or

deteriorate in service.

(i) FAA approved test instruments must be used for maintenance of the ISMLS

facility.

(j) The mean corrective maintenance time of the ISMLS equipment may not

exceed 0.5 hours, with a maximum corrective maintenance time of not greater

than 1.5 hours. This measure applies to failures of the monitor, transmitter

and associated antenna assemblies, limited to unscheduled outage and out-of-

tolerance conditions.

(k) The mean time between failures of the ISMLS equipment may not be less

than 1,500 hours. This measure applies to unscheduled outages, out-of-

tolerance conditions, and failures of the monitor, transmitter, and

associated antenna assemblies.

(l) Inspection consists of an examination of the ISMLS equipment to ensure

that unsafe operating conditions do not exist.

(m) Monitoring of the ISMLS radiated signal must ensure a high degree of

integrity and minimize the requirements for ground and flight inspection. The

monitor must be checked periodically during the in-service test evaluation

period for calibration and stability. These tests and ground checks of glide

slope, localizer, and marker beacon radiation characteristics must be

conducted in accordance with the maintenance requirements of this section.

Sec. 171.275   Reports.

The owner of the ISMLS facility or his maintenance representative must make

the following reports at the indicated time to the appropriate FAA Regional

Office where the facility is located.

(a) Facility Equipment Performance and Adjustment Data (FAA Form 198). The

FAA Form 198 shall be filled out by the owner or his maintenance

representative with the equipment adjustments and meter readings as of the

time of facility commissioning. One copy must be kept in the permanent

records of the facility and two copies must be sent to the appropriate FAA

Regional Office. The owner or his maintenance representative must revise the

FAA Form 198 data after any major repair, modernization, or retuning to

reflect an accurate record of facility operation and adjustment. In the event

the data are revised, the owner or his maintenance representative shall

notify the appropriate FAA Regional Office of such revisions, and forward

copies of the revisions to the appropriate FAA Regional Office.

(b) Facility Maintenance Log (FAAForm 6030--1). FAA Form 6030-1 is a

permanent record of all the activities required to maintain the ISMLS

facility. The entries must include all malfunctions met in maintaining the

facility including information on the kind of work and adjustments made,

equipment failures, causes (if determined) and corrective action taken. In

addition, the entries must include completion of periodic maintenance

required to maintain the facility. The owner or his maintenance

representative must keep the original of each form at the facility and send a

copy to the appropriate FAA Regional Office at the end of each month in which

it is prepared. However, where an FAA approved remote monitoring system is

installed which precludes the need for periodic maintenance visits to the

facility, monthly reports from the remote monitoring system control point

must be forwarded to the appropriate FAA Regional Office, and a hard copy

retained at the control point.

(c) Technical Performance Record (FAA Form 418). FAA Form 418 contains a

record of system parameters, recorded on each scheduled visit to the

facility. The owner or his maintenance representative shall keep the original

of each month's record at the facility and send a copy of the form to the

appropriate FAA Regional Office.

Subpart J--Microwave Landing System (MLS)

Source: Docket No. 20669, 51 FR 33177, Sept. 18, 1986, unless otherwise

noted.

Sec. 171.301   Scope.

This subpart sets forth minimum requirements for the approval,

installation, operation and maintenance of non-Federal Microwave Landing

System (MLS) facilities that provide the basis for instrument flight rules

(IFR) and air traffic control procedures.

Sec. 171.303   Definitions.

As used in this subpart:

"Auxiliary data" means data transmitted in addition to basic data that

provide ground equipment siting information for use in refining airborne

position calculations and other supplementary information.

"Basic data" means data transmitted by the ground equipment that are

associated directly with the operation of the landing guidance system.

"Beam center" means the midpoint between the -3 dB points on the leading

and trailing edges of the scanning beam main lobe.

"Beamwidth" means the width of the scanning beam main lobe measured at the

-3 dB points and defined in angular units on the boresight, in the horizontal

plane for the azimuth function and in the vertical plane for the elevation

function.

"Clearance guidance sector" means the volume of airspace, inside the

coverage sector, within which the azimuth guidance information provided is

not proportional to the angular displacement of the aircraft, but is a

constant fly-left or fly-right indication of the direction relative to the

approach course the aircraft should proceed in order to enter the

proportional guidance sector.

"Control Motion Noise (CMN)" means those fluctuations in the guidance which

affect aircraft attitude, control surface motion, column motion, and wheel

motion. Control motion noise is evaluated by filtering the flight error

record with a band-pass filter which has corner frequencies at 0.3 radian/sec

and 10 radians/sec for azimuth data and 0.5 radian/sec and 10 radians/sec for

elevation data.

"Data rate" means the average number of times per second that transmissions

occur for a given function.

"Differential Phase Shift Keying (DPSK)" means differential phase

modulation of the radio frequency carrier with relative phase states of 0

degree or 180 degrees.

"Failure" means the inability of an item to perform within previously

specified limits.

"Guard time" means an unused period of time provided in the transmitted

signal format to allow for equipment tolerances.

"Integrity" means that quality which relates to the trust which can be

placed in the correctness of the information supplied by the facility.

"Mean corrective time" means the average time required to correct an

equipment failure over a given period, after a service technician reaches the

facility.

"Mean course error" means the mean value of the azimuth error along a

specified radial of the azimuth function.

"Mean glide path error" means the mean value of the elevation error along a

specified glidepath of the elevation function.

"Mean-time-between-failures (MTBF)" means the average time between

equipment failures over a given period.

"Microwave Landing System (MLS)" means the MLS selected by ICAO for

international standardization.

"Minimum glidepath" means the lowest angle of descent along the zero degree

azimuth that is consistent with published approach procedures and obstacle

clearance criteria.

"MLS Approach Reference Datum" is a point at a specified height located

vertically above the intersection of the runway centerline and the threshold.

"MLS back azimuth reference datum" means a point 15 meters (50 feet) above

the runway centerline at the runway midpoint.

"MLS datum point" means a point defined by the intersection of the runway

centerline with a vertical plane perpendicular to the centerline and passing

through the elevation antenna phase center.

"Out of coverage indication (OCI)" means a signal radiated into areas

outside the intended coverage sector, where required, to specifically prevent

invalid removal of an airborne warning indication in the presence of

misleading guidance information.

"Path Following Error (PFE)" means the guidance perturbations which could

cause aircraft displacement from the desired course or glidepath. It is

composed of the path following noise and of the mean course error in the case

of azimuth functions, or the mean glidepath error in the case of elevation

functions. Path following errors are evaluated by filtering the flight error

record with a second order low pass filter which has a corner frequency at

0.5 radian/sec for azimuth data or 1.5 radians/sec for elevation data.

"Path following noise (PFN)" means that portion of the guidance signal

error which could cause displacement from the actual mean course line or mean

glidepath as appropriate.

"Split-site ground station" means the type of ground station in which the

azimuth portion of the ground station is located near the stop end of the

runway, and the elevation portion is located near the approach end.

"Time division multiplex (TDM)" means that each function is transmitted on

the same frequency in time sequence, with a distinct preamble preceding each

function transmission.

Sec. 171.305  Requests for IFR procedure.

(a) Each person who requests an IFR procedure based on an MLS facility

which that person owns must submit the following information with that

request:

(1) A description of the facility and evidence that the equipment meets the

performance requirements of Secs. 171.309, 171.311, 171.313, 171.315,

171.317, 171.319, and 171.321 and is fabricated and installed in accordance

with Sec. 171.323.

(2) A proposed procedure for operating the facility.

(3) A proposed maintenance organization and a maintenance manual that meets

the requirements of Sec. 171.325.

(4) A statement of intent to meet the requirements of this subpart.

(5) A showing that the facility has an acceptable level of operational

reliability and an acceptable standard of performance. Previous equivalent

operational experience with a facility with identical design and operational

characteristics will be considered in showing compliance with this

subparagraph.

(b) FAA inspects and evaluates the MLS facility; it advises the owner of

the results, and of any required changes in the MLS facility or in the

maintenance manual or maintenance organization. The owner must then correct

the deficiencies, if any, and operate the MLS facility for an in-service

evaluation by the FAA.

Sec. 171.307  Minimum requirements for approval.

(a) The following are the minimum requirements that must be met before the

FAA approves an IFR procedure for a non-Federal MLS facility:

(1) The performance of the MLS facility, as determined by flight and ground

inspection conducted by the FAA, must meet the requirements of Secs. 171.309,

171.311, 171.313, 171.315, 171.317, 171.319, and 171.321.

(2) The fabrication and installation of the equipment must meet the

requirements of Sec. 171.323.

(3) The owner must agree to operate and maintain the MLS facility in

accordance with Sec. 171.325.

(4) The owner must agree to furnish operational records as set forth in

Sec. 171.327 and agree to allow the FAA to inspect the facility and its

operation whenever necessary.

(5) The owner must assure the FAA that he will not withdraw the MLS

facility from service without the permission of the FAA.

(6) The owner must bear all costs of meeting the requirements of this

section and of any flight or ground inspection made before the MLS facility

is commissioned.

(b) [Reserved]

Sec. 171.309  General requirements.

The MLS is a precision approach and landing guidance system which provides

position information and various ground-to-air data. The position information

is provided in a wide coverage sector and is determined by an azimuth angle

measurement, an elevation angle measurement and a range (distance)

measurement.

(a) An MLS constructed to meet the requirements of this subpart must

include:

(1) Approach azimuth equipment, associated monitor, remote control and

indicator equipment.

(2) Approach elevation equipment, associated monitor, remote control and

indicator equipment.

(3) A means for the encoding and transmission of essential data words,

associated monitor, remote control and indicator equipment. Essential data

are basic data words 1, 2, 3, 4, and 6 and auxiliary data words A1, A2 and

A3.

(4) Distance measuring equipment (DME), associated monitor, remote control

and indicator equipment.

(5) Remote controls for paragraphs (a) (1), (2), (3), and (4) of this

section must include as a minimum on/off and reset capabilities and may be

integrated in the same equipment.

(6) At locations where a VHF marker beacon (75 MHz) is already installed,

it may be used in lieu of the DME equipment.

(b) In addition to the equipment required in paragraph (a) of this section

the MLS may include:

(1) Back azimuth equipment, associated monitor, remote control and

indicator equipment. When Back Azimuth is provided, a means for transmission

of Basic Data Word 5 and Auxiliary Data Word A4 shall also be provided.

(2) A wider proportional guidance sector which exceeds the minimum

specified in Secs. 171.313 and 171.317.

(3) Precision DME, associated monitor, remote control and indicator

equipment.

(4) VHF marker beacon (75 MHz), associated monitor, remote control and

indicator equipment.

(5) The MLS signal format will accommodate additional functions (e.g.,

flare elevation) which may be included as desired. Remote controls for

paragraphs (b) (1), (3) and (4) of this section must include as a minimum on/

off and reset capabilities, and may be integrated in the same equipment.

(6) Provisions for the encoding and transmission of additional auxiliary

data words, associated monitor, remote control and indicator equipment.

(c) MLS ground equipment must be designed to operate on a nominal 120/240

volt, 60 Hz, 3-wire single phase AC power source and must meet the following

service conditions:

(1) AC line parameters, DC voltage, elevation and duty:

120 VAC nominal value--102 V to 138 V (+/-1 V)*

240 VAC nominal value--204 V to 276 V (+/-2 V)*

60 Hz AC line frequency--57 Hz to 63 Hz (+/-0.2 Hz)*

*Note: Where discrete values of the above frequency or voltages are

specified for testing purposes, the tolerances given in parentheses indicated

by an asterisk apply to the test instruments used to measure these

parameters.

Elevation--0 to 3000 meters (10,000 feet) above sea level

Duty--Continuous, unattended

(2) Ambient conditions within the shelter for electronic equipment

installed in shelters are:

Temperature, -10 deg.C to +50 deg.C

Relative humidity, 5% to 90%

(3) Ambient conditions for electronic equipment and all other equipment

installed outdoors (for example, antenna, field detectors, and shelters):

Temperature, -50 deg.C to +70 deg.C

Relative humidity, 5% to 100%

(4) All equipment installed outdoors must operate satisfactorily under the

following conditions:

Wind Velocity: The ground equipment shall remain within monitor limits with

wind velocities of up to 70 knots from such directions that the velocity

component perpendicular to runway centerline does not exceed 35 knots. The

ground equipment shall withstand winds up to 100 knots from any direction

without damage.

Hail Stones: 1.25 centimeters (1/2  inch) diameter.

Rain: Provide required coverage with rain falling at a rate of 50 millimeters

(2 inches) per hour, through a distance of 9 kilometers (5 nautical miles)

and with rain falling at the rate of 25 millimeters (1 inch) per hour for

the additional 28 kilometers (15 nautical miles).

Ice Loading: Encased in 1.25 centimeters (1/2  inch) radial thickness of

clear ice.

Antenna Radome De-Icing: Down to -6 deg.C (20 deg.F) and wind up to 35 knots.

(d) The transmitter frequencies of an MLS must be in accordance with the

frequency plan approved by the FAA.

(e) The DME component listed in paragraph (a)(4) of this section must

comply with the minimum standard performance requirements specified in

Subpart G of this part.

(f) The marker beacon components listed in paragraph (b)(4) of this section

must comply with the minimum standard performance requirements specified in

Subpart H of this part.

Sec. 171.311   Signal format requirements.

The signals radiated by the MLS must conform to the signal format in which

angle guidance functions and data functions are transmitted sequentially on

the same C-band frequency. Each function is identified by a unique digital

code which initializes the airborne receiver for proper processing. The

signal format must meet the following minimum requirements:

(a) Frequency assignment. The ground components (except DME/Marker Beacon)

must operate on a single frequency assignment or channel, using time division

multiplexing. These components must be capable of operating on any one of the

200 channels spaced 300 KHz apart with center frequencies from 5031.0 MHz to

5090.7 MHz and with channel numbering as shown in Table 1a. The operating

radio frequencies of all ground components must not vary by more than +/-10

KHz from the assigned frequency. Any one transmitter frequency must not vary

more than +/-50 Hz in any one second period. The MLS angle/data and DME

equipment must operate on one of the paired channels as shown in Table 1b.

Table 1a--Frequency Channel Plan

Channel No.       Frequency (MHz)

500                  5031.0

501                  5031.3

502                  5031.6

503                  5031.9

504                  5032.2

505                  5032.5

506                  5032.8

507                  5033.1

508                  5033.4

509                  5033.7

510                  5034.0

511                  5034.3

*        *        *        *        *

598                  5060.4

599                  5060.7

600                  5061.0

601                  5061.3

*        *        *        *        *

698                  5090.4

699                  5090.7

Table 1b--Channels

Channel pairing

MLS

VHF    angle   MLS

freq.   freq.   Ch.

DME No.       MHz     MHz    No.

* 1X

** 1Y

* 2X

** 2Y

* 3X

** 3Y

* 4X

** 4Y

* 5X

** 5Y

* 6X

** 6Y

* 7X

** 7Y

* 8X

** 8Y

* 9X

** 9Y

* 10X

** 10Y

* 11X

** 11Y

* 12X

** 12Y

* 13X

** 13Y

* 14X

** 14Y

* 15X

** 15Y

* 16X

** 16Y

<gradient> 17X  108.00

17Y             108.05  5043.0  540

17Z                     5043.3  541

18X             108.10  5031.0  500

18W                     5031.3  501

18Y             108.15  5043.6  542

18Z                     5043.9  543

19X             108.20

19Y             108.25  5044.2  544

19Z                     5044.5  545

20X             108.30  5031.6  502

20W                     5031.9  503

20Y             108.35  5044.8  546

20Z                     5045.1  547

21X             108.40

21Y             108.45  5045.4  548

21Z                     5045.7  549

22X             108.50  5032.2  504

22W                     5032.5  505

22Y             108.55  5046.0  550

22Z                     5046.3  551

23X             108.60

23Y             108.65  5046.6  552

23Z                     5046.9  553

24X             108.70  5032.8  506

24W                     5033.1  507

24Y             108.75  5047.2  554

24Z                     5047.5  555

25X             108.80

25Y             108.85  5047.8  556

25Z                     5048.1  557

26X             108.90  5033.4  508

26W                     5033.7  509

26Y             108.95  5048.4  558

26Z                     5048.7  559

27X             109.00

27Y             109.05  5049.0  560

27Z                     5049.3  561

28X             109.10  5034.0  510

28W                     5034.3  511

28Y             109.15  5049.6  562

28Z                     5049.9  563

29X             109.20

29Y             109.25  5050.2  564

29Z                     5050.5  565

30X             109.30  5034.6  512

30W                     5034.9  513

30Y             109.35  5050.8  566

30Z                     5051.1  567

31X             109.40

31Y             109.45  5051.4  568

31Z                     5051.7  569

32X             109.50  5035.2  514

32W                     5035.5  515

32Y             109.55  5052.0  570

32Z                     5052.3  571

33X             109.60

33Y             109.65  5052.6  572

33Z                     5052.9  573

34X             109.70  5035.8  516

34W                     5036.1  517

34Y             109.75  5053.2  574

34Z                     5053.5  575

35X             109.80

35Y             109.85  5053.8  576

35Z                     5054.1  577

36X             109.90  5036.4  518

36W                     5036.7  519

36Y             109.95  5054.4  578

36Z                     5054.7  579

37X             110.00

37Y             110.05  5055.0  580

37Z                     5055.3  581

38X             110.10  5037.0  520

38W                     5037.3  521

38Y             110.15  5055.6  582

38Z                     5055.9  583

39X             110.20

39Y             110.25  5056.2  584

39Z                     5056.5  585

40X             110.30  5037.6  522

40W                     5037.9  523

40Y             110.35  5056.8  586

40Z                     5057.1  587

41X             110.40

41Y             110.45  5057.4  588

41Z                     5057.7  589

42X             110.50  5038.2  524

42W                     5038.5  525

42Y             110.55  5058.0  590

42Z                     5058.3  591

43X             110.60

43Y             110.65  5058.6  592

43Z                     5058.9  593

44X             110.70  5038.8  526

44W                     5039.1  527

44Y             110.75  5059.2  594

44Z                     5059.5  595

45X             110.80

45Y             110.85  5059.8  596

45Z                     5060.1  597

46X             110.90  5039.4  528

46W                     5039.7  529

46Y             110.95  5060.4  598

46Z                     5060.7  599

47X             111.00

47Y             111.05  5061.0  600

47Z                     5061.3  601

48X             111.10  5040.0  530

48W                     5040.3  531

48Y             111.15  5061.6  602

48Z                     5061.9  603

49X             111.20

49Y             111.25  5062.2  604

49Z                     5062.5  605

50X             111.30  5040.6  532

50W                     5040.9  533

50Y             111.35  5062.8  606

50Z                     5063.1  607

51X             111.40

51Y             111.45  5063.4  608

51Z                     5063.7  609

52X             111.50  5041.2  534

52W                     5041.5  535

52Y             111.55  5064.0  610

52Z                     5064.3  611

53X             111.60

53Y             111.65  5064.6  612

53Z                     5064.9  613

54X             111.70  5041.8  536

54W                     5042.1  537

54Y             111.75  5065.2  614

54Z                     5065.5  615

55X             111.80

55Y             111.85  5065.8  616

55Z                     5066.1  617

56X             111.90  5042.4  538

56W                     5042.7  539

56Y             111.95  5066.4  618

56Z                     5066.7  619

57X             112.00

57Y             112.05

58X             112.10

58Y             112.15

59X             112.20

59Y             122.25

** 60X

** 60Y

** 61X

** 61Y

** 62X

** 62Y

** 63X

** 63Y

** 64X

** 64Y

** 65X

** 65Y

** 66X

** 66Y

** 67X

** 67Y

** 68X

** 68Y

** 69X

** 69Y

70X             112.30

** 70Y          112.35

71X             112.40

** 71Y          112.45

72X             112.50

** 72Y          112.55

73X             112.60

** 73Y          112.65

74X             112.70

** 74Y          112.75

75X             112.80

** 75Y          112.85

76X             112.90

** 76Y          112.95

77X             113.00

** 77Y          113.05

78X             113.10

** 78Y          113.15

79X             113.20

** 79Y          113.25

80X             113.30

80Y             113.35  5067.0  620

80Z                     5067.3  621

81X             113.40

81Y             113.45  5067.6  622

81Z                     5067.9  623

82X             113.50

82Y             113.55  5068.2  624

82Z                     5068.5  625

83X             113.60

83Y             113.65  5068.8  626

83Z                     5069.1  627

84X             113.70

84Y             113.75  5069.4  628

84Z                     6069.7  629

85X             113.80

85Y             113.85  5070.0  630

85Z                     5070.3  631

86X             113.90

86Y             113.95  5070.6  632

86Z                     5070.9  633

87X             114.00

87Y             114.05  5071.2  634

87Z                     5071.5  635

88X             114.10

88Y             114.15  5071.8  636

88Z                     5072.1  637

89X             114.20

89Y             114.25  5072.4  638

89Z                     5072.7  639

90X             114.30

90Y             114.35  5073.0  640

90Z                     5073.3  641

91X             114.40

91Y             114.45  5073.6  642

91Z                     5073.9  643

92X             114.50

92Y             114.55  5074.2  644

92Z                     5074.5  645

93X             114.60

93Y             114.65  5074.8  646

93Z                     5075.1  647

94X             114.70

94Y             114.75  5075.4  648

94Z                     5075.7  649

95X             114.80

95Y             114.85  5076.0  650

95Z                     5076.3  651

96X             114.90

96Y             114.95  5076.6  652

96Z                     5076.9  653

97X             115.00

97Y             115.05  5077.2  654

97Z                     5077.5  655

98X             115.10

98Y             115.15  5077.8  656

98Z                     5078.1  657

99X             115.20

99Y             115.25  5078.4  658

99Z                     5078.7  659

100X            115.30

100Y            115.35  5079.0  660

100Z                    5079.3  661

101X            115.40

101Y            115.45  5079.6  662

101Z                    5079.9  663

102X            115.50

102Y            115.55  5080.2  664

102Z                    5080.5  665

103X            115.60

103Y            115.65  5080.B  666

103Z                    5081.1  667

104X            115.70

104Y            115.75  5081.4  668

104Z                    5081.7  669

105X            115.80

105Y            115.85  5082.0  670

105Z                    5082.3  671

106X            115.90

106Y            115.95  5082.6  672

106Z                    5082.9  673

107X            116.00

107Y            116.05  5083.2  674

107Z                    5083.5  675

108X            116.10     508

108Y            116.15  5083.8  676

108Z                    5084.1  677

109X            116.20

109Y            116.25  5084.4  678

109Z                    5084.7  679

110X            116.30

110Y            116.35  5085.0  680

110Z                    5085.3  681

111X            116.40

111Y            116.45  5086.6  682

111Z                    5085.9  683

112X            116.50

112Y            116.55  5086.2  684

112Z                    5086.5  685

113X            116.60

113Y            116.65  5086.8  686

113Z                    5087.1  687

114X            116.70

114Y            116.75  5087.4  688

114Z                    5087.7  689

115X            116.80

115Y            116.85  5088.0  690

115Z                    5088.3  691

116X            116.90

116Y            116.95  5088.6  692

116Z                    5088.9  693

117X            117.00

117Y            117.05  5089.2  694

117Z                    5089.5  695

118X            117.10

118Y            117.15  5089.8  696

118Z                    5090.1  697

119X            117.20

119Y            117.25  5090.4  698

119Z                    5090.7  699

120X            117.30

120Y            117.35

121X            117.40

121Y            117.45

122X            117.50

122Y            117.55

123X            117.60

123Y            117.65

124X            117.70

** 124Y         117.75

125X            117.80

** 125Y         117.85

126X            117.90

** 126Y         117.95

[ ...Table continues... ]

DME parameters

Interrogation

Pulse codes            Reply

DME/P Mode

Pulse

DME/N     IA      FA           codes

Freq.  micro-  micro-  micro-  Freq.  micro-

MHz     s       s       s      MHz     s

1025      12                    962      12

1025      36                   1088      30

1026      12                    963      12

1026      36                   1089      30

1027      12                    964      12

1027      36                   1090      30

1028      12                    965      12

1028      36                   1091      30

1029      12                    966      12

1029      36                   1092      30

1030      12                    967      12

1030      36                   1093      30

1031      12                    968      12

1031      36                   1094      30

1032      12                    969      12

1032      36                   1095      30

1033      12                    970      12

1033      36                   1096      30

1034      12                    971      12

1034      36                   1097      30

1035      12                    972      12

1035      36                   1098      30

1036      12                    973      12

1036      36                   1099      30

1037      12                    974      12

1037      36                   1100      30

1038      12                    975      12

1038      36                   1101      30

1039      12                    976      12

1039      36                   1102      30

1040      12                    977      12

1040      36                   1103      30

1041      12                    978      12

1041      36      36      42   1104      30

1041              21      27   1104      15

1042      12      12      18    979      12

1042              24      30    979      24

1042      36      36      42   1105      30

1042              21      27   1105      15

1043      12                    980      12

1043      36      36      42   1106      30

1043              21      27   1106      15

1044      12      12      18    981      12

1044              24      30    981      24

1044      36      36      42   1107      30

1044              21      27   1107      15

1045      12                    982      12

1045      36      36      42   1108      30

1045              21      27   1108      15

1046      12      12      18    983      12

1046              24      30    983      24

1046      36      36      42   1109      30

1046              21      27   1109      15

1047      12                    984      12

1047      36      36      42   1110      30

1047              21      27   1110      15

1048      12      12      18    985      12

1048              24      30    985      24

1048      36      36      42   1111      30

1048              21      27   1111      15

1049      12                    986      12

1049      36      36      42   1112      30

1049              21      27   1112      15

1050      12      12      18    987      12

1050              24      30    987      24

1050      36      36      42   1113      30

1050              21      27   1113      15

1051      12                    988      12

1051      36      36      42   1114      30

1051              21      27   1114      15

1052      12      12      18    989      12

1052              24      30    989      24

1052      36      36      42   1115      30

1052              21      27   1115      15

1053      12                    990      12

1053      36      36      42   1116      30

1043              21      27   1116      15

1054      12      12      18    991      12

1054              24      30    991      24

1054      36      36      42   1117      30

1054              21      27   1117      15

1055      12                    992      12

1055      36      36      42   1118      30

1055              21      27   1118      15

1056      12      12      18    993      12

1056              24      30    993      24

1056      36      36      42   1119      30

1056              21      27   1119      15

1057      12                    994      12

1057      36      36      42   1120      30

1057              21      27   1120      15

1058      12      12      18    995      12

1058              24      30    995      24

1058      36      36      42   1121      30

1058              21      27   1121      15

1059      12                    996      12

1059      36      36      42   1122      30

1059              21      27   1122      15

1060      12      12      18    997      12

1060              24      30    997      24

1060      36      36      42   1123      30

1060              21      27   1123      15

1061      12                    998      12

1061      36      36      42   1124      30

1061              21      27   1124      15

1062      12      12      18    999      12

1062              24      30    999      24

1062      36      36      42   1125      30

1062              21      27   1125      15

1063      12                   1000      12

1063      36      36      42   1126      30

1063              21      27   1126      15

1064      12      12      18   1001      12

1064              24      30   1001      24

1064      36      36      42   1127      30

1064              21      27   1127      15

1065      12                   1002      12

1065      36      36      42   1128      30

1065              21      27   1128      15

1066      12      12      18   1003      12

1066              24      30   1003      24

1066      36      36      42   1129      30

1066              21      27   1129      15

1067      12                   1004      12

1067      36      36      42   1130      30

1067              21      27   1130      15

1068      12      12      18   1005      12

1068              24      30   1005      24

1068      36      36      42   1131      30

1068              21      27   1131      15

1069      12                   1006      12

1069      36      36      42   1132      30

1069              21      27   1132      15

1070      12      12      18   1007      12

1070              24      30   1007      24

1070      36      36      42   1133      30

1070              21      27   1133      15

1071      12                   1008      12

1071      36      36      42   1134      30

1071              21      27   1134      15

1072      12      12      18   1009      12

1072              24      30   1009      24

1072      36      36      42   1135      30

1072              21      27   1135      15

1073      12                   1010      12

1073      36      36      42   1136      30

1073              21      27   1136      15

1074      12      12      18   1011      12

1074              24      30   1011      24

1074      36      36      42   1137      30

1074              21      27   1137      15

1075      12                   1012      12

1075      36      36      42   1138      30

1075              21      27   1138      15

1076      12      12      18   1013      12

1076              24      30   1013      24

1076      36      36      42   1139      30

1076              21      27   1139      15

1077      12                   1014      12

1077      36      36      42   1140      30

1077              21      27   1140      15

1078      12      12      18   1015      12

1078              24      30   1015      24

1078      36      36      42   1141      30

1078              21      27   1141      15

1079      12                   1016      12

1079      36      36      42   1142      30

1079              21      27   1142      15

1080      12      12      18   1017      12

1080              24      30   1017      24

1080      36      36      42   1143      30

1080              21      27   1143      15

1081      12                   1018      12

1081      36                   1144      30

1082      12                   1019      12

1082      36                   1145      30

1083      12                   1020      12

1083      36                   1146      30

1084      12                   1021      12

1084      36                   1147      30

1085      12                   1022      12

1085      36                   1148      30

1086      12                   1023      12

1086      36                   1149      30

1037      12                   1024      12

1087      36                   1150      30

1088      12                   1151      12

1088      36                   1025      30

1089      12                   1152      12

1089      36                   1026      30

1090      12                   1153      12

1090      36                   1027      30

1091      12                   1154      12

1091      36                   1028      30

1092      12                   1155      12

1092      36                   1029      30

1093      12                   1156      12

1093      36                   1030      30

1094      12                   1157      12

1094      36                   1031      30

1095      12                   1158      12

1095      36                   1032      30

1096      12                   1159      12

1096      36                   1033      30

1097      12                   1160      12

1097      36                   1034      30

1098      12                   1161      12

1098      36                   1035      30

1099      12                   1162      12

1099      36                   1036      30

1100      12                   1163      12

1100      36                   1037      30

1101      12                   1164      12

1101      36                   1038      30

1102      12                   1165      12

1102      36                   1039      30

1103      12                   1166      12

1103      36                   1040      30

1104      12                   1167      12

1104      36      36      42   1041      30

1104              21      27   1041      15

1105      12                   1168      12

1105      36      36      42   1042      30

1005              21      27   1042      15

1106      12                   1169      12

1106      36      36      42   1043      30

1106              21      27   1043      15

1107      12                   1170      12

1107      36      36      42   1044      30

1107              21      27   1044      15

1108      12                   1171      12

1108      36      36      42   1045      30

1108              21      27   1045      15

1109      12                   1172      12

1109      36      36      42   1046      30

1109              21      27   1046      15

1110      12                   1173      12

1110      36      36      42   1047      30

1110              21      27   1047      15

1111      12                   1174      12

1111      36      36      42   1048      30

1111              21      27   1048      15

1112      12                   1175      12

1112      36      36      42   1049      30

1112              21      27   1049      15

1113      12                   1176      12

1113      36      36      42   1050      30

1113              21      27   1050      15

1114      12                   1177      12

1114      36      36      42   1051      30

1114              21      27   1051      15

1115      12                   1178      12

1115      36      36      42   1052      30

1115              21      27   1052      15

1116      12                   1179      12

1116      36      36      42   1053      30

1116              21      27   1053      15

1117      12                   1180      12

1117      36      36      42   1054      30

1117              21      27   1054      15

1118      12                   1181      12

1118      36      36      42   1055      30

1118              21      27   1055      15

1119      12                   1182      12

1119      36      36      42   1056      30

1119              21      27   1056      15

1120      12                   1183      12

1120      36      36      42   1057      30

1120              21      27   1057      15

1121      12                   1184      12

1121      36      36      42   1058      30

1121              21      27   1058      15

1122      12                   1185      12

1122      36      36      42   1059      30

1122              21      27   1059      15

1123      12                   1186      12

1123      36      36      42   1060      30

1123              21      27   1060      15

1124      12                   1187      12

1124      36      36      42   1061      30

1124              21      27   1061      15

1125      12                   1188      12

1125      36      36      42   1062      30

1125              21      27   1062      15

1126      12                   1189      12

1126      36      36      42   1063      30

1126              21      27   1063      15

1127      12                   1190      12

1127      36      36      42   1064      30

1127              21      27   1064      19

1128      12                   1191      12

1128      36      36      42   1065      30

1128              21      27   1065      19

1129      12                   1192      12

1129      36      36      42   1066      30

1129              21      27   1066      15

1130      12                   1193      12

1130      36      36      42   1067      30

1130              21      27   1067      15

1131      12                   1194      12

1131      36      36      42   1068      30

1131              21      27   1068      15

1132      12                   1195      12

1132      36      36      42   1069      30

1132              21      27   1069      15

1133      12                   1196      12

1133      36      36      42   1070      30

1133              21      27   1070      15

1134      12                   1197      12

1134      36      36      42   1071      30

1134              21      27   1071      15

1135      12                   1198      12

1135      36      36      42   1072      30

1135              21      27   1072      15

1136      12                   1199      12

1136      36      36      42   1073      30

1136              21      27   1073      15

1137      12                   1200      12

1137      36      36      42   1074      30

1137              21      27   1074      15

1138      12                   1201      12

1138      36      36      42   1075      30

1138              21      27   1075      15

1139      12                   1202      12

1139      36      36      42   1076      30

1139              21      27   1076      15

1140      12                   1203      12

1140      36      36      42   1077      30

1140              21      27   1077      15

1141      12                   1204      12

1141      36      36      42   1078      30

1141              21      27   1078      15

1142      12                   12.5      12

1142      36      36      42   1079      30

1142              21      27   1079      12

1143      12                   1206      12

1143      36      36      42   1080      30

1143              21      27   1080      15

1144      12                   1207      12

1144      36                   1081      30

1145      12                   1208      12

1145      36                   1082      30

1146      12                   1209      12

1146      36                   1083      30

1147      12                   1210      12

1147      36                   1084      30

1148      12                   1211      12

1148      36                   1085      30

1149      12                   1212      12

1149      36                   1086      30

1150      12                   1213      12

1150      36                   1087      30

Notes:

* These channels are reserved exclusively

for national allotments.

** These channels may be used for national

allotment on a secondary basis. The primary

reason for reserving these channels is to

provide protection for the secondary

Surveillance Radar (SSR) system.

<gradient> 108.0 MHz is not scheduled for

assignment to ILS service. The associated

DME operating channel No. 17X may be

assigned to the emergency service.

(b) Polarization. (1) The radio frequency emissions from all ground

equipment must be nominally vertically polarized. Any horizontally polarized

radio frequency emission component from the ground equipment must not have

incorrectly coded angle information such that the limits specified in

paragraphs (b) (2) and (3) of this section are exceeded.

(2) Rotation of the receiving antenna thirty degrees from the vertically

polarized position must not cause the path following error to exceed the

allowed error at that location.

(c) Modulation requirements. Each function transmitter must be capable of

DPSK and continuous wave (CW) modulations of the RF carrier which have the

following characteristics.

(1) DPSK. The DPSK signal must have the following characteristics:

bit rate          15.625 KHz

bit length        64 microseconds

logic "0"         no phase transition

logic "1"         phase transition

phase transition  less than 10 microseconds

phase tolerance   +/- 10 degrees

The phase shall advance (or retard) monotonically throughout the transition

region. Amplitude modulation during the phase transition period shall not be

used.

[ ...Illustration appears here... ]

Figure 1. -DPSK Phase Characteristic

(2) CW. The CW pulse transmissions and the CW angle transmissions as may be

required in the signal format of any function must have characteristics such

that the requirements of paragraph (d) of this section are met.

(d) Radio frequency signal spectrum. The transmitted signal must be such

that during the transmission time, the mean power density above a height of

600 meters (2000 feet) does not exceed -100.5 dBW/m2 for angle guidance and -

95.5 dBW/m2 for data, as measured in a 150 KHz bandwidth centered at a

frequency of 840 KHz or more from the assigned frequency.

(e) Synchronization. Synchronization between the azimuth and elevation

components is required and, in split-site configurations, would normally be

accomplished by landline interconnections. Synchronization monitoring must be

provided to preclude function overlap.

(f) Transmission rates. Angle guidance and data signals must be transmitted

at the following average repetition rates:

Average data

Function           rate (Hertz)

Approach Azimuth                13+/-0.5

High Rate Approach Azimuth  /1/ 39+/-1.5

Approach Elevation              39+/-1.5

Back Azimuth                  6.5+/-0.25

Basic Data                         (/2/)

Auxiliary Data                     (/3/)

/1/ The higher rate is recommended for

azimuth scanning antennas with

beamwidths greater than two degrees. It

should be noted that the time available

in the signal format for additional

functions is limited when the higher

rate is used.

/2/ Refer to Table 8a.

/3/ Refer to Table 8c.

(g) Transmission sequences. Sequences of angle transmissions which will

generate the required repetition rates are shown in Figures 2 and 3.

[ ...Illustration appears here... ]

Figure 2. Transmission sequence pair which provide for all MLS angle

guidance functions.

[ ...Illustration appears here... ]

Figure 3. Transmission sequence pair which provides for the MLS high rate

approach azimuth angle guidance function.

(h) TDM cycle. The time periods between angle transmission sequences must

be varied so that exact repetitions do not occur within periods of less than

0.5 second in order to protect against synchronous interference. One such

combination of sequences is shown in Figure 4 which forms a full multiplex

cycle. Data may be transmitted during suitable open times within or between

the sequences.

[ ...Illustration appears here... ]

Figure 4. A complete function multiplex cycle.

(i) Function Formats (General). Each angle function must contain the

following elements: a preamble; sector signals; and a TO and FRO angle scan

organized as shown in Figure 5a. Each data function must contain a preamble

and a data transmission period organized as shown in Figure 5b.

[ ...Illustration appears here... ]

Figure 5 - Function format.

(1) Preamble format. The transmitted angle and date functions must use the

preamble format shown in Figure 6. This format consists of a carrier

acquisition period of unmodulated CW transmission followed by a receiver

synchronization code and a function identification code. The preamble timing

must be in accordance with Table 2.

[ ...Illustration appears here... ]

Figure 6 - Preamble organization.

(i) Digital codes. The coding used in the preamble for receiver

synchronization is a Barker code logic 11101. The time of the last phase

transition midpoint in the code shall be the receiver reference time (see

Table 2). The function identification codes must be as shown in Table 3. The

last two bits (I11 and I12) of the code are parity bits obeying the

equations:

I6+I7+I8+I9+I10+I11=Even

I6+I8+I10+I12=Even

(ii) Data modulation. The digital code portions of the preamble must be

DPSK modulated in accordance with Sec. 171.311(c)(1) and must be transmitted

throughout the function coverage volume.

(2) Angle function formats. The timing of the angle transmissions must be

in accordance with Tables 4a, 4b, and 5. The actual timing of the TO and FRO

scans must be as required to meet the accuracy requirements of Secs. 171.313

and 171.317.

(i) Preamble. Must be in accordance with requirements of Sec.

171.311(i)(1).

Table 2--Preamble Timing /1/

Event time slot begins

at--

15.625

kHz

clock

pulse         Time

Event              (number)  (milliseconds)

Carrier acquisition:

(CW transmission)                    0               0

Receiver reference time code:

I1=1                                13           0.832

I2=1                                14           0.896

I3=1                                15           0.960

I4=0                                16           1.024

I5=1                                17       /2/ 1.088

Function identification:

I6                                  18           1.152

I7                                  19           1.216

I8                                  20           1.280

I9                                  21           1.344

I10 (see table 1)                   22           1.408

I11                                 23           1.472

I12                                 24           1.536

END PREAMBLE                        25           1.600

/1/ Applies to all functions transmitted.

/2/ Reference time for receiver synchronization for all

function timing.

Table 3--Function Identification Codes

Code

Function           I6  I7  I8  I9  I10  I11  I12

Approach azimuth             0   0   1   1    0    0    1

High rate approach azimuth   0   0   1   0    1    0    0

Approach elevation           1   1   0   0    0    0    1

Back azimuth                 1   0   0   1    0    0    1

Basic data 1                 0   1   0   1    0    0    0

Basic data 2                 0   1   1   1    1    0    0

Basic data 3                 1   0   1   0    0    0    0

Basic data 4                 1   0   0   0    1    0    0

Basic data 5                 1   1   0   1    1    0    0

Dasic data 6                 0   0   0   1    1    0    1

Auxiliary data A             1   1   1   0    0    1    0

Auxiliary data B             1   0   1   0    1    1    1

Auxiliary data C             1   1   1   1    0    0    0

(ii) Sector signals. In all azimuth formats, sector signals must be

transmitted to provide Morse Code identification, airborne antenna selection,

and system test signals. These signals are not required in the elevation

formats. In addition, if the signal from an installed ground component

results in a valid indication in an area where no valid guidance should

exist, OCI signals must be radiated as provided for in the signal format (see

Tables 4a, 4b, and 5). The sector signals are defined as follows:

(A) Morse Code. DPSK transmissions that will permit Morse Code facility

identification in the aircraft by a four letter code starting with the letter

"M" must be included in all azimuth functions. They must be transmitted and

repeated at approximately equal intervals, not less than six times per

minute, during which time the ground subsystem is available for operational

use. When the transmissions of the ground subsystem are not available, the

identification signal must be suppressed. The audible tone in the aircraft is

started by setting the Morse Code bit to logic "1" and stopped by a logic "0"

(see Tables 4a and 4b). The identification code characteristics must conform

to the following: the dot must be between 0.13 and 0.16 second in duration,

and the dash between 0.39 and 0.48 second. The duration between dots and/or

dashes must be one dot plus or minus 10%. The duration between characters

(letters) must not be less than three dots. When back azimuth is provided,

the code shall be transmitted by the approach azimuth and back azimuth within

plus or minus 0.08 seconds.

(B) Airborne antenna selection. A signal for airborne antenna selection

shall be transmitted as a "zero" DPSK signal lasting for a six-bit period

(see Tables 4a and 4b).

Table 4a--Approach Azimuth Function timing

Event time slot begins

at--

15.625

kHz

clock

pulse         Time

Event                  (number)  (milliseconds)

Preamble                                      0               0

Morse code                                   25           1.600

Antenna select                               26           1.664

Rear OCI                                     32           2.048

Left OCI                                     34           2.176

Right OCI                                    36           2.304

To test                                      38           2.432

To scan /1/                                  40           2.560

Pause                                                     8.760

Midscan point                                             9.060

FRO scan /1/                                              9.360

FRO test                                                 15.560

End Function (Airborne)                                  15.688

End guard time; end function (ground)                    15.900

/1/The actual commencement and completion of the TO and the FRO

scan transmissions are dependent on the amount of proportional

guidance provided. The time slots provided shall accommodate a

maximum scan of plus or minus 62.0 degrees. Scan timing shall

be compatible with accuracy requirements.

Table 4b--High Rate Approach Azimuth and Back Azimuth

Function Timing

Event time slot begins

at--

15.625

kHz

clock

pulse         Time

Event                  (number)  (milliseconds)

Preamble                                      0               0

Morse Code                                   25           1.600

Antenna select                               26           1.664

Rear OCI                                     32           2.048

Left OCI                                     34           2.176

Right OCI                                    36           2.304

To test                                      38           2.432

To scan /1/                                  40           2.560

Pause                                                     6.760

Midscan point                                             7.060

FRO scan /1/                                              7.360

FRO test pulse                                           11.560

End function (airborne)                                  11.688

End guard time; end function (ground)                    11.900

/1/ The actual commencement and completion of the TO and the

FRO scan transmissions are dependent on the amount of

proportional guidance provided. The time slots provided will

accommodate a maximum scan of plus or minus 42.0 degrees. Scan

timing shall be compatible with accuracy requirements.

(C) OCI. Where OCI pulses are used, they must be: (1) greater than any

guidance signal in the OCI sector; (2) at least 5 dB less than the level of

the scanning beam within the proportional guidance sector; and (3) for

azimuth functions with clearance signals, at least 5 dB less than the level

of the left (right) clearance pulses within the left (right) clearance

sector.

Table 5--Approach Elevation Function Timing

Event time slot begins

at:

15.625

kHz

clock

pluse         Time

Event                  (number)  (milliseconds)

Preamble                                      0               0

Processor pause                              25           1.600

OCI                                          27           1.728

To scan /1/                                  29           1.856

Pause                                                     3.406

Midscan point                                             3.606

FRO scan /1/                                              3.806

End function (airborne)                                   5.356

End guard time; end function (ground)                     5.600

/1/ The actual commencement and completion of the TO and FRO

scan transmissions are dependent upon the amount of

proportional guidance provided. The time slots provided will

accommodate a maximum scan of -1.5 degrees to +29.5 degrees.

Scan timing shall be compatible with accuracy requirements.

The duration of each pulse measured at the half amplitude point shall be at

least 100 microseconds, and the rise and fall times shall be less then 10

microseconds. It shall be permissible to sequentially transmit two pulses in

each out-of-coverage indication time slot. Where pulse pairs are used, the

duration of each pulse shall be at least 50 microseconds, and the rise and

fall times shall be less then 10 microseconds. The transmission of out-of-

coverage indication pulses radiated from antennas with overlapping coverage

patterns shall be separated by at least 10 microseconds.

Note: If desired, two pulses may be sequentially transmitted in each OCI

time slot. Where pulse pairs are used, the duration of each pulse must be 45

(+/-5) microseconds and the rise and fall times must be less than 10

microseconds.

(D) System test. Time slots are provided in Tables 4a and 4b to allow

radiation of TO and FRO test pulses. However, radiation of these pulses is

not required since the characteristics of these pulses have not yet been

standardized.

(iii) Angle encoding. The encoding must be as follows:

(A) General. Azimuth and elevation angles are encoded by scanning a narrow

beam between the limits of the proportional coverage sector first in one

direction (the TO scan) and then in the opposite direction (the FRO scan).

Angular information must be encoded by the amount of time separation between

the beam centers of the TO and FRO scanning beam pulses. The TO and FRO

transmissions must be symmetrically disposed about the midscan point listed

in Tables 4a, 4b, 5, and 7. The midscan point and the center of the time

interval between the TO and FRO scan transmissions must coincide with a

tolerance of +/-10 microseconds. Angular coding must be linear with angle and

properly decoded using the formula:

V

u= - (T0 -t)

2

where:

u=Receiver angle in degrees.

V=Scan velocity in degrees per microsecond.

T0=Time separation in microseconds between TO and FRO beam centers

corresponding to zero degrees.

t=Time separation in microseconds between TO and FRO beam centers.

The timing requirements are listed in Table 6 and illustrated in Figure 7.

[ ...Illustration appears here... ]

Figure 7. Azimuth Angle Scan Timing (Not to Scale)

(B) Azimuth angle encoding. Each guidance angle transmitted must consist of

a clockwise TO scan followed by a counterclockwise FRO scan as viewed from

above the antenna. For approach azimuth functions, increasing angle values

must be in the direction of the TO scan; for the back azimuth function,

increasing angle values must be in the direction of the FRO scan. The antenna

has a narrow beam in the plane of the scan direction and a broad beam in the

orthogonal plane which fills the vertical coverage.

(C) Elevation angle encoding. The radiation from elevation equipment must

produce a beam which scans from the horizon up to the highest elevation angle

and then scans back down to the horizon. The antenna has a narrow beam in the

plane of the scan direction and a broad beam in the orthogonal plane which

fills the horizontal coverage. Elevation angles are defined from the

horizontal plane containing the antenna phase center; positive angles are

above the horizontal and zero angle is along the horizontal.

(iv) Clearance guidance. The timing of the clearance pulses must be in

accordance with Figure 8. For azimuth elements with proportional coverage of

less than +/-40 degrees (+/-20 degrees for back azimuth), clearance guidance

information must be provided by transmitting pulses in a TO and FRO format

adjacent to the stop/start times of the scanning beam signal. The fly-right

clearance pulses must represent positive angles and the fly-left clearance

pulses must represent negative angles. The duration of each clearance pulse

must be 50 microseconds with a tolerance of +/-5 microseconds. The

transmitter switching time between the clearance pulses and the scanning beam

transmissions must not exceed 10 microseconds. The rise time at the edge of

each clearance pulse must be less than 10 microseconds. Within the fly-right

clearance guidance section, the fly-right clearance guidance signal shall

exceed scanning beam antenna sidelobes and other guidance and OCI signals by

at least 5 dB; within the fly-left clearance guidance sector, the fly left

clearance guidance signal shall exceed scanning beam antenna sidelobes and

all other guidance and OCI signals by at least 5 dB; within the proportional

guidance sector, the clearance guidance signals shall be at least 5dB below

the proportional guidance signal. Optionally, clearance guidance may be

provided by scanning throughout the approach guidance sector. For angles

outside the approach azimuth proportional coverage limits as set in Basic

Data Word One (Basic Data Word 5 for back azimuth), proper decode and display

of clearance guidance must occur to the limits of the guidance region. Where

used, clearance pulses shall be transmitted adjacent to the scanning beam

signals at the edges of proportional coverage as shown in Figure 8. The

proportional coverage boundary shall be established at one beamwidth inside

the scan start/stop angles, such that the transition between scanning beam

and clearance signals occurs outside the proportional coverage sector. When

clearance pulses are provided in conjunction with a narrow beamwidth (e.g.,

one degree) scanning antenna, the scanning beam antenna shall radiate for 15

microseconds while stationary at the scan start/stop angles.

(3) Data function format. Basic data words provide equipment

characteristics and certain siting information. Basic data words must be

transmitted from an antenna located at the approach azimuth or back azimuth

site which provides coverage throughout the appropriate sector. Data function

timing must be in accordance with Table 7a.

Table 6--Angle Scan Timing Constants

Max

value                           Pause

of       To    V(deg/    Tm     time     Tt

Function           t(usec)  (usec)  usec)   (usec)  (usec)  (usec)

Approach azimuth             13,000   6,800    0.02   7,972     600  13,128

High rate approach azimuth    9,000   4,800    0.02   5,972     600   9,128

Approach elevation            3,500   3,350    0.02   2,518     400     N/A

Back azimuth                  9,000   4,800   -0.02   5,972     600   9,128

Table 7a--Basic Data Function Timing

Event time slot begins

at:/1/

15.625

kHz

clock

pulse         Time

Event                  (number)  (milliseconds)

Preamble                                      0               0

Data transmission (bits I13-I30)             25           1.600

Parity transmission (bits I31-I32)           43           2.752

End function (airborne)                      45           2.880

End guard time: end function (ground)                     3.100

/1/ The previous event time slot ends at this time.

Table 7b--Auxiliary Data Function Timing--(Digital)

Event time slot begins

at:

15.625

kHz

clock

pulse         Time

Event                  (number)  (milliseconds)

Preamble                                      0               0

Address transmission (bits I13-I20)          25           1.600

Data transmission: (bits I21-I69)            33           2.112

Parity transmission (bits I70-I76)           82           5.248

End function (airborne)                      89           5.696

End guard time; end function (ground)                     5.900

Table 7c--Auxiliary Data Function Timing--(Alphanumeric)

Event time slot begins

at:

15.615

kHz

clock

pulse         Time

Event                  (number)  (milliseconds)

Preamble                                      0               0

Address transmission (bits I13-I20)          25           1.600

Data transmission: (bits I21-I76             33           2.112

End function (airborne)                      89           5.696

End guard time; (end function ground)                     5.900

(i) Preamble. Must be in accordance with requirements of Sec.

171.311(i)(1).

(ii) Data transmissions. Basic data must be transmitted using DPSK

modulation. The content and repetition rate of each basic data word must be

in accordance with Table 8a. For data containing digital information, binary

number 1 must represent the lower range limit with increments in binary steps

to the upper range limit shown in Table 8a. Data containing digital

information shall be transmitted with the least significant bit first.

(j) Basic Data word requirements. Basic Data shall consist of the items

specified in Table 8a. Basic Data word contents shall be defined as follows:

(1) Approach azimuth to threshold distance shall represent the minimum

distance between the Approach Azimuth antenna phase center and the vertical

plane perpendicular to the centerline which contains the landing threshold.

(2) Approach azimuth proportional coverage limit shall represent the limit

of the sector in which proportional approach azimuth guidance is transmitted.

[ ...Illustration appears here... ]

Figure 8.  Clearance Pulse Timing for Azimuth Functions

Table 8a--Basic Data Words

LSB       Data bit

Data bit NZ            Data item definition      value        value

Basic Data Word No. 1

1                          Preamble                        N/A  1

2                                                               1

3                                                               1

4                                                               0

5                                                               1

6                                                               0

7                                                               1

8                                                               0

9                                                               1

10                                                              0

11                                                              0

12                                                              0

13                         Approach azimuth to            100m  100m

threshold distance (Om-

630m)

14                                                              200m

15                                                              400m

16                                                              800m

17                                                              1600m

18                                                              3200m

19                         Approach azimuth             2 deg.  -2 deg.

proportional coverage

limit (negative limit)

(0 deg. to -62 deg.)

20                                                              -4 deg.

21                                                              -8 deg.

22                                                              -16 deg.

23                                                              -32 deg.

24                         Approach azimuth             2 deg.  2 deg.

proportional coverage

limit (positive limit)

(0 deg. to +62 deg.)

25                                                              4 deg.

26                                                              8 deg.

27                                                              16 deg.

28                                                              32 deg.

29                         Clearance signal type           N/A  0=pulse; 1=SB

30                         Spare                                Transmit zero

31                         Parity: (13+14+15. . .+30       N/A  N/A

+31=odd)

32                         Parity: (14+16+18. . .+30       N/A  N/A

+32=odd)

Note 1: Transmit throughout the Approach Azimuth guidance sector at intervals

of 1.0 seconds or less.

Note 2: The all zero state of the data field represents the lower limit of

the absolute value of the coded parameter unless otherwise noted.

Basic Data Word No. 2

1                          Preamble                        N/A  1

2                                                               1

3                                                               1

4                                                               0

5                                                               1

6                                                               0

7                                                               1

8                                                               1

9                                                               1

10                                                              1

11                                                              0

12                                                              0

13                         Minimum glide path (2.0    0.1 deg.  0.1 deg.

deg. to 14.7 deg.)

14                                                              0.2 deg.

15                                                              0.4 deg.

16                                                              0.8 deg.

17                                                              1.6 deg.

18                                                              3.2 deg.

19                                                              6.4 deg.

20                         Back azimuth status                  see note 4

21                         DME status                           see note 6

22

23                         Approach azimuth status              see note 4

24                         Approach azimuth status              see lnote 4

25                         Spare                                Transmit zero

26                         ......do                                 Do.

27                         ......do                                 Do.

28                         ......do                                 Do.

29                         ......do                                 Do.

30                         ......do                                 Do.

31                         Parity: (13+14+15. . .+30       N/A  N/A

+31)=odd)

32                         Parity: (14+16+18. . .+30       N/A  N/A

+32=odd)

Note 1: Transmit throughout the Approach Azimuth guidance sector at intervals

of 0.16 seconds or less.

Note 2: The all zero state of the data field represents the lower limit of

the absolute range of the coded parameter unless otherwise noted.

Basic Data Word No. 3

1                          Preamble                        N/A  1

2                                                               1

3                                                               1

4                                                               0

5                                                               1

6                                                               1

7                                                               0

8                                                               1

9                                                               0

10                                                              0

11                                                              0

12                                                              0

13                         Approach azimuth           0.5 deg.  0.5 deg.

beamwidth (0.5 deg.-4.0

deg.) See note 7

14                                                              1.0 deg.

15                                                              2.0 deg.

16                         Approach elevation         0.5 deg.  0.5 deg.

beamwidth (0.5 deg. to

2.5 deg.) See note 7

17                                                              1.0 deg.

18                         Note: values greater than            2.0 deg.

2.5 deg. are invalid

19                         DME distance (Om to           12.5m  12.5m

6387.5m

20                                                              25.0m

21                                                              50.0m

22                                                              100.0m

23                                                              200.0m

24                                                              400.0m

25                                                              800.0m

26                                                              1600.0m

27                                                              3200.0m

28                         Spare                                Transmit zero

29                         ......do                                 Do.

30                         ......do                                 Do.

31                         Parity: (13+14+15. . .+30

+31=odd)

32                         Parity: (14+16+18. . .+30       N/A  N/A

+32=odd)

Note 1: Transmit throughout the Approach Azimuth guidance sector at intervals

of 1.0 seconds or less.

Note 2: The all zero state of the data field represents the lower limit of

the absolute range of the coded parameter unless otherwise noted.

Basic Data Word No. 4

1                          Preamble                        N/A  1

2                                                               1

3                                                               1

4                                                               0

5                                                               1

6                                                               1

7                                                               0

8                                                               0

9                                                               0

10                                                              1

11                                                              0

12                                                              0

13                         Approach azimuth magnetic    1 deg.  1 deg.

orientation (0 deg. to

359 deg.)

14                                                              2 deg.

15                                                              4 deg.

16                                                              8 deg.

17                                                              16 deg.

18                                                              32 deg.

19                                                              64 deg.

20                                                              128 deg.

21                                                              256 deg.

22                         Back azimuth magnetic        1 deg.  1 deg.

orientation (0 deg. to

359 deg.)

23                                                              2 deg.

24                                                              4 deg.

25                                                              8 deg.

26                                                              16 deg.

27                                                              32 deg.

28                                                              64 deg.

29                                                              128 deg.

30                                                              256 deg.

31                         Parity: (13+14+15. . .+30       N/A  N/A

+31=odd)

32                         Parity: (14+16+18. . .+30       N/A  N/A

+32=odd)

Note 1: Transmit at intervals of 1.0 second or less throughout the Approach

Azimuth guidance sector, except when Back Azimuth guidance is provided. See

Note 8.

Note 2: The all zero state of the data field represents the lower limit of

the absolute range of the coded parameter unless otherwise noted.

Basic Data Word No. 5

1                          Preamble                        N/A  1

2                                                               1

3                                                               1

4                                                               0

5                                                               1

6                                                               1

7                                                               1

8                                                               0

9                                                               1

10                                                              1

11                                                              0

12                                                              0

13                         Back azimuth proportional    2 deg.  -2 deg.

coverage negative limit

(0 deg. to -42 deg.)

14                                                              -4 deg.

15                                                              -8 deg.

16                                                              -16 deg.

17                                                              -32 deg.

18                         Back azimuth proportional    2 deg.  2 deg.

coverage positive limit

(0 deg. to +42 deg.)

19                                                              4 deg.

20                                                              8 deg.

21                                                              16 deg.

22                                                              32 deg.

23                         Back azimuth beamwidth     0.5 deg.  0.5 deg.

(0.5 deg. to 4.0 deg.)

See note 7

24                                                              1.0 deg.

25                                                              2.0 deg.

26                         Back azimuth status                  See Note 10

27                         ......do                                 Do.

28                         ......do                                 Do.

29                         ......do                                 Do.

30                         ......do                                 Do.

31                         Parity: (13+14+15. . .+30       N/A  N/A

+31=odd)

32                         Parity: (14+16+18. . .+30       N/A  N/A

+32=odd)

Note 1: Transmit only when Back Azimuth guidance is provided. See note 9.

Note 2: The all zero state of the data filed represents the lower limit of

the absolute range of the coded parameter unless otherwise noted.

Basic Data Word No. 6

1                          Preamble                        N/A  1

2                                                               1

3                                                               1

4                                                               0

5                                                               1

6                                                               0

7                                                               0

8                                                               0

9                                                               1

10                                                              1

11                                                              0

12                                                              1

(13-                       MLS ground equipment

30)                        identification (Note 3)

13                         Character 2                     N/A  B1

14                                                              B2

15                                                              B3

16                                                              B4

17                                                              B5

18                                                              B6

19                         Character 3                     N/A  B1

20                                                              B2

21                                                              B3

22                                                              B4

23                                                              B5

24                                                              B6

25                         Character 4                     N/A  B1

26                                                              B2

27                                                              B3

28                                                              B4

29                                                              B5

30                                                              B6

31                         Parity: (13+14+15. . .+30       N/A  N/A

+31=odd)

32                         Parity: (14+16+18. . .+30       N/A  N/A

+32=odd)

Note 1: Transmit at intervals of 1.0 second or less throughout the Approach

Azimuth guidance sector, except when Back Azimuth guidance is provided. See

note 8.

Note 3: Characters are encoded using

the International Alphabet Number 5,

(IA-5):

Note 4: Coding for status bit:

0=Function not radiated, or radiated in

test mode (not reliable for

navigation).

1=Function radiated in normal mode (for

Back Azimuth, this also indicates that

a Back Azimuth transmission follows).

Note 5: Date items which are not

applicable to a particular ground

equipment shall be transmitted as all

zeros.

Note 6: Coding for status bits:

I21  I22

0      0  DME transponder inoperative or not available.

1      0  Only IA mode or DME/N available.

0      0  FA mode, Standard 1, available.

1      1  FA mode, Standard 2, available.

Note 7: The value coded shall be the actual beamwidth

(as defined in Sec. 171.311 (j)(9) rounded to the

nearest 0.5 degree.

Note 8: When back Azimuth guidance is provided, Data

Words 4 and 6 shall be transmitted at intervals of 1.33

seconds or less throughout the Approach Azimuth

coverage and 4 seconds or less throughout the Back

Azimuth coverage.

Note 9: When Back Azimuth guidance is provided, Data

Word 5 shall be transmitted at an interval of 1.33

seconds or less through out the Back Azimuth coverage

sector and 4 seconds or less throughout the Approach

Azimuth coverage sector.

Note 10: Coding for status bit:

0=Function not radiated, or radiated in test mode (not

reliable for navigation).

1=Function radiated in normal mode.

(4) Minimum glidepath the lowest angle of descent along the zero degree

azimuth that is consistent with published approach procedures and obstacle

clearance criteria.

(5) Back azimuth status shall represent the operational status of the Back

Azimuth equipment.

(6) DME status shall represent the operational status of the DME equipment.

(7) Approach azimuth status shall represent the operational status of the

approach azimuth equipment.

(8) Approach elevation status shall represent the operational status of the

approach elevation equipment.

(9) Beamwidth the width of the scanning beam main lobe measured at the -3

dB points and defined in angular units on the antenna boresight, in the

horizontal plane for the azimuth function and in the vertical plane for the

elevation function.

(10) DME distance shall represent the minimum distance between the DME

antenna phase center and the vertical plane perpendicular to the runway

centerline which contains the MLS datum point.

(11) Approach azimuth magnetic orientation shall represent the angle

measured in the horizontal plane clockwise from Magnetic North to the zero-

degree angle guidance radial originating from the approach azimuth antenna

phase center. The vertex of the measured angle shall be at the approach

azimuth antenna phase center.

Note: For example, this data item would be encoded 090 for an approach

azimuth antenna serving runway 27 (assuming the magnetic heading is 270

degrees) when sited such that the zero degree radial is parallel to

centerline.

(12) Back azimuth magnetic orientation shall represent the angle measured

in the horizontal plane clockwise from Magnetic North to the zero-degree

angle guidance radial originating from the Back Azimuth antenna. The vertex

of the measured angle shall be at the Back Azimuth antenna phase center.

Note: For example, this data item would be encoded 270 for a Back Azimuth

Antenna serving runway 27 (assuming the magnetic heading is 270 degrees) when

sited such that the zero degree radial is parallel to centerline.

(13) Back azimuth proportional coverage limit shall represent the limit of

the sector in which proportional back azimuth guidance is transmitted.

(14) MLS ground equipment identification shall represent the last three

characters of the system identification specified in Sec. 171.311(i)(2). The

characters shall be encoded in accordance with International Alphabet No. 5

(IA-5) using bits b1 through b6.

Note: Bit b7 of this code may be reconstructed in the airborne receiver by

taking the complement of bit b6.

(k) Residual radiation. The residual radiation of a transmitter associated

with an MLS function during time intervals when it should not be transmitting

shall not adversely affect the reception of any other function. The residual

radiation of an MLS function at times when another function is radiating

shall be at least 70 dB below the level provided when transmitting.

(l) Symmetrical scanning. The TO and FRO scan transmissions shall be

symmetrically disposed about the mid-scan point listed in Tables 4a, 4b and

5. The mid-scan point and the center of the time interval between the TO and

FRO scan shall coincide with a tolerance of plus or minus 10 microseconds.

(m) Auxiliary data--(1) Addresses. Three function identification codes are

reserved to indicate transmission of Auxiliary Data A, Auxiliary Data B, and

Auxiliary Data C. Auxiliary Data A contents are specified below, Auxiliary

Data B contents are reserved for future use, and Auxiliary Data C contents

are reserved for national use. The address codes of the auxiliary data words

shall be as shown in Table 8b.

(2) Organization and timing. The organization and timing of digital

auxiliary data must be as specified in Table 7b. Data containing digital

information must be transmitted with the least significant bit first.

Alphanumeric data characters must be encoded in accordance with the 7-unit

code character set as defined by the American National Standard Code for

Information Interchange (ASCII). An even parity bit is added to each

character. Alphanumeric data must be transmitted in the order in which they

are to be read. The serial transmission of a character must be with the lower

order bit transmitted first and the parity bit transmitted last. The timing

for alphanumeric auxiliary data must be as shown in Table 7c.

(3) Auxiliary Data A content: The data items specified in Table 8c are

defined as follows:

(i) Approach azimuth antenna offset shall represent the minimum distance

between the Approach Azimuth antenna phase center and the vertical plane

containing the runway centerline.

(ii) Approach azimuth to MLS datum point distance shall represent the

minimum distance between the Approach Azimuth antenna phase center and the

vertical plane perpendicular to the centerline which contains the MLS datum

point.

(iii) Approach azimuth alignment with runway centerline shall represent the

minimum angle between the approach azimuth antenna zero-degree guidance plane

and the runway certerline.

(iv) Approach azimuth antenna coordinate system shall represent the

coordinate system (planar or conical) of the angle data transmitted by the

approach azimuth antenna.

(v) Approach elevation antenna offset shall represent the minimum distance

between the elevation antenna phase center and the vertical plane containing

the runway centerline.

(vi) MLS datum point to threshold distance shall represent the distance

measured along the runway centerline from the MLS datum point to the runway

threshold.

(vii) Approach elevation antenna height shall represent the height of the

elevation antenna phase center relative to the height of the MLS datum point.

(viii) DME offset shall represent the minimum distance between the DME

antenna phase center and the vertical plane containing the runway centerline.

(ix) DME to MLS datum point distance shall represent the minimum distance

between the DME antenna phase center and the vertical plane perpendicular to

the centerline which contains the MLS datum point.

(x) Back azimuth antenna offset shall represent the minimum distance

between the back azimuth antenna phase center and the vertical plane

containing the runway centerline.

(xi) Back azimuth to MLS datum point distance shall represent the minimum

distance between the Back Azimuth antenna and the vertical plane

perpendicular to the centerline which contains the MLS datum point.

(xii) Back azimuth antenna alignment with runway centerline shall represent

the minimum angle between the back azimuth antenna zero-degree guidance plane

and the runway centerline.

Sec. 171.313  Azimuth performance requirements.

This section prescribes the performance requirements for the azimuth

equipment of the MLS as follows:

(a) Approach azimuth coverage requirements. The approach azimuth equipment

must provide guidance information in at least the following volume of space

(see Figure 9):

Table 8b---Auxiliary Data Word Address

Codes

No.  I13  I14  I15  I16  I17  I18  I19  I20

1.    0    0    0    0    0    1    1    1

2.    0    0    0    0    1    0    1    0

3.    0    0    0    0    1    1    0    1

4.    0    0    0    1    0    0    1    1

5.    0    0    0    1    0    1    0    0

6.    0    0    0    1    1    0    0    1

7.    0    0    0    1    1    1    1    0

8.    0    0    1    0    0    0    1    0

9.    0    0    1    0    0    1    0    1

10.    0    0    1    0    1    0    0    0

11.    0    0    1    0    1    1    1    1

12.    0    0    1    1    0    0    0    1

13.    0    0    1    1    0    1    1    0

14.    0    0    1    1    1    0    1    1

15.    0    0    1    1    1    1    0    0

16.    0    1    0    0    0    0    1    1

17.    0    1    0    0    0    1    0    0

18.    0    1    0    0    1    0    0    1

19.    0    1    0    0    1    1    1    0

20.    0    1    0    1    0    0    0    0

21.    0    1    0    1    0    1    1    1

22.    0    1    0    1    1    0    1    0

23.    0    1    0    1    1    1    0    1

24.    0    1    1    0    0    0    0    1

25.    0    1    1    0    0    1    1    0

26.    0    1    1    0    1    0    1    1

27.    0    1    1    0    1    1    0    0

28.    0    1    1    1    0    0    1    0

29.    0    1    1    1    0    1    0    1

30.    0    1    1    1    1    0    0    0

31.    0    1    1    1    1    1    1    1

32.    1    0    0    0    0    0    1    0

33.    1    0    0    0    0    1    0    1

34.    1    0    0    0    1    0    0    0

35.    1    0    0    0    1    1    1    1

36.    1    0    0    1    0    0    0    1

37.    1    0    0    1    0    1    1    0

38.    1    0    0    1    1    0    1    1

39.    1    0    0    1    1    1    0    0

40.    1    0    1    0    0    0    0    0

41.    1    0    1    0    0    1    1    1

42.    1    0    1    0    1    0    1    0

43.    1    0    1    0    1    1    0    1

44.    1    0    1    1    0    0    1    1

45.    1    0    1    1    0    1    0    0

46.    1    0    1    1    1    0    0    1

47.    1    0    1    1    1    1    1    0

48.    1    1    0    0    0    0    0    1

49.    1    1    0    0    0    1    1    0

50.    1    1    0    0    1    0    1    1

51.    1    1    0    0    1    1    0    0

52.    1    1    0    1    0    0    1    0

53.    1    1    0    1    0    1    0    1

54.    1    1    0    1    1    0    0    0

55.    1    1    0    1    1    1    1    1

56.    1    1    1    0    0    0    1    1

57.    1    1    1    0    0    1    0    0

58.    1    1    1    0    1    0    0    1

59.    1    1    1    0    1    1    1    0

60.    1    1    1    1    0    0    0    0

61.    1    1    1    1    0    1    1    1

62.    1    1    1    1    1    0    1    0

63.    1    1    1    1    1    1    0    1

64.    0    0    0    0    0    0    0    0

Note 1: Parity bits I19 and I20 are chosen to satisfy the equations:

I13+I14+I15+I16+I17+I18+I19=EVEN

I14+I16+I18+I20=EVEN

Table 8c--Auxiliary Data

Word                          Maximun time

(See                             between                             Least

note                 Type of  transmissions  Bits    Range of     significant

6)   Data content    data      (Seconds)    used     values          bit

A1    Preamble       Digital            1.0    12

Address                                   8

Approach                                 10  -511 m to              1 m

azimuth                                      +511 m (See

antenna                                      note 3)

offset

Approach                                 13  0 m to 8 191           1 m

azimuth to                                   m

MLS datum

point

distance

Approach                                 12  -20.47 deg.      0.01 deg.

azimuth                                      to 20.47

antenna                                      deg. (See

alignment                                    note 3)

with runway

centerline

Approach                                  1  (See note 2)

azimuth

antenna

coordinate

system

Spare                                    13

Parity                                    7  (See note 1)

A2    Preamble       Digital            1.0    12

Address                                   8

Approach                                 10  -511 m to              1 m

elevation                                    +511 m (See

antenna                                      note 3)

offset

MLS datum                                10  0 m to 1 023           1 m

point to                                     m

threshold

distance

Approach                                  7  -6.3 m to            0.1 m

elevation                                    +6.3 m (See

antenna                                      note 3)

height

Spare                                    22

Parity                                    7  (See note 1)

A3    Preamble       Digital   (See note 4)    12

Address                                   8

DME offset                               10  -511 m to              1 m

+511 m

DME to MLS                               14  -8 191 m to            1 m

datum point                                  +8 191 m

distance                                     (See note 3)

Spare                                    25

Parity                                    7  (See note 1)

A4    Preamble       Digital   (See note 5)    12

Address                                   8

Back azimuth                             10  -511 m to              1 m

antenna                                      +511 m (See

note 3)

Back azimuth                             11  0 m to 2 047           1 m

to MLS datum                                 m

point

distance

Back azimuth                             12  -20.47 deg.      0.01 deg.

antenna                                      to 20.47

alignment                                    deg. (See

with runway                                  note 3)

centerline

Spare                                    16

Parity                                    7  (See note 1)

Note 1: Parity bits I70 to I76 are chosen to satisfy the equations which

follow:

For BIT I70:

Even=(I13+...+I18)+I20+I22+I24+I25+I28+I29+I31+I32+I33+I35+I36+I38+I41+I44+I4

For BIT I71:

Even=(I14+...+I19)+I21+I23+I25+I26+I29+I30+I32+I33+I34+I36+I37+I39+I42+I45+I4

For BIT I72:

Even=(I15+...+I20)+I22+I24+I26+I27+I30+I31+I33+I34+I35+I37+I38+I40+I43+I46+I4

For BIT I73:

Even=(I16+...+I21)+I23+I25+I27+I28+I31+I32+I34+I35+I36+I38+I39+I41+I44+I47+I4

For BIT I74:

Even=(I17+...+I22)+I24+I26+I28+I29+I32+I33+I35+I36+I37+I39+I40+I42+I45+I48+I4

For BIT I75:

Even=(I13+...+I17)+I19+I21+I23+I24+I27+I28+I30+I31+I32+I34+I35+I37+I40+I43+I4

For BIT I76:

Even=I13+I14+...+I75+I76

Note 2</= Code for I56 is: 0=conical; 1=planar.

Note 3: The convention for the coding of negative numbers is as follows: -MSB

is the sign bit; 0=+; 1=-.

-Other bits represent the absolute value.

The convention for the antenna location is as follows: As viewed from the MLS

approach reference datum looking toward the datum point, a positive number

shall represent a location to the right of the runway centerline (lateral

offset) or above the runway (vertical offset), or towards the stop end of the

runway (longitudinal distance).

The convention for the antenna alignment is as follows: As viewed from above,

a positive number shall represent clockwise rotation from the runway

centerline to the respective zero-degree guidance plane.

Note 4: Data Word A3 is transmitted at intervals of 1.0 seconds or less

throughout the approach Azimuth coverage sector, except when back Azimuth

guidance is provided. Where back Azimuth is provided transmit at intervals of

1.33 seconds or less throughout the approach Azimuth sector and 4.0 seconds

or less throughout the back Azimuth coverage sector.

Note 5: When back Azimuth guidance is provided, transmit at intervals of 1.33

seconds or less throughout the back Azimuth coverage sector and 4.0 seconds

or less throughout the approach Azimuth coverage sector.

Note 6: The designation "A1" represents the function identification code for

"Auxiliary Data A" and address code number 1.

[ ...Illustration appears here... ]

Figure 9. Approach Azimuth/Data Coverage

(1) Horizontally within a sector plus or minus 40 degrees about the runway

centerline originating at the datum point and extending in the direction of

the approach to 20 nautical miles from the runway threshold. The minimum

proportional guidance sector must be plus or minus 10 degrees about the

runway centerline. Clearance signals must be used to provide the balance of

the required coverage, where the proportional sector is less than plus or

minus 40 degrees. When intervening obstacles prevent full coverage, the +/-40

deg. guidance sector can be reduced as required. For systems providing +/-60

deg. lateral guidance the coverage requirement is reduced to 14 nm beyond +/-

40 deg..

(2) Vertically between:

(i) A conical surface originating 2.5 meters (8 feet) above the runway

centerline at threshold inclined at 0.9 degree above the horizontal.

(ii) A conical surface originating at the azimuth ground equipment antenna

inclined at 15 degrees above the horizontal to a height of 6000 meters

(20,000 feet).

(iii) Where intervening obstacles penetrate the lower surface, coverage

need be provided only to the minimum line of sight.

(3) Runway region:

(i) Proportional guidance horizontally within a sector 45 meters (150 feet)

each side of the runway centerline beginning at the stop end and extending

parallel with the runway centerline in the direction of the approach to join

the approach region. This requirement does not apply to offset azimuth

installations.

(ii) Vertically between a horizontal surface which is 2.5 meters (8 feet)

above the farthest point of runway centerline which is in line of sight of

the azimuth antenna, and in a conical surface originating at the azimuth

ground equipment antenna inclined at 20 degrees above the horizontal up to a

height to 600 meters (2000 feet). This requirement does not apply to offset

azimuth installations.

(4) Within the approach azimuth coverage sector defined in paragraphs (a)

(1), and (2) and (3) of this section, the power densities must not be less

than those shown in Table 9 but the equipment design must also allow for:

(i) Transmitter power degradation from normal by -1.5 dB;

Table 9--Minimum Power Density Within Coverage Boundaries

(dBW/m2)

Angle signals for

various antenna

beamwidths

Data     1    1.5     2      3    Clearance

Function           signals  deg.  deg.  deg.   deg.    signals

Approach azimuth              -89.5   -88        -85.5    -82        -88

High rate approach azimuth    -89.5   -88          -88  -86.5        -88

Back azimuth                  -89.5   -88        -85.5    -82        -88

Approach elevation            -89.5   -88   -88    -88

(ii) Rain loss of -2.2 dB at the longitudinal coverage extremes.

(b) Siting requirements. The approach azimuth antenna system must, except

as allowed in paragraph (c) of this section:

(1) Be located on the extension of the centerline of the runway beyond the

stop end;

(2) Be adjusted so that the zero degree azimuth plane will be a vertical

plane which contains the centerline of the runway served;

(3) Have the minimum height necessary to comply with the coverage

requirements prescribed in paragraph (a) of this section;

(4) Be located at a distance from the stop end of the runway that is

consistent with safe obstruction clearance practices;

(5) Not obscure any light of an approach lighting system; and

(6) Be installed on frangible mounts or beyond the 300 meter (1000 feet)

light bar.

(c) On runways where limited terrain prevents the azimuth antenna from

being positioned on the runway centerline extended, and the cost of the land

fill or a tall tower antenna support is prohibitive, the azimuth antenna may

be offset.

(d) Antenna coordinates. The scanning beams transmitted by the approach

azimuth equipment within +/-40 deg. of the centerline may be either conical

or planar.

(e) Approach azimuth accuracy. (1) The system and subsystem errors shall

not exceed those listed in Table 10 at the approach reference datum.

At the approach reference datum, temporal sinusoidal noise components shall

not exceed 0.025 degree peak in the frequency band 0.01 Hz to 1.6 Hz, and the

CMN shall not exceed 0.10 degree. From the approach reference datum to the

coverage limit, the PFE, PFN and CMN limits, expressed in angular terms,

shall be allowed to linearly increase as follows:

(i) With distance along the runway centerline extended, by a factor of 1.2

for the PFE and PFN limits and to +/-0.10 degree for the CMN limits.

(ii) With azimuth angle, by a factor of 1.5 at the +/-40 degree and a

factor of 2.0 at the +/-60 degree azimuth angles for the PFE, PFN and CMN

limits.

(iii) With elevation angle from +9 degrees to +15 degrees, by a factor of

1.5 for the PFE and PFN limits.

(iv) Maximum angular limits. The PFE limits shall not exceed +/-0.25 degree

in any coverage region below an elevation angle of +9 degrees nor exceed +/-

0.50 degree in any coverage region above that elevation angle. The CMN limits

shall not exceed +/-0.10 degree in any coverage region within +/-10 degrees

of runway centerline extended nor exceed +/-0.20 degree in any other region

within coverage.

Note: It is desirable that the CMN not exceed +/-0.10 degree throughout the

coverage.

(f) Approach azimuth antenna characteristics are as follows:

(1) Drift. Any azimuth angle as encoded by the scanning beam at any point

within the proportional coverage must not vary more than +/-0.07 degree over

the range of service conditions specified in Sec. 171.309(d) without the use

of internal environmental controls. Multipath effects are excluded from this

requirement.

(2) Beam pointing errors. The azimuth angle as encoded by the scanning beam

at any point within +/-0.5 degree of the zero degree azimuth must not deviate

from the true azimuth angle at that point by more than +/-.05 degree.

Multipath and drift effects are excluded from this requirement.

Table 10--Approach Azimuth Accuracies at the Approach

Reference Datum

Angular error (degrees)

Error                                                   Airborne

type             System            Ground subsystem     subsystem

PFE    +/-20 ft. (6.1m) /1/,/2/    +/-0.118 deg. /3/  +/-0.017 deg.

CMN    +/-10.5 ft. (3.2m) /1/,/4/  +/-0.030 deg.      +/-0.050 deg.

Notes:

/1/ Includes errors due to ground and airborne equipment and

propagation effects.

/2/ The system PFN component must not exceed +/-3.5 meters (11.5

feet).

/3/ The mean (bias) error component contributed by the ground

equipment should not exceed +/-10 feet.

/4/ The system control motion noise must not exceed 0.1 degree.

/5/ The airborne subsystem angular errors are provided for

information only.

(3) Antenna alignment. The antenna must be equipped with suitable optical,

electrical or mechanical means or any combination of the three, to bring the

zero degree azimuth radial into coincidence with the approach reference datum

(for centerline siting) with a maximum error of 0.02 degree. Additionally,

the azimuth antenna bias adjustment must be electronically steerable at least

to the monitor limits in steps not greater than 0.005 degree.

(4) Antenna far field patterns in the plane of scan. On boresight, the

azimuth antenna mainlobe pattern must conform to Figure 10, and the beamwidth

must be such that, in the installed environment, no significant lateral

reflections of the mainlobe exist along the approach course. In any case the

beamwidth must not exceed three degrees. Anywhere within coverage the -3 dB

width of the antenna mainlobe, while scanning normally, must not be less than

25 microseconds (0.5 degree) or greater than 250 microseconds (5 degrees).

The antenna mainlobe may be allowed to broaden from the value at boresight by

a factor of 1/cosu, where u is the angle off boresight. The sidelobe levels

must be as follows:

(i) Dynamic sidelobe levels. With the antenna scanning normally, the

dynamic sidelobe level that is detected by a receiver at any point within the

proportional coverage sector must be down at least 10 dB from the peak of the

main beam. Outside the coverage sector, the radiation from the the scanning

beam antenna must be of such a nature that receiver warning will not be

removed or suitable OCI signals must be provided.

(ii) Effective sidelobe levels. With the antenna scanning normally, the

sidelobe levels in the plane of scan must be such that, in the installed

environment, the CMN contributed by sidelobe reflections will not exceed the

angular equivalent of 9 feet at approach reference datum over the required

range of aircraft approach speeds.

[ ...Illustration appears here... ]

Figure 10. Far Field Dynamic Signal in Space

(5) Antenna far field pattern in the vertical plane. The azimuth antenna

free space radiation pattern below the horizon must have a slope of at least

-8 dB/degree at the horizon and all sidelobes below the horizon must be at

least 13 dB below the pattern peak. The antenna radiation pattern above the

horizon must satisfy both the system coverage requirements and the spurious

radiation requirement.

(6) Data antenna. The data antenna must have horizontal and vertical

patterns as required for its function.

(g) Back azimuth coverage requirements. The back azimuth equipment where

used must provide guidance information in at least the following volume of

space (see Figure 11):

(1) Horizontally within a sector +/-40 degrees about the runway centerline

originating at the back azimuth ground equipment antenna and extending in the

direction of the missed approach at least to 20 nautical miles from the

runway stop end. The minimum proportional guidance sector must be +/-10

degrees about the runway centerline. Clearance signals must be used to

provide the balance of the required coverage where the proportional sector is

less than +/-40 degrees.

(2) Vertically in the runway region between:

(i) A horizontal surface 2.5 meters (8 feet) above the farthest point of

runway centerline which is in line of sight of the azimuth antenna, and,

(ii) A conical surface originating at the azimuth ground equipment antenna

inclined at 20 degrees above the horizontal up to a height of 600 meters

(2000 feet).

(3) Vertically in the back azimuth region between:

(i) A conical surface originating 2.5 meters (8 feet) above the runway stop

end, included at 0.9 degree above the horizontal, and,

(ii) A conical surface orginating at the missed approach azimuth ground

equipment antenna, inclined at 15 degrees above the horizontal up to a height

of 1500 meters (5000 feet).

(iii) Where obstacles penetrate the lower coverage limits, coverage need be

provided only to minimum line of sight.

(4) Within the back azimuth coverage sector defined in paragraph (q) (1),

(2), and (3) of this section the power densities must not be less than those

shown in Table 9, but the equipment design must also allow for:

(i) Transmitter power degradation from normal -1.5 dB.

(ii) Rain loss of -2.2 dB at the longitudinal coverage extremes.

(h) Back azimuth siting. The back azimuth equipment antenna must:

(1) Normally be located on the extension of the runway centerline at the

threshold end;

(2) Be adjusted so that the vertical plane containing the zero degree

course line contains the back azimuth reference datum;

(3) Have minimum height necessary to comply with the course requirements

prescribed in paragraph (g) of this section;

(4) Be located at a distance from the threshold end that is consistent with

safe obstruction clearance practices;

(5) Not obscure any light of an approach lighting system; and

(6) Be installed on frangible mounts or beyond the 300 meter (1000 feet)

light bar.

(i) Back azimuth antenna coordinates. The scanning beams transmitted by the

back azimuth equipment may be either conical or planar.

(j) Back azimuth accuracy. The requirements specified in Sec. 171.313(e)

apply except that the reference point is the back azimuth reference datum.

(k) Back azimuth antenna characteristics. The requirements specified in

Sec. 171.313(f) apply.

(l) Scanning conventions. Figure 12 shows the approach azimuth and back

azimuth scanning conventions.

(m) False guidance. False courses which can be acquired and tracked by an

aircraft shall not exist anywhere either inside or outside of the MLS

coverage sector. False courses which exist outside of the minimum coverage

sector may be suppressed by the use of OCI.

Note: False courses may be due to (but not limited to) MLS airborne

receiver acquisition of the following types of false guidance: reflections of

the scanning beam, scanning beam antenna sidelobes and grating lobes, and

incorrect clearance.

Sec. 171.315  Azimuth monitor system requirements.

(a) The approach azimuth or back azimuth monitor system must cause the

radiation to cease and a warning must be provided at the designated control

point if any of the following conditions persist for longer than the periods

specified:

[ ...Illustration appears here... ]

Figure 11. Back Azimuth/Data Coverage

[ ...Illustration appears here... ]

Figure 12. Azimuth Guidance Functions Scanning Conventions

[ ...Illustration appears here... ]

Figure 12. Azimuth Guidance Functions Scanning Conventions

[ ...Illustration appears here... ]

Figure 12. Azimuth Guidance Functions Scanning Conventions

[ ...Illustration appears here... ]

Figure 12. Azimuth Guidance Functions Scanning Conventions

(1) There is a change in the ground equipment contribution to the mean

course error component such that the path following error at the reference

datum or in the direction of any azimuth radial, exceeds the limits specified

in Secs. 171.313(e)(1) or 171.313(j) for a period of more than one second.

Note: The above requirement and the requirement to limit the ground

equipment mean error to +/-10 ft. can be satisfied by the following

procedure. The integral monitor alarm limit should be set to the angular

equivalent of +/-10 ft. at the approach reference datum. This will limit the

electrical component of the mean course error to +/-10 ft. The field monitor

alarm limit should be set such that with the mean course error at the alarm

limit the total allowed PFE is not exceeded on any commissioned approach

course from the limit of coverage to an altitude of 100 feet.

(2) There are errors in two consecutive transmissions of Basic Data Words

1, 2, 4 or 5.

(3) There is a reduction in the radiated power to a level not less than

that specified in Sec.171.313(a)(4) or Sec. 171.313(g)(4) for a period of

more than one second.

(4) There is an error in the preamble DPSK transmissions which occurs more

than once in any one second period.

(5) There is an error in the time division multiplex synchronization of a

particular azimuth function that the requirement specified in Sec. 171.311(e)

is not satisfied and if this condition persists for more than one second.

(6) A failure of the monitor is detected.

(b) Radiation of the following fuctions must cease and a warning provided

at the designated control point if there are errors in 2 consecutive

transmissions:

(1) Morse Code Identification,

(2) Basic Data Words 3 and 6,

(3) Auxiliary Data Words.

(c) The period during which erroneous guidance information is radiated must

not exceed the periods specified in Sec. 171.315(a). If the fault is not

cleared within the time allowed, the ground equipment must be shut down.

After shutdown, no attempt must be made to restore service until a period of

20 seconds has elapsed.

Sec. 171.317   Approach elevation performance requirements.

This section prescribes the performance requirements for the elevation

equipment components of the MLS as follows:

(a) Elevation coverage requirements. The approach elevation facility must

provide proportional guidance information in at least the following volume of

space (see Figure 13):

(1) Laterally within a sector originating at the datum point which is at

least equal to the proportional guidance sector provided by the approach

azimuth ground equipment.

(2) Longitudinally from 75 meters (250 feet) from the datum point to 20

nautical miles from threshold in the direction of the approach.

(3) Vertically within the sector bounded by:

(i) A surface which is the locus of points 2.5 meters (8 feet) above the

runway surface;

(ii) A conical surface originating at the datum point and inclined 0.9

degree above the horizontal and,

(iii) A conical surface originating at the datum point and inclined at 15.0

degrees above the horizontal up to a height of 6000 meters (20,000 feet).

[ ...Illustration appears here... ]

Figure 13. Approach Elevation Coverage

Where the physical characteristics of the approach region prevent

theachievement of the standards under paragraphs (a) (1), (2), and (3) of

this section, guidance need not be provided below a conical surface

originating at the elevation antenna and inclined 0.9 degree above the line

of sight.

(4) Within the elevation coverage sector defined in paragraphs (a) (1), (2)

and (3) of this section, the power densities must not be less than those

shown in Table 9, but the equipment design must also allow for:

(i) Transmitter power degradation from normal by -1.5 dB.

(ii) Rain loss of -2.2 dB at the coverage extremes.

(b) Elevation siting requirements. The Elevation Antenna System must:

(1) Be located as close to runway centerline as possible (without violating

obstacle clearance criteria).

(2) Be located near runway threshold such that the asymptote of the minimum

glidepath crosses the threshold of the runway at the Approach Reference Datum

height. Normally, the minimum glidepath should be 3 degrees and the Approach

Reference Datum height should be 50 feet. However, there are circumstances

where other glideslopes and reference datum heights are appropriate. Some of

these instances are discussed in FAA Order 8260.34 (Glide Slope Threshold

Crossing Height Requirements) and Order 8260.3 (IFR Approval of MLS.)

(3) Be located such that the MLS Approach Reference Datum and ILS Reference

Datum heights are coincident within a tolerance of 3 feet when MLS is

installed on a runway already served by an ILS. This requirement applies only

if the ILS glide slope is sited such that the height of the reference datum

meets the requirements of FAA Order 8260.34.

(c) Antenna coordinates. The scanning beams transmitted by the elevation

subsystem must be conical.

(d) Elevation accuracy. (1) The accuracies shown in Table 13 are required

at the approach reference datum. From the approach reference datum to the

coverage limit, the PFE, PFN and CMN limits shall be allowed to linearly

increase as follows:

(i) With distance along the runway centerline extended at the minimum glide

path angle, by a factor of 1.2 for the PFE and PFN limits and to +/-0.10

degree for the CMN limits;

(ii) With azimuth angle, from runway centerline extended to the coverage

extreme, by a factor of 1.2 for the PFE and PFN limits and by a factor of 2.0

for the CMN limits;

(iii) With increasing elevation angles from +3 degrees to +15 degrees, by a

factor of 2.0 for the PFE and PFN limits;

Table 13--Elevation Accuracies at the

Approach Reference Datum

Angular error

(degrees)

Airborne

Error                     Ground    subsystem

type        System       subsystem     /4/

PFE    /1/ /2/ +/-0.133      (/3/)   +/-0.017

CMN         /1/+/-0.050   +/-0.020   +/-0.010

Notes:

/1/ Includes errors due to ground and

airborne equipment and propagation effects.

/2/ The system PFN component must not exceed

+/-0.087 degree.

/3/ The mean (bias) error component

contributed by the ground equipment should

not exceed +/-0.067 degree.

/4/ The airborne subsystem angular errors are

provided for information only.

(iv) With decreasing elevation angle from +3 degrees (or 60% of the minimum

glide path angle, whichever is less) to the coverage extreme, by a factor of

3 for the PFE, PFN and CMN limits; and

(v) Maximum angular limits. the CMN limits shall not exceed +/-0.10 degree

in any coverage region within +/-10 degrees laterally of runway centerline

extended which is above the elevation angle specified in (iv) above.

Note: It is desirable that the CMN not exceed +/-0.10 degree throughout the

coverage region above the elevation angle specified in paragraph (d)(1)(iv)

of this section.

(2) The system and ground subsystem accuracies shown in Table 13 are to be

demonstrated at commissioning as maximum error limits. Subsequent to

commissioning, the accuracies are to be considered at 95% probability limits.

(e) Elevation antenna characteristics are as follows:

(1) Drift. Any elevation angle as encoded by the scanning beam at any point

within the coverage sector must not vary more than 0.04 degree over the range

of service conditions specified in Sec. 171.309(d) without the use of

internal environmental controls. Multipath effects are excluded from this

requirement.

(2) Beam pointing errors. The elevation angle as encoded by the scanning

beam at any point within the coverage sector must not deviate from the true

elevation angle at that point by more than +/-0.04 degree for elevation

angles from 2.5  deg. to 3.5  deg.. Above 3.5  deg. these errors may linearly

increase to +/-0.1 degree at 7.5  deg.. Multipath and drift effects are

excluded from this requirement.

(3) Antenna alignment. The antenna must be equipped with suitable optical,

electrical, or mechanical means or any combination of the three, to align the

lowest operationally required glidepath to the true glidepath angle with a

maximum error of 0.01 degree. Additionally, the elevation antenna bias

adjustment must be electronically steerable at least to the monitor limits in

steps not greater than 0.005 degrees.

(4) Antenna far field patterns in the plane of scan. On the lowest

operationally required glidepath, the antenna mainlobe pattern must conform

to Figure 10, and the beamwidth must be such that in the installed

environment, no significant ground reflections of the mainlobe exist. In any

case, the beamwidth must not exceed 2 degrees. The antenna mainlobe may be

allowed to broaden from the value at boresight by a factor of 1/cosu, where u

is the angle of boresight. Anywhere within coverage, the -3 dB width of the

antenna mainlobe, while scanning normally, must not be less than 25

microseconds (0.5 degrees) or greater than 250 microseconds (5 degrees). The

sidelobe levels must be as follows:

(i) Dynamic sidelobe levels. With the antenna scanning normally, the

dynamic sidelobe level that is detected by a receiver at any point within the

proportional coverage sector must be down at least 10 dB from the peak of the

mainlobe. Outside the proportional coverage sector, the radiation from the

scanning beam antenna must be of such a nature that receiver warnings will

not be removed or a suitable OCI signal must be provided.

(ii) Effective sidelobe levels. With the antenna scanning normally, the

sidelobe levels in the plane of scan must be such that, when reflected from

the ground, the resultant PFE along any glidepath does not exceed 0.083

degrees.

(5) Antenna far field pattern in the horizontal plane. The horizontal

pattern of the antenna must gradually de-emphasize the signal away from

antenna boresight. Typically, the horizontal pattern should be reduced by at

least 3 dB at 20 degrees off boresight and by at least 6 dB at 40 degrees off

boresight. Depending on the actual multipath conditions, the horizontal

radiation patterns may require more or less de-emphasis.

(6) Data antenna. The data antenna must have horizontal and vertical

patterns as required for its function.

(f) False guidance. False courses which can be acquired and tracked by an

aircraft shall not exist anywhere either inside or outside of the MLS

coverage sector. False courses which exist outside of the minimum coverage

sector may be suppressed by the use of OCI.

Note: False courses may be due to (but not limited to) MLS airborne

receiver acquisition of the following types of false guidance: reflections of

the scanning beam and scanning beam antenna sidelobes and grating lobes.

Sec. 171.319  Approach elevation monitor system requirements.

(a) The monitor system must act to ensure that any of the following

conditions do not persist for longer than the periods specified when:

(1) There is a change in the ground component contribution to the mean

glidepath error component such that the path following error on any glidepath

exceeds the limits specified in Sec. 171.317(d) for a period of more than one

second.

Note: The above requirement and the requirement to limit the ground

equipment mean error to +/-0.067 degree can be satisfied by the following

procedure. The integral monitor alarm limit should be set to +/-0.067 degree.

This will limit the electrical component of mean glidepath error to +/-0.067

degree. The field monitor alarm limit should be set such that with the mean

glidepath error at the alarm limit the total allowed PFE is not exceeded on

any commissioned glidepath from the limit of coverage to an altitude of 100

feet.

(2) There is a reduction in the radiated power to a level not less than

that specified in Sec. 171.317(a)(4) for a period of more than one second.

(3) There is an error in the preamble DPSK transmission which occurs more

than once in any one second period.

(4) There is an error in the time division multiplex synchronization of a

particular elevation function such that the requirement specified in Sec.

171.311(e) is not satisfied and this condition persists for more than one

second.

(5) A failure of the monitor is detected.

(b) The period during which erroneous guidance information is radiated must

not exceed the periods specified in Sec. 171.319(a). If the fault is not

cleared within the time allowed, radiation shall cease. After shutdown, no

attempt must be made to restore service until a period of 20 seconds has

elapsed.

Sec. 171.321  DME and marker beacon performance requirements.

(a) The DME equipment must meet the performance requirements prescribed in

Subpart G of the part. This subpart imposes requirements that performance

features must comply with International Standards and Recommended Practices,

Aeronautical Telecommunications, Vol. I of Annex 10 to ICAO. It is available

from ICAO, Aviation Building, 1080 University Street, Montreal 101, Quebec,

Canada, Attention: Distribution Officer and also available for inspection at

the Office of the Federal Register Information Center, Room 8301, 1100 L

Street, NW., Washington, DC 20408.

(b) MLS marker beacon equipment must meet the performance requirements

prescribed in Subpart H of this Part. This subpart imposes requirements that

performance features must comply with International Standards and Recommended

Practices, Aeronautical Telecommuncations, Vol. I of Annex 10 to ICAO.

Sec. 171.323  Fabrication and installation requirements.

(a) The MLS facility must be permanent and must be located, constructed,

and installed in accordance with best commercial engineering practices, using

applicable electric and safety codes and Federal Communications Commission

(FCC) licensing requirements and siting requirements of Secs. 171.313(b) and

171.317(b).

(b) The MLS facility components must utilize solid state technology except

that traveling wave tube amplifiers (TWTA) may be used. A maximum level of

common modularity must be provided along with diagnostics to facilitate

maintenance and troubleshooting.

(c) An approved monitoring capability must be provided which indicates the

status of the equipment at the site and at a remotely located maintenance

area, with monitor capability that provides pre-alarm of impending system

failures. This monitoring feature must be capable of transmitting the status

and pre-alarm over standard phone lines to a remote section. In the event the

sponsor requests the FAA to assume ownership of the facility, the monitoring

feature must also be capable of interfacing with FAA remote monitoring

requirements. This requirement may be complied with by the addition of

optional software and/or hardware in space provided in the original

equipment.

(d) The mean corrective maintenance time of the MLS equipment must be equal

to or less than 0.5 hours with a maximum corrective maintenance time not to

exceed 1.5 hours. This measure applies to correction of unscheduled failures

of the monitor, transmitter and associated antenna assemblies, limited to

unscheduled outage and out of tolerance conditions.

(e) The mean-time-between-failures of the MLS angle system must not be less

than 1,500 hours. This measure applies to unscheduled outage, out-of-

tolerance conditions, and failures of the monitor, transmitter, and

associated antenna assemblies.

(f) The MLS facility must have a reliable source of suitable primary power,

either from a power distribution system or locally generated. Adequate power

capacity must be provided for the operation of the MLS as well as the test

and working equipment of the MLS.

(g) The MLS facility must have a continuously engaged or floating battery

power source for the continued normal operation of the ground station

operation if the primary power fails. A trickle charge must be supplied to

recharge the batteries during the period of available primary power. Upon

loss and subsequent restoration of power, the battery must be restored to

full charge within 24 hours. When primary power is applied, the state of the

battery charge must not affect the operation of the MLS ground station. The

battery must allow continuation of normal operation of the MLS facility for

at least 2 hours without the use of additional sources of power. When the

system is operating from the battery supply without prime power, the radome

deicers and the environmental system need not operate. The equipment must

meet all specification requirements with or without batteries installed.

(h) There must be a means for determining, from the ground, the performance

of the system including antenna, both initially and periodically.

(i) The facility must have, or be supplemented by ground, air or landline

communications services. At facilities within or immediately adjacent to air

traffic control areas, that are intended for use as instrument approach aids

for an airport, there must be ground air communications or reliable

communications (at least a landline telephone) from the airport to the

nearest FAA air traffic control or communication facility. Compliance with

this paragraph need not be shown at airports where an adjacent FAA facility

can communicate with aircraft on the ground at the airport and during the

entire proposed instrument approach procedure. In addition, at low traffic

density airports within or immediately adjacent to air traffic control zones

or areas, and where extensive delays are not a factor, the requirements of

this paragraph may be reduced to reliable communications from the airport to

the nearest FAA air traffic control or communications facility. If the

adjacent FAA facility can communicate with aircraft during the proposed

instrument approach procedure down to the airport surface or at least down to

the minimum approach altitude, this would require at least a landline

telephone.

(j) The location of the phase center for all antennas must be clearly

marked on the antenna enclosures.

(k) The latitude, longitude and mean sea level elevation of all MLS

antennas, runway threshold and runway stop end must be determined by survey

with an accuracy of +/-3 meters (+/-10 feet) laterally and +/-0.3 meter (+/-

1.0 foot) vertically. The relative lateral and vertical offsets of all

antenna phase centers, and both runway ends must be determined with an

accuracy of +/-0.3 meter (+/-1.0 foot) laterally and +/-0.03 meter (+/-0.1

foot) vertically. The owner must bear all costs of the survey. The results of

this survey must be included in the "operations and maintenance" manual

required by section 171.325 of this subpart and will be noted on FAA Form 198

required by Sec. 171.327.

EFFECTIVE DATE NOTE: Amdt. 171-16, 56 FR 65665, Dec. 17, 1991, revised

paragraph (i) of Sec. 171.323 effective September 16, 1993. For the

convenience of the user, the revised text is set forth as follows:

Sec. 171.323   Fabrication and installation requirements.

* * * * *

(i) The facility must have, or be supplemented by, ground, air, or landline

communications services. At facilities within or immediately adjacent to

controlled airspace, that are intended for use as instrument approach aids

for an airport, there must be ground air communications or reliable

communications (at least a landline telephone) from the airport to the

nearest FAA air traffic control or communication facility. Compliance with

this paragraph need not be shown at airports where an adjacent FAA facility

can communicate with aircraft on the ground at the airport and during the

entire proposed instrument approach procedure. In addition, at low traffic

density airports within or immediately adjacent to controlled airspace, and

where extensive delays are not a factor, the requirements of this paragraph

may be reduced to reliable communications from the airport to the nearest FAA

air traffic control or communications facility. If the adjacent FAA facility

can communicate with aircraft during the proposed instrument approach

procedure down to the airport surface or at least down to the minimum en

route altitude, this would require at least a landline telephone.

Sec. 171.325   Maintenance and operations requirements.

(a) The owner of the facility must establish an adequate maintenance system

and provide MLS qualified maintenance personnel to maintain the facility at

the level attained at the time it was commissioned. Each person who maintains

a facility must meet the FCC licensing requirements and demonstrate that he

has the special knowledge and skills needed to maintain an MLS facility,

including proficiency in maintenance procedures and the use of specialized

test equipment.

(b) In the event of out-of-tolerance conditions or malfunctions, as

evidenced by receiving two successive pilot reports, the owner must close the

facility by encasing radiation, and issue a "Notice to Airmen" (NOTAM) that

the facility is out of service.

(c) The owner must prepare, and obtain approval of, an operations and

maintenance manual that sets forth mandatory procedures for operations,

periodic maintenance, and emergency maintenance, including instructions on

each of the following:

(1) Physical security of the facility.

(2) Maintenance and operations by authorized persons.

(3) FCC licensing requirements for operations and maintenance personnel.

(4) Posting of licenses and signs.

(5) Relations between the facility and FAA air traffic control facilities,

with a description of the boundaries of controlled airspace over or near the

facility, instructions for relaying air traffic control instructions and

information, if applicable, and instructions for the operation of an air

traffic advisory service if the facility is located outside of controlled

airspace.

(6) Notice to the Administrator of any suspension of service.

(7) Detailed and specific maintenance procedures and servicing guides

stating the frequency of servicing.

(8) Air-ground communications, if provided, expressly written or

incorporating appropriate sections of FAA manuals by reference.

(9) Keeping the station logs and other technical reports, and the

submission of reports required by Sec. 171.327.

(10) Monitoring of the MLS facility.

(11) Inspections by United States personnel.

(12) Names, addresses, and telephone numbers of persons to be notified in

an emergency.

(13) Shutdowns for periodic maintenance and issuing of NOTAM for routine or

emergency shutdowns.

(14) Commissioning of the MLS facility.

(15) An acceptable procedure for amending or revising the manual.

(16) An explanation of the kinds of activities (such as construction or

grading) in the vicinity of the MLS facility that may require shutdown or

recertification of the MLS facility by FAA flight check.

(17) Procedures for conducting a ground check of the azimuth and elevation

alignment.

(18) The following information concerning the MLS facility:

(i) Facility component locations with respect to airport layout, instrument

runways, and similar areas.

(ii) The type, make and model of the basic radio equipment that provides

the service including required test equipment.

(iii) The station power emission, channel, and frequency of the azimuth,

elevation, DME, marker beacon, and associated compass locators, if any.

(iv) The hours of operation.

(v) Station identification call letters and method of station

identification and the time spacing of the identification.

(vi) A description of the critical parts that may not be changed, adjusted,

or repaired without an FAA flight check to confirm published operations.

(d) The owner or his maintenance representative must make a ground check of

the MLS facility periodically in accordance with procedures approved by the

FAA at the time of commissioning, and must report the results of the checks

as provided in Sec. 171.327.

(e) The only modifications permitted are those that are submitted to FAA

for approval by the MLS equipment manufacturer. The owner or sponsor of the

facility must incorporate these modifications in the MLS equipment.

Associated changes must also be made to the operations and maintenance manual

required in paragraph (c) of this section. This and all other corrections and

additions to this operations and maintenance manual must also be submitted to

FAA for approval.

(f) The owner or the owner's maintenance representative must participate in

inspections made by the FAA.

(g) The owner must ensure the availability of a sufficient stock of spare

parts, including solid state components, or modules to make possible the

prompt replacement of components or modules that fail or deteriorate in

service.

(h) FAA approved test instruments must be used for maintenance of the MLS

facility.

(i) Inspection consists of an examination of the MLS equipment to ensure

that unsafe operating conditions do not exist.

(j) Monitoring of the MLS radiated signal must ensure a high degree of

integrity and minimize the requirements for ground and flight inspection. The

monitor must be checked daily during the in-service test evaluation period

(96 hour burn in) for calibration and stability. These tests and ground

checks or azimuth, elevation, DME, and marker beacon radiation

characteristics must be conducted in accordance with the maintenance

requirements of this section.

Sec. 171.327  Operational records.

The owner of the MLS facility or his maintenance representative must submit

the following operational records at the indicated time to the appropriate

FAA regional office where the facility is located.

(a) Facility Equipment Performance & Adjustment Data (FAA Form 198). The

FAA Form 198 shall be filled out by the owner or his maintenance

representative with the equipment adjustments and meter readings as of the

time of facility commissioning. One copy must be kept in the permanent

records of the facility and two copies must be sent to the appropriate FAA

regional office. The owner or his maintenance representative must revise the

FAA Form 198 data after any major repair, modernization, or retuning to

reflect an accurate record of facility operation and adjustment.

(b) Facility Maintenance Log (FAA Form 6030-1). FAA Form 6030-1 is

permanent record of all the activities required to maintain the MLS facility.

The entries must include all malfunctions met in maintaining the facility

including information on the kind of work and adjustments made, equipment

failures, causes (if determined) and corrective action taken. In addition,

the entries must include completion of periodic maintenance required to

maintain the facility. The owner or his maintenance representative must keep

the original of each form at the facility and send a copy to the appropriate

FAA regional office at the end of each month in which it is prepared.

However, where an FAA approved remote monitoring system is installed which

precludes the need for periodic maintenance visits to the facility, monthly

reports from the remote monitoring system control point must be forwarded to

the appropriate FAA regional office, and a hard copy retained at the control

point.

(c) Technical Performance Record (FAA Form 6830 (formerly FAA Form 418)).

This form contains a record of system parameters as specified in the

manufacturer's equipment manual. This data will be recorded on each scheduled

visit to the facility. The owner or his maintenance representative shall keep

the original of each record at the facility and send a copy of the form to

the appropriate FAA regional office.