PART 21--CERTIFICATION PROCEDURES FOR PRODUCTS AND PARTS

Special Federal Aviation Regulations

SFAR No. 26

Special Federal Aviation Regulation No. 27-5 [Removed. 55 FR

32859, Aug. 10, 1990]

SFAR No. 27-5 [Removed. 55 FR 32860, Aug. 10, 1990]

SFAR No. 29-4

SFAR No. 41

Subpart A--General

Sec. 21.1 Applicability.

Sec. 21.2 Falsification of applications, reports, or records.

Sec. 21.3 Reporting of failures, malfunctions, and defects.

Sec. 21.5 Airplane or Rotorcraft Flight Manual.

Subpart B--Type Certificates

Sec. 21.11 Applicability.

Sec. 21.13 Eligibility.

Sec. 21.15 Application for type certificate.

Sec. 21.16 Special conditions.

Sec. 21.17 Designation of applicable regulations.

Sec. 21.19 Changes requiring a new type certificate.

Sec. 21.21 Issue of type certificate: normal, utility,

acrobatic, commuter, and transport category aircraft; manned

free balloons; special classes of aircraft; aircraft

engines; propellers.

Sec. 21.23 [Reserved]

Sec. 21.24 Issuance of type certificate: primary category

aircraft.

Sec. 21.25 Issue of type certificate: Restricted category

aircraft.

Sec. 21.27 Issue of type certificate: surplus aircraft of the

Armed Forces.

Sec. 21.29 Issue of type certificate: import products.

Sec. 21.31 Type design.

Sec. 21.33 Inspection and tests.

Sec. 21.35 Flight tests.

Sec. 21.37 Flight test pilot.

Sec. 21.39 Flight test instrument calibration and correction

report.

Sec. 21.41 Type certificate.

Sec. 21.43 Location of manufacturing facilities.

Sec. 21.45 Privileges.

Sec. 21.47 Transferability.

Sec. 21.49 Availability.

Sec. 21.50 Instructions for continued airworthiness and

manufacturer's maintenance manuals having airworthiness

limitations sections.

Sec. 21.51 Duration.

Sec. 21.53 Statement of conformity.

Subpart C--Provisional Type Certificates

Sec. 21.71 Applicability.

Sec. 21.73 Eligibility.

Sec. 21.75 Application.

Sec. 21.77 Duration.

Sec. 21.79 Transferability.

Sec. 21.81 Requirements for issue and amendment of Class I

provisional type certificates.

Sec. 21.83 Requirements for issue and amendment of Class II

provisional type certificates.

Sec. 21.85 Provisional amendments to type certificates.

Subpart D--Changes to Type Certificates

Sec. 21.91 Applicability.

Sec. 21.93 Classification of changes in type design.

Sec. 21.95 Approval of minor changes in type design.

Sec. 21.97 Approval of major changes in type design.

Sec. 21.99 Required design changes.

Sec. 21.101 Designation of applicable regulations.

Subpart E--Supplemental Type Certificates

Sec. 21.111 Applicability.

Sec. 21.113 Requirement of supplemental type certificate.

Sec. 21.115 Applicable requirements.

Sec. 21.117 Issue of supplemental type certificates.

Sec. 21.119 Privileges.

Subpart F--Production Under Type Certificate Only

Sec. 21.121 Applicability.

Sec. 21.123 Production under type certificate.

Sec. 21.125 Production inspection system: Materials Review

Board.

Sec. 21.127 Tests: aircraft.

Sec. 21.128 Tests: aircraft engines.

Sec. 21.129 Tests: propellers.

Sec. 21.130 Statement of conformity.

Subpart G--Production Certificates

Sec. 21.131 Applicability.

Sec. 21.133 Eligibility.

Sec. 21.135 Requirements for issuance.

Sec. 21.137 Location of manufacturing facilities.

Sec. 21.139 Quality control.

Sec. 21.143 Quality control data requirements; prime

manufacturer.

Sec. 21.147 Changes in quality control system.

Sec. 21.149 Multiple products.

Sec. 21.151 Production limitation record.

Sec. 21.153 Amendment of the production certificates.

Sec. 21.155 Transferability.

Sec. 21.157 Inspections and tests.

Sec. 21.159 Duration.

Sec. 21.161 Display.

Sec. 21.163 Privileges.

Sec. 21.165 Responsibility of holder.

Subpart H -- Airworthiness Certificates

Sec. 21.171 Applicability.

Sec. 21.173 Eligibility.

Sec. 21.175 Airworthiness certificates: classification.

Sec. 21.177 Amendment or modification.

Sec. 21.179 Transferability.

Sec. 21.181 Duration.

Sec. 21.182 Aircraft identification.

Sec. 21.183 Issue of standard airworthiness certificates for

normal, utility, acrobatic, commuter, and transport category

aircraft; manned free balloons; and special classes of

aircraft.

Sec. 21.184 Issue of special airworthiness certificates for

primary category aircraft.

Sec. 21.185 Issue of airworthiness certificates for restricted

category aircraft.

Sec. 21.187 Issue of multiple airworthiness certification.

Sec. 21.189 Issue of airworthiness certificate for limited

category aircraft.

Sec. 21.191 Experimental certificates.

Sec. 21.193 Experimental certificates: general.

Sec. 21.195 Experimental certificates: Aircraft to be used for

market surveys, sales demonstrations, and customer crew

training.

Sec. 21.197 Special flight permits.

Sec. 21.199 Issue of special flight permits.

Subpart I--Provisional Airworthiness Certificates

Sec. 21.211 Applicability.

Sec. 21.213 Eligibility.

Sec. 21.215 Application.

Sec. 21.217 Duration.

Sec. 21.219 Transferability.

Sec. 21.221 Class I provisional airworthiness certificates.

Sec. 21.223 Class II provisional airworthiness certificates.

Sec. 21.225 Provisional airworthiness certificates

corresponding with provisional amendments to type

certificates.

Subpart J--Delegation Option Authorization Procedures

Sec. 21.231 Applicability.

Sec. 21.235 Application.

Sec. 21.239 Eligibility.

Sec. 21.243 Duration.

Sec. 21.245 Maintenance of eligibility.

Sec. 21.247 Transferability.

Sec. 21.249 Inspections.

Sec. 21.251 Limits of applicability.

Sec. 21.253 Type certificates: application.

Sec. 21.257 Type certificates: issue.

Sec. 21.261 Equivalent safety provisions.

Sec. 21.267 Production certificates.

Sec. 21.269 Export airworthiness approvals.

Sec. 21.271 Airworthiness approval tags.

Sec. 21.273 Airworthiness certificates other than experimental.

Sec. 21.275 Experimental certificates.

Sec. 21.277 Data review and service experience.

Sec. 21.289 Major repairs, rebuilding and alteration.

Sec. 21.293 Current records.

Subpart K--Approval of Materials, Parts, Processes, and Appliances

Sec. 21.301 Applicability.

Sec. 21.303 Replacement and modification parts.

Sec. 21.305 Approval of materials, parts, processes, and

appliances.

Subpart L--Export Airworthiness Approvals

Sec. 21.321 Applicability.

Sec. 21.323 Eligibility.

Sec. 21.325 Export airworthiness approvals.

Sec. 21.327 Application.

Sec. 21.329 Issue of export certificates of airworthiness for

Class I products.

Sec. 21.331 Issue of airworthiness approval tags for Class II

products.

Sec. 21.333 Issue of export airworthiness approval tags for

Class III products.

Sec. 21.335 Responsibilities of exporters.

Sec. 21.337 Performance of inspections and overhauls.

Sec. 21.339 Special export airworthiness approval for aircraft.

Subpart M--Designated Alteration Station Authorization Procedures

Sec. 21.431 Applicability.

Sec. 21.435 Application.

Sec. 21.439 Eligibility.

Sec. 21.441 Procedure manual.

Sec. 21.443 Duration.

Sec. 21.445 Maintenance of eligibility.

Sec. 21.447 Transferability.

Sec. 21.449 Inspections.

Sec. 21.451 Limits of applicability.

Sec. 21.461 Equivalent safety provisions.

Sec. 21.463 Supplemental type certificates.

Sec. 21.473 Airworthiness certificates other than experimental.

Sec. 21.475 Experimental certificates.

Sec. 21.477 Data review and service experience.

Sec. 21.493 Current records.

Subpart N--Approval of Engines, Propellers, Materials, Parts, and

Appliances: Import

Sec. 21.500 Approval of engines and propellers.

Sec. 21.502 Approval of materials, parts, and appliances.

Subpart O--Technical Standard Order Authorizations

Sec. 21.601 Applicability.

Sec. 21.603 TSO marking and privileges.

Sec. 21.605 Application and issue.

Sec. 21.607 General rules governing holders of TSO

authorizations.

Sec. 21.609 Approval for deviation.

Sec. 21.611 Design changes.

Sec. 21.613 Recordkeeping requirements.

Sec. 21.615 FAA inspection.

Sec. 21.617 Issue of letters of TSO design approval: import

appliances.

Sec. 21.619 Noncompliance.

Sec. 21.621 Transferability and duration.

SFAR No. 26

Contrary provisions of Secs. 21.29, 21.500, and 21.502 of the Federal

Aviation Regulations notwithstanding--

1. A type certificate may be issued under Sec. 21.29 for an aircraft

engine or propeller manufactured in a foreign country with which the United

States has a currently effective bilateral agreement for the acceptance of

powered aircraft for export and import and that is to be imported into the

United States if (i) a type certificate has previously been issued by the

Administrator for a product manufactured in that country which is of the same

kind as the product for which a type certificate is requested; and (ii) the

Administrator determines that the design standards and practices, the quality

control standards, and the certification procedures utilized by such country

for the particular product being imported are the equivalent of those

required in the United States.

2. Aircraft engines, propellers, materials, parts (including

subassemblies), or appliances (hereinafter referred to as aircraft

components), manufactured in a foreign country with which the United States

has a currently effective bilateral agreement for the acceptance of powered

aircraft may be approved under Sec. 21.500 or Sec. 21.502, as applicable, if

(i) an approval has previously been issued by the Administrator for an

aircraft component manufactured in that country which is of the same kind as

the aircraft component for which approval is requested; and (ii) the

Administrator determines that the quality control standards and the

certification and approval procedures utilized by such country for the

particular aircraft component being imported are the equivalent of those

required in the United States.

3. Aircraft subassemblies not covered under paragraph 2 that are to be

incorporated on aircraft designed and manufactured in the United States, and

that are manufactured in a foreign country with which the United States has a

currently effective bilateral agreement for the acceptance of powered

aircraft may be approved if (i) an approval has previously been issued by the

Administrator for any other subassembly manufactured in that country; (ii)

the Administrator determines that the quality control standards and the

certification and approval procedures utilized by that country for the

particular subassembly being imported are equivalent to those required in the

United States; and (iii) the competent aeronautical authorities of the

country certify that the subassembly meets the applicable design

requirements.

4. Appropriate procedures for the execution of the provisions contained in

paragraphs 1, 2, and 3 of the regulation may be embodied in agreements

between the Administrator and the competent aeronautical authorities of the

country of manufacture of the aircraft component.

5. In the event that the Administrator determines that the quality control

standards and certification and approval procedures being utilized in the

foreign countries to which this regulation is applicable no longer meet the

quality control and certification and approval requirements equivalent to

those required in the United States, the approval given under this regulation

for the import into the United States of those aeronautical products covered

by the regulation will be terminated.

6. After October 1, 1977, this special regulation applies only to aircraft

components and subassemblies produced in Japan pursuant to contracts between

Japanese manufacturers and United States product manufacturers entered into

prior to October 1, 1977.

This special regulation shall terminate December 1, 1977, unless sooner

rescinded or superseded.

[35 FR 12749, Aug. 12, 1970, as amended at 40 FR 28604, July 8, 1975; 41 FR

27954, July 8, 1976; Doc. No. 10492, 42 FR 35634, July 11, 1977; Amdt. 26-11,

42 FR 54408, Oct. 6, 1977]

Special Federal Aviation Regulation No. 27-5  [Removed.  55 FR 32859,

Aug. 10, 1990]

EDITORIAL NOTE: For the convenience of the user, the removed text is

set out below.

Special Federal Aviation Regulation No. 27-5--Fuel Venting and Exhaust

Emission Requirements for Turbine Engine Powered Airplanes

Sec. 1  General Requirements.

(a) This SFAR provides for the approval or acceptance by the Administrator

of the FAA or the Administrator of the EPA of testing and sampling methods,

analytical techniques, and related equipment not identical to those specified

in this part. Before either approves or accepts any such alternate,

equivalent, or otherwise nonidentical procedures or equipment, the

Administrator of the FAA or the Administrator of the EPA shall consult with

the other in determining whether or not the action requires rulemaking under

sections 231 and 232 of the Clean Air Act, as amended, consistent with the

responsibilities of the Administrator of the EPA and the Secretary of

Transportation under sections 231 and 232 of the Act. (42 U.S.C. 7571, 7572).

(b) U.S. airplanes. This SFAR applies to civil airplanes that are powered

by aircraft gas turbine engines of the classes specified herein and that have

U.S. Standard Airworthiness Certificates.

(c) Foreign airplanes. Pursuant to the definition of "aircraft" in 40 CFR

87.1(a), this SFAR applies to civil airplanes that are powered by aircraft

gas turbine engines of the classes specified herein and that have foreign

airworthiness certificates that are equivalent to U.S. Standard Airworthiness

Certificates. This includes only those foreign civil airplanes that, if

registered in the United States, would be required by applicable Federal

Aviation Regulations to have a U.S. standard airworthiness certificate in

order to conduct the operations intended for the airplane.

Pursuant to 40 CFR 87.3, this SFAR does not apply where inconsistent with

an obligation assumed by the United States to a foreign country in a treaty,

convention, or agreement.

Sec. 3  Relation to 40 CFR Part 87.

(a) Reference in this regulation to 40 CFR Part 87 refers to Title 40 of

the Code of Federal Regulations, Chapter I--Environmental Protection Agency,

Part 87, Control of Air Pollution from Aircraft and Aircraft Engines (40 CFR

Part 87), originally issued on July 6, 1973, and published in the Federal

Register (38 FR 19088) on July 17, 1973, as revised effective January 31,

1983 (47 FR 58462), as amended effective January 20, 1983 (48 FR 2716).

(b) This SFAR contains regulations to ensure compliance with certain

standards in Environmental Protection Agency (EPA), 40 CFR Part 87. If EPA

takes any action, including the issuance of an exemption or issuance of a

revised or alternate procedure, test method, or other regulation, the effect

of which is to relax, or delay the effective date of any provision of 40 CFR

Part 87 that is made applicable to an aircraft under this SFAR, the new

relaxed EPA requirement, upon its effective date, is incorporated into this

SFAR and supersedes the provisions of this SFAR that are based on the

provisions of 40 CFR Part 87 that were relaxed by such action.

(c) Unless otherwise stated, all terminology and abbreviations in this SFAR

that are defined in 40 CFR Part 87 have the meaning specified in that part,

and all terms in 40 CFR Part 87 that are not defined in that Part but that

are used in this SFAR have the meaning given them in the Clean Air Act, as

amended by 42 USC Sec. 7401.

(d) All interpretations of 40 CFR Part 87 that are rendered by EPA also

apply to this SFAR.

(e) Consistent with subsection (c) of this section, the following

definitions apply:

(1) "Administrator of the FAA" means the Administrator of the Federal

Aviation Administration or any other officer or employee of the Federal

Aviation Administration to whom the authority involved may be delegated.

(2) "Administrator of the EPA" means the Administrator of the Environmental

Protection Agency and any other officer or employee of the Environmental

Protection Agency to whom the authority involved may be delegated.

Sec. 5  Additional EPA approvals and procedures.

(a) If EPA, under 40 CFR 87.3(a) approves or accepts any testing and

sampling procedures or methods, analytical techniques, and related equipment

not identical to those specified in EPA Part 87, this SFAR requires a showing

that such alternate, equivalent, or otherwise nonidentical procedures have

been complied with, and that such alternate equipment was used to show

compliance, unless the applicant elects to comply fully with 40 CFR Part 87.

(b) If the Administrator of the EPA after consultation with the

Administrator of the FAA, prescribes special test procedures for any aircraft

or aircraft engine that is not susceptible to satisfactory testing by the

procedures in 40 CFR Part 87, this SFAR requires a showing that those special

test procedures have been complied with.

(c) Wherever 40 CFR Part 87 requires agreement, acceptance, or approval by

the Administrator of EPA, this SFAR requires a showing that such agreement or

approval has been obtained.

Sec. 7  Relation to State and local regulations.

(a) Pursuant to 42 U.S.C. Sec. 7573, no state or political subdivision

thereof may adopt or attempt to enforce any standard respecting emissions of

any air pollutant from any aircraft or engine thereof unless that standard is

identical to a standard made applicable to the aircraft by the terms of this

SFAR.

(b) If EPA, by regulation or exemption, relaxes a provision of 40 CFR Part

87, no state or political subdivision thereof may adopt or attempt to enforce

the terms of this SFAR that are superseded by the relaxed requirement.

(c) Consistent with Sec. 87.6 of 40 CFR Part 87, if the Administrator of

the FAA determines that any emission control regulation cannot be safely

applied to an aircraft, that provision may not be adopted or enforced against

that aircraft by a state or political subdivision thereof, even if it is in

this SFAR.

(d) If any provision of this SFAR is rendered inapplicable to a foreign

aircraft as provided in 40 CFR 87.3(c) and section 1(b) of this SFAR, that

provision may not be adopted or enforced against that foreign aircraft by a

state or political subdivision thereof.

Sec. 9  Petitions for rulemaking and exemption.

(a) Notwithstanding Part 11 of the Federal Aviation Regulations (14 CFR

Part 11), all petitions for rulemaking involving either the substance of an

emission standard prescribed by EPA that is incorporated in this SFAR, or the

compliance date for such standard or procedure, must be submitted to EPA.

Information copies of such petitions are invited by the FAA.

(b) Petitions for rulemaking or exemption involving provisions of this SFAR

that do not effect the substance or the compliance date of an emission

standard or test procedure that is prescribed by EPA, and petitions for

exemptions from the regulations in this SFAR issued under the authority

section 232 of the Clean Air Act by which the Administrator, by virtue of the

delegation from the Secretary of Transportation in 49 CFR section 1.47(g),

and after consultation with the Administrator of the EPA, prescribes

regulations to insure compliance with the EPA's regulations prescribed in 40

CFR 87.7(a)(4), (b), (c) and (d) (issued under the authority of Sec. 231 of

the Clean Air Act) are subject to Part 11 of the Federal Aviation Regulations

(14 CFR Part 11).

(c) Any engine for which a petition for exemption has been granted pursuant

to subsection (b) of this section must be reported to the FAA by serial

number and type certificate number on or before the last day of the month

following the date of manufacture as defined in section 12 of this SFAR.

(d) Notwithstanding any other requirement of 14 CFR Sec. 45.13, an aircraft

engine specifically exempted from the applicable requirements of this SFAR

pursuant to subsection (b) of this section must have the designator EXEMPT,

or an equivalent designator approved by the Administrator of the FAA,

inscribed as part of the identification data of the engine.

(e) Exemptions granted pursuant to subsection (b) of this section are not

transferable to any other engine.

(f) Requests for flights conducted under the authority of the exemption

provisions of 40 CFR 87.7(a)(2) must comply with the procedural requirements

set forth in 14 CFR Sec. 21.199.

Sec. 10  Incorporation by reference.

(a) General. This SFAR prescribes certain standards and procedures which

are not set forth in full text in the rule. On November 22, 1983, those

standards and procedures were approved by the Office of the Federal Register

for incorporation by reference pursuant to 5 U.S.C. 552(a) and 1 CFR Part 51.

(b) Changes to incorporated matter. Incorporated matter which is subject to

subsequent change is incorporated by reference according to the specific

reference and to the identification statement. Adoption of any subsequent

change in incorporated matter that impacts compliance with standards and

procedures is made under 14 CFR Part 11 and 1 CFR Part 51.

(c) Identification statement. The complete title or description which

identifies each published matter incorporated by reference in this SFAR is as

follows:

(1) ICAO Annex 16--Environmental Protection, Volume II--Aircraft Engine

Emissions Appendices 2 through 6 dated 18 February 1982. This document can be

obtained from the International Civil Aviation Organization, P.O. Box 400,

Succursale: Place de L'Aviation Internationale, 1000 Sherbrooke Street West,

Montreal, Quebec, Canada H3A 2R2 at $3.00 per copy.

(d) Availability for inspection. A copy of each publication incorporated by

reference in this SFAR is available for public inspection at the following

locations:

(1) FAA Office of the Chief Counsel, Rules Docket, Room 916, Federal

Aviation Administration Headquarters Building, 800 Independence Avenue, S.W.,

Washington, D.C.

(2) Department of Transportation, Branch Library, Room 930, Federal

Aviation Administration Headquarters Building, 800 Independence Avenue, S.W.,

Washington, D.C.

(3) The respective offices of the Federal Aviation Administration as

follows:

(i) New England Regional Office, 12 New England Executive Park, Burlington,

Massachusetts 01803.

(ii) New York Aircraft Certification Office, 181 South Franklin Avenue,

Room 202, Valley Stream, New York 11581.

(iii) Atlanta Aircraft Certification Office, 1669 Phoenix Parkway, Suite

210, Atlanta, GA 30349.

(iv) Chicago Aircraft Certification Office, 2300 East Devon, Des Plaines,

Illinois 60018.

(v) Central Regional Office, 601 East Twelfth Street, Kansas City, Missouri

64106.

(vi) Southwest Regional Office, 4400 Blue Mound Road, Fort Worth, Texas

76101.

(vii) Denver Aircraft Certification Field Office, 10455 East 25th Avenue,

Aurora, Colorado 80010.

(viii) Northwest Regional Office, 17900 Pacific Highway South, Seattle,

Washington 98168.

(ix) Anchorage Aircraft Certification Field Office, 222 W. 7th Avenue, NZ14,

Anchorage, Alaska 99513.

(x) Los Angeles Aircraft Certification Office, 3229 East Spring Street,

Long Beach, CA 90806.

(xi) Brussels Aircraft Certification Staff, 15 Rue de la Loi, Room B-1040,

Brussels, Belgium.

(xii) FAA Representative, Office of U.S. Consulate General, Avenida

Presidente Wilson, 147 Rio de Janeiro, Brazil.

(4) The Office of the Federal Register, Room 8401, 1100 "L" Street, NW.,

Washington, D.C.

Sec. 11  Compliance with airworthiness regulations.

It must be shown that the airplane meets the airworthiness regulations

constituting the type certification basis of the airplane under all

conditions in which compliance with this SFAR is shown.

Sec. 12  Date of manufacture.

As used in this SFAR, unless 40 CFR Part 87 or the Clean Air Act, as

amended, requires otherwise, the date on which an aircraft engine is

"manufactured" is the date on which that individual engine is originally

approved by the FAA, or by a foreign country of manufacture, for installation

on an aircraft. Notwithstanding any other requirements of 14 CFR 45.13,

beginning January 1, 1984, the date of manufacture (month and year) must be

included as part of the identification data of engines manufactured on or

after that date.

Sec. 13  Engine classes and test configuration.

(a) Consistent with section 3(c) of this SFAR, the following definitions in

40 CFR 87.1 apply:

(1) "Aircraft engine" means a propulsion engine which is installed in or

which is manufactured for installation in an aircraft.

(2) "Class TP" means all aircraft turboprop engines.

(3) "Class TF" mean all turbofan or turbojet aircraft engines except

engines of Class T3, T8, and TSS.

(4) "Class T3" means all aircraft gas turbine engines of the JT3D model

family.

(5) "Class T8 means all aircraft gas turbine engines of the JT8D model

family.

(6) "Class TSS" means all aircraft gas turbine engines employed for

propulsion of aircraft designed to operate at supersonic flight speeds.

(7) "Commercial aircraft engine" means any aircraft engine used or intended

for use by an "air carrier" (including those engaged in "intrastate air

transportation") or a "commercial operator" (including those engaged in

"intrastate air transportation") as these terms are defined in the Federal

Aviation Act and the Federal Aviation Regulations.

(8) "Commercial aircraft gas turbine engine" means a turboprop, turbofan,

or turbojet commercial aircraft engine.

(b) As prescribed in 40 CFR Part 87, the complete engine as configured for

final acceptance testing, including all accessories that might reasonably be

expected to influence emissions to the atmosphere excluding auxiliary

gearbox-mounted components required to drive aircraft systems and service air

bleed, must be functional for all testing under this SFAR.

Sec. 14  Compliance.

(a) Compliance with the fuel venting emissions requirements of this SFAR

that apply beginning on February 1, 1974, and beginning January 1, 1975, may

be shown by any means of compliance, applied to the airframe of the engine,

that prevents the intentional discharge of fuel from fuel nozzle manifolds

after the engines are shut down. Acceptable means of compliance include the

following:

(1) Incorporation of an FAA approved system that recirculates the fuel back

into the fuel system.

(2) Capping or securing the pressurization and drain valve.

(3) Manually draining the fuel from a holding tank into a container.

(b) Continued compliance with the exhaust emissions requirements of this

SFAR that became effective on February 1, 1974, January 1, 1975, January 1,

1978, and January 1, 1984, for engines for which the type design has been

shown to meet those requirements, is demonstrated if the engine is maintained

in accordance with applicable maintenance requirements for 14 CFR Chapter I.

All methods of demonstrating compliance and all model designations previously

having been found acceptable to the Administrator of the FAA shall be deemed

to continue to be an acceptable demonstration of compliance with the specific

standards for which they were approved.

(c) Each applicant must allow the Administrator of the FAA to make, or

witness, any test necessary to determine compliance with the applicable

provisions of this SFAR.

(d) An acceptable alternative to testing every engine is described in

Appendix 6 to Volume II--Aircraft Engine Emissions, of ICAO Annex 16--

Environmental Protection, 18 February 1982, incorporated herein by reference

as indicated in Section 10. Other methods of demonstrating compliance may be

approved by the Administrator of the FAA with the concurrence of the

Administrator of the EPA.

Sec. 15  Type certificates.

(a) Notwithstanding Part 21 of the Federal Aviation Regulations, and

irrespective of the date of application, no type certificate is issued, on

and after the dates specified in subparagraphs (a)(1) through (a)(5) of this

section, for the airplanes specified therein, unless:

(1) For airplanes powered by engines of Class TF, Class TP, Class T3, Class

T8, or Class TSS, the airframe or engine complies with the fuel venting

emissions requirements and related test procedures of 40 CFR Part 87 that

became effective on February 1, 1974, and January 1, 1975;

(2) For airplanes powered by engines of Class T8, each engine complies with

the exhaust emissions requirements and related test procedures of 40 CFR Part

87 that became effective on February 1, 1974;

(3) For airplanes powered by engines of Class TF that have a rated power of

29,000 pounds thrust or greater, each engine complies with the exhaust

emissions requirements and related test procedures of 40 CFR Part 87 that

became effective on January 1, 1976;

(4) For airplanes powered by engines of Class T3, each engine manufactured

on or after January 1, 1978, complies with the exhaust emissions requirements

and related test procedures of 40 CFR Part 87 that became effective on

January 1, 1978; and

(5) For airplanes powered by engines of Class TP, Class TF, Class T3, Class

T8, and Class TSS--each engine manufactured on or after January 1, 1984,

complies with the exhaust emissions requirements and related test procedures

of 40 CFR Part 87 that apply beginning January 1, 1984.

(b) Notwithstanding Part 21 of the Federal Aviation Regulations, and

irrespective of the date of application, no type certificate is issued, on

and after the dates specified in paragraphs (a)(2), (a)(3) and (a)(4) of this

section, for an engine specified therein unless the engine complies with that

subparagraph.

(c) Notwithstanding Part 21 of the Federal Aviation Regulations and

irrespective of the date of application, no type certificate is issued on or

after January 1, 1984, for any Class TP, Class TF, Class T3, Class T8, or

Class TSS engine unless the engine complies with the exhaust emissions

requirements and related test procedures of 40 CFR Part 87 that apply

beginning January 1, 1984.

Sec. 17  Supplemental or amended type certificates.

(a) Notwithstanding Part 21 of the Federal Aviation Regulations, and

irrespective of the date of application, no supplemental or amended type

certificate is issued on and after the dates specified in paragraphs (a)(1)

through (a)(5) of this section, for the airplanes specified therein, unless:

(1) For airplanes powered by engines of Class TF, Class TP, Class T3, Class

T8, or Class TSS, the airframe or engine complies with the fuel venting

emissions requirements and related test procedures of 40 CFR Part 87 that

became effective on February 1, 1974, and January 1, 1975;

(2) For airplanes powered by engines of Class T8, each engine complies with

the exhaust emissions requirements and related test procedures of 40 CFR Part

87 that became effective on February 1, 1974;

(3) For airplanes powered by engines of Class TF that have a rated power of

29,000 pounds thrust or greater, each engine complies with the exhaust

emissions requirements and related test procedures of 40 CFR Part 87 that

became effective on January 1, 1976;

(4) For airplanes powered by engines of Class T3, each engine manufactured

on or after January 1, 1978, complies with the exhaust emissions requirements

and related test procedures of 40 CFR Part 87 that became effective on

January 1, 1978; and

(5) For airplanes powered by engines of Class TP, Class TF, Class T3, Class

T8, and Class TSS--each engine manufactured on or after January 1, 1984,

complies with the exhaust emissions requirements and related test procedures

of 40 CFR Part 87 that apply beginning January 1, 1984.

(b) Notwithstanding Part 21 of the Federal Aviation Regulations, and

irrespective of the date of application, no supplemental or amended type

certificate is issued, on or after the dates specified in paragraphs (a)(2),

(a)(3), and (a)(4) of this section, for an engine specified therein, unless

the engine complies with that paragraph.

(c) Notwithstanding Part 21 of the Federal Aviation Regulations and

irrespective of the date of application, no supplemental or amended type

certificate is issued on or after January 1, 1984, for any Class TP, Class

TF, Class T3, Class T8, or Class TSS engine unless the engine complies with

the exhaust emissions requirements and related test procedures of 40 CFR Part

87 that apply beginning January 1, 1984.

Sec. 19  Engine approvals.

(a) On and after the dates specified in this paragraph, and notwithstanding

Part 21 of the Federal Aviation Regulations, no original FAA approval for

installation of an engine on aircraft covered by this SFAR (including FAA

acceptance of an equivalent original approval issued on or before January 1,

1978, by a foreign country of manufacture) is made by the FAA:

(1) For an engine of Class T8, unless the engine complies with the exhaust

emissions requirements and related test procedures of 40 CFR Part 87 that

became effective on February 1, 1974;

(2) For an engine of Class TF that has a rated power of 29,000 pounds

thrust or greater, unless the engine complies with the exhaust emissions

requirements and related test procedures of 40 CFR Part 87 that became

effective on January 1, 1976;

(3) For an engine of Class T3, unless each engine manufactured on or after

January 1, 1978, complies with the exhaust emissions requirements and related

test procedures of 40 CFR Part 87 that became effective on January 1, 1978;

and

(4) For engines of Class TP, Class TF, Class T3, Class T8, or Class TSS,

unless each engine manufactured on or after January 1, 1984, complies with

the exhaust emissions requirements and related test procedures of 40 CFR Part

87 that apply beginning January 1, 1984.

Sec. 20  Identification Data and Recording Requirements.

(a) On or after January 1, 1984, notwithstanding 14 CFR Part 45, no

original FAA approval for installation of an engine on an airplane (including

FAA acceptance of an equivalent original approval issued on or after January

1, 1984, by a foreign country of manufacture) is made by the FAA, for an

engine covered by this SFAR unless that engine has secured to it an

identification plate displaying the date of manufacture and a designation,

approved by the Administrator of the FAA, that indicates compliance with

applicable provisions of 40 CFR Part 87. Approved indicators include COMPLY,

EXEMPT, and NON-US as appropriate.

(b) On or after January 1, 1984, notwithstanding 14 CFR Part 45, no person

may secure an identification plate to any airplane engine covered by this

SFAR unless that engine complies with the applicable provisions of this SFAR.

(c) On or after January 1, 1984, notwithstanding any other requirement of

14 CFR Part 43, for any engine granted an exemption pursuant to any

applicable provision of 40 CFR 87.7(a)(1), (a)(4), (b), (c), or (d), an

indication of the type of exemption and the reason for the grant must be

noted in the permanent powerplant record, that accompanies the engine, at the

time of manufacture of the engine.

(d) On or after January 1, 1984, notwithstanding any other requirement of

14 CFR Part 43, for any engine with a rated thrust in excess of 26.7

kilonewtons which does not comply with the hydrocarbon emissions standard of

40 CFR Part 87, the statement "May not be used as a commercial aircraft

engine" must be noted in the permanent powerplant record, that accompanies

the engine, at the time of manufacture of the engine.

(e) On or after January 1, 1984, notwithstanding any other requirement of

14 CFR Part 43, for any aircraft assembled with an engine that has been

granted an exemption pursuant to 40 CFR Sec. 87.7(a)(1), the notation "the

aircraft may not be operated within the United States", or an equivalent

notation approved by the Administrator of the FAA, must be inserted in the

aircraft logbook, or alternate equivalent document, at the time of

installation of the engine.

Sec. 21  Standard airworthiness certificates.

Notwithstanding Part 21 of the Federal Aviation Regulations, and

irrespective of the date of application, no standard airworthiness

certificate is issued, on and after the dates specified in paragraphs (a)

through (e) of this section, for the airplanes specified therein, unless:

(a) For airplanes powered by engines of Class TP, Class TF, Class T3, Class

T8, or Class TSS the airframe or engine complies with the fuel venting

emissions requirement and related test procedures of 40 CFR Part 87 that

became effective on February 1, 1974, and January 1, 1975;

(b) For airplanes powered by engines of Class T8, each engine complies with

the exhaust emissions requirements and related test procedures of 40 CFR Part

87 that became effective on February 1, 1974;

(c) For airplanes powered by engines of Class TF that have a rated power of

29,000 pounds thrust or greater, each engine complies with the exhaust

emissions requirements and related test procedures of 40 CFR Part 87 that

became effective on January 1, 1976;

(d) For airplanes powered by engines of Class T3, each engine manufactured

on or after January 1, 1978, complies with the exhaust emissions requirements

and related test procedures of 40 CFR Part 87 that became effective on

January 1, 1978; and

(e) For airplanes powered by engines of Class TP, Class TF, Class T3, Class

T8, and Class TSS--each engine manufactured on or after January 1, 1984,

complies with the exhaust emissions requirements and related test procedures

of 40 CFR Part 87 that apply beginning January 1, 1984.

Sec. 25  Operation.

On and after the dates specified in paragraphs (a) through (e) of this

section, no person may, within the United States, operate an airplane

specified in those paragraphs unless:

(a) For airplanes powered by engines of Class TP, Class TF, Class T3, Class

T8, or Class TSS, the airframe or the engine complies with the fuel venting

emissions requirements and related test procedures of 40 CFR Part 87 that

became effective on February 1, 1974, and January 1, 1975;

(b) For airplanes powered by engines of Class T8, each engine complies with

the exhaust emissions requirements and related test procedures of 40 CFR Part

87 that became effective on February 1, 1974;

(c) For airplanes powered by engines of Class TF that have a rated power of

29,000 pounds thrust or greater, each engine complies with the exhaust

emissions requirements and related test procedures of 40 CFR Part 87 that

became effective on January 1, 1976;

(d) For airplanes powered by engines of Class T3, each engine

manufacturered on or after January 1, 1978, complies with the exhaust

emissions requirements and related test procedures of 40 CFR Part 87 that

became effective on January 1, 1978; and

(e) For airplanes powered by engines of Class TP, Class TF, Class T3, Class

T8, and Class TSS--each engine manufactured on or after January 1, 1984,

complies with the exhaust emissions and related test procedures of 40 CFR

Part 87 that apply beginning January 1, 1984.

[48 FR 56739, Dec. 23, 1983, as amended by SFAR 27-6, 51 FR 10613, Mar. 28,

1986; SFAR 27-5, 54 FR 39289, Sept. 25, 1989]

*****************************************************************************

55 FR 32856, No. 155, Aug. 10, 1990

SUMMARY: This final rule codifies as new part 34 all of the applicable

aircraft engine fuel venting and exhaust emission requirements of Special

Federal Aviation Regulation (SFAR) 27-5, and the test procedures specified

under the regulations implementing the Clean Air Act. This rule consolidates

all of the requirements and test procedures into this part, and inserts into

other affected parts the requirements to comply with new part 34. New part 34

does not alter any of the requirements specified under SFAR 27-5 or the

regulations implementing the Clean Air Act.

EFFECTIVE DATE: September 10, 1990.

*****************************************************************************

SFAR No. 27-5  [Removed.  55 FR 32860, Aug. 10, 1990]

EDITORIAL NOTE: For the convenience of the user, the removed text is

set out below.

SFAR No. 27-5--Fuel Venting and Exhaust Emission Requirements for Turbine

Engine Powered Airplanes

EDITORIAL NOTE: For the text of SFAR No. 27, see Part 11 of this chapter.

*****************************************************************************

55 FR 32856, No. 155, Aug. 10, 1990

SUMMARY: This final rule codifies as new part 34 all of the applicable

aircraft engine fuel venting and exhaust emission requirements of Special

Federal Aviation Regulation (SFAR) 27-5, and the test procedures specified

under the regulations implementing the Clean Air Act. This rule consolidates

all of the requirements and test procedures into this part, and inserts into

other affected parts the requirements to comply with new part 34. New part 34

does not alter any of the requirements specified under SFAR 27-5 or the

regulations implementing the Clean Air Act.

EFFECTIVE DATE: September 10, 1990.

*****************************************************************************

SFAR No. 29-4

Limited IFR Operations of Rotorcraft

1. Contrary provisions of Parts 21, 27, and 29 of the Federal Aviation

Regulations notwithstanding, an operator of a rotorcraft that is not

otherwise certificated for IFR operations may conduct an approved limited IFR

operation in the rotorcraft when--

(a) FAA approval for the operation has been issued under paragraph 2 of

this SFAR;

(b) The operator complies with all conditions and limitations established

by this SFAR and the approval; and

(c) A copy of the approval and this SFAR are set forth as a supplement to

the Rotorcraft Flight Manual.

2. FAA approval for the operation of a rotorcraft in limited IFR operations

may be issued when the following conditions are met:

(a) The operation is approved as part of the FAA study of limited

rotorcraft IFR operations.

(b) Specific FAA approval has been obtained for the following:

(i) The rotorcraft (make, model, and serial number).

(ii) The flightcrew.

(iii) The procedures to be followed in the operation of the rotorcraft

under IFR and the equipment that must be operable during such operations.

(c) The conditions and limitations necessary for the safe operation of the

rotorcraft in limited IFR operations have been established, approved, and

incorporated into the operating limitations section of the Rotorcraft Flight

Manual.

3. An approval issued under paragraph 2 of this Special Federal Aviation

Regulation and the change to the Rotorcraft Flight Manual specified in

paragraph 2(c) of this Special Federal Aviation Regulation constitute a

supplemental type certificate for each rotorcraft approved under paragraph 2

of this SFAR. The supplemental type certificate will remain in effect until

the approval to operate issued under the Special Federal Aviation Regulation

is surrendered, revoked, or otherwise terminated.

4. Notwithstanding Sec. 91.167(a)(3) of the Federal Aviation Regulations, a

person may operate a rotorcraft in a limited IFR operation approved under

paragraph 2(a) of the Special Federal Aviation Regulation with enough fuel to

fly, after reaching the alternate airport, for not less than 30 minutes, when

that period of time has been approved.

5. Expiration.

(a) New applications for limited IFR rotorcraft operations under SFAR No.

29 may be submitted for approval until, but not including, the effective date

of Amendment No. 1 of the Rotorcraft Regulatory Review Program. On and after

the effective date of Amendment No. 1 of the Rotorcraft Regulatory Review

Program, all applicants for certification of IFR rotorcraft operations must

comply with the applicable provisions of the Federal Aviation Regulations.

(b) This Special Federal Aviation Regulation will terminate when all

approvals issued under Special Federal Aviation Regulation No. 29 are

surrendered, revoked, or otherwise terminated.

[Doc. No. 14237, 48 FR 632, Jan. 6, 1983, as amended at 54 FR 34329, Aug. 18,

1989]

Effective Date Note: At 54 FR 34329, Aug. 18, 1989, Part 21, SFAR 29-4,

paragraph 4 was amended by changing the cross reference "Sec. 91.23(a)(3)" to

"91.167", effective, August 18, 1990.

SFAR No. 41

1. Applicability.

(a) Contrary provisions of Parts 21 and 23 of the Federal Aviation

Regulations notwithstanding, an applicant is entitled to an amended or

supplemental type certificate in the normal category for a reciprocating or

turbopropeller-powered multiengine small airplane originally type

certificated prior to October 17, 1979 in accordance with Part 23 of the

Federal Aviation Regulations in effect on March 13, 1971, or later, that is

to be certificated with a passenger seating configuration, excluding pilot

seats, of 10 seats or more (but not more than 19 seats) at a maximum

certificated takeoff weight of 12,500 pounds or less, if the applicant

complies with--

(1) The regulations incorporated in the type certificate; and

(2) The requirements of Appendix A of Part 135 of the Federal Aviation

Regulations in effect on September 26, 1978, except that the landing distance

must be determined for standard atmosphere at each weight, altitude, and wind

within the operating limits established by the applicant in accordance with

Sec. 23.75(a) of this chapter in effect on September 26, 1978. Instead of a

gliding approach specified in Sec. 23.75(a), the landing may be preceded by a

steady approach down to the 50-foot height at a gradient of descent not

greater than 5.2 percent (3 deg.) at a calibrated airspeed not less than

1.3Vs1.

(b) Contrary provisions of Parts 1, 21, 23, 91, 121, and 135 of the Federal

Aviation Regulations notwithstanding, an applicant is entitled to an amended

or supplemental type certificate in the normal category for a reciprocating

or turbopropeller powered multiengine airplane originally type certified

prior to October 17, 1979 that is to be certificated with a maximum takeoff

weight in excess of 12,500 pounds, a specified maximum zero fuel weight to be

established by the applicant, and, where requested by the applicant, an

increase in passenger seating configuration, (but not more than 19 passenger

seats), if the applicant complies with--

(1) The regulations incorporated in the type certificate;

(2) The requirements of Appendix A of Part 135 of the Federal Aviation

Regulations in effect on September 26, 1978 with the exceptions specified in

section 5 of this Special Federal Aviation Regulation; and

(3) The additional requirements specified in sections 7 through 14 of this

Special Federal Aviation Regulation applicable to takeoff weights in excess

of 12,500 pounds.

(c) Contrary provisions of Part 1 of the Federal Aviation Regulations

notwithstanding, an airplane certificated under paragraph (b) of this section

is considered to be a small airplane for purposes of Parts 21, 23, 36, 121,

135, and 139 of the Federal Aviation Regulations, and a large airplane for

purposes of Parts 61 and 91. Compliance with the small airplane provisions of

Part 36 of the Federal Aviation Regulations must be shown at the maximum

certificated takeoff weight approved under this Special Federal Aviation

Regulation.

2. Eligibility. Any person may apply for a supplemental type certificate

(or an amended type certificate in the case of a type certificate holder)

under this Special Federal Aviation Regulation.

3. Production limitation. An amended or supplemental type certificate

issued pursuant to section 1.(b) of this Special Federal Aviation Regulation

is effective for the purpose of obtaining an original or an amended

airworthiness certificate, until October 17, 1991 unless the type certificate

is sooner surrendered, suspended, revoked, or terminated.

4. Restrictions. For airplanes certificated under section 1.(b) of this

Special Federal Aviation Regulation--

(a) The maximum zero fuel weight of the airplane must be established as an

operating limitation; and

(b) Except as provided in paragraph (c) of this section the airworthiness

certificate shall be endorsed "This airplane at weights in excess of 5,700 kg

does not meet the airworthiness requirements of ICAO, as prescribed by Annex

8 of the Convention on International Civil Aviation."

(c) An applicant is entitled to type certificate amendment or a

supplemental type certificate that shows compliance with ICAO Annex 8 if the

airplane meets SFAR 41 and the following requirements prescribed by the

Administrator in effect on October 17, 1979:

(1) At each weight, altitude, and temperature within the operational limits

selected by the applicant--

(i) For approach climb performance, comply with Secs. 25.121(d) and

25.1533(a)(2).

(ii) For takeoff performance, comply with Secs. 25.105(d), 25.111,

25.113(a), and 25.115.

(2) For gust loads design at rough air gust speed VB, comply with Secs.

25.335(d), 25.341(a)(1), and 25.351(b).

(3) For smoke evacuation design, comply with Sec. 25.831(d).

(4) For engine rotation and restarting design, comply with Sec. 25.903 (c)

and (e).

(5) For engine cooling design, comply with Sec. 25.1521(e).

5. Exceptions. For purposes of obtaining an amended or supplemental type

certificate under section 1.(b) of this Special Federal Aviation Regulation,

the following exceptions apply. All references in this section to specific

sections of Parts 23 and 25 of this chapter are to those in effect on

September 26, 1978 if no other date is given:

(a) Compliance with section 1 of Appendix A of Part 135 of the Federal

Aviation Regulations is not required.

(b) Compliance may be shown with the applicable regulations incorporated in

the type certificate in lieu of the requirements of Appendix A of Part 135 of

the Federal Aviation Regulations for takeoff weights of 12,500 pounds or

less, if the airplane was type certificated--

(1) Under FAR Part 23 in effect prior to Amendment 23-10 and the airplane

is to be used only in FAR Part 91 operations;

(2) Before July 19, 1970, in the normal category with a passenger seating

configuration, excluding any pilot seat, of 10 seats or more, (but not to

exceed 19 passenger seats), and meets special conditions issued by the

Administrator for airplanes intended for use in operations under FAR Part

135; or

(3) Before July 19, 1970, in the normal category with a passenger seating

configuration, excluding any pilot seat, of 10 seats or more, (but not to

exceed 19 passenger seats), and meets the additional airworthiness standards

in Special Federal Aviation Regulation No. 23.

(c) In lieu of compliance with sections 7., and 19.(c) of Appendix A of

Part 135 of the Federal Aviation Regulations, comply with the following at

takeoff weights in excess of 12,500 pounds:

Landing

(a) The landing distance must be determined for standard atmosphere at each

weight, altitude, and wind within the operational limits established by the

applicant in accordance with Sec. 23.75(a) of this chapter. Instead of a

gliding approach specified in Sec. 23.75(a), the landing may be preceded by a

steady approach down to the 50-foot height at a gradient of descent not

greater than 5.2 percent (3 deg.) at a calibrated airspeed not less than 1.3

VS1.

(b) The landing distance data must include correction factors for not more

than 50 percent of the nominal wind components along the landing path

opposite to the direction of landing, and not less than 150 percent of the

nominal wind components along the landing path in the direction of landing.

(d) In lieu of compliance with section 28 of Appendix A of Part 135 of the

Federal Aviation Regulations, comply with the following:

Fatigue evaluation of flight structure. Unless it is shown that the

structure, operating stress levels, materials, and expected use are

comparable from a fatigue standpoint to a similar design which has had

substantial satifactory service experience, the strength, detail design, and

the fabrication of those parts of the wing, wing carrythrough, vertical fin,

horizortal stabilizer, and attaching structure whose failure would be

catastrophic must be evaluted under either--

(a) A fatigue strength investigation in which the structure is shown by

analysis, tests, or both, to be able to withstand the repeated loads of

variable magnitude expected in service. Analysis alone is acceptable only

when it is conservative and applied to simple structures; or

(b) A fail-safe strength investigation in which it is shown by analysis,

tests, or both, that catastrophic failure of the structure is not probable

after fatigue, or obvious partial failure, of a principal structural element,

and that the remaining structure is able to withstand a static ultimate load

factor of 75 percent of the critical limit load factor at VC. These loads

must be multiplied by a factor of 1.15 unless the dynamic effects of failure

under static load are otherwise considered.

(e) In lieu of compliance with section 32 of Appendix A of Part 135 of the

Federal Aviation Regulations, comply with the following:

Doors and exits. The airplane must meet the requirements of Secs.23.783 and

23.807 (a)(3), (b), and (c) of this chapter, and in addition the following

requirements:

(a) Each cabin must have at least one easily accessible external door.

(b) There must be a means to lock and safeguard each external door against

opening in flight (either inadvertently by persons or as a result of

mechanical failure or failure of a single structural element). Each external

door must be operable from both the inside and the outside, even though

persons may be crowded against the door on the inside of the airplane. Inward

opening doors may be used if there are means to prevent occupants from

crowding against the door to an extent that would interfere with the opening

of the door. The means of opening must be simple and obvious and must be

arranged and marked so that it can be readily located and operated, even in

darkness. Auxiliary locking devices may be used.

(c) Each external door must be reasonably free from jamming as a result of

fuselage deformation in a minor crash.

(d) Each external door must be located where persons using it will not be

endangered by the propellers when appropriate operating procedures are used.

(e) There must be a provision for direct visual inspection of the locking

mechanism by crewmembers to determine whether external doors, for which the

initial opening movement is outward (including passenger, crew, service, and

cargo doors), are fully locked. In addition, there must be a visual means to

signal to appropriate crewmembers when normally used external doors are

closed and fully locked.

(f) Cargo and service doors not suitable for use as exits in an emergency

need only meet paragraph (e) of section 5(e) of this regulation and be

safeguarded against opening in flight as a result of mechanical failure or

failure of a single structural element.

(g) The passenger entrance door must qualify as a floor level emergency

exit. If an integral stair is installed at such a passenger entry door, the

stair must be designed so that when subjected to the inertia forces specified

in Sec. 23.561 of this chapter, and following the collapse of one or more

legs of the landing gear, it will not interfere to an extent that will reduce

the effectiveness of emergency egress through the passenger entry door. Each

additional required emergency exit except floor level exits must be located

over the wing or must be provided with acceptable means to assist the

occupants in descending to the ground. In addition to the passenger entrance

door--

(1) For a total passenger seating capacity of 15 or less, an emergency

exit, as defined in Sec. 23.807(b) of this chapter, is required on each side

of the cabin; and

(2) For a total passenger seating capacity of 16 through 19, three

emergency exits, as defined in Sec. 23.807(b) of this chapter, are required

with one on the same side as the door and two on the side opposite the door.

(h) An evacuation demonstration must be conducted utilizing the maximum

number of occupants for which certification is desired. It must be conducted

under simulated night conditions utilizing only the emergency exits on the

most critical side of the aircraft. The participants must be representative

of average airline passengers with no prior practice or rehearsal for the

demonstration. Evacuation must be completed within 90 seconds.

(i) Each emergency exit must be marked with the word "Exit" by a sign which

has white letters 1 inch high on a red background 2 inches high, be self-

illuminated or independently internally electrically illuminated, and have a

minimum luminescence (brightness) of at least 160 microlamberts. The colors

may be reversed if the passenger compartment illumination is essentially the

same.

(j) Access to window type emergency exits may not be obstructed by seats or

seat backs.

(k) The width of the main passenger aisle at any point between seats must

equal or exceed the values in the following table:

Minimum main

passenger aisle

width

Less

than 25

Number of     inches   25 inches

passenger      from     and more

seats       floor    from floor

10 through 19  9 inches  15 inches.

(f) In lieu of compliance with Section 45 of Appendix A of Part 135 of the

Federal Aviation Regulations, comply with Sec. 23.954 of this chapter.

(g) In lieu of compliance with Section 56 of Appendix A of Part 135 of the

Federal Aviation Regulations, comply with the following:

Cowlings. The airplane must be designed and constructed so that no fire

originating in any engine compartment can enter, either through openings or

by burn through, any other region where it would create additional hazards.

(h) In lieu of complaince with Section 57 of Appendix A of Part 135 of the

Federal Aviation Regulations, comply with Sec. 25.863 of this chapter.

6. Additional requirements--general. The additional requirements specified

in sections 7 through 14 apply to the certification of airplanes pursuant to

section 1.(b) of this Special Federal Aviation Regulation.

7. Compartment interiors.

(a) If smoking is to be prohibited, there must be a placard so stating, and

if smoking is to be allowed--

(1) There must be an adequate number of self-contained removable ashtrays;

and

(2) Where the crew compartment is separated from the passenger compartment,

there must be at least one sign (using either letters or symbols) notifying

all passengers when smoking is prohibited. Signs which notify when smoking is

prohibited must--

(i) Be legible to each passenger seated in the passenger cabin under all

probable lighting conditions; and

(ii) When illuminated, be so constructed that the crew can turn them on and

off.

(b) Each disposal receptacle for towels, paper, or waste must be fully

enclosed and constructed of at least fire resistant materials, and must

contain fires likely to occur in it under normal use. The ability of the

disposal receptacle to contain those fires under all probable conditions of

wear, misalignment, and ventilation expected in service must be demonstrated

by test. A placard containing the legible words "No Cigarette Disposal" must

be located on or near each disposal receptacle door.

(c) Lavatories must have "No Smoking" or "No Smoking in Lavatory" placards

located conspicuously on each side of the entry door, and self-contained

removable ashtrays located conspicuously on or near the entry side of each

lavatory door, except that one ashtray may serve more than one lavatory door

if it can be seen from the cabin side of each lavatory door served. The

placards must have red letters at least one-half inch high on a white

background at least one inch high. (A "No smoking" symbol may be included on

the placard).

(d) There must be at least one hand fire extinguisher conventiently located

in the pilot compartment.

(e) There must be at least one hand fire extinguisher conventiently located

in the passenger compartment.

8. Landing gear. Comply with Sec. 25.721(a)(2), (b), and (c) of this

chapter in effect on September 26, 1978.

9. Fuel system components crashworthiness. Comply with Secs. 25.963(d) and

25.994 of this chapter in effect on September 26, 1978.

10. Shutoff means. Comply with Sec. 23.1189 of this chapter in effect on

September 26, 1978.

11. Fire detector and extinguishing systems--(a) Fire detector systems. (1)

There must be a means which ensures the prompt detection of a fire in an

engine compartment.

(2) Each fire detector must be constructed and installed to withstand the

vibration, inertia, and other loads to which it may be subjected in

operation.

(3) No fire detector may be affected by any oil, water, other fluids, or

fumes that might be present.

(4) There must be means to allow the crew to check, in flight, the function

of each fire detector electric circuit.

(5) Wiring and other components of each fire detector system in an engine

compartment must be at least fire resistant.

(b) Fire extinguishing systems. (1) Except for combustor, turbine, and tail

pipe sections of turbine engine installations that contain lines or

components carrying flammable fluids or gases for which it is shown that a

fire originating in these sections can be controlled, there must be a fire

extinguisher system serving each engine compartment.

(2) The fire extinguishing system, the quantity of the extinguishing agent,

the rate of discharge, and the discharge distribution must be adequate to

extinguish fires. An individual "one shot" system may be used.

(3) The fire-extinguishing system for a nacelle must be able to

simultaneously protect each compartment of the nacelle for which protection

is provided.

12. Fire extinguishing agents. Comply with Sec. 25.1197 of this chapter in

effect on September 26, 1978.

13. Extinguishing agent containers. Comply with Sec. 25.1199 of this

chapter in effect on September 26, 1978.

14. Fire extinguishing system materials. Comply with Sec. 25.1201 of this

chapter in effect on September 26, 1978.

15. Expiration. This Special Federal Aviation Regulation terminates on

September 13, 1983, unless sooner rescinded or superseded.

[Doc. No. 18315, 44 FR 53729, Sept. 17, 1979; 45 FR 25047, Apr. 14, 1980; 45

FR 80973, Dec. 8, 1980, as amended by Doc. No. 21716, 47 FR 35153, Aug. 12,

1982]

Subpart A--General

Sec. 21.1  Applicability.

(a) This part prescribes--

(1) Procedural requirements for the issue of type certificates and changes

to those certificates; the issue of production certificates; the issue of

airworthiness certificates; and the issue of export airworthiness approvals.

(2) Rules governing the holders of any certificate specified in paragraph

(a)(1) of this section; and

(3) Procedural requirements for the approval of certain materials, parts,

processes, and appliances.

(b) For the purposes of this part, the word "product" means an aircraft,

aircraft engine, or propeller. In addition, for the purposes of Subpart L

only, it includes components and parts of aircraft, of aircraft engines, and

of propellers; also parts, materials, and appliances, approved under the

Technical Standard Order system.

[Doc. No. 5085, 29 FR 14563, Oct. 24, 1964, as amended by Amdt. 21-2, 30 FR

8465, July 2, 1965; Amdt. 21-6, 30 FR 11379, Sept. 8, 1965]

Sec. 21.2   Falsification of applications, reports, or records.

(a) No person shall make or cause to be made--

(1) Any fraudulent or intentionally false statement on any application for

a certificate or approval under this part;

(2) Any fraudulent or intentionally false entry in any record or report

that is required to be kept, made, or used to show compliance with any

requirement for the issuance or the exercise of the privileges of any

certificate or approval issued under this part;

(3) Any reproduction for a fraudulent purpose of any certificate or

approval issued under this part.

(4) Any alteration of any certificate or approval issued under this part.

(b) The commission by any person of an act prohibited under paragraph (a)

of this section is a basis for suspending or revoking any certificate or

approval issued under this part and held by that person.

[57 FR 41367, Sept. 9, 1992]

*****************************************************************************

57 FR 41360, No. 175, Sept. 9, 1992

SUMMARY: This final rule establishes a new primary category of aircraft,

and new simplified procedures for type, production, and airworthiness

certification, and associated maintenance procedures. Aircraft in this

category are of simple design intended exclusively for pleasure and personal

use. Primary category aircraft (airplanes, gliders, rotorcraft, manned free

balloons, etc.) may be unpowered or powered by a single, naturally aspirated

engine, with a 61-knot or less stall speed limitation for airplanes and a 6-

pound per square foot main rotor disc loading limitation for rotorcraft.

Primary category aircraft may have a maximum certificated weight of no more

than 2,700 pounds, maximum seating capacity of four, and unpressurized

cabins. Although these aircraft may be available for rental and flight

instruction under certain conditions, the carrying of persons or property for

hire is prohibited. This final rule also adds a new section addressing the

falsification of documents submitted as part of certification for products

and parts.

EFFECTIVE DATE: December 31, 1992.

*****************************************************************************

Sec. 21.3  Reporting of failures, malfunctions, and defects.

(a) Except as provided in paragraph (d) of this section, the holder of a

Type Certificate (including a Supplemental Type Certificate), a Parts

Manufacturer Approval (PMA), or a TSO authorization, or the licensee of a

Type Certificate shall report any failure, malfunction, or defect in any

product, part, process, or article manufactured by it that it determines has

resulted in any of the occurrences listed in paragraph (c) of this section.

(b) The holder of a Type Certificate (including a Supplemental Type

Certificate), a Parts Manufacturer Approval (PMA), or a TSO authorization, or

the licensee of a Type of Certificate shall report any defect in any product,

part, or article manufactured by it that has left its quality control system

and that it determines could result in any of the occurrences listed in

paragraph (c) of this section.

(c) The following occurrences must be reported as provided in paragraphs

(a) and (b) of this section:

(1) Fires caused by a system or equipment failure, malfunction, or defect.

(2) An engine exhaust system failure, malfunction, or defect which causes

damage to the engine, adjacent aircraft structure, equipment, or components.

(3) The accumulation or circulation of toxic or noxious gases in the crew

compartment or passenger cabin.

(4) A malfunction, failure, or defect of a propeller control system.

(5) A propeller or rotorcraft hub or blade structural failure.

(6) Flammable fluid leakage in areas where an ignition source normally

exists.

(7) A brake system failure caused by structural or material failure during

operation.

(8) A significant aircraft primary structural defect or failure caused by

any autogenous condition (fatigue, understrength, corrosion, etc.).

(9) Any abnormal vibration or buffeting caused by a structural or system

malfunction, defect, or failure.

(10) An engine failure.

(11) Any structural or flight control system malfunction, defect, or

failure which causes an interference with normal control of the aircraft for

which derogates the flying qualities.

(12) A complete loss of more than one electrical power generating system or

hydraulic power system during a given operation of the aircraft.

(13) A failure or malfunction of more than one attitude, airspeed, or

altitude instrument during a given operation of the aircraft.

(d) The requirements of paragraph (a) of this section do not apply to--

(1) Failures, malfunctions, or defects that the holder of a Type

Certificate (including a Supplemental Type Certificate), Parts Manufacturer

Approval (PMA), or TSO authorization, or the licensee of a Type Certificate--

(i) Determines were caused by improper maintenance, or improper usage;

(ii) Knows were reported to the FAA by another person under the Federal

Aviation Regulations; or

(iii) Has already reported under the accident reporting provisions of Part

430 of the regulations of the National Transportation Safety Board.

(2) Failures, malfunctions, or defects in products, parts, or articles

manufactured by a foreign manufacturer under a U.S. Type Certificate issued

under Sec. 21.29 or Sec. 21.617, or exported to the United States under Sec.

21.502.

(e) Each report required by this section--

(1) Shall be made to the Aircraft Certification Office in the region in

which the person required to make the report is located within 24 hours after

it has determined that the failure, malfunction, or defect required to be

reported has occurred. However, a report that is due on a Saturday or a

Sunday may be delivered on the following Monday and one that is due on a

holiday may be delivered on the next workday;

(2) Shall be transmitted in a manner and form acceptable to the

Administrator and by the most expeditious method available; and

(3) Shall include as much of the following information as is available and

applicable:

(i) Aircraft serial number.

(ii) When the failure, malfunction, or defect is associated with an article

approved under a TSO authorization, the article serial number and model

designation, as appropriate.

(iii) When the failure, malfunction, or defect is associated with an engine

or propeller, the engine or propeller serial number, as appropriate.

(iv) Product model.

(v) Identification of the part, component, or system involved. The

identification must include the part number.

(vi) Nature of the failure, malfunction, or defect.

(f) Whenever the investigation of an accident or service difficulty report

shows that an article manufactured under a TSO authorization is unsafe

because of a manufacturing or design defect, the manufacturer shall, upon

request of the Administrator, report to the Administrator the results of its

investigation and any action taken or proposed by the manufacturer to correct

that defect. If action is required to correct the defect in existing

articles, the manufacturer shall submit the data necessary for the issuance

of an appropriate airworthiness directive to the Manager of the Aircraft

Certification Office for the geographic area of the FAA regional office in

the region in which it is located.

[Amdt. 21-36, 35 FR 18187, Nov. 28, 1970, as amended by Amdt. 21-37, 35 FR

18450, Dec. 4, 1970; Amdt. 21-50, 45 FR 38346, June 9, 1980; Amdt. 21-67, 54

FR 39291, Sept. 25, 1989]

Sec. 21.5  Airplane or Rotorcraft Flight Manual.

(a) With each airplane or rotorcraft that was not type certificated with an

Airplane or Rotorcraft Flight Manual and that has had no flight time prior to

March 1, 1979, the holder of a Type Certificate (including a Supplemental

Type Certificate) or the licensee of a Type Certificate shall make available

to the owner at the time of delivery of the aircraft a current approved

Airplane or Rotorcraft Flight Manual.

(b) The Airplane or Rotorcraft Flight Manual required by paragraph (a) of

this section must contain the following information:

(1) The operating limitations and information required to be furnished in

an Airplane or Rotorcraft Flight Manual or in manual material, markings, and

placards, by the applicable regulations under which the airplane or

rotorcraft was type certificated.

(2) The maximum ambient atmospheric temperature for which engine cooling

was demonstrated must be stated in the performance information section of the

Flight Manual, if the applicable regulations under which the aircraft was

type certificated do not require ambient temperature on engine cooling

operating limitations in the Flight Manual.

[Amdt. 21-46, 43 FR 2316, Jan. 16, 1978]

Subpart B--Type Certificates

Source: Docket No. 5085, 29 FR 14564, Oct. 24, 1964, unless otherwise

noted.

Sec. 21.11  Applicability.

This subpart prescribes--

(a) Procedural requirements for the issue of type certificates for

aircraft, aircraft engines, and propellers; and

(b) Rules governing the holders of those certificates.

Sec. 21.13  Eligibility.

Any interested person may apply for a type certificate.

[Amdt. 21-25, 34 FR 14068, Sept. 5, 1969]

Sec. 21.15  Application for type certificate.

(a) An application for a type certificate is made on a form and in a manner

prescribed by the Administrator and is submitted to the appropriate Aircraft

Certification Office.

(b) An application for an aircraft type certificate must be accompanied by

a three-view drawing of that aircraft and available preliminary basic data.

(c) An application for an aircraft engine type certificate must be

accompanied by a description of the engine design features, the engine

operating characteristics, and the proposed engine operating limitations.

[Doc. No. 5085, 29 FR 14564, Oct. 24, 1964, as amended by Amdt. 21-40, 39 FR

35459, Oct. 1, 1974; Amdt. 21-67, 54 FR 39291, Sept. 25, 1989]

Sec. 21.16  Special conditions.

If the Administrator finds that the airworthiness regulations of this

subchapter do not contain adequate or appropriate safety standards for an

aircraft, aircraft engine, or propeller because of a novel or unusual design

feature of the aircraft, aircraft engine or propeller, he prescribes special

conditions and amendments thereto for the product. The special conditions are

issued in accordance with Part 11 of this chapter and contain such safety

standards for the aircraft, aircraft engine or propeller as the Administrator

finds necessary to establish a level of safety equivalent to that established

in the regulations.

[Amdt. 21-19, 32 FR 17851, Dec. 13, 1967; as amended by Amdt. 21-51, 45 FR

60170, Sept. 11, 1980]

Sec. 21.17  Designation of applicable regulations.

(a) Except as provided in Sec. 23.2, Sec. 25.2, Sec. 27.2, Sec. 29.2,

and in parts 34 and 36 of this chapter, an applicant for a type certificate

must show that the aircraft, aircraft engine, or propeller concerned meets--

(1) The applicable requirements of this subchapter that are effective on

the date of application for that certificate unless--

(i) Otherwise specified by the Administrator; or

(ii) Compliance with later effective amendments is elected or required

under this section; and

(2) Any special conditions prescribed by the Administrator.

(b) For special classes of aircraft, including the engines and propellers

installed thereon (e.g., gliders, airships, and other nonconventional

aircraft), for which airworthiness standards have not been issued under this

subchapter, the applicable requirements will be the portions of those other

airworthiness requirements contained in Parts 23, 25, 27, 29, 31, 33, and 35

found by the Administrator to be appropriate for the aircraft and applicable

to a specific type design, or such airworthiness criteria as the

Administrator may find provide an equivalent level of safety to those parts.

(c) An application for type certification of a transport category aircraft

is effective for 5 years and an application for any other type certificate is

effective for 3 years, unless an applicant shows at the time of application

that his product requires a longer period of time for design, development,

and testing, and the Administrator approves a longer period.

(d) In a case where a type certificate has not been issued, or it is clear

that a type certificate will not be issued, within the time limit established

under paragraph (c) of this section, the applicant may--

(1) File a new application for a type certificate and comply with all the

provisions of paragraph (a) of this section applicable to an original

application; or

(2) File for an extension of the original application and comply with the

applicable airworthiness requirements of this subchapter that were effective

on a date, to be selected by the applicant, not earlier than the date which

precedes the date of issue of the type certificate by the time limit

established under paragraph (c) of this section for the original application.

(e) If an applicant elects to comply with an amendment to this subchapter

that is effective after the filing of the application for a type certificate,

he must also comply with any other amendment that the Administrator finds is

directly related.

(f) For primary category aircraft, the requirements are:

(1) The applicable airworthiness requirements contained in parts 23, 27,

31, 33, and 35 of this subchapter, or such other airworthiness criteria as

the Administrator may find appropriate and applicable to the specific design

and intended use and provide a level of safety acceptable to the

Administrator.

(2) The noise standards of part 36 applicable to primary category aircraft.

[Doc. No. 5085, 29 FR 14564, Oct. 24, 1964, as amended by Amdt. 21-19, 32 FR

17851, Dec. 13, 1967; Amdt. 21-24, 34 FR 364, Jan. 10, 1969; Amdt. 21-42, 40

FR 1033, Jan. 6, 1975; Amdt. 21-58, 50 FR 46877, Nov. 13, 1985; Amdt. 21-60,

52 FR 8042, Mar. 13, 1987; Amdt. 21-68, 55 FR 32860, Aug. 10, 1990; Amdt.

21-69, 56 FR 41051, Aug. 16, 1991; Amdt. 21-70, 57 FR 41367, Sept. 9, 1992]

*****************************************************************************

57 FR 41360, No. 175, Sept. 9, 1992

SUMMARY: This final rule establishes a new primary category of aircraft, and

new simplified procedures for type, production, and airworthiness

certification, and associated maintenance procedures. Aircraft in this

category are of simple design intended exclusively for pleasure and personal

use. Primary category aircraft (airplanes, gliders, rotorcraft, manned free

balloons, etc.) may be unpowered or powered by a single, naturally aspirated

engine, with a 61-knot or less stall speed limitation for airplanes and a 6-

pound per square foot main rotor disc loading limitation for rotorcraft.

Primary category aircraft may have a maximum certificated weight of no more

than 2,700 pounds, maximum seating capacity of four, and unpressurized

cabins. Although these aircraft may be available for rental and flight

instruction under certain conditions, the carrying of persons or property for

hire is prohibited. This final rule also adds a new section addressing the

falsification of documents submitted as part of certification for products

and parts.

EFFECTIVE DATE: December 31, 1992.

*****************************************************************************

Sec. 21.19  Changes requiring a new type certificate.

Any person who proposes to change a product must make a new application for

a type certificate if--

(a) The Administrator finds that the proposed change in design,

configuration, power, power limitations (engines), speed limitations

(engines), or weight is so extensive that a substantially complete

investigation of compliance with the applicable regulations is required;

(b) In the case of a normal, utility, acrobatic, commuter or transport

category aircraft, the proposed change is--

(1) In the number of engines or rotors; or

(2) To engines or rotors using different principles of propulsion or to

rotors using different principles of operation;

(c) In the case of an aircraft engine, the proposed change is in the

principle of operation; or

(d) In the case of propellers, the proposed change is in the number of

blades or principle of pitch change operation.

[Doc. No. 5085, 29 FR 14564, Oct. 24, 1964, as amended by Amdt. 23-34, 52 FR

1835, Jan. 15, 1987]

Sec. 21.21  Issue of type certificate: normal, utility, acrobatic, commuter,

and transport category aircraft; manned free balloons; special classes of

aircraft; aircraft engines; propellers.

An applicant is entitled to a type certificate for an aircraft in the

normal, utility, acrobatic, commuter, or transport category, or for a manned

free balloon, special class of aircraft, or an aircraft engine or propeller,

if--

(a) The product qualifies under Sec. 21.27; or

(b) The applicant submits the type design, test reports, and computations

necessary to show that the product to be certificated meets the applicable

airworthiness, aircraft noise, fuel venting, and exhaust emission

requirements of the Federal Aviation Regulations and any special conditions

prescribed by the Administrator, and the Administrator finds--

(1) Upon examination of the type design, and after completing all tests and

inspections, that the type design and the product meet the applicable noise,

fuel venting, and emissions requirements of the Federal Aviation Regulations,

and further finds that they meet the applicable airworthiness requirements of

the Federal Aviation Regulations or that any airworthiness provisions not

complied with are compensated for by factors that provide an equivalent level

of safety; and

(2) For an aircraft, that no feature or characteristic makes it unsafe for

the category in which certification is requested.

[Doc. No. 5085, 29 FR 14564, Oct. 24, 1964, as amended by Amdt. 21-15, 32 FR

3735, Mar. 4, 1967; Amdt. 21-27, 34 FR 18368, Nov. 18, 1969; Amdt. 21-60, 52

FR 8042, Mar. 13, 1987; Amdt. 21-68, 55 FR 32860, Aug. 10, 1990]

*****************************************************************************

55 FR 32856, No. 155, Aug. 10, 1990

SUMMARY: This final rule codifies as new part 34 all of the applicable

aircraft engine fuel venting and exhaust emission requirements of Special

Federal Aviation Regulation (SFAR) 27-5, and the test procedures specified

under the regulations implementing the Clean Air Act. This rule consolidates

all of the requirements and test procedures into this part, and inserts into

other affected parts the requirements to comply with new part 34. New part 34

does not alter any of the requirements specified under SFAR 27-5 or the

regulations implementing the Clean Air Act.

EFFECTIVE DATE: September 10, 1990.

*****************************************************************************

Sec. 21.23  [Reserved]

Sec. 21.24  Issuance of type certificate: primary category aircraft.

(a) The applicant is entitled to a type certificate for an aircraft in the

primary category if--

(1) The aircraft--

(i) Is unpowered; is an airplane powered by a single, naturally aspirated

engine with a 61-knot or less Vso stall speed as defined in Sec. 23.49; or is

a rotorcraft with a 6-pound per square foot main rotor disc loading

limitation, under sea level standard day conditions;

(ii) Weighs not more than 2,700 pounds;

(iii) Has a maximum seating capacity of not more than four persons,

including the pilot; and

(iv) Has an unpressurized cabin.

(2) The applicant has submitted--

(i) Except as provided by paragraph (c) of this section, a statement, in a

form and manner acceptable to the Administrator, certifying that: the

applicant has completed the engineering analysis necessary to demonstrate

compliance with the applicable airworthiness requirements; the applicant has

conducted appropriate flight, structural, propulsion, and systems tests

necessary to show that the aircraft, its components, and its equipment are

reliable and function properly; the type design complies with the

airworthiness standards and noise requirements established for the aircraft

under Sec. 21.17(f); and no feature or characteristic makes it unsafe for its

intended use;

(ii) The flight manual required by Sec. 21.5(b), including any information

required to be furnished by the applicable airworthiness standards;

(iii) Instructions for continued airworthiness in accordance with Sec.

21.50(b); and

(iv) A report that: summarizes how compliance with each provision of the

type certification basis was determined; lists the specific documents in

which the type certification data information is provided; lists all

necessary drawings and documents used to define the type design; and lists

all the engineering reports on tests and computations that the applicant must

retain and make available under Sec. 21.49 to substantiate compliance with

the applicable airworthiness standards.

(3) The Administrator finds that--

(i) The aircraft complies with those applicable airworthiness requirements

approved under Sec. 21.17(f) of this part; and

(ii) The aircraft has no feature or characteristic that makes it unsafe for

its intended use.

(b) An applicant may include a special inspection and preventive

maintenance program as part of the aircraft's type design or supplemental

type design.

(c) For aircraft manufactured outside of the United States in a country

with which the United States has a bilateral airworthiness agreement for the

acceptance of these aircraft, and from which the aircraft is to be imported

into the United States--

(1) The statement required by paragraph (a)(2)(i) of this section must be

made by the civil airworthiness authority of the exporting country; and

(2) The required manuals, placards, listings, instrument markings, and

documents required by paragraphs (a) and (b) of this section must be

submitted in English.

[Amdt. 21-70, 57 FR 41367, Sept. 9, 1992]

*****************************************************************************

57 FR 41360, No. 175, Sept. 9, 1992

SUMMARY: This final rule establishes a new primary category of aircraft, and

new simplified procedures for type, production, and airworthiness

certification, and associated maintenance procedures. Aircraft in this

category are of simple design intended exclusively for pleasure and personal

use. Primary category aircraft (airplanes, gliders, rotorcraft, manned free

balloons, etc.) may be unpowered or powered by a single, naturally aspirated

engine, with a 61-knot or less stall speed limitation for airplanes and a 6-

pound per square foot main rotor disc loading limitation for rotorcraft.

Primary category aircraft may have a maximum certificated weight of no more

than 2,700 pounds, maximum seating capacity of four, and unpressurized

cabins. Although these aircraft may be available for rental and flight

instruction under certain conditions, the carrying of persons or property for

hire is prohibited. This final rule also adds a new section addressing the

falsification of documents submitted as part of certification for products

and parts.

EFFECTIVE DATE: December 31, 1992.

*****************************************************************************

Sec. 21.25  Issue of type certificate: Restricted category aircraft.

(a) An applicant is entitled to a type certificate for an aircraft in the

restricted category for special purpose operations if he shows compliance

with the applicable noise requirements of Part 36 of this chapter, and if he

shows that no feature or characteristic of the aircraft makes it unsafe when

it is operated under the limitations prescribed for its intended use, and

that the aircraft--

(1) Meets the airworthiness requirements of an aircraft category except

those requirments that the Administrator finds inappropriate for the special

purpose for which the aircraft is to be used; or

(2) Is of a type that has been manufactured in accordance with the

requirements of and accepted for use by, an Armed Force of the United States

and has been later modified for a special purpose.

(b) For the purposes of this section, "special purpose operations"

includes--

(1) Agricultural (spraying, dusting, and seeding, and livestock and

predatory animal control);

(2) Forest and wildlife conservation;

(3) Aerial surveying (photography, mapping, and oil and mineral

exploration);

(4) Patrolling (pipelines, power lines, and canals);

(5) Weather control (cloud seeding);

(6) Aerial advertising (skywriting, banner towing, airborne signs and

public address systems); and

(7) Any other operation specified by the Administrator.

[Doc. No. 5085, 29 FR 14564, Oct. 24, 1964, as amended by Amdt. 21-42, 40 FR

1033, Jan. 6, 1975]

Sec. 21.27  Issue of type certificate: surplus aircraft of the Armed Forces.

(a) Except as provided in paragraph (b) of this section an applicant is

entitled to a type certificate for an aircraft in the normal, utility,

acrobatic, commuter, or transport category that was designed and constructed

in the United States, accepted for operational use, and declared surplus by,

an Armed Force of the United States, and that is shown to comply with the

applicable certification requirements in paragraph (f) of this section.

(b) An applicant is entitled to a type certificate for a surplus aircraft

of the Armed Forces of the United States that is a counterpart of a

previously type certificated civil aircraft, if he shows compliance with the

regulations governing the original civil aircraft type certificate.

(c) Aircraft engines, propellers, and their related accessories installed

in surplus Armed Forces aircraft, for which a type certificate is sought

under this section, will be approved for use on those aircraft if the

applicant shows that on the basis of the previous military qualifications,

acceptance, and service record, the product provides substantially the same

level of airworthiness as would be provided if the engines or propellers were

type certificated under Part 33 or 35 of the Federal Aviation Regulations.

(d) The Administrator may relieve an applicant from strict compliance with

a specific provision of the applicable requirements in paragraph (f) of this

section, if the Administrator finds that the method of compliance proposed by

the applicant provides substantially the same level of airworthiness and that

strict compliance with those regulations would impose a severe burden on the

applicant. The Administrator may use experience that was satisfactory to an

Armed Force of the United States in making such a determination.

(e) The Administrator may require an applicant to comply with special

conditions and later requirements than those in paragraphs (c) and (f) of

this section, if the Administrator finds that compliance with the listed

regulations would not ensure an adequate level of airworthiness for the

aircraft.

(f) Except as provided in paragraphs (b) through (e) of this section, an

applicant for a type certificate under this section must comply with the

appropriate regulations listed in the following table:

Date accepted for

operational use by the

Armed Forces of the      Regulations that apply

Type of aircraft           United States                  /1/

Small reciprocating-      Before May 16, 1956       CAR Part 3, as effective

engine powered            After May 15, 1956        May 15, 1956.

airplanes                                           CAR Part 3, or FAR Part

23.

Small turbine engine-     Before Oct. 2, 1959       CAR Part 3, as effective

powered airplanes         After Oct. 1, 1959        Oct. 1, 1959.

CAR Part 3 or FAR Part

23.

Commuter category         After (Feb. 17, 1987)

airplanes                 FAR Part 23 as of (Feb.

17, 1987).

Large reciprocating-      Before Aug. 26, 1955      CAR Part 4b, as

engine powered            After Aug. 25, 1959       effective Aug. 25,

airplanes                                           1955.

CAR Part 4b or FAR Part

25.

Large turbine engine-     Before Oct. 2, 1959       CAR Part 4b, as

powered airplanes         After Oct. 1, 1959        effective Oct. 1, 1959.

CAR Part 4b or FAR Part

25.

Rotorcraft with maximum

certificated takeoff

weight of:

6,000 pounds or less     Before Oct. 2, 1959       CAR Part 6, as effective

After Oct. 1, 1959        Oct. 1, 1959.

CAR Part 6, or FAR Part

27.

Over 6,000 pounds        Before Oct. 2, 1959       CAR Part 7, as effective

After Oct. 1, 1959        Oct. 1, 1959.

CAR Part 7, or FAR Part

29.

/1/ Where no specific date is listed, the applicable regulations are those

in effect on the date that the first aircraft of the particular model was

accepted for operational use by the Armed Forces.

[Doc. No. 5085, 29 FR 14564, Oct. 24, 1964, as amended by Amdt. 21-59, 52 FR

1835, Jan. 15, 1987; 52 FR 7262, Mar. 9, 1987]

Sec. 21.29  Issue of type certificate: import products.

(a) A type certificate may be issued for a product that is manufactured in

a foreign country with which the United States has an agreement for the

acceptance of these products for export and import and that is to be imported

into the United States if--

(1) The country in which the product was manufactured certifies that the

product has been examined, tested, and found to meet--

(i) The applicable aircraft noise, fuel venting and exhaust emissions

requirements of this subchapter as designated in Sec. 21.17, or the

applicable aircraft noise, fuel venting and exhaust emissions requirements of

the country in which the product was manufactured, and any other requirements

the Administrator may prescribe to provide noise, fuel venting and exhaust

emission levels no greater than those provided by the applicable aircraft

noise, fuel venting, and exhaust emission requirements of this subchapter as

designated in Sec. 21.17; and

(ii) The applicable airworthiness requirements of this subchapter as

designated in Sec. 21.17, or the applicable airworthiness requirements of the

country in which the product was manufactured and any other requirements the

Administrator may prescribe to provide a level of safety equivalent to that

provided by the applicable airworthiness requirements of this subchapter as

designated in Sec. 21.17;

(2) The applicant has submitted the technical data, concerning aircraft

noise and airworthiness, respecting the product required by the

Administrator; and

(3) The manuals, placards, listings, and instrument markings required by

the applicable airworthiness (and noise, where applicable) requirements are

presented in the English language.

(b) A product type certificated under this section is considered to be type

certificated under the noise standards of part 36, and the fuel venting and

exhaust emission standards of part 34, of the Federal Aviation Regulations

where compliance therewith is certified under paragraph (a)(1)(i) of this

section, and under the airworthiness standards of that part of the Federal

Aviation Regulations with which compliance is certified under paragraph

(a)(1)(ii) of this section or to which an equivalent level of safety is

certified under paragraph (a)(1)(ii) of this section.

[Amdt. 21-27, 34 FR 18363, Nov. 18, 1969, as amended by Amdt. 21-68, 55 FR

32860, Aug. 10, 1990; 55 FR 37287, Sept. 10, 1990]

*****************************************************************************

55 FR 32856, No. 155, Aug. 10, 1990

SUMMARY: This final rule codifies as new part 34 all of the applicable

aircraft engine fuel venting and exhaust emission requirements of Special

Federal Aviation Regulation (SFAR) 27-5, and the test procedures specified

under the regulations implementing the Clean Air Act. This rule consolidates

all of the requirements and test procedures into this part, and inserts into

other affected parts the requirements to comply with new part 34. New part 34

does not alter any of the requirements specified under SFAR 27-5 or the

regulations implementing the Clean Air Act.

EFFECTIVE DATE: September 10, 1990.

*****************************************************************************

Sec. 21.31  Type design.

The type design consists of--

(a) The drawings and specifications, and a listing of those drawings and

specifications, necessary to define the configuration and the design features

of the product shown to comply with the requirements of that part of this

subchapter applicable to the product;

(b) Information on dimensions, materials, and processes necessary to define

the structural strength of the product;

(c) The Airworthiness Limitations section of the Instructions for Continued

Airworthiness as required by Parts 23, 25, 27, 29, 31, 33, and 35 of this

chapter, or as otherwise required by the Administrator; and as specified in

the applicable airworthiness criteria for special classes of aircraft

defined in Sec. 21.17(b); and

(d) For primary category aircraft, if desired, a special inspection and

preventive maintenance program designed to be accomplished by an

appropriately rated and trained pilot-owner.

(e) Any other data necessary to allow, by comparison, the determination of

the airworthiness, noise characteristics, fuel venting, and exhaust emissions

(where applicable) of later products of the same type.

[Doc. No. 5085, 29 FR 14564, Oct. 24, 1964, as amended by Amdt. 21-27, 34 FR

18363, Nov. 18, 1969; Amdt. 21-51, 45 FR 60170, Sept. 11, 1980; Amdt. 21-60,

52 FR 8042, Mar. 13, 1987; Amdt. 21-68, 55 FR 32860, Aug. 10, 1990; Amdt.

21-70, 57 FR 41368, Sept. 9, 1992]

*****************************************************************************

57 FR 41360, No. 175, Sept. 9, 1992

SUMMARY: This final rule establishes a new primary category of aircraft, and

new simplified procedures for type, production, and airworthiness

certification, and associated maintenance procedures. Aircraft in this

category are of simple design intended exclusively for pleasure and personal

use. Primary category aircraft (airplanes, gliders, rotorcraft, manned free

balloons, etc.) may be unpowered or powered by a single, naturally aspirated

engine, with a 61-knot or less stall speed limitation for airplanes and a 6-

pound per square foot main rotor disc loading limitation for rotorcraft.

Primary category aircraft may have a maximum certificated weight of no more

than 2,700 pounds, maximum seating capacity of four, and unpressurized

cabins. Although these aircraft may be available for rental and flight

instruction under certain conditions, the carrying of persons or property for

hire is prohibited. This final rule also adds a new section addressing the

falsification of documents submitted as part of certification for products

and parts.

EFFECTIVE DATE: December 31, 1992.

*****************************************************************************

Sec. 21.33  Inspection and tests.

(a) Each applicant must allow the Administrator to make any inspection and

any flight and ground test necessary to determine compliance with the

applicable requirements of the Federal Aviation Regulations. However, unless

otherwise authorized by the Administrator--

(1) No aircraft, aircraft engine, propeller, or part thereof may be

presented to the Administrator for test unless compliance with paragraphs

(b)(2) through (b)(4) of this section has been shown for that aircraft,

aircraft engine, propeller, or part thereof; and

(2) No change may be made to an aircraft, aircraft engine, propeller, or

part thereof between the time that compliance with paragraphs (b)(2) through

(b)(4) of this section is shown for that aircraft, aircraft engine,

propeller, or part thereof and the time that it is presented to the

Administrator for test.

(b) Each applicant must make all inspections and tests necessary to

determine--

(1) Compliance with the applicable airworthiness, aircraft noise, fuel

venting, and exhaust emission requirements;

(2) That materials and products conform to the specifications in the type

design;

(3) That parts of the products conform to the drawings in the type design;

and

(4) That the manufacturing processes, construction and assembly conform to

those specified in the type design.

[Doc. No. 5085, 29 FR 14564, Oct. 24, 1964, as amended by Amdt. 21-17, 32 FR

14926, Oct. 28, 1967; Amdt. 21-27, 34 FR 18363, Nov. 18, 1969; Amdt. 21-44,

41 FR 55463, Dec. 20, 1976; Amdt. 21-68, 55 FR 32860, Aug. 10, 1990]

*****************************************************************************

55 FR 32856, No. 155, Aug. 10, 1990

SUMMARY: This final rule codifies as new part 34 all of the applicable

aircraft engine fuel venting and exhaust emission requirements of Special

Federal Aviation Regulation (SFAR) 27-5, and the test procedures specified

under the regulations implementing the Clean Air Act. This rule consolidates

all of the requirements and test procedures into this part, and inserts into

other affected parts the requirements to comply with new part 34. New part 34

does not alter any of the requirements specified under SFAR 27-5 or the

regulations implementing the Clean Air Act.

EFFECTIVE DATE: September 10, 1990.

*****************************************************************************

Sec. 21.35  Flight tests.

(a) Each applicant for an aircraft type certificate (other than under Secs.

21.24 through 21.29) must make the tests listed in paragraph (b) of this

section. Before making the tests the applicant must show--

(1) Compliance with the applicable structural requirements of this

subchapter;

(2) Completion of necessary ground inspections and tests;

(3) That the aircraft conforms with the type design; and

(4) That the Administrator received a flight test report from the applicant

(signed, in the case of aircraft to be certificated under Part 25 [New] of

this chapter, by the applicant's test pilot) containing the results of his

tests.

(b) Upon showing compliance with paragraph (a) of this section, the

applicant must make all flight tests that the Administrator finds necessary--

(1) To determine compliance with the applicable requirements of this

subchapter; and

(2) For aircraft to be certificated under this subchapter, except gliders

and except airplanes of 6,000 lbs. or less maximum certificated weight that

are to be certificated under Part 23 of this chapter, to determine whether

there is reasonable assurance that the aircraft, its components, and its

equipment are reliable and function properly.

(c) Each applicant must, if practicable, make the tests prescribed in

paragraph (b)(2) of this section upon the aircraft that was used to show

compliance with--

(1) Paragraph (b)(1) of this section; and

(2) For rotorcraft, the rotor drive endurance tests prescribed in Sec.

27.923 or Sec. 29.923 of this chapter, as applicable.

(d) Each applicant must show for each flight test (except in a glider or a

manned free balloon) that adequate provision is made for the flight test crew

for emergency egress and the use of parachutes.

(e) Except in gliders and manned free balloons, an applicant must

discontinue flight tests under this section until he shows that corrective

action has been taken, whenever--

(1) The applicant's test pilot is unable or unwilling to make any of the

required flight tests; or

(2) Items of noncompliance with requirements are found that may make

additional test data meaningless or that would make further testing unduly

hazardous.

(f) The flight tests prescribed in paragraph (b)(2) of this section must

include--

(1) For aircraft incorporating turbine engines of a type not previously

used in a type certificated aircraft, at least 300 hours of operation with a

full complement of engines that conform to a type certificate; and

(2) For all other aircraft, at least 150 hours of operation.

[Doc. No. 5085, 29 FR 14564, Oct. 24, 1964, as amended by Amdt. No. 21-40, 39

FR 35459, Oct. 1, 1974; Amdt. 21-51, 45 FR 60170, Sept. 11, 1980; Amdt.

21-70, 57 FR 41368, Sept. 9, 1992]

*****************************************************************************

57 FR 41360, No. 175, Sept. 9, 1992

SUMMARY: This final rule establishes a new primary category of aircraft, and

new simplified procedures for type, production, and airworthiness

certification, and associated maintenance procedures. Aircraft in this

category are of simple design intended exclusively for pleasure and personal

use. Primary category aircraft (airplanes, gliders, rotorcraft, manned free

balloons, etc.) may be unpowered or powered by a single, naturally aspirated

engine, with a 61-knot or less stall speed limitation for airplanes and a 6-

pound per square foot main rotor disc loading limitation for rotorcraft.

Primary category aircraft may have a maximum certificated weight of no more

than 2,700 pounds, maximum seating capacity of four, and unpressurized

cabins. Although these aircraft may be available for rental and flight

instruction under certain conditions, the carrying of persons or property for

hire is prohibited. This final rule also adds a new section addressing the

falsification of documents submitted as part of certification for products

and parts.

EFFECTIVE DATE: December 31, 1992.

*****************************************************************************

Sec. 21.37  Flight test pilot.

Each applicant for a normal, utility, acrobatic, commuter, or transport

category aircraft type certificate must provide a person holding an

appropriate pilot certificate to make the flight tests required by this part.

[Doc. No. 5085, 29 FR 14564, Oct. 24, 1964, as amended by Amdt. 21-59, 52 FR

1835, Jan. 15, 1987]

Sec. 21.39  Flight test instrument calibration and correction report.

(a) Each applicant for a normal, utility, acrobatic, commuter, or transport

category aircraft type certificate must submit a report to the Administrator

showing the computations and tests required in connection with the

calibration of instruments used for test purposes and in the correction of

test results to standard atmospheric conditions.

(b) Each applicant must allow the Administrator to conduct any flight tests

that he finds necessary to check the accuracy of the report submitted under

paragraph (a) of this section.

[Doc. No. 5085, 29 FR 14564, Oct. 24, 1964, as amended by Amdt. 21-59, 52 FR

1835, Jan. 15, 1987]

Sec. 21.41  Type certificate.

Each type certificate is considered to include the type design, the

operating limitations, the certificate data sheet, the applicable regulations

of this subchapter with which the Administrator records compliance, and any

other conditions or limitations prescribed for the product in this

subchapter.

Sec. 21.43  Location of manufacturing facilities.

Except as provided in Sec. 21.29, the Administrator does not issue a type

certificate if the manufacturing facilities for the product are located

outside of the United States, unless the Administrator finds that the

location of the manufacturer's facilities places no undue burden on the FAA

in administering applicable airworthiness requirements.

Sec. 21.45  Privileges.

The holder or licensee of a type certificate for a product may--

(a) In the case of aircraft, upon compliance with Secs. 21.173 through

21.189, obtain airworthiness certificates;

(b) In the case of aircraft engines or propellers, obtain approval for

installation or certified aircraft;

(c) In the case of any product, upon compliance with Secs. 21.133 through

21.163, obtain a production certificate for the type certificated product;

(d) Obtain approval of replacement parts for that product.

Sec. 21.47  Transferability.

A type certificate may be transferred to or made available to third persons

by licensing agreements. Each grantor shall, within 30 days after the

transfer of a certificate or execution or termination of a licensing

agreement, notify in writing the appropriate Aircraft Certification Office.

The notification must state the name and address of the transferee or

licensee, date of the transaction, and in the case of a licensing agreement,

the extent of authority granted the licensee.

Sec. 21.49  Availability.

The holder of a type certificate shall make the certificate available for

examination upon the request of the Administrator or the National

Transportation Safety Board.

[Doc. No. 5085, 29 FR 14564, Oct. 24, 1964, as amended by Doc. No. 8084, 32

FR 5769, Apr. 11, 1967]

Sec. 21.50  Instructions for continued airworthiness and manufacturer's

maintenance manuals having airworthiness limitations sections.

(a) The holder of a type certificate for a rotorcraft for which a

Rotorcraft Maintenance Manual containing an "Airworthiness Limitations"

section has been issued under Sec. 27.1529 (a)(2) or Sec. 29.1529 (a)(2) of

this chapter, and who obtains approval of changes to any replacement time,

inspection interval, or related procedure in that section of the manual,

shall make those changes available upon request to any operator of the same

type of rotorcraft.

(b) The holder of a design approval, including either the type certificate

or supplemental type certificate for an aircraft, aircraft engine, or

propeller for which application was made after January 28, 1981, shall

furnish at least one set of complete Instructions for Continued

Airworthiness, prepared in accordance with Secs. 23.1529, 25.1529, 27.1529,

29.1529, 31.82, 33.4, or 35.4 of this chapter, or as specified in the

applicable airworthiness criteria for special classes of aircraft defined in

Sec. 21.17(b), as applicable, to the owner of each type of aircraft, aircraft

engine, or propeller upon its delivery, or upon issuance of the first

standard airworthiness certificate for the affected aircraft, whichever

occurs later, and thereafter make those instructions available to any other

person required by this chapter to comply with any of the terms of these

instructions. In addition, changes to the Instructions for Continued

Airworthiness shall be made available to any person required by this chapter

to comply with any of those instructions.

[Amdt. No. 21-23, 33 FR 14105, Sept. 18, 1968, as amended by Amdt. No 21-51,

45 FR 60170, Sept. 11, 1980; Amdt. 21-60, 52 FR 8042, Mar. 13, 1987]

Sec. 21.51  Duration.

A type certificate is effective until surrendered, suspended, revoked, or a

termination date is otherwise established by the Administrator.

Sec. 21.53  Statement of conformity.

(a) Each applicant must submit a statement of conformity (FAA Form 317) to

the Administrator for each aircraft engine and propeller presented to the

Administrator for type certification. This statement of conformity must

include a statement that the aircraft engine or propeller conforms to the

type design therefor.

(b) Each applicant must submit a statement of conformity to the

Administrator for each aircraft or part thereof presented to the

Administrator for tests. This statement of conformity must include a

statement that the applicant has complied with Sec. 21.33(a) (unless

otherwise authorized under that paragraph).

[Amdt. 21-17, 32 FR 14926, Oct. 28, 1967]

Subpart C--Provisional Type Certificates

Source: Docket No. 5085, 29 FR 14566, Oct. 24, 1964, unless otherwise

noted.

Sec. 21.71  Applicability.

This subpart prescribes--

(a) Procedural requirements for the issue of provisional type certificates,

amendments to provisional type certificates, and provisional amendments to

type certificates; and

(b) Rules government the holders of those certificates.

Sec. 21.73  Eligibility.

(a) Any manufacturer of aircraft manufactured within the United States who

is a United States citizen may apply for Class I or Class II provisional type

certificates, for amendments to provisional type certificates held by him,

and for provisional amendments to type certificates held by him.

(b) Any manufacturer of aircraft manufactured in a foreign country with

which the United States has an agreement for the acceptance of those aircraft

for export and import may apply for a Class II provisional type certificate,

for amendments to provisional type certificates held by him, and for

provisional amendments to type certificates held by him.

(c) An aircraft engine manufacturer who is a United States citizen and who

has altered a type certificated aircraft by installing different type

certificated aircraft engines manufactured by him within the United States

may apply for a Class I provisional type certificate for the aircraft, and

for amendments to Class I provisional type certificates held by him, if the

basic aircraft, before alteration, was type certificated in the normal,

utility, acrobatic, commuter, or transport category.

[Doc. No. 5085, 29 FR 14566, Oct. 24, 1964, as amended by Amdt. 21-12, 31 FR

13380, Oct. 15, 1966; Amdt. 21-59, 52 FR 1836, Jan. 15, 1987]

Sec. 21.75   Application.

Applications for provisional type certificates, for amendments thereto, and

for provisional amendments to type certificates must be submitted to the

Manager of the Aircraft Certification Office for the geographic area in which

the applicant is located (or in the case of European, African, Middle East

Region, the Manager, Aircraft Engineering Division), and must be accompanied

by the pertinent information specified in this subpart.

[Amdt. 21-67, 54 FR 39291, Sept. 25, 1989]

Sec. 21.77  Duration.

(a) Unless sooner surrendered, superseded, revoked, or otherwise

terminated, provisional type certificates and amendments thereto are

effective for the periods specified in this section.

(b) A Class I provisional type certificate is effective for 24 months after

the date of issue.

(c) A Class II provisional type certificate is effective for twelve months

after the date of issue.

(d) An amendment to a Class I or Class II provisional type certificate is

effective for the duration of the amended certificate.

(e) A provisional amendment to a type certificate is effective for six

months after its approval or until the amendment of the type certificate is

approved, whichever is first.

[Doc. No. 5085, 29 FR 14566, Oct. 24, 1964 as amended by Amdt. 21-7, 30 FR

14311, Nov. 16, 1965]

Sec. 21.79  Transferability.

Provisional type certificates are not transferable.

Sec. 21.81  Requirements for issue and amendment of Class I provisional type

certificates.

(a) An applicant is entitled to the issue or amendment of a Class I

provisional type certificate if he shows compliance with this section and the

Administrator finds that there is no feature, characteristic, or condition

that would make the aircraft unsafe when operated in accordance with the

limitations established in paragraph (e) of this section and in Sec. 91.317

of this chapter.

(b) The applicant must apply for the issue of a type or supplemental type

certificate for the aircraft.

(c) The applicant must certify that--

(1) The aircraft has been designed and constructed in accordance with the

airworthiness requirements applicable to the issue of the type or

supplemental type certificate applied for;

(2) The aircraft substantially meets the applicable flight characteristic

requirements for the type or supplemental type certificate applied for; and

(3) The aircraft can be operated safely under the appropriate operating

limitations specified in paragraph (a) of this section.

(d) The applicant must submit a report showing that the aircraft had been

flown in all maneuvers necessary to show compliance with the flight

requirements for the issue of the type or supplemental type certificate

applied for, and to establish that the aircraft can be operated safely in

accordance with the limitations contained in this subchapter.

(e) The applicant must establish all limitations required for the issue of

the type or supplemental type certificate applied for, including limitations

on weights, speeds, flight maneuvers, loading, and operation of controls and

equipment unless, for each limitation not so established, appropriate

operating restrictions are established for the aircraft.

(f) The applicant must establish an inspection and maintenance program for

the continued airworthiness of the aircraft.

(g) The applicant must show that a prototyp@aircraft has been flown for a@

least 50 hours under an experimental certificate issued under Secs. 21.191

through 21.195, or under the auspices of an Armed Force of the United States.

However, in the case of an amendment to a provisional type certificate, the

Administrator may reduce the number of required flight hours.

[Doc. No. 5085, 29 FR 14566, Oct. 24, 1964, as amended by Amdt. 21-66, 54 FR

34329, Aug. 18, 1989]

Effective Date Note: At 54 FR 34329, Aug. 18, 1989, Sec. 21.81(a) was

amended by changing the cross reference "Sec. 91.41" to "91.317", effective

August 18, 1990.

Sec. 21.83  Requirements for issue and amendment of Class II provisional type

certificates.

(a) An applicant who manufactures aircraft within the United States is

entitled to the issue or amendment of a Class II provisional type certificate

if he shows compliance with this section and the Administrator finds that

there is no feature, characteristic, or condition that would make the

aircraft unsafe when operated in accordance with the limitations in paragraph

(h) of this section, and Secs. 91.317 and 121.207 of this chapter.

(b) An applicant who manufactures aircraft in a country with which the

United States has an agreement for the acceptance of those aircraft for

export and import is entitled to the issue or amendment of a Class II

provisional type certificate if the country in which the aircraft was

manufactured certifies that the applicant has shown compliance with this

section, that the aircraft meets the requirements of paragraph (f) of this

section and that there is no feature, characteristic, or condition that would

make the aircraft unsafe when operated in accordance with the limitations in

paragraph (h) of this section and Secs. 91.317 and 121.207 of this chapter.

(c) The applicant must apply for a type certificate, in the transport

category, for the aircraft.

(d) The applicant must hold a U.S. type certificate for at least one other

aircraft in the same transport category as the subject aircraft.

(e) The FAA's official flight test program or the flight test program

conducted by the authorities of the country in which the aircraft was

manufactured, with respect to the issue of a type certificate for that

aircraft, must be in progress.

(f) The applicant or, in the case of a foreign manufactured aircraft, the

country in which the aircraft was manufactured, must certify that--

(1) The aircraft has been designed and constructed in accordance with the

airworthiness requirements applicable to the issue of the type certificate

applied for;

(2) The aircraft substantially complies with the applicable flight

characteristic requirements for the type certificate applied for; and

(3) The aircraft can be operated safely under the appropriate operating

limitations in this subchapter.

(g) The applicant must submit a report showing that the aircraft has been

flown in all maneuvers necessary to show compliance with the flight

requirements for the issue of the type certificate and to establish that the

aircraft can be operated safely in accordance with the limitations in this

subchapter.

(h) The applicant must prepare a provisional aircraft flight manual

containing all limitations required for the issue of the type certificate

applied for, including limitations on weights, speeds, flight maneuvers,

loading, and operation of controls and equipment unless, for each limitation

not so established, appropriate operating restrictions are established for

the aircraft.

(i) The applicant must establish an inspection and maintenance program for

the continued airworthiness of the aircraft.

(j) The applicant must show that a prototype aircraft has been flown for at

least 100 hours. In the case of an amendment to a provisional type

certificate, the Administrator may reduce the number of required flight

hours.

[Amdt. 21-12, 31 FR 13386, Oct. 15, 1966, as amended by Amdt. 21-66, 54 FR

34329, Aug. 18, 1989]

Effective Date Note: At 54 FR 34329, Aug. 18, 1989, Sec. 21.81(a) and (b)

were amended by changing the cross reference "Sec. 91.41" to "Sec. 91.317" in

each paragraph, effective August 18, 1990.

Sec. 21.85  Provisional amendments to type certificates.

(a) An applicant who manufactures aircraft within the United States is

entitled to a provisional amendment to a type certificate if he shows

compliance with this section and the Administrator finds that there is no

feature, characteristic, or condition that would make the aircraft unsafe

when operated under the appropriate limitations contained in this subchapter.

(b) An applicant who manufactures aircraft in a foreign country with which

the United States has an agreement for the acceptance of those aircraft for

export and import is entitled to a provisional amendment to a type

certificate if the country in which the aircraft was manufactured certifies

that the applicant has shown compliance with this section, that the aircraft

meets the requirements of paragraph (e) of this section and that there is no

feature, characteristic, or condition that would make the aircraft unsafe

when operated under the appropriate limitations contained in this subchapter.

(c) The applicant must apply for an amendment to the type certificate.

(d) The FAA's official flight test program or the flight test program

conducted by the authorities of the country in which the aircraft was

manufactured, with respect to the amendment of the type certificate, must be

in progress.

(e) The applicant or, in the case of foreign manufactured aircraft, the

country in which the aircraft was manufactured, must certify that--

(1) The modification involved in the amendment to the type certificate has

been designed and constructed in accordance with the airworthiness

requirements applicable to the issue of the type certificate for the

aircraft;

(2) The aircraft substantially complies with the applicable flight

characteristic requirements for the type certificate; and

(3) The aircraft can be operated safely under the appropriate operating

limitations in this subchapter.

(f) The applicant must submit a report showing that the aircraft

incorporating the modifications involved has been flown in all maneuvers

necessary to show compliance with the flight requirements applicable to those

modifications and to establish that the aircraft can be operated safely in

accordance with the limitations specified in Secs. 91.317 and 121.207 of this

chapter.

(g) The applicant must establish and publish, in a provisional aircraft

flight manual or other document and on appropriate placards, all limitations

required for the issue of the type certificate applied for, including weight,

speed, flight maneuvers, loading, and operation of controls and equipment,

unless, for each limitation not so established, appropriate operating

restrictions are established for the aircraft.

(h) The applicant must establish an inspection and maintenance program for

the continued airworthiness of the aircraft.

(i) The applicant must operate a prototype aircraft modified in accordance

with the corresponding amendment to the type certificate for the number of

hours found necessary by the Administrator.

[Amdt. 21-12, 31 FR 13388, Oct. 15, 1966, as amended by Amdt. 21-66, 54 FR

34329, Aug. 18, 1989]

Effective Date Note: At 54 FR 34329, Aug. 18, 1989, Sec. 21.85(f) was

amended by changing the cross reference "Sec. 91.41" to "Sec. 91.317",

effective August 18, 1990.

Subpart D--Changes to Type Certificates

Source: Docket No. 5085, 29 FR 14567, Oct. 24, 1964, unless otherwise

noted.

Sec. 21.91  Applicability.

This subpart prescribes procedural requirements for the approval of changes

to type certificates.

Sec. 21.93  Classification of changes in type design.

(a) In addition to changes in type design specified in paragraph (b) of

this section, changes in type design are classified as minor and major. A

"minor change" is one that has no appreciable effect on the weight, balance,

structural strength, reliability, operational characteristics, or other

characteristics affecting the airworthiness of the product. All other changes

are "major changes" (except as provided in paragraph (b) of this section).

(b) For the purpose of complying with Part 36 of this chapter, and except

as provided in paragraphs (b)(2), (b)(3), and (b)(4) of this section, any

voluntary change in the type design of an aircraft that may increase the

noise levels of that aircraft is an "acoustical change" (in addition to being

a minor or major change as classified in paragraph (a) of this section) for

the following aircraft:

(1) Transport category large airplanes.

(2) Turbojet powered airplanes (regardless of category). For airplanes to

which this paragraph applies, "acoustical changes" do not include changes in

type design that are limited to one of the following--

(i) Gear down flight with one or more retractable landing gear down during

the entire flight, or

(ii) Spare engine and nacelle carriage external to the skin of the airplane

(and return of the pylon or other external mount), or

(iii) Time-limited engine and/or nacelle changes, where the change in type

design specifies that the airplane may not be operated for a period of more

than 90 days unless compliance with the applicable acoustical change

provisions of Part 36 of this chapter is shown for that change in type

design.

(3) Propeller driven commuter category and small airplanes in the primary,

normal, utility, acrobatic, transport, and restricted categories, except for

airplanes that are:

(i) Designated for "agricultural aircraft operations" (as defined in Sec.

137.3 of this chapter, effective January 1, 1966) to which Sec. 36.1583 of

this chapter does not apply, or

(ii) Designated for dispensing fire fighting materials to which Sec.

36.1583 of this chapter does not apply, or

(iii) U.S. registered, and that had flight time prior to January 1, 1955 or

(iv) Land configured aircraft reconfigured with floats or skis. This

reconfiguration does not permit further exception from the requirements of

this section upon any acoustical change not enumerated in Sec. 21.93(b).

(4) Helicopters, (except for those helicopters that are designated

exclusively for "agricultural aircraft operations", as defined in Sec. 137.3

of this chapter, as effective on January 1, 1966, for dispensing fire

fighting materials, or for carrying external loads, as defined in Sec.

133.1(b) of this chapter, as effective on December 20, 1976). For helicopters

to which this paragraph applies, "acoustical changes" include the following

type design changes:

(i) Any changes to, or removal of, a muffler or other component designed

for noise control.

(ii) Any other design or configuration change (including a change in the

operating limitations of the aircraft) that, based on FAA-approved analytical

or test data, the Administrator determines may result in an increase in noise

level.

(c) For purposes of complying with part 34 of this chapter, any voluntary

change in the type design of the airplane or engine which may increase fuel

venting or exhaust emissions is an "emissions change."

[Amdt. 21-27, 34 FR 18363, Nov. 18, 1969, as amended by Amdt. 21-42, 40 FR

1033, Jan. 6, 1975; Amdt. 21-47, 43 FR 28419, June 29, 1978; Amdt. 21-56, 47

FR 758, Jan. 7, 1982; Amdt. 21-61, 53 FR 3539, Feb. 5, 1988; Amdt. 21-62, 53

FR 16365, May 6, 1988; Amdt. 21-63, 53 FR 47399, Nov. 22, 1988; Amdt. 21-68,

55 FR 32860, Aug. 10, 1990; Amdt. 21-70, 57 FR 41368, Sept. 9, 1992]

*****************************************************************************

57 FR 41360, No. 175, Sept. 9, 1992

SUMMARY: This final rule establishes a new primary category of aircraft, and

new simplified procedures for type, production, and airworthiness

certification, and associated maintenance procedures. Aircraft in this

category are of simple design intended exclusively for pleasure and personal

use. Primary category aircraft (airplanes, gliders, rotorcraft, manned free

balloons, etc.) may be unpowered or powered by a single, naturally aspirated

engine, with a 61-knot or less stall speed limitation for airplanes and a 6-

pound per square foot main rotor disc loading limitation for rotorcraft.

Primary category aircraft may have a maximum certificated weight of no more

than 2,700 pounds, maximum seating capacity of four, and unpressurized

cabins. Although these aircraft may be available for rental and flight

instruction under certain conditions, the carrying of persons or property for

hire is prohibited. This final rule also adds a new section addressing the

falsification of documents submitted as part of certification for products

and parts.

EFFECTIVE DATE: December 31, 1992.

*****************************************************************************

Sec. 21.95  Approval of minor changes in type design.

Minor changes in a type design may be approved under a method acceptable to

the Administrator before submitting to the Administrator any substantiating

or descriptive data.

Sec. 21.97  Approval of major changes in type design.

(a) In the case of a major change in type design, the applicant must submit

substantiating data and necessary descriptive data for inclusion in the type

design.

(b) Approval of a major change in the type design of an aircraft engine is

limited to the specific engine configuration upon which the change is made

unless the applicant identifies in the necessary descriptive data for

inclusion in the type design the other configurations of the same engine type

for which approval is requested and shows that the change is compatible with

the other configurations.

[Amdt. 21-40, 39 FR 35459, Oct. 1, 1974]

Sec. 21.99  Required design changes.

(a) When an Airworthiness Directive is issued under Part 39 the holder of

the type certific@e for the product concerned must--

(1) If the Administrator finds that design changes are necessary to correct

the unsafe condition of the product, and upon his request, submit appropriate

design changes for approval; and

(2) Upon approval of the design changes, make available the descriptive

data covering the changes to all operators of products previously

certificated under the type certificate.

(b) In a case where there are no current unsafe conditions, but the

Administrator or the holder of the type certificate finds through service

experience that changes in type design will contribute to the safety of the

product, the holder of the type certificate may submit appropriate design

changes for approval. Upon approval of the changes, the manufacturer shall

make information on the design changes available to all operators of the same

type of product.

[Doc. No. 5085, 29 FR 14567, Oct. 24, 1964, as amended by Amdt. 21-3, 30 FR

8826, July 24, 1965]

Sec. 21.101  Designation of applicable regulations.

(a) Except as provided in Secs. 23.2, 25.2, 27.2, 29.2 and parts 34 and

36 of this chapter, an applicant for a change to a type certificate must

comply with either--

(1) The regulations incorporated by reference in the type certificate; or

(2) The applicable regulations in effect on the date of the application,

plus any other amendments the Administrator finds to be directly related.

(b) If the Administrator finds that a proposed change consists of a new

design or a substantially complete redesign of a component, equipment

installation, or system installation, and that the regulations incorporated

by reference in the type certificate for the product do not provide adequate

standards with respect to the proposed change, the applicant must comply

with--

(1) The applicable provisions of this subchapter, in effect on the date of

the application for the change, that the Administrator finds necessary to

provide a level of safety equal to that established by the regulations

incorporated by reference in the type certificate for the product; and

(2) Any special conditions, and amendments to those special conditions,

prescribed by the Administrator to provide a level of safety equal to that

established by the regulations incorporated by reference in the type

certificate for the product.

(c) Unless otherwise required by Sec. 21.19(a), an applicant for a change

to a type certificate for a transport category airplane involving the

replacement of reciprocating engines with the same number of turbopropeller

powerplants must comply with the requirements of Part 25 of this chapter

applicable to the airplane as type certificated with reciprocating engines,

and with the following:

(1) The certification performance requirements prescribed in Secs. 25.101

through 25.125 and 25.149, 25.1533, 25.1583, and 25.1587.

(2) The powerplant requirements of Part 25 of this chapter applicable to

turbopropeller engine-powered airplanes.

(3) The requirements of Part 25 of this chapter for the standardization of

cockpit controls and instruments, unless the Administrator finds that

compliance with a particular detailed requirement would be impractical and

would not contribute materially to standardization.

(4) Any other requirement of Part 25 of this chapter applicable to

turbopropeller engine-powered airplanes that the Administrator finds to be

related to the changes in engines and that are necessary to ensure a level of

safety equal to that of the airplane certificated with reciprocating engines.

For each new limitation established with respect to weight, speed, or

altitude that is significantly altered from those approved for the airplane

with reciprocating engines, the applicant must show compliance with the

requirements of Part 25 of this chapter applicable to the limitations being

changed.

[Doc. No. 5085, 29 FR 14567, Oct. 24, 1964, as amended by Amdt. 21-16, 32 FR

13262, Sept. 20, 1967; Amdt. 21-19, 32 FR 17851, Dec. 13, 1967; Amdt. 21-27,

34 FR 18363, Nov. 18, 1969; Amdt. 21-42, 40 FR 1033, Jan. 6, 1975; Amdt. 21-

58, 50 FR 46877, Nov. 13, 1985; Amdt. 21-68, 55 FR 32860, Aug. 10, 1990;

Admt. 21-69, 56 FR 41051, Aug. 16, 1991]

*****************************************************************************

56 FR 41048, No. 159, Aug. 16, 1991

SUMMARY: This final rule amends the airworthiness and operating

regulations to require installation and use of shoulder harnesses at all

seats of rotorcraft manufactured after September 16, 1992. These amendments

respond to a safety recommendation from the National Transportation Safety

Board and are intended to enhance protection of occupants in rotorcraft.

DATES: Effective date: September 16, 1991.

Compliance date: September 16, 1992.

*****************************************************************************

Subpart E--Supplemental Type Certificates

Source: Docket No. 5085, 29 FR 14568, Oct. 24, 1964, unless otherwise

noted.

Sec. 21.111  Applicability.

This subpart prescribes procedural requirements for the issue of

supplemental type certificates.

Sec. 21.113  Requirement of supplemental type certificate.

Any person who alters a product by introducing a major change in type

design, not great enough to require a new application for a type certificate

under Sec. 21.19, shall apply to the Administrator for a supplemental type

certificate, except that the holder of a type certificate for the product may

apply for amendment of the original type certificate. The application must be

made in a form and manner prescribed by the Administrator.

Sec. 21.115  Applicable requirements.

(a) Each applicant for a supplemental type certificate must show that the

altered product meets applicable airworthiness requirements as specified in

paragraphs (a) and (b) of Sec. 21.101 and, in the case of an acoustical

change described in Sec. 21.93(b), show compliance with the applicable noise

requirements of part 36 of this chapter and, in the case of an emissions

change described in Sec. 21.93(c), show compliance with the applicable fuel

venting and exhaust emissions requirements of part 34 of this chapter.

(b) Each applicant for a supplemental type certificate must meet Secs.

21.33 and 21.53 with respect to each change in the type design.

[Amdt. 21-17, 32 FR 14927, Oct. 28, 1967, as amended by Amdt. 21-42, 40 FR

1033, Jan. 6, 1975; Amdt. 21-52A, 45 FR 79009, Nov. 28, 1980; Amdt. 21-61, 53

FR 3540, Feb. 5, 1988; Amdt. 21-68, 55 FR 32860, Aug. 10, 1990; Amdt. 21-71,

57 FR 42854, Sept. 16, 1992]

*****************************************************************************

57 FR 42846, No. 180, Sept. 16, 1992

SUMMARY: This final rule adds a new appendix to the noise standards

regulations. The new appendix provides for an alternative noise certification

procedure for primary, normal, transport, and restricted category helicopters

not exceeding 6,000 pounds maximum takeoff weight. The new appendix is an

optional alternative to existing helicopter noise requirements and is not an

additional regulatory requirement. Applicants for certification may

demonstrate compliance with the noise standards of either Appendix H or the

less costly but more stringent new Appendix J. The new certification

procedure is intended to provide regulatory relief to manufacturers of light

helicopters by substantially reducing the costs of demonstrating compliance

with the noise regulations.

DATES: Effective September 11, 1992. The incorporation by reference of

certain publications listed in the rule is approved by the Director of the

Federal Register as of September 16, 1992.

*****************************************************************************

Sec. 21.117  Issue of supplemental type certificates.

(a) An applicant is entitled to a supplemental type certificate if he meets

the requirements of Secs. 21.113 and 21.115.

(b) A supplemental type certificate consists of--

(1) The approval by the Administrator of a change in the type design of the

product; and

(2) The type certificate previously issued for the product.

Sec. 21.119  Privileges.

The holder of a supplemental type certificate may--

(a) In the case of aircraft, obtain airworthiness certificates;

(b) In the case of other products, obtain approval for installation on

certificated aircraft; and

(c) Obtain a production certificate for the change in the type design that

was approved by that supplemental type certificate.

Subpart F--Production Under Type Certificate Only

Source: Docket No. 5085, 29 FR 14568, Oct. 24, 1964, unless otherwise

noted.

Sec. 21.121  Applicability.

This subpart prescribes rules for production under a type certificate only.

Sec. 21.123  Production under type certificate.

Each manufacturer of a product being manufactured under a type certificate

only shall--

(a) Make each product available for inspection by the Administrator;

(b) Maintain at the place of manufacture the technical data and drawings

necessary for the Administrator to determine whether the product and its

parts conform to the type design;

(c) Except as otherwise authorized by the Aircraft Certification

Directorate Manager for the geographic area which the manufacturer is

located, for products manufactured more than 6 months after the date of issue

of the type certificate, establish and maintain an approved production

inspection system that insures that each product conforms to the type design

and is in condition for safe operation; and

(d) Upon the establishment of the approved production inspection system (as

required by paragraph (c) of this section) submit to the Administrator a

manual that describes that system and the means for making the determinations

required by Sec. 21.125(b).

[Doc. No. 5085, 29 FR 14568, Oct. 24, 1964, as amended by Amdt. 21-34, 35 FR

13008, Aug. 15, 1970; Amdt. 21-51, 45 FR 60170, Sept. 11, 1980; Amdt. 21-67,

54 FR 39291, Sept. 25, 1989]

Sec. 21.125  Production inspection system: Materials Review Board.

(a) Each manufacturer required to establish a production inspection system

by Sec. 21.123(c) shall--

(1) Establish a Materials Review Board (to include representatives from the

inspection and engineering departments) and materials review procedures; and

(2) Maintain complete records of Materials Review Board action for at least

two years.

(b) The production inspection system required in Sec. 21.123(c) must

provide a means for determining at least the following:

(1) Incoming materials, and bought or subcontracted parts, used in the

finished product must be as specified in the type design data, or must be

suitable equivalents.

(2) Incoming materials, and bought or subcontracted parts, must be properly

identified if their physical or chemical properties cannot be readily and

accurately determined.

(3) Materials subject to damage and deterioration must be suitably stored

and adequately protected.

(4) Processes affecting the quality and safety of the finished product must

be accomplished in accordance with acceptable industry or United States

specifications.

(5) Parts and components in process must be inspected for conformity with

the type design data at points in production where accurate determinations

can be made.

(6) Current design drawings must be readily available to manufacturing and

inspection personnel, and used when necessary.

(7) Design changes, including material substitutions, must be controlled

and approved before being incorporated in the finished product.

(8) Rejected materials and parts must be segregated and identified in a

manner that precludes installation in the finished product.

(9) Materials and parts that are withheld because of departures from design

data or specifications, and that are to be considered for installation in the

finished product, must be processed through the Materials Review Board. Those

materials and parts determined by the Board to be serviceable must be

properly identified and reinspected if rework or repair is necessary.

Materials and parts rejected by the Board must be marked and disposed of to

ensure that they are not incorporated in the final product.

(10) Inspection records must be maintained, identified with the completed

product where practicable, and retained by the manufacturer for at least two

years.

Sec. 21.127  Tests: aircraft.

(a) Each person manufacturing aircraft under a type certificate only shall

establish an approved production flight test procedure and flight check-off

form, and in accordance with that form, flight test each aircraft produced.

(b) Each production flight test procedure must include the following:

(1) An operational check of the trim, controllability, or other flight

characteristics to establish that the production aircraft has the same range

and degree of control as the prototype aircraft.

(2) An operational check of each part or system operated by the crew while

in flight to establish that, during flight, instrument readings are within

normal range.

(3) A determination that all instruments are properly marked, and that all

placards and required flight manuals are installed after flight test.

(4) A check of the operational characteristics of the aircraft on the

ground.

(5) A check on any other items peculiar to the aircraft being tested that

can best be done during the ground or flight operation of the aircraft.

Sec. 21.128  Tests: aircraft engines.

(a) Each person manufacturing aircraft engines under a type certificate

only shall subject each engine (except rocket engines for which the

manufacturer must establish a sampling technique) to an acceptable test run

that includes the following:

(1) Break-in runs that include a determination of fuel and oil consumption

and a determination of power characteristics at rated maximum continuous

power or thrust and, if applicable, at rated takeoff power or thrust.

(2) At least five hours of operation at rated maximum continuous power or

thrust. For engines having a rated takeoff power or thrust higher than rated

maximum continuous power or thrust, the five-hour run must include 30 minutes

at rated takeoff power or thrust.

(b) The test runs required by paragraph (a) of this section may be made

with the engine appropriately mounted and using current types of power and

thrust measuring equipment.

[Doc. No. 5085, 29 FR 14568, Oct. 24, 1964, as amended by Amdt. 21-5, 32 FR

3735, Mar. 4, 1967]

Sec. 21.129  Tests: propellers.

Each person manufacturing propellers under a type certificate only shall

give each variable pitch propeller an acceptable functional test to determine

if it operates properly throughout the normal range of operation.

Sec. 21.130  Statement of conformity.

Each holder or licensee of a type certificate only, for a product

manufactured in the United States, shall, upon the initial transfer by him of

the ownership of such product manufactured under that type certificate, or

upon application for the original issue of an aircraft airworthiness

certificate or an aircraft engine or propeller airworthiness approval tag

(FAA Form 8130-3), give the Administrator a statement of conformity (FAA Form

317). This statement must be signed by an authorized person who holds a

responsible position in the manufacturing organization, and must include--

(a) For each product, a statement that the product conforms to its type

certificate and is in condition for safe operation;

(b) For each aircraft, a statement that the aircraft has been flight

checked; and

(c) For each aircraft engine or variable pitch propeller, a statement that

the engine or propeller has been subjected by the manufacturer to a final

operational check.

However, in the case of a product manufactured for an Armed Force of the

United States, a statement of conformity is not required if the product has

been accepted by that Armed Force.

[Amdt. 21-25, 34 FR 14068, Sept. 5, 1969]

Subpart G--Production Certificates

Source: Docket No. 5085, 29 FR 14569, Oct. 24, 1964, unless otherwise

noted.

Sec. 21.131  Applicability.

This subpart prescribes procedural requirements for the issue of production

certificates and rules governing the holders of those certificates.

Sec. 21.133  Eligibility.

(a) Any person may apply for a production certificate if he holds, for the

product concerned, a--

(1) Current type certificate;

(2) Right to the benefits of that type certificate under a licensing

agreement; or

(3) Supplemental type certificate.

(b) Each application for a production certificate must be made in a form

and manner prescribed by the Administrator.

Sec. 21.135  Requirements for issuance.

An applicant is entitled to a production certificate if the Administrator

finds, after examination of the supporting data and after inspection of the

organization and production facilities, that the applicant has complied with

Secs. 21.139 and 21.143.

Sec. 21.137  Location of manufacturing facilities.

The Administrator does not issue a production certificate if the

manufacturing facilities concerned are located outside the United States,

unless the Administrator finds no undue burden on the United States in

administering the applicable requirements of the Federal Aviation Act of 1958

or of the Federal Aviation Regulations.

Sec. 21.139  Quality control.

The applicant must show that he has established and can maintain a quality

control system for any product, for which he requests a production

certificate, so that each article will meet the design provisions of the

pertinent type certificate.

Sec. 21.143  Quality control data requirements; prime manufacturer.

(a) Each applicant must submit, for approval, data describing the

inspection and test procedures necessary to ensure that each article produced

conforms to the type design and is in a condition for safe operation,

including as applicable--

(1) A statement describing assigned responsibilities and delegated

authority of the quality control organization, together with a chart

indicating the functional relationship of the quality control organization to

management and to other organizational components, and indicating the chain

of authority and responsibility within the quality control organization;

(2) A description of inspection procedures for raw materials, purchased

items, and parts and assemblies produced by manufacturers' suppliers

including methods used to ensure acceptable quality of parts and assemblies

that cannot be completely inspected for conformity and quality when delivered

to the prime manufacturer's plant;

(3) A description of the methods used for production inspection of

individual parts and complete assemblies, including the identification of any

special manufacturing processes involved, the means used to control the

processes, the final test procedure for the complete product, and, in the

case of aircraft, a copy of the manufacturer's production flight test

procedures and checkoff list;

(4) An outline of the materials review system, including the procedure for

recording review board decisions and disposing of rejected parts;

(5) An outline of a system for informing company inspectors of current

changes in engineering drawings, specifications, and quality control

procedures; and

(6) A list or chart showing the location and type of inspection stations.

(b) Each prime manufacturer shall make available to the Administrator

information regarding all delegation of authority to suppliers to make major

inspections of parts or assemblies for which the prime manufacturer is

responsible.

[Doc. No. 5085, 29 FR 14569, Oct. 24, 1964, as amended by Amdt. 21-51, 45 FR

60170, Sept. 11, 1980]

Sec. 21.147  Changes in quality control system.

After the issue of a production certificate, each change to the quality

control system is subject to review by the Administrator. The holder of a

production certificate shall immediately notify the Administrator, in writing

of any change that may affect the inspection, conformity, or airworthiness of

the product.

Sec. 21.149  Multiple products.

The Administrator may authorize more than one type certificated product to

be manufactured under the terms of one production certificate, if the

products have similar production characteristics.

Sec. 21.151  Production limitation record.

A production limitation record is issued as part of a production

certificate. The record lists the type certificate of every product that the

applicant is authorized to manufacture under the terms of the production

certificate.

Sec. 21.153  Amendment of the production certificates.

The holder of a production certificate desiring to amend it to add a type

certificate or model, or both, must apply therefor in a form and manner

prescribed by the Administrator. The applicant must comply with the

applicable requirements of Secs. 21.139, 21.143, and 21.147.

Sec. 21.155  Transferability.

A production certificate is not transferable.

Sec. 21.157  Inspections and tests.

Each holder of a production certificate shall allow the Administrator to

make any inspections and tests necessary to determine compliance with the

applicable regulations in this subchapter.

Sec. 21.159  Duration.

A production certificate is effective until surrendered, suspended,

revoked, or a termination date is otherwise established by the Administrator,

or the location of the manufacturing facility is changed.

Sec. 21.161  Display.

The holder of a production certificate shall display it prominently in the

main office of the factory in which the product concerned is manufactured.

Sec. 21.163    Privileges.

(a) The holder of a production certificate may--

(1) Obtain an aircraft airworthiness certificate without further showing,

except that the Administrator may inspect the aircraft for conformity with

the type design; or

(2) In the case of other products, obtain approval for installation on type

certificated aircraft.

(b) Notwithstanding the provisions of Sec. 147.3 of this chapter, the

holder of a production certificate for a primary category aircraft, or for a

normal, utility, or acrobatic category aircraft of a type design that is

eligible for a special airworthiness certificate in the primary category

under Sec. 21.184(c), may--

(1) Conduct training for persons in the performance of a special inspection

and preventive maintenance program approved as a part of the aircraft's type

design under Sec. 21.24(b), provided the training is given by a person

holding a mechanic certificate with appropriate airframe and powerplant

ratings issued under part 65 of this chapter; and

(2) Issue a certificate of competency to persons successfully completing

the approved training program, provided the certificate specifies the

aircraft make and model to which the certificate applies.

[Amdt. 21-70, 57 FR 41368, Sept. 9, 1992]

*****************************************************************************

57 FR 41360, No. 175, Sept. 9, 1992

SUMMARY: This final rule establishes a new primary category of aircraft, and

new simplified procedures for type, production, and airworthiness

certification, and associated maintenance procedures. Aircraft in this

category are of simple design intended exclusively for pleasure and personal

use. Primary category aircraft (airplanes, gliders, rotorcraft, manned free

balloons, etc.) may be unpowered or powered by a single, naturally aspirated

engine, with a 61-knot or less stall speed limitation for airplanes and a 6-

pound per square foot main rotor disc loading limitation for rotorcraft.

Primary category aircraft may have a maximum certificated weight of no more

than 2,700 pounds, maximum seating capacity of four, and unpressurized

cabins. Although these aircraft may be available for rental and flight

instruction under certain conditions, the carrying of persons or property for

hire is prohibited. This final rule also adds a new section addressing the

falsification of documents submitted as part of certification for products

and parts.

EFFECTIVE DATE: December 31, 1992.

*****************************************************************************

Sec. 21.165  Responsibility of holder.

The holder of a production certificate shall--

(a) Maintain the quality control system in conformity with the data and

procedures approved for the production certificate; and

(b) Determine that each part and each completed product, including primary

category aircraft assembled under a production certificate by another person

from a kit provided by the holder of the production certificate, submitted

for airworthiness certification or approval conforms to the approved design

and is in a condition for safe operation.

[Doc. No. 5085, 29 FR 14569, Oct. 24, 1964, as amended by Amdt. 21-64, 53 FR

48521, Dec. 1, 1988; Amdt. 21-70, 57 FR 41368, Sept. 9, 1992]

*****************************************************************************

57 FR 41360, No. 175, Sept. 9, 1992

SUMMARY: This final rule establishes a new primary category of aircraft, and

new simplified procedures for type, production, and airworthiness

certification, and associated maintenance procedures. Aircraft in this

category are of simple design intended exclusively for pleasure and personal

use. Primary category aircraft (airplanes, gliders, rotorcraft, manned free

balloons, etc.) may be unpowered or powered by a single, naturally aspirated

engine, with a 61-knot or less stall speed limitation for airplanes and a 6-

pound per square foot main rotor disc loading limitation for rotorcraft.

Primary category aircraft may have a maximum certificated weight of no more

than 2,700 pounds, maximum seating capacity of four, and unpressurized

cabins. Although these aircraft may be available for rental and flight

instruction under certain conditions, the carrying of persons or property for

hire is prohibited. This final rule also adds a new section addressing the

falsification of documents submitted as part of certification for products

and parts.

EFFECTIVE DATE: December 31, 1992.

*****************************************************************************

Subpart H -- Airworthiness Certificates

Sec. 21.171  Applicability.

This subpart prescribes procedural requirements for the issue of

airworthiness certificates.

Sec. 21.173  Eligibility.

Any registered owner of a U.S.-registered aircraft (or the agent of the

owner) may apply for an airworthiness certificate for that aircraft. An

application for an airworthiness certificate must be made in a form and

manner acceptable to the Administrator, and may be submitted to any FAA

office.

[Amdt. 21-26, 34 FR 15244, Sept. 30, 1969]

Sec. 21.175  Airworthiness certificates: classification.

(a) Standard airworthiness certificates are airworthiness certificates

issued for aircraft type certificated in the normal, utility, acrobatic,

commuter, or transport category, and for manned free balloons, and for

aircraft designated by the Administrator as special classes of aircraft.

(b) Special airworthiness certificates are primary restricted, limited,

and provisional airworthiness certificates, special flight permits, and

experimental certificates.

[Amdt. 21-21, 33 FR 6858, May 7, 1968, as amended by Amdt. 21-60, 52 FR 8043,

Mar. 13, 1987; Amdt. 21-70, 57 FR 41368, Sept. 9, 1992]

*****************************************************************************

57 FR 41360, No. 175, Sept. 9, 1992

SUMMARY: This final rule establishes a new primary category of aircraft, and

new simplified procedures for type, production, and airworthiness

certification, and associated maintenance procedures. Aircraft in this

category are of simple design intended exclusively for pleasure and personal

use. Primary category aircraft (airplanes, gliders, rotorcraft, manned free

balloons, etc.) may be unpowered or powered by a single, naturally aspirated

engine, with a 61-knot or less stall speed limitation for airplanes and a 6-

pound per square foot main rotor disc loading limitation for rotorcraft.

Primary category aircraft may have a maximum certificated weight of no more

than 2,700 pounds, maximum seating capacity of four, and unpressurized

cabins. Although these aircraft may be available for rental and flight

instruction under certain conditions, the carrying of persons or property for

hire is prohibited. This final rule also adds a new section addressing the

falsification of documents submitted as part of certification for products

and parts.

EFFECTIVE DATE: December 31, 1992.

*****************************************************************************

Sec. 21.177  Amendment or modification.

An airworthiness certificate may be amended or modified only upon

application to the Administrator.

Sec. 21.179  Transferability.

An airworthiness certificate is transferred with the aircraft.

Sec. 21.181  Duration.

(a) Unless sooner surrendered, suspended, revoked, or a termination date is

otherwise established by the Administrator, airworthiness certificates are

effective as follows:

(1) Standard airworthiness certificates, special airworthiness

certificates-primary category, and airworthiness certificates issued for

restricted or limited category aircraft are effective as long as the

maintenance, preventive maintenance, and alterations are performed in

accordance with Parts 43 and 91 of this chapter and the aircraft are

registered in the United States.

(2) A special flight permit is effective for the period of time specified

in the permit.

(3) An experimental certificate for research and development, showing

compliance with regulations, crew training, or market surveys is effective

for one year after the date of issue or renewal unless a shorter period is

prescribed by the Administrator. The duration of amateur-built, exhibition,

and air-racing experimental certificates will be unlimited unless the

Administrator finds for good cause that a specific period should be

established.

(b) The owner, operator, or bailee of the aircraft shall, upon request,

make it available for inspection by the Administrator.

(c) Upon suspension, revocation, or termination by order of the

Administrator of an airworthiness certificate, the owner, operator, or bailee

of an aircraft shall, upon request, surrender the certificate to the

Administrator.

[Amdt. 21-21, 33 FR 6858, May 7, 1968, as amended by Amdt. 21-49, 44 FR

46781, Aug. 9, 1979; Amdt. 21-70, 57 FR 41368, Sept. 9, 1992]

*****************************************************************************

57 FR 41360, No. 175, Sept. 9, 1992

SUMMARY: This final rule establishes a new primary category of aircraft, and

new simplified procedures for type, production, and airworthiness

certification, and associated maintenance procedures. Aircraft in this

category are of simple design intended exclusively for pleasure and personal

use. Primary category aircraft (airplanes, gliders, rotorcraft, manned free

balloons, etc.) may be unpowered or powered by a single, naturally aspirated

engine, with a 61-knot or less stall speed limitation for airplanes and a 6-

pound per square foot main rotor disc loading limitation for rotorcraft.

Primary category aircraft may have a maximum certificated weight of no more

than 2,700 pounds, maximum seating capacity of four, and unpressurized

cabins. Although these aircraft may be available for rental and flight

instruction under certain conditions, the carrying of persons or property for

hire is prohibited. This final rule also adds a new section addressing the

falsification of documents submitted as part of certification for products

and parts.

EFFECTIVE DATE: December 31, 1992.

*****************************************************************************

Sec. 21.182  Aircraft identification.

(a) Except as provided in paragraph (b) of this section, each applicant for

an airworthiness certificate under this subpart must show that his aircraft

is identified as prescribed in Sec. 45.11.

(b) Paragraph (a) of this section does not apply to applicants for the

following:

(1) A special flight permit.

(2) An experimental certificate for an aircraft that is not amateur-built

or kit-built.

(3) A change from one airworthiness classification to another, for an

aircraft already identified as prescribed in Sec. 45.11.

[Amdt. 21-13, 32 FR 188, Jan. 10, 1967, as amended by Amdt. 21-51, 45 FR

60170, Sept. 11, 1980; Amdt. 21-70, 57 FR 41368, Sept. 9, 1992]

*****************************************************************************

57 FR 41360, No. 175, Sept. 9, 1992

SUMMARY: This final rule establishes a new primary category of aircraft, and

new simplified procedures for type, production, and airworthiness

certification, and associated maintenance procedures. Aircraft in this

category are of simple design intended exclusively for pleasure and personal

use. Primary category aircraft (airplanes, gliders, rotorcraft, manned free

balloons, etc.) may be unpowered or powered by a single, naturally aspirated

engine, with a 61-knot or less stall speed limitation for airplanes and a 6-

pound per square foot main rotor disc loading limitation for rotorcraft.

Primary category aircraft may have a maximum certificated weight of no more

than 2,700 pounds, maximum seating capacity of four, and unpressurized

cabins. Although these aircraft may be available for rental and flight

instruction under certain conditions, the carrying of persons or property for

hire is prohibited. This final rule also adds a new section addressing the

falsification of documents submitted as part of certification for products

and parts.

EFFECTIVE DATE: December 31, 1992.

*****************************************************************************

Sec. 21.183  Issue of standard airworthiness certificates for normal,

utility, acrobatic, commuter, and transport category aircraft; manned

free balloons; and special classes of aircraft.

(a) New aircraft manufactured under a production certificate. An applicant

for a standard airworthiness certificate for a new aircraft manufactured

under a production certificate is entitled to a standard airworthiness

certificate without further showing, except that the Administrator may

inspect the aircraft to determine conformity to the type design and condition

for safe operation.

(b) New aircraft manufactured under type certificate only. An applicant for

a standard airworthiness certificate for a new aircraft manufactured under a

type certificate only is entitled to a standard airworthiness certificate

upon presentation, by the holder or licensee of the type certificate, of the

statement of conformity prescribed in Sec. 21.130 if the Administrator finds

after inspection that the aircraft conforms to the type design and is in

condition for safe operation.

(c) Import aircraft. An applicant for a standard airworthiness certificate

for an import aircraft type certificated in accordance with Sec. 21.29 is

entitled to an airworthiness certificate if the country in which the aircraft

was manufactured certifies, and the Administrator finds, that the aircraft

conforms to the type design and is in condition for safe operation.

(d) Other aircraft. An applicant for a standard airworthiness certificate

for aircraft not covered by paragraphs (a) through (c) of this section is

entitled to a standard airworthiness certificate if--

(1) He presents evidence to the Administrator that the aircraft conforms to

a type design approved under a type certificate or a supplemental type

certificate and to applicable Airworthiness Directives;

(2) The aircraft (except an experimentally certificated aircraft that

previously had been issued a different airworthiness certificate under this

section) has been inspected in accordance with the performance rules for 100-

hour inspections set forth in Sec. 43.15 of this chapter and found airworthy

by--

(i) The manufacturer;

(ii) The holder of a repair station certificate as provided in Part 145 of

this chapter;

(iii) The holder of a mechanic certificate as authorized in Part 65 of this

chapter; or

(iv) The holder of a certificate issued under Part 121 or 127 of this

chapter, and having a maintenance and inspection organization appropriate to

the aircraft type; and

(3) The Administrator finds after inspection, that the aircraft conforms to

the type design, and is in condition for safe operation.

(e) Noise requirements. Notwithstanding all other provisions of this

section, the following must be complied with for the original issuance of a

standard airworthiness certificate:

(1) For transport category large airplanes and turbojet powered airplanes

that have not had any flight time before the dates specified in Sec. 36.1(d),

no standard airworthiness certificate is originally issued under this section

unless the Administrator finds that the type design complies with the noise

requirements in Sec. 36.1(d) in addition to the applicable airworthiness

requirements in this section. For import airplanes, compliance with this

paragraph is shown if the country in which the airplane was manufactured

certifies, and the Administrator finds, that Sec. 36.1(d) (or the applicable

airplane noise requirements of the country in which the airplane was

manufactured and any other requirements the Administrator may prescribe to

provide noise levels no greater than those provided by compliance with Sec.

36.1(d)) and paragraph (c) of this section are complied with.

(2) For normal, utility, acrobatic, commuter, or transport category

propeller driven small airplanes (except for those airplanes that are

designed for "agricultural aircraft operations" (as defined in Sec. 137.3 of

this chapter, as effective on January 1, 1966) or for dispensing fire

fighting materials to which Sec. 36.1583 of this chapter does not apply) that

have not had any flight time before the applicable date specified in Part 36

of this chapter, no standard airworthiness certificate is originally issued

under this section unless the applicant shows that the type design complies

with the applicable noise requirements of Part 36 of this chapter in addition

to the applicable airworthiness requirements in this section. For import

airplanes, compliance with this paragraph is shown if the country in which

the airplane was manufactured certifies, and the Administrator finds, that

the applicable requirements of Part 36 of this chapter (or the applicable

airplane noise requirements of the country in which the airplane was

manufactured and any other requirements the Administrator may prescribe to

provide noise levels no greater than those provided by compliance with the

applicable requirements of Part 36 of this chapter) and paragraph (c) of this

section are complied with.

(f) Passenger emergency exit requirements. Notwithstanding all other

provisions of this section, each applicant for issuance of a standard

airworthiness certificate for a transport category airplane manufactured

after October 16, 1987, must show that the airplane meets the requirements of

Sec. 25.807(c)(7) in effect on July 24, 1989. For the purposes of this

paragraph, the date of manufacture of an airplane is the date the inspection

acceptance records reflect that the airplane is complete and meets the FAA-

approved type design data.

(g) Fuel venting and exhaust emission requirements. Notwithstanding all

other provisions of this section, and irrespective of the date of

application, no airworthiness certificate is issued, on and after the dates

specified in part 34 for the airplanes specified therein, unless the airplane

complies with the applicable requirements of that part.

[Amdt. 21-17, 32 FR 14927, Oct. 28, 1967, as amended by Amdt. 21-20, 33 FR

3055, Feb. 16, 1968; Amdt. 21-25, 34 FR 14068, Sept. 5, 1969; Amdt. 21-42, 40

FR 1033, Jan. 6, 1975; Amdt. 21-47, 43 FR 28419, June 29, 1978; Amdt. 21-52,

45 FR 67066, Oct. 9, 1980; Amdt. 21-59, 52 FR 1836, Jan. 15, 1987; Amdt. 21-

60, 52 FR 8043, Mar. 13, 1987; Amdt. 21-65, 54 FR 26695, June 23, 1989; Amdt.

21-68, 55 FR 32860, Aug. 10, 1990]

*****************************************************************************

55 FR 32856, No. 155, Aug. 10, 1990

SUMMARY: This final rule codifies as new part 34 all of the applicable

aircraft engine fuel venting and exhaust emission requirements of Special

Federal Aviation Regulation (SFAR) 27-5, and the test procedures specified

under the regulations implementing the Clean Air Act. This rule consolidates

all of the requirements and test procedures into this part, and inserts into

other affected parts the requirements to comply with new part 34. New part 34

does not alter any of the requirements specified under SFAR 27-5 or the

regulations implementing the Clean Air Act.

EFFECTIVE DATE: September 10, 1990.

*****************************************************************************

Sec. 21.184    Issue of special airworthiness certificates for primary

category aircraft.

(a) New primary category aircraft manufactured under a production

certificate. An applicant for an original, special airworthiness certificate-

primary category for a new aircraft that meets the criteria of Sec.

21.24(a)(1), manufactured under a production certificate, including aircraft

assembled by another person from a kit provided by the holder of the

production certificate and under the supervision and quality control of that

holder, is entitled to a special airworthiness certificate without further

showing, except that the Administrator may inspect the aircraft to determine

conformity to the type design and condition for safe operation.

(b) Imported aircraft. An applicant for a special airworthiness

certificate-primary category for an imported aircraft type certificated under

Sec. 21.29 is entitled to a special airworthiness certificate if the civil

airworthiness authority of the country in which the aircraft was manufactured

certifies, and the Administrator finds after inspection, that the aircraft

conforms to an approved type design that meets the criteria of Sec.

21.24(a)(1) and is in a condition for safe operation.

(c) Aircraft having a current standard airworthiness certificate. An

applicant for a special airworthiness certificate-primary category, for an

aircraft having a current standard airworthiness certificate that meets the

criteria of Sec. 21.24(a)(1), may obtain the primary category certificate in

exchange for its standard airworthiness certificate through the supplemental

type certification process. For the purposes of this paragraph, a current

standard airworthiness certificate means that the aircraft conforms to its

approved normal, utility, or acrobatic type design, complies with all

applicable airworthiness directives, has been inspected and found airworthy

within the last 12 calendar months in accordance with Sec. 91.409(a)(1) of

this chapter, and is found to be in a condition for safe operation by the

Administrator.

(d) Other aircraft. An applicant for a special airworthiness certificate-

primary category for an aircraft that meets the criteria of Sec. 21.24(a)(1),

and is not covered by paragraph (a), (b), or (c) of this section, is entitled

to a special airworthiness certificate if--

(1) The applicant presents evidence to the Administrator that the aircraft

conforms to an approved primary, normal, utility, or acrobatic type design,

including compliance with all applicable airworthiness directives;

(2) The aircraft has been inspected and found airworthy within the past 12

calendar months in accordance with Sec. 91.409(a)(1) of this chapter and;

(3) The aircraft is found by the Administrator to conform to an approved

type design and to be in a condition for safe operation.

(e) Multiple-category airworthiness certificates in the primary category

and any other category will not be issued; a primary category aircraft may

hold only one airworthiness certificate.

[Amdt. 21-70, 57 FR 41368, Sept. 9, 1992; 57 FR 43776, Sept. 22, 1992]

*****************************************************************************

57 FR 41360, No. 175, Sept. 9, 1992

SUMMARY: This final rule establishes a new primary category of aircraft, and

new simplified procedures for type, production, and airworthiness

certification, and associated maintenance procedures. Aircraft in this

category are of simple design intended exclusively for pleasure and personal

use. Primary category aircraft (airplanes, gliders, rotorcraft, manned free

balloons, etc.) may be unpowered or powered by a single, naturally aspirated

engine, with a 61-knot or less stall speed limitation for airplanes and a 6-

pound per square foot main rotor disc loading limitation for rotorcraft.

Primary category aircraft may have a maximum certificated weight of no more

than 2,700 pounds, maximum seating capacity of four, and unpressurized

cabins. Although these aircraft may be available for rental and flight

instruction under certain conditions, the carrying of persons or property for

hire is prohibited. This final rule also adds a new section addressing the

falsification of documents submitted as part of certification for products

and parts.

EFFECTIVE DATE: December 31, 1992.

*****************************************************************************

Sec. 21.185  Issue of airworthiness certificates for restricted category

aircraft.

(a) Aircraft manufactured under a production certificate or type

certificate only. An applicant for the original issue of a restricted

category airworthiness certificate for an aircraft type certificated in the

restricted category, that was not previously type certificated in any other

category, must comply with the appropriate provisions of Sec. 21.183.

(b) Other aircraft. An applicant for a restricted category airworthiness

certificate for an aircraft type certificated in the restricted category,

that was either a surplus aircraft of the Armed Forces or previously type

certificated in another category, is entitled to an airworthiness certificate

if the aircraft has been inspected by the Administrator and found by him to

be in a good state of preservation and repair and in a condition for safe

operation.

(c) Import aircraft. An applicant for the original issue of a restricted

category airworthiness certificate for an import aircraft type certificated

in the restricted category only in accordance with Sec. 21.29 is entitled to

an airworthiness certificate if the country in which the aircraft was

manufactured certifies, and the Administrator finds, that the aircraft

conforms to the type design and is in a condition for safe operation.

(d) Noise requirements. For propeller-driven small airplanes (except

airplanes designed for "agricultural aircraft operations," as defined in Sec.

137.3 of this chapter, as effective on January 1, 1966, or for dispensing

fire fighting materials) that have not had any flight time before the

applicable date specified in Part 36 of this chapter, and notwithstanding the

other provisions of this section, no original restricted category

airworthiness certificate is issued under this section unless the

Administrator finds that the type design complies with the applicable noise

requirements of Part 36 of this chapter in addition to the applicable

airworthiness requirements of this section. For import airplanes, compliance

with this paragraph is shown if the country in which the airplane was

manufactured certifies, and the Administrator finds, that the applicable

requirements of Part 36 of this chapter (or the applicable airplane noise

requirements of the country in which the airplane was manufactured and any

other requirements the Administrator may prescribe to provide noise levels no

greater than those provided by compliance with the applicable requirements of

Part 36 of this chapter) and paragraph (c) of this section are complied with.

[Amdt. 21-10, 31 FR 9211, July 6, 1966; as amended by Amdt. 21-32, 35 FR

10202, June 23, 1970; Amdt. 21-42, 40 FR 1034, Jan. 6, 1975]

Sec. 21.187  Issue of multiple airworthiness certification.

(a) An applicant for an airworthiness certificate in the restricted

category, and in one or more other categories except primary category, is

entitled to the certificate, if--

(1) He shows compliance with the requirements for each category, when the

aircraft is in the configuration for that category; and

(2) He shows that the aircraft can be converted from one category to

another by removing or adding equipment by simple mechanical means.

(b) The operator of an aircraft certificated under this section shall have

the aircraft inspected by the Administrator, or by a certificated mechanic

with an appropriate airframe rating, to determine airworthiness each time the

aircraft is converted from the restricted category to another category for

the carriage of passengers for compensation or hire, unless the Administrator

finds this unnecessary for safety in a particular case.

(c) The aircraft complies with the applicable requirements of part 34.

[Doc. No. 5085, 29 FR 14569, Oct. 24, 1964, as amended by Amdt. 21-68, 55

FR 32860, Aug. 10, 1990; Amdt. 21-70, 57 FR 41368, Sept. 9, 1992]

*****************************************************************************

57 FR 41360, No. 175, Sept. 9, 1992

SUMMARY: This final rule establishes a new primary category of aircraft, and

new simplified procedures for type, production, and airworthiness

certification, and associated maintenance procedures. Aircraft in this

category are of simple design intended exclusively for pleasure and personal

use. Primary category aircraft (airplanes, gliders, rotorcraft, manned free

balloons, etc.) may be unpowered or powered by a single, naturally aspirated

engine, with a 61-knot or less stall speed limitation for airplanes and a 6-

pound per square foot main rotor disc loading limitation for rotorcraft.

Primary category aircraft may have a maximum certificated weight of no more

than 2,700 pounds, maximum seating capacity of four, and unpressurized

cabins. Although these aircraft may be available for rental and flight

instruction under certain conditions, the carrying of persons or property for

hire is prohibited. This final rule also adds a new section addressing the

falsification of documents submitted as part of certification for products

and parts.

EFFECTIVE DATE: December 31, 1992.

*****************************************************************************

Sec. 21.189  Issue of airworthiness certificate for limited category

aircraft.

(a) An applicant for an airworthiness certificate for an aircraft in the

limited category is entitled to the certificate when--

(1) He shows that the aircraft has been previously issued a limited

category type certificate and that the aircraft conforms to that type

certificate; and

(2) The Administrator finds, after inspection (including a flight check by

the applicant), that the aircraft is in a good state of preservation and

repair and is in a condition for safe operation.

(b) The Administrator prescribes limitations and conditions necessary for

safe operation.

[Doc. No. 5085, 29 FR 14570, Oct. 24, 1964, as amended by Amdt. 21-4, 30 FR

9437, July 29, 1965]

Sec. 21.191  Experimental certificates.

Experimental certificates are issued for the following purposes:

(a) Research and development. Testing new aircraft design concepts, new

aircraft equipment, new aircraft installations, new aircraft operating

techniques, or new uses for aircraft.

(b) Showing compliance with regulations.  Conducting flight tests and other

operations to show compliance with the airworthiness regulations including

flights to show compliance for issuance of type and supplemental type

certificates, flights to substantiate major design changes, and flights to

show compliance with the function and reliability requirements of the

regulations.

(c) Crew training. Training of the applicant's flight crews.

(d) Exhibition. Exhibiting the aircraft's flight capabilities, performance,

or unusual characteristics at air shows, motion picture, television, and

similar productions, and the maintenance of exhibition flight proficiency,

including (for persons exhibiting aircraft) flying to and from such air shows

and productions.

(e) Air racing. Participating in air races, including (for such

participants) practicing for such air races and flying to and from racing

events.

(f) Market surveys. Use of aircraft for purposes of conducting market

surveys, sales demonstrations, and customer crew training only as provided in

Sec. 21.195.

(g) Operating amateur-built aircraft.  Operating an aircraft the major

portion of which has been fabricated and assembled by persons who undertook

the construction project solely for their own education or recreation.

(h) Operating kit-built aircraft. Operating a primary category aircraft

that meets the criteria of Sec. 21.24(a)(1) that was assembled by a person

from a kit manufactured by the holder of a production certificate for that

kit, without the supervision and quality control of the production

certificate holder under Sec. 21.184(a).

[Amdt. 21-21, 38 FR 6858, May 7, 1968, as amended by Amdt. 21-57, 49 FR

39651, Oct. 9, 1984; Amdt. 21-70, 57 FR 41369, Sept. 9, 1992]

*****************************************************************************

57 FR 41360, No. 175, Sept. 9, 1992

SUMMARY: This final rule establishes a new primary category of aircraft, and

new simplified procedures for type, production, and airworthiness

certification, and associated maintenance procedures. Aircraft in this

category are of simple design intended exclusively for pleasure and personal

use. Primary category aircraft (airplanes, gliders, rotorcraft, manned free

balloons, etc.) may be unpowered or powered by a single, naturally aspirated

engine, with a 61-knot or less stall speed limitation for airplanes and a 6-

pound per square foot main rotor disc loading limitation for rotorcraft.

Primary category aircraft may have a maximum certificated weight of no more

than 2,700 pounds, maximum seating capacity of four, and unpressurized

cabins. Although these aircraft may be available for rental and flight

instruction under certain conditions, the carrying of persons or property for

hire is prohibited. This final rule also adds a new section addressing the

falsification of documents submitted as part of certification for products

and parts.

EFFECTIVE DATE: December 31, 1992.

*****************************************************************************

Sec. 21.193  Experimental certificates: general.

An applicant for an experimental certificate must submit the following

information:

(a) A statement, in a form and manner prescribed by the Administrator

setting forth the purpose for which the aircraft is to be used.

(b) Enough data (such as photographs) to identify the aircraft.

(c) Upon inspection of the aircraft, any pertinent information found

necessary by the Administrator to safeguard the general public.

(d) In the case of an aircraft to be used for experimental purposes--

(1) The purpose of the experiment;

(2) The estimated time or number of flights required for the experiment;

(3) The areas over which the experiment will be conducted; and

(4) Except for aircraft converted from a previously certificated type

without appreciable change in the external configuration, three-view drawings

or three-view dimensioned photographs of the aircraft.

Sec. 21.195  Experimental certificates: Aircraft to be used for market

surveys, sales demonstrations, and customer crew training.

(a) A manufacturer of aircraft manufactured within the United States may

apply for an experimental certificate for an aircraft that is to be used for

market surveys, sales demonstrations, or customer crew training.

(b) A manufacturer of aircraft engines who has altered a type certificated

aircraft by installing different engines, manufactured by him within the

United States, may apply for an experimental certificate for that aircraft to

be used for market surveys, sales demonstrations, or customer crew training,

if the basic aircraft, before alteration, was type certificated in the

normal, acrobatic, commuter, or transport category.

(c) A person who has altered the design of a type certificated aircraft may

apply for an experimental certificate for the altered aircraft to be used for

market surveys, sales demonstrations, or customer crew training if the basic

aircraft, before alteration, was type certificated in the normal, utility,

acrobatic, or transport category.

(d) An applicant for an experimental certificate under this section is

entitled to that certificate if, in addition to meeting the requirements of

Sec. 21.193--

(1) He has established an inspection and maintenance program for the

continued airworthiness of the aircraft; and

(2) He shows that the aircraft has been flown for at least 50 hours, or for

at least 5 hours if it is a type certificated aircraft which has been

modified.

[Amdt. 21-21, 33 FR 6858, May 7, 1968, as amended by Amdt. 21-28, 35 FR 2818,

Feb. 11, 1970; Amdt. 21-57, 49 FR 39651, Oct. 9, 1984; Amdt. 21-59, 52 FR

1836, Jan. 15, 1987]

Sec. 21.197  Special flight permits.

(a) A special flight permit may be issued for an aircraft that may not

currently meet applicable airworthiness requirements but is capable of safe

flight, for the following purposes:

(1) Flying the aircraft to a base where repairs, alterations, or

maintenance are to be performed, or to a point of storage.

(2) Delivering or exporting the aircraft.

(3) Production flight testing new production aircraft.

(4) Evacuating aircraft from areas of impending danger.

(5) Conducting customer demonstration flights in new production aircraft

that have satisfactorily completed production flight tests.

(b) A special flight permit may also be issued to authorize the operation

of an aircraft at a weight in excess of its maximum certificated takeoff

weight for flight beyond the normal range over water, or over land areas

where adequate landing facilities or appropriate fuel is not available. The

excess weight that may be authorized under this paragraph is limited to the

additional fuel, fuel-carrying facilities, and navigation equipment necessary

for the flight.

(c) Upon application, as prescribed in Secs. 121.79, 127.27, and 135.17 of

this chapter, a special flight permit with a continuing authorization may be

issued for aircraft that may not meet applicable airworthiness requirements

but are capable of safe flight for the purpose of flying aircraft to a base

where maintenance or alterations are to be performed. The permit issued under

this paragraph is an authorization, including conditions and limitations for

flight, which is set forth in the certificate holder's operations

specifications. The permit issued under this paragraph may be issued to--

(1) Certificate holders authorized to conduct operations under Part 121 or

Part 127 of this chapter; or

(2) Certificate holders authorized to conduct operations under Part 135 for

those aircraft they operate and maintain under a continuous airworthiness

maintenance program prescribed by Sec. 135.411 (a)(2) or (b) of that part.

The permit issued under this paragraph is an authorization, including any

conditions and limitations for flight, which is set forth in the certificate

holder's operations specifications.

[Doc. No. 5085, 29 FR 14570, Oct. 24, 1964, as amended by Amdt. 21-21, 33 FR

6859, May 7, 1968; Amdt. 21--51, 45 FR 60170, Sept. 11, 1980; Amdt. 21-54, 46

FR 37878, July 23, 1981]

Sec. 21.199  Issue of special flight permits.

(a) Except as provided in Sec. 21.197(c), an applicant for a special flight

permit must submit a statement in a form and manner prescribed by the

Administrator, indicating--

(1) The purpose of the flight.

(2) The proposed itinerary.

(3) The crew required to operate the aircraft and its equipment, e.g.,

pilot, co-pilot, navigator, etc.

(4) The ways, if any, in which the aircraft does not comply with the

applicable airworthiness requirements.

(5) Any restriction the applicant considers necessary for safe operation of

the aircraft.

(6) Any other information considered necessary by the Administrator for the

purpose of prescribing operating limitations.

(b) The Administrator may make, or require the applicant to make

appropriate inspections or tests necessary for safety.

[Doc. No. 5085, 29 FR 14570, Oct. 24, 1964, as amended by Amdt. 21-21, 33 FR

6859, May 7, 1968; Amdt. 21-22, 33 FR 11901, Aug. 22, 1968]

Subpart I--Provisional Airworthiness Certificates

Source: Docket No. 5085, 29 FR 14571, Oct. 24, 1964, unless otherwise

noted.

Sec. 21.211  Applicability.

This subpart prescribes procedural requirements for the issue of

provisional airworthiness certificates.

Sec. 21.213  Eligibility.

(a) A manufacturer who is a United States citizen may apply for a Class I

or Class II provisional airworthiness certificate for aircraft manufactured

by him within the U.S.

(b) Any holder of an air carrier operating certificate under Part 121 or

Part 127 of this chapter who is a United States citizen may apply for a Class

II provisional airworthiness certificate for transport category aircraft that

meet either of the following:

(1) The aircraft has a current Class II provisional type certificate or an

amendment thereto.

(2) The aircraft has a current provisional amendment to a type certificate

that was preceded by a corresponding Class II provisional type certificate.

(c) An aircraft engine manufacturer who is a United States citizen and who

has altered a type certificated aircraft by installing different type

certificated engines, manufactured by him within the United States, may apply

for a Class I provisional airworthiness certificate for that aircraft, if the

basic aircraft, before alteration, was type certificated in the normal,

utility, acrobatic, commuter, or transport category.

[Doc. No. 5085, 29 FR 14571, Oct. 24, 1964, as amended by Amdt. 21-59, 52 FR

1836, Jan. 15, 1987]

Sec. 21.215   Application.

Application for provisional airworthiness certificates must be submitted to

the Manufacturing Inspection District Office in the geographic area in which

the manufacturer or air carrier is located. The application must be

accompanied by the pertinent information specified in this subpart.

[Amdt. 21-67, 54 FR 39291, Sept. 25, 1989]

Sec. 21.217  Duration.

Unless sooner surrendered, superseded, revoked, or otherwise terminated,

provisional airworthiness certificates are effective for the duration of the

corresponding provisional type certificate, amendment to a provisional type

certificate, or provisional amendment to the type certificate.

Sec. 21.219  Transferability.

Class I provisional airworthiness certificates are not transferable. Class

II provisional airworthiness certificates may be transferred to an air

carrier eligible to apply for a certificate under Sec. 21.213(b).

Sec. 21.221  Class I provisional airworthiness certificates.

(a) Except as provided in Sec. 21.225, an applicant is entitled to a Class

I provisional airworthiness certificate for an aircraft for which a Class I

provisional type certificate has been issued if--

(1) He meets the eligibility requirements of Sec. 21.213 and he complies

with this section; and

(2) The Administrator finds that there is no feature, characteristic or

condition of the aircraft that would make the aircraft unsafe when operated

in accordance with the limitations established in Secs. 21.81(e) and 91.317

of this subchapter.

(b) The manufacturer must hold a provisional type certificate for the

aircraft.

(c) The manufacturer must submit a statement that the aircraft conforms to

the type design corresponding to the provisional type certificate and has

been found by him to be in safe operating condition under all applicable

limitations.

(d) The aircraft must be flown at least five hours by the manufacturer.

(e) The aircraft must be supplied with a provisional aircraft flight manual

or other document and appropriate placards containing the limitations

established by Secs. 21.81(e) and 91.317.

[Doc. No. 5085, 29 FR 14571, Oct. 24, 1964, as amended by Amdt. 21-66, 54 FR

34329, Aug. 18, 1989]

Effective Date Note: At Amdt. 21-66, 54 FR 34329, Aug. 18, 1989, Sec.

21.221(a) and (e) was amended by changing the cross reference "Sec. 91.41" to

"Sec. 91.317", effective August 18, 1990.

Sec. 21.223  Class II provisional airworthiness certificates.

(a) Except as provided in Sec. 21.225, an applicant is entitled to a Class

II provisional airworthiness certificate for an aircraft for which a Class II

provisional type certificate has been issued if--

(1) He meets the eligibility requirements of Sec. 21.213 and he complies

with this section; and

(2) The Administrator finds that there is no feature, characteristic, or

condition of the aircraft that would make the aircraft unsafe when operated

in accordance with the limitations established in Secs. 21.83(h), 91.317, and

121.207 of this chapter.

(b) The applicant must show that a Class II provisional type certificate

for the aircraft has been issued to the manufacturer.

(c) The applicant must submit a statement by the manufacturer that the

aircraft has been manufactured under a quality control system adequate to

ensure that the aircraft conforms to the type design corresponding with the

provisional type certificate.

(d) The applicant must submit a statement that the aircraft has been found

by him to be in a safe operating condition under the applicable limitations.

(e) The aircraft must be flown at least five hours by the manufacturer.

(f) The aircraft must be supplied with a provisional aircraft flight manual

containing the limitations established by Secs. 21.83(h), 91.317, and 121.207

of this chapter.

[Doc. No. 5085, 29 FR 14571, Oct. 24, 1964, as amended by Amdt. 21-12, 31 FR

13389, Oct. 15, 1966; Amdt. 21-66, 54 FR 34329, Aug. 18, 1989]

Effective Date Note: At Amdt. 21-66, 54 FR 34329, Aug. 18, 1989, Sec.

21.223(a)(2) and (f) were amended by changing the cross reference "Sec.

91.41" to "Sec. 91.317".

Sec. 21.225  Provisional airworthiness certificates corresponding with

provisional amendments to type certificates.

(a) An applicant is entitled to a Class I or a Class II provisional

airworthiness certificate, for an aircraft, for which a provisional amendment

to the type certificate has been issued, if--

(1) He meets the eligibility requirements of Sec. 21.213 and he complies

with this section; and

(2) The Administrator finds that there is no feature, characteristic, or

condition of the aircraft, as modified in accordance with the provisionally

amended type certificate, that would make the aircraft unsafe when operated

in accordance with the applicable limitations established in Secs. 21.85(g),

91.317, and 121.207 of this chapter.

(b) The applicant must show that the modification was made under a quality

control system adequate to ensure that the modification conforms to the

provisionally amended type certificate.

(c) The applicant must submit a statement that the aircraft has been found

by him to be in a safe operating condition under the applicable limitations.

(d) The aircraft must be flown at least five hours by the manufacturer.

(e) The aircraft must be supplied with a provisional aircraft flight manual

or other document and appropriate placards containing the limitations

required by Secs. 21.85(g), 91.317, and 121.207 of this chapter.

[Doc. No. 5085, 29 FR 14571, Oct. 24, 1964, as amended by Amdt. 21-12, 31 FR

13389, Oct. 15, 1966; Amdt. 21-66, 54 FR 34329, Aug. 18, 1989]

Effective Date Note: At Amdt. 21-66, 54 FR 34329, Aug. 18, 1989, Sec.

21.225(a)(2) and (e) were amended by changing the cross reference "Sec.

91.41" to "Sec. 91.317", effective August 18, 1990.

Subpart J--Delegation Option Authorization Procedures

Source: Amdt. 21-5, 30 FR 11375, Sept. 8, 1965, unless otherwise noted.

Sec. 21.231  Applicability.

This subpart prescribes procedures for--

(a) Obtaining and using a delegation option authorization for type,

production, and airworthiness certification (as applicable) of--

(1) Small airplanes and small gliders;

(2) Commuter category airplanes;

(3) Normal category rotorcraft;

(4) Turbojet engines of not more than 1,000 pounds thrust;

(5) Turbopropeller and reciprocating engines of not more than 500 brake

horsepower; and

(6) Propellers manufactured for use on engines covered by paragraph (a)(4)

of this section; and

(b) Issuing airworthiness approval tags for engines, propellers, and parts

of products covered by paragraph (a) of this section.

[Amdt. 21-5, 30 FR 11375, Sept. 8, 1965, as amended by Amdt. 21-59, 52 FR

1836, Jan. 15, 1987]

Sec. 21.235  Application.

(a) An application for a delegation option authorization must be submitted,

in a form and manner prescribed by the Administrator, to the Aircraft

Certification Office for the area in which the manufacturer is located.

(b) The application must include the names, signatures, and titles of the

persons for whom authorization to sign airworthiness certificates, repair and

alteration forms, and inspection forms is requested.

[Doc. No. 5085, 29 FR 14574, Oct. 24, 1964, as amended by Amdt. 21-67, 54 FR

39291, Sept. 25, 1989]

Sec. 21.239  Eligibility.

To be eligible for a delegation option authorization, the applicant must--

(a) Hold a current type certificate, issued to him under the standard

procedures, for a product type certificated under the same part as the

products for which the delegation option authorization is sought;

(b) Hold a current production certificate issued under the standard

procedures;

(c) Employ a staff of engineering, flight test, production and inspection

personnel who can determine compliance with the applicable airworthiness

requirements of this chapter; and

(d) Meet the requirements of this subpart.

Sec. 21.243  Duration.

A delegation option authorization is effective until it is surrendered or

the Administrator suspends, revokes, or otherwise terminates it.

Sec. 21.245  Maintenance of eligibility.

The holder of a delegation option authorization shall continue to meet the

requirements for issue of the authorization or shall notify the Administrator

within 48 hours of any change (including a change of personnel) that could

affect the ability of the holder to meet those requirements.

Sec. 21.247  Transferability.

A delegation option authorization is not transferable.

Sec. 21.249  Inspections.

Upon request, each holder of a delegation option authorization and each

applicant shall let the Administrator inspect his organization, facilities,

product, and records.

Sec. 21.251  Limits of applicability.

(a) Delegation option authorizations apply only to products that are

manufactured by the holder of the authorization.

(b) Delegation option authorizations may be used for--

(1) Type certification;

(2) Changes in the type design of products for which the manufacturer

holds, or obtains, a type certificate;

(3) The amendment of a production certificate held by the manufacturer to

include additional models or additional types for which he holds or obtains a

type certificate; and

(4) The issue of--

(i) Experimental certificates for aircraft for which the manufacturer has

applied for a type certificate or amended type certificate under Sec. 21.253,

to permit the operation of those aircraft for the purpose of research and

development, crew training, market surveys, or the showing of compliance with

the applicable airworthiness requirements;

(ii) Airworthiness certificates (other than experimental certificates) for

aircraft for which the manufacturer holds a type certificate and holds or is

in the process of obtaining a production certificate;

(iii) Airworthiness approval tags (FAA Form 8130-3) for engines and

propellers for which the manufacturer holds a type certificate and holds or

is in the process of obtaining a production certificate; and

(iv) Airworthiness approval tags (FAA Form 8130-3) for parts of products

covered by this section.

(c) Delegation option procedures may be applied to one or more types

selected by the manufacturer, who must notify the FAA of each model, and of

the first serial number of each model manufactured by him under the

delegation option procedures. Other types or models may remain under the

standard procedures.

(d) Delegation option authorizations are subject to any additional

limitations prescribed by the Administrator after inspection of the

applicant's facilities or review of the staff qualifications.

[Amdt. 21-5, 30 FR 11375, Sept. 8, 1965, as amended by Amdt. 21-31, 35 FR

7292, May 9, 1970; Amdt. 21-43, 40 FR 2576, Jan. 14, 1975]

Sec. 21.253  Type certificates: application.

(a) To obtain, under the delegation option authorization, a type

certificate for a new product or an amended type certificate, the

manufacturer must submit to the Administrator--

(1) An application for a type certificate (FAA Form 312);

(2) A statement listing the airworthiness requirements of this chapter (by

part number and effective date) that the manufacturer considers applicable;

(3) After determining that the type design meets the applicable

requirements, a statement certifying that this determination has been made;

(4) After placing the required technical data and type inspection report in

the technical data file required by Sec. 21.293(a)(1)(i), a statement

certifying that this has been done;

(5) A proposed type certificate data sheet; and

(6) An Aircraft Flight Manual (if required) or a summary of required

operating limitations and other information necessary for safe operation of

the product.

Sec. 21.257  Type certificates: issue.

An applicant is entitled to a type certificate for a product manufactured

under a delegation option authorization if the Administrator finds that the

product meets the applicable airworthiness, noise, fuel venting, and exhaust

emission requirements (including applicable acoustical change or emissions

change requirements in the case of changes in type design).

[Doc. No. 25613, Amdt. 21-68, 55 FR 32860, Aug. 10, 1990]

*****************************************************************************

55 FR 32856, No. 155, Aug. 10, 1990

SUMMARY: This final rule codifies as new part 34 all of the applicable

aircraft engine fuel venting and exhaust emission requirements of Special

Federal Aviation Regulation (SFAR) 27-5, and the test procedures specified

under the regulations implementing the Clean Air Act. This rule consolidates

all of the requirements and test procedures into this part, and inserts into

other affected parts the requirements to comply with new part 34. New part 34

does not alter any of the requirements specified under SFAR 27-5 or the

regulations implementing the Clean Air Act.

EFFECTIVE DATE: September 10, 1990.

*****************************************************************************

Sec. 21.261  Equivalent safety provisions.

The manufacturer shall obtain the Administrator's concurrence on the

application of all equivalent safety provisions applied under Sec. 21.21.

Sec. 21.267  Production certificates.

To have a new model or new type certificate listed on his production

certificate (issued under Subpart G of this part), the manufacturer must

submit to the Administrator--

(a) An application for an amendment to the production certificate;

(b) After determining that the production certification requirements of

Subpart G, with respect to the new model or type, are met, a statement

certifying that this determination has been made;

(c) A statement identifying the type certificate number under which the

product is being manufactured; and

(d) After placing the manufacturing and quality control data required by

Sec. 21.143 with the data required by Sec. 21.293(a)(1)(ii), a statement

certifying that this has been done.

Sec. 21.269  Export airworthiness approvals.

The manufacturer may issue export airworthiness approvals.

Sec. 21.271  Airworthiness approval tags.

(a) A manufacturer may issue an airworthiness approval tag (FAA Form 8130-

3) for each engine and propeller covered by Sec. 21.251(b)(4), and may issue

an airworthiness approval tag for parts of each product covered by that

section, if he finds, on the basis of inspection and operation tests, that

those products conform to a type design for which he holds a type certificate

and are in condition for safe operation.

(b) When a new model has been included on the Production Limitation Record,

the production certification number shall be stamped on the engine or

propeller identification data place instead of issuing an airworthiness

approval tag.

[Amdt. 21-5, 30 FR 11375, Sept. 8, 1965, as amended by Amdt. 21-43, 40 FR

2577, Jan. 14, 1975]

Sec. 21.273  Airworthiness certificates other than experimental.

(a) The manufacturer may issue an airworthiness certificate for aircraft

manufactured under a delegation option authorization if he finds, on the

basis of the inspection and production flight check, that each aircraft

conforms to a type design for which he holds a type certificate and is in a

condition for safe operation.

(b) The manufacturer may authorize any employee to sign airworthiness

certificates if that employee--

(1) Performs, or is in direct charge of, the inspection specified in

paragraph (a) of this section; and

(2) Is listed on the manufacturer's application for the delegation option

authorization, or on amendments thereof.

[Amdt. 21-5, 30 FR 11375, Sept. 8, 1965, as amended by Amdt. 21-18, 32 FR

15472, Nov. 7, 1967]

Sec. 21.275  Experimental certificates.

(a) The manufacturer shall, before issuing an experimental certificate,

obtain from the Administration any limitations and conditions that the

Administrator considers necessary for safety.

(b) For experimental certificates issued by the manufacturer, under this

subpart, for aircraft for which the manufacturer holds the type certificate

and which have undergone changes to the type design requiring flight test,

the manufacturer may prescribe any operating limitations that he considers

necessary.

Sec. 21.277  Data review and service experience.

(a) If the Administrator finds that a product for which a type certificate

was issued under this subpart does not meet the applicable airworthiness

requirements, or that an unsafe feature or characteristic caused by a defect

in design or manufacture exists, the manufacturer, upon notification by the

Administrator, shall investigate the matter and report to the Administrator

the results of the investigation and the action, if any, taken or proposed.

(b) If corrective action by the user of the product is necessary for safety

because of any noncompliance or defect specified in paragraph (a) of this

section, the manufacturer shall submit the information necessary for the

issue of an Airworthiness Directive under Part 39.

Sec. 21.289  Major repairs, rebuilding and alteration.

For types covered by a delegation option authorization, a manufacturer

may--

(a) After finding that a major repair or major alteration meets the

applicable airworthiness requirements of this chapter, approve that repair or

alteration; and

(b) Authorize any employee to execute and sign FAA Form 337 and make

required log book entries if that employee--

(1) Inspects, or is in direct charge of inspecting, the repair, rebuilding,

or alteration; and

(2) Is listed on the application for the delegation option authorization,

or on amendments thereof.

Sec. 21.293  Current records.

(a) The manufacturer shall maintain at his factory, for each product type

certificated under a delegation option authorization, current records

containing the following:

(1) For the duration of the manufacturing operating under the delegation

option authorization--

(i) A technical data file that includes the type design drawings,

specifications, reports on tests prescribed by this part, and the original

type inspection report and amendments to that report;

(ii) The data (including amendments) required to be submitted with the

original application for each production certificate; and

(iii) A record of any rebuilding and alteration performed by the

manufacturer on products manufactured under the delegation option

authorization.

(2) For 2 years--

(i) A complete inspection record for each product manufactured, by serial

number, and data covering the processes and tests to which materials and

parts are subjected; and

(ii) A record of reported service difficulties.

(b) The records and data specified in paragraph (a) of this section shall

be--

(1) Made available, upon the Administrator's request, for examination by

the Administrator at any time; and

(2) Identified and sent to the Administrator as soon as the manufacturer no

longer operates under the delegation option procedures.

Subpart K--Approval of Materials, Parts, Processes, and Appliances

Source: Docket No. 5085, 29 FR 14574, Oct. 24, 1964, unless otherwise

noted.

Sec. 21.301  Applicability.

This subpart prescribes procedural requirements for the approval of certain

materials, parts, processes, and appliances.

Sec. 21.303  Replacement and modification parts.

(a) Except as provided in paragraph (b) of this section, no person may

produce a modification or replacement part for sale for installation on a

type certificated product unless it is produced pursuant to a Parts

Manufacturer Approval issued under this subpart.

(b) This section does not apply to the following:

(1) Parts produced under a type or production certificate.

(2) Parts produced by an owner or operator for maintaining or altering his

own product.

(3) Parts produced under an FAA Technical Standard Order.

(4) Standard parts (such as bolts and nuts) conforming to established

industry or U.S. specifications.

(c) An application for a Parts Manufacturer Approval is made to the

Regional Office of the region in which the manufacturing facility is located

and must include the following:

(1) The identity of the product on which the part is to be installed.

(2) The name and address of the manufacturing facilities at which these

parts are to be manufactured.

(3) The design of the part, which consists of--

(i) Drawings and specifications necessary to show the configuration of the

part; and

(ii) Information on dimensions, materials, and processes necessary to

define the structural strength of the part.

(4) Test reports and computations necessary to show that the design of the

part meets the airworthiness requirements of the Federal Aviation Regulations

applicable to the product on which the part is to be installed, unless the

applicant shows that the design of the part is identical to the design of a

part that is covered under a type certificate. If the design of the part was

obtained by a licensing agreement, evidence of that agreement must be

furnished.

(d) An applicant is entitled to a Parts Manufacturer Approval for a

replacement or modification part if--

(1) The Administrator finds, upon examination of the design and after

completing all tests and inspections, that the design meets the airworthiness

requirements of the Federal Aviation Regulations applicable to the product on

which the part is to be installed; and

(2) He submits a statement certifying that he has established the

fabrication inspection system required by paragraph (h) of this section.

(e) Each applicant for a Parts Manufacturer Approval must allow the

Administrator to make any inspection or test necessary to determine

compliance with the applicable Federal Aviation Regulations. However, unless

otherwise authorized by the Administrator--

(1) No part may be presented to the Administrator for an inspection or test

unless compliance with paragraphs (f) (2) through (4) of this section has

been shown for that part; and

(2) No change may be made to a part between the time that compliance with

paragraphs (f) (2) through (4) of this section is shown for that part and the

time that the part is presented to the Administrator for the inspection or

test.

(f) Each applicant for a Parts Manufacturer Approval must make all

inspections and tests necessary to determine--

(1) Compliance with the applicable airworthiness requirements;

(2) That materials conform to the specifications in the design;

(3) That the part conforms to the drawings in the design; and

(4) That the fabrication processes, construction, and assembly conform to

those specified in the design.

(g) The Administrator does not issue a Parts Manufacturer Approval if the

manufacturing facilities for the part are located outside of the United

States, unless the Administrator finds that the location of the manufacturing

facilities places no burden on the FAA in administering applicable

airworthiness requirements.

(h) Each holder of a Parts Manufacturer Approval shall establish and

maintain a fabrication inspection system that ensures that each completed

part conforms to its design data and is safe for installation on applicable

type certificated products. The system shall include the following:

(1) Incoming materials used in the finished part must be as specified in

the design data.

(2) Incoming materials must be properly identified if their physical and

chemical properties cannot otherwise be readily and accurately determined.

(3) Materials subject to damage and deterioration must be suitably stored

and adequately protected.

(4) Processes affecting the quality and safety of the finished product must

be accomplished in accordance with acceptable specifications.

(5) Parts in process must be inspected for conformity with the design data

at points in production where accurate determination can be made. Statistical

quality control procedures may be employed where it is shown that a

satisfactory level of quality will be maintained for the particular part

involved.

(6) Current design drawings must be readily available to manufacturing and

inspection personnel, and used when necessary.

(7) Major changes to the basic design must be adequately controlled and

approved before being incorporated in the finished part.

(8) Rejected materials and components must be segregated and identified in

such a manner as to preclude their use in the finished part.

(9) Inspection records must be maintained, identified with the completed

part, where practicable, and retained in the manufacturer's file for a period

of at least 2 years after the part has been completed.

(i) A Parts Manufacturer Approval issued under this section is not

transferable and is effective until surrendered or withdrawn or otherwise

terminated by the Administrator.

(j) The holder of a Parts Manufacturer Approval shall notify the FAA in

writing within 10 days from the date the manufacturing facility at which the

parts are manufactured is relocated or expanded to include additional

facilities at other locations.

(k) Each holder of a Parts Manufacturer Approval shall determine that each

completed part conforms to the design data and is safe for installation on

type certificated products.

[Amdt. 21-38, 37 FR 10659, May 26, 1972, as amended by Amdt. 21-41, 39 FR

41965, Dec. 4, 1974]

Sec. 21.305  Approval of materials, parts, processes, and appliances.

Whenever a material, part, process, or appliance is required to be approved

under this chapter, it may be approved--

(a) Under a Parts Manufacturer Approval issued under Sec. 21.303;

(b) Under a Technical Standard Order issued by the Administrator. Advisory

Circular 20-110 contains a list of Technical Standard Orders that may be used

to obtain approval. Copies of the Advisory Circular may be obtained from the

U.S. Department of Transportation, Publication Section (M-443.1), Washington,

D.C. 20590;

(c) In conjunction with type certification procedures for a product; or

(d) In any other manner approved by the Administrator.

[Amdt. 21-38, 37 FR 10659, May 26, 1972, as amended by Amdt. 21-50, 45 FR

38346, June 9, 1980]

Subpart L--Export Airworthiness Approvals

Source: Amdt. 21-2, 30 FR 8465, July 2, 1965, unless otherwise noted.

Sec. 21.321  Applicability.

(a) This subpart prescribes--

(1) Procedural requirements for the issue of export airworthiness

approvals; and

(2) Rules governing the holders of those approvals.

(b) For the purposes of this subpart--

(1) A Class I product is a complete aircraft, aircraft engine, or

propeller, which--

(i) Has been type certificated in accordance with the applicable Federal

Aviation Regulations and for which Federal Aviation Specifications or type

certificate data sheets have been issued; or

(ii) Is identical to a type certificated product specified in paragraph

(b)(1)(i) of this section in all respects except as is otherwise acceptable

to the civil aviation authority of the importing state.

(2) A Class II product is a major component of a Class I product (e.g.,

wings, fuselages, empennage assemblies, landing gears, power transmissions,

control surfaces, etc), the failure of which would jeopardize the safety of a

Class I product; or any part, material, or appliance, approved and

manufactured under the Technical Standard Order (TSO) system in the "C"

series.

(3) A Class III product is any part or component which is not a Class I or

Class II product and includes standard parts, i.e., those designated as AN,

NAS, SAE, etc.

(4) The words "newly overhauled" when used to describe a product means that

the product has not been operated or placed in service, except for functional

testing, since having been overhauled, inspected and approved for return to

service in accordance with the applicable Federal Aviation Regulations.

[Amdt. 21-2, 30 FR 11375, July 2, 1965, as amended by Amdt. 21-48, 44 FR

15649, Mar. 15, 1979]

Sec. 21.323  Eligibility.

(a) Any exporter or his authorized representative may obtain an export

airworthiness approval for a Class I or Class II product.

(b) Any manufacturer may obtain an export airworthiness approval for a

Class III product if the manufacturer--

(1) Has in his employ a designated representative of the Administrator who

has been authorized to issue that approval; and

(2) Holds for that product--

(i) A production certificate;

(ii) An approved production inspection system;

(iii) An FAA Parts Manufacturer Approval (PMA); or

(iv) A Technical Standard Order authorization.

Sec. 21.325  Export airworthiness approvals.

(a) Kinds of approvals. (1) Export airworthiness approval of Class I

products is issued in the form of Export Certificates of Airworthiness, FAA

Form 8130-4. Such a certificate does not authorize the operation of aircraft.

(2) Export airworthiness approval of Class II and III products is issued in

the form of Airworthiness Approval Tags, FAA Form 8130-3.

(b) Products which may be approved.  Export airworthiness approvals are

issued for--

(1) New aircraft that are assembled and that have been flight-tested, and

other Class I products located in the United States, except that export

airworthiness approval may be issued for any of the following without

assembly or flight-test:

(i) A small airplane type certificated under Part 3 or 4a of the Civil Air

Regulations, or Part 23 of the Federal Aviation Regulations, and manufactured

under a production certificate;

(ii) A glider type certificated under Sec. 21.23 of this part and

manufactured under a production certificate; or

(iii) A normal category rotorcraft type certificated under Part 6 of the

Civil Air Regulations or Part 27 of the Federal Aviation Regulations and

manufactured under a production certificate.

(2) Used aircraft possessing a valid U.S. airworthiness certificate, or

other used Class I products that have been maintained in accordance with the

applicable CAR's or FAR's and are located in a foreign country, if the

Administrator finds that the location places no undue burden upon the FAA in

administering the provisions of this regulation.

(3) Class II and III products that are manufactured and located in the

United States.

(c) Export airworthiness approval exceptions. If the export airworthiness

approval is issued on the basis of a written statement by the importing state

as provided for in Sec. 21.327(e)(4), the requirements that are not met and

the differences in configuration, if any, between the product to be exported

and the related type certificated product, are listed on the export

airworthiness approval as exceptions.

[Amdt. 21-2, 30 FR 8465, July 2, 1965, as amended by Amdt. 21-14, 32 FR 2999,

Feb. 17, 1967; Amdt. 21-43, 40 FR 2577, Jan. 14, 1975; Amdt. 21-48, 44 FR

15649, Mar. 15, 1979]

Sec. 21.327  Application.

(a) Except as provided in paragraph (b) of this section, an application for

export airworthiness approval for a Class I or Class II product is made on a

form and in a manner prescribed by the Administrator and is submitted to the

appropriate Flight Standards District Office or to the nearest international

field office.

(b) A manufacturer holding a production certificate may apply orally to the

appropriate Flight Standards District Office or the nearest international

field office for export airworthiness approval of a Class II product approved

under his production certificate.

(c) Application for export airworthiness approval of Class III products is

made to the designated representative of the Administrator authorized to

issue those approvals.

(d) A separate application must be made for--

(1) Each aircraft;

(2) Each engine and propeller, except that one application may be made for

more than one engine or propeller, if all are of the same type and model and

are exported to the same purchaser and country; and

(3) Each type of Class II product, except that one application may be used

for more than one type of Class II product when--

(i) They are separated and identified in the application as to the type and

model of the related Class I product; and

(ii) They are to be exported to the same purchaser and country.

(e) Each application must be accompanied by a written statement from the

importing country that will validate the export airworthiness approval if the

product being exported is--

(1) An aircraft manufactured outside the United States and being exported

to a country with which the United States has a reciprocal agreement

concerning the validation of export certificates;

(2) An unassembled aircraft which has not been flight-tested;

(3) A product that does not meet the special requirement of the importing

country; or

(4) A product that does not meet a requirement specified in Secs. 21.329,

21.331, or 21.333, as applicable, for the issuance of an export airworthiness

approval. The written statement must list the requirements not met.

(f) Each application for export airworthiness approval of a Class I product

must include, as applicable:

(1) A Statement of Conformity, FAA Form 8130-9, for each new product that

has not been manufactured under a production certificate.

(2) A weight and balance report, with a loading schedule when applicable,

for each aircraft in accordance with Part 43 of this chapter. For transport

aircraft and commuter category airplanes this report must be based on an

actual weighing of the aircraft within the preceding twelve months, but after

any major repairs or alterations to the aircraft. Changes in equipment not

classed as major changes that are made after the actual weighing may be

accounted for on a "computed" basis and the report revised accordingly.

Manufacturers of new nontransport category airplanes, normal category

rotorcraft, and gliders may submit reports having computed weight and balance

data, in place of an actual weighing of the aircraft, if fleet weight control

procedures approved by the FAA have been established for such aircraft. In

such a case, the following statement must be entered in each report: "The

weight and balance data shown in this report are computed on the basis of

Federal Aviation Administration approved procedures for establishing fleet

weight averages." The weight and balance report must include an equipment

list showing weights and moment arms of all required and optional items of

equipment that are included in the certificated empty weight.

(3) A maintenance manual for each new product when such a manual is

required by the applicable airworthiness rules.

(4) Evidence of compliance with the applicable airworthiness directives. A

suitable notation must be made when such directives are not complied with.

(5) When temporary installations are incorporated in an aircraft for the

purpose of export delivery, the application form must include a general

description of the installations together with a statement that the

installation will be removed and the aircraft restored to the approved

configuration upon completion of the delivery flight.

(6) Historical records such as aircraft and engine log books, repair and

alteration forms, etc., for used aircraft and newly overhauled products.

(7) For products intended for overseas shipment, the application form must

describe the methods used, if any, for the preservation and packaging of such

products to protect them against corrosion and damage while in transit or

storage. The description must also indicate the duration of the effectiveness

of such methods.

(8) The Airplane or Rotorcraft Flight Manual when such material is required

by the applicable airworthiness regulations for the particular aircraft.

(9) A statement as to the date when title passed or is expected to pass to

a foreign purchaser.

(10) The data required by the special requirements of the importing

country.

[Amdt. 21-2, 30 FR 8465, July 2, 1965, as amended by Doc. No. 8084, 32 FR

5769, Apr. 11, 1967; Amdt. 21-48, 44 FR 15650, Mar. 15, 1979; Amdt. 21-59, 52

FR 1836, Jan. 15, 1987]

Sec. 21.329  Issue of export certificates of airworthiness for Class I

products.

An applicant is entitled to an export certificate of airworthiness for a

Class I product if that applicant shows at the time the product is submitted

to the Administrator for export airworthiness approval that it meets the

requirements of paragraphs (a) through (f) of this section, as applicable,

except as provided in paragraph (g) of this section:

(a) New or used aircraft manufactured in the United States must meet the

airworthiness requirement for a standard U.S. airworthiness certificate under

Sec. 21.183, or meet the airworthiness certification requirements for a

"restricted" airworthiness certificate under Sec. 21.185.

(b) New or used aircraft manufactured outside the United States must have a

valid U.S. standard airworthiness certificate.

(c) Used aircraft must have undergone an annual type inspection and be

approved for return to service in accordance with Part 43 of this chapter.

The inspection must have been performed and properly documented within 30

days before the date the application is made for an export certificate of

airworthiness. In complying with this paragraph, consideration may be given

to the inspections performed on an aircraft maintained in accordance with a

continuous airworthiness maintenance program under Part 121 or 127 of this

chapter or a progressive inspection program under Part 91 of this chapter,

within the 30 days prior to the date the application is made for an export

certificate of airworthiness.

(d) New engines and propellers must conform to the type design and must be

in a condition for safe operation.

(e) Used engines and propellers which are not being exported as part of a

certificated aircraft must have been newly overhauled.

(f) The special requirements of the importing country must have been met.

(g) A product need not meet a requirement specified in paragraphs (a)

through (f) of this section, as applicable, if acceptable to the importing

country and the importing country indicates that acceptability in accordance

with Sec. 21.327(e)(4) of this part.

[Amdt. 21-2, 30 FR 8465, July 2, 1965, as amended by Amdt. 21-8, 31 FR 2421,

Feb. 5, 1966; Amdt. 21-9, 31 FR 3336, Mar. 3, 1966; Amdt. 21-48, 44 FR 15650,

Mar. 15, 1979]

Sec. 21.331  Issue of airworthiness approval tags for Class II products.

(a) An applicant is entitled to an export airworthiness approval tag for

Class II products if that applicant shows, except as provided in paragraph

(b) of this section, that--

(1) The products are new or have been newly overhauled and conform to the

approved design data;

(2) The products are in a condition for safe operation;

(3) The products are identified with at least the manufacturer's name, part

number, model designation (when applicable), and serial number or equivalent;

and

(4) The products meet the special requirements of the importing country.

(b) A product need not meet a requirement specified in paragraph (a) of

this section if acceptable to the importing country and the importing country

indicates that acceptability in accordance with Sec. 21.327(e)(4) of this

part.

[Amdt. 21-2, 30 FR 8465, July 2, l965, as amended by Amdt. 21-48, 44 FR

15650, Mar. 15, 1979]

Sec. 21.333  Issue of export airworthiness approval tags for Class III

products.

(a) An applicant is entitled to an export airworthiness approval tag for

Class III products if that applicant shows, except as provided in paragraph

(b) of this section, that--

(1) The products conform to the approved design data applicable to the

Class I or Class II product of which they are a part;

(2) The products are in a condition for safe operation; and

(3) The products comply with the special requirements of the importing

country.

(b) A product need not meet a requirement specified in paragraph (a) of

this section if acceptable to the importing country and the importing country

indicates that acceptability in accordance with Sec. 21.327(e)(4) of this

part.

[Amdt. 21-2, 30 FR 8465, July 2, l965, as amended by Amdt. 21-48, 44 FR

15650, Mar. 15, 1979]

Sec. 21.335  Responsibilities of exporters.

Each exporter receiving an export airworthiness approval for a product

shall--

(a) Forward to the air authority of the importing country all documents and

information necessary for the proper operation of the products being

exported, e.g., Flight Manuals, Maintenance Manuals, Service Bulletins, and

assembly instructions, and such other material as is stipulated in the

special requirements of the importing country. The documents, information,

and material may be forwarded by any means consistent with the special

requirements of the importing country;

(b) Forward the manufacturer's assembly instructions and an FAA-approved

flight test checkoff form to the air authority of the importing country when

unassembled aircraft are being exported. These instructions must be in

sufficient detail to permit whatever rigging, alignment, and ground testing

is necessary to ensure that the aircraft will conform to the approved

configuration when assembled;

(c) Remove or cause to be removed any temporary installation incorporated

on an aircraft for the purpose of export delivery and restore the aircraft to

the approved configuration upon completion of the delivery flight;

(d) Secure all proper foreign entry clearances from all the countries

involved when conducting sales demonstrations or delivery flights; and

(e) When title to an aircraft passes or has passed to a foreign purchaser--

(1) Request cancellation of the U.S. registration and airworthiness

certificates, giving the date of transfer of title, and the name and address

of the foreign owner;

(2) Return the Registration and Airworthiness Certificates, AC Form 8050.3

and FAA Form 8100-2, to the FAA; and

(3) Submit a statement certifying that the United States' identification

and registration numbers have been removed from the aircraft in compliance

with Sec. 45.33.

[Amdt. 21-2, 30 FR 8465, July 2, 1965, as amended by Amdt. 21-48, 44 FR

15650, Mar. 15, 1979]

Sec. 21.337  Performance of inspections and overhauls.

Unless otherwise provided for in this subpart, each inspection and overhaul

required for export airworthiness approval of Class I and Class II products

must be performed and approved by one of the following:

(a) The manufacturer of the product.

(b) An appropriately certificated domestic repair station.

(c) An appropriately certificated foreign repair station having adequate

overhaul facilities, and maintenance organization appropriate to the product

involved, when the product is a Class I product located in a foreign country

and an international office of Flight Standards Service has approved the use

of such foreign repair station.

(d) The holder of an inspection authorization as provided in Part 65 of

this chapter.

(e) An air carrier, when the product is one that the carrier has maintained

under its own or another air carrier's continuous airworthiness maintenance

program and maintenance manuals as provided in Part 121 or 127 of this

chapter.

(f) A commercial operator, when the product is one that the operator has

maintained under its continuous airworthiness maintenance program and

maintenance manual as provided in Part 121 of this chapter.

[Amdt. 21-2, 30 FR 8465, July 2, 1965, as amended by Amdt. 21-8, 31 FR 2421,

Feb. 5, 1966]

Sec. 21.339  Special export airworthiness approval for aircraft.

A special export certificate of airworthiness may be issued for an aircraft

located in the United States that is to be flown to several foreign countries

for the purpose of sale, without returning the aircraft to the United States

for the certificate if--

(a) The aircraft possesses either--

(1) A standard U.S. certificate of airworthiness; or

(2) A special U.S. certificate of airworthiness in the restricted category

issued under Sec. 21.185;

(b) The owner files an application as required by Sec. 21.327 except that

items 3 and 4 of the application (FAA Form 8130-1) need not be completed;

(c) The aircraft is inspected by the Administrator before leaving the

United States and is found to comply with all the applicable requirements;

(d) A list of foreign countries in which it is intended to conduct sales

demonstrations, together with the expected dates and duration of such

demonstration, is included in the application;

(e) For each prospective importing country, the applicant shows that--

(1) He has met that country's special requirements, other than those

requiring that documents, information, and materials be furnished; and

(2) He has the documents, information, and materials necessary to meet the

special requirements of that country; and

(f) All other requirements for the issuance of a Class I export certificate

of airworthiness are met.

[Amdt. 21-12, 31 FR 12565, Sept. 23, 1966, as amended by Amdt. 21-43, 40 FR

2577, Jan. 14, 1975; Amdt. 21-55, 46 FR 44737, Sept. 8, 1981]

Subpart M--Designated Alteration Station Authorization Procedures

Source: Amdt. 21-6, 30 FR 11379, Sept. 8, 1965; 30 FR 11849, Sept. 16,

1965, unless otherwise noted.

Sec. 21.431  Applicability.

(a) This subpart prescribes Designated Alteration Station (DAS)

authorization procedures for--

(1) Issuing supplemental type certificates;

(2) Issuing experimental certificates; and

(3) Amending standard airworthiness certificates.

(b) This subpart applies to domestic repair stations, air carriers (except

air taxi operators), commercial operators of large aircraft, and

manufacturers of products.

Sec. 21.435  Application.

The applicant for a DAS authorization must submit an application, in

writing and signed by an official of the applicant, to the Aircraft

Certification Office responsible for the geographic area in which the

applicant is located. The application must contain--

(a) The repair station certificate number held by the repair station

applicant, and the current ratings covered by the certificate;

(b) The air carrier or commercial operator operating certificate number

held by the air carrier or commercial operator applicant, and the products

that it may operate and maintain under the certificate;

(c) A statement by the manufacturer applicant of the products for which he

holds the type certificate;

(d) The names, signatures, and titles of the persons for whom authorization

to issue supplemental type certificates or experimental certificates, or

amend airworthiness certificates, is requested; and

(e) A description of the applicant's facilities, and of the staff with

which compliance with Sec. 21.439(a)(4) is to be shown.

[Amdt. 21-6, 30 FR 11379, Sept. 8, 1965; 30 FR 11849, Sept. 16, 1965, as

amended by Amdt. 21-67, 54 FR 39291, Sept. 25, 1989]

Sec. 21.439  Eligibility.

(a) To be eligible for a DAS authorization, the applicant must--

(1) Hold a current domestic repair station certificate under Part 145, or

air carrier or commercial operator operating certificate under Part 121;

(2) Be a manufacturer of a product for which it has alteration authority

under Sec. 43.3(i) of this subchapter;

(3) Have adequate maintenance facilities and personnel, in the United

States, appropriate to the products that it may operate and maintain under

its certificate; and

(4) Employ, or have available, a staff of engineering, flight test, and

inspection personnel who can determine compliance with the applicable

airworthiness requirements of this chapter.

(b) At least one member of the staff required by paragraph (a)(4) of this

section must have all of the following qualifications:

(1) A thorough working knowledge of the applicable requirements of this

chapter.

(2) A position, on the applicant's staff, with authority to establish

alteration programs that ensure that altered products meet the applicable

requirements of this chapter.

(3) At least one year of satisfactory experience in direct contact with the

FAA (or its predecessor agency (CAA)) while processing engineering work for

type certification or alteration projects.

(4) At least eight years of aeronautical engineering experience (which may

include the one year required by paragraph (b)(3) of this section).

(5) The general technical knowledge and experience necessary to determine

that altered products, of the types for which a DAS authorization is

requested, are in condition for safe operation.

Sec. 21.441  Procedure manual.

(a) No DAS may exercise any authority under this subpart unless it submits,

and obtains approval of, a procedure manual containing--

(1) The procedures for issuing STCs; and

(2) The names, signatures, and responsibilities of officials and of each

staff member required by Sec. 21.439(a)(4), identifying those persons who--

(i) Have authority to make changes in procedures that require a revision to

the procedure manual; and

(ii) Are to conduct inspections (including conformity and compliance

inspections) or approve inspection reports, prepare or approve data, plan or

conduct tests, approve the results of tests, amend airworthiness

certificates, issue experimental certificates, approve changes to operating

limitations or Aircraft Flight Manuals, and sign supplemental type

certificates.

(b) No DAS may continue to perform any DAS function affected by any change

in facilities or staff necessary to continue to meet the requirements of Sec.

21.439, or affected by any change in procedures from those approved under

paragraph (a) of this section, unless that change is approved and entered in

the manual. For this purpose, the manual shall contain a log-of-revisions

page with space for the identification of each revised item, page, or date,

and the signature of the person approving the change for the Administrator.

Sec. 21.443  Duration.

(a) A DAS authorization is effective until it is surrendered or the

Administrator suspends, revokes, or otherwise terminates it.

(b) The DAS shall return the authorization certificate to the Administrator

when it is no longer effective.

Sec. 21.445  Maintenance of eligibility.

The DAS shall continue to meet the requirements for issue of the

authorization or shall notify the Administrator within 48 hours of any change

(including a change of personnel) that could affect the ability of the DAS to

meet those requirements.

Sec. 21.447  Transferability.

A DAS authorization is not transferable.

Sec. 21.449  Inspections.

Upon request, each DAS and each applicant shall let the Administrator

inspect his facilities, products, and records.

Sec. 21.451  Limits of applicability.

(a) DAS authorizations apply only to products--

(1) Covered by the ratings of the repair station applicant;

(2) Covered by the operating certificate and maintenance manual of the air

carrier or commercial operator applicant; and

(3) For which the manufacturer applicant has alteration authority under

Sec. 43.3(i) of this subchapter.

(b) DAS authorizations may be used for--

(1) The issue of supplemental type certificates;

(2) The issue of experimental certificates for aircraft that--

(i) Are altered by the DAS under a supplemental type certificate issued by

the DAS; and

(ii) Require flight tests in order to show compliance with the applicable

airworthiness requirements of this chapter; and

(3) The amendment of standard airworthiness certificates for aircraft

altered under this subpart.

(c) DAS authorizations are subject to any additional limitations prescribed

by the Administrator after inspection of the applicant's facilities or review

of the staff qualifications.

(d) Notwithstanding any other provision of this subpart, a DAS may not

issue a supplemental type certificate involving the exhaust emissions change

requirements of part 34 or the acoustical change requirements of part 36 of

this chapter until the Administrator finds that those requirements are met.

[Amdt. 21-6, 30 FR 11379, Sept. 8, 1965; 30 FR 11849, Sept. 16, 1965, as

amended by Amdt. 21-42, 40 FR 1034, Jan. 6, 1975; Amdt. 21-68, 55 FR 32860,

Aug. 10, 1990]

*****************************************************************************

55 FR 32856, No. 155, Aug. 10, 1990

SUMMARY: This final rule codifies as new part 34 all of the applicable

aircraft engine fuel venting and exhaust emission requirements of Special

Federal Aviation Regulation (SFAR) 27-5, and the test procedures specified

under the regulations implementing the Clean Air Act. This rule consolidates

all of the requirements and test procedures into this part, and inserts into

other affected parts the requirements to comply with new part 34. New part 34

does not alter any of the requirements specified under SFAR 27-5 or the

regulations implementing the Clean Air Act.

EFFECTIVE DATE: September 10, 1990.

*****************************************************************************

Sec. 21.461  Equivalent safety provisions.

The DAS shall obtain the Administrator's concurrence on the application of

all equivalent safety provisions applied under Sec. 21.21.

Sec. 21.463  Supplemental type certificates.

(a) For each supplemental type certificate issued under this subpart, the

DAS shall follow the procedure manual prescribed in Sec. 21.441 and shall,

before issuing the certificate--

(1) Submit to the Administrator a statement describing--

(i) The type design change;

(ii) The airworthiness requirements of this chapter (by part and effective

date) that the DAS considers applicable; and

(iii) The proposed program for meeting the applicable airworthiness

requirements;

(2) Find that each applicable airworthiness requirement is met; and

(3) Find that the type of product for which the STC is to be issued, as

modified by the supplemental type design data upon which the STC is based, is

of proper design for safe operation.

(b) Within 30 days after the date of issue of the STC, the DAS shall submit

to the Administrator--

(1) Two copies of the STC;

(2) One copy of the design data approved by the DAS and referred to in the

STC;

(3) One copy of each inspection and test report; and

(4) Two copies of each revision to the Aircraft Flight Manual or to the

operating limitations, and any other information necessary for safe operation

of the product.

Sec. 21.473  Airworthiness certificates other than experimental.

For each amendment made to a standard airworthiness certificate under this

subpart, the DAS shall follow the procedure manual prescribed in Sec. 21.441

and shall, before making that amendment--

(a) Complete each flight test necessary to meet the applicable

airworthiness requirements of this chapter;

(b) Find that each applicable airworthiness requirement of this chapter is

met; and

(c) Find that the aircraft is in condition for safe operation.

Sec. 21.475  Experimental certificates.

The DAS shall, before issuing an experimental certificate, obtain from the

Administrator any limitations and conditions that the Administrator considers

necessary for safety.

Sec. 21.477  Data review and service experience.

(a) If the Administrator finds that a product for which an STC was issued

under this subpart does not meet the applicable airworthiness requirements,

or that an unsafe feature or characteristic caused by a defect in design or

manufacture exists, the DAS, upon notification by the Administrator, shall

investigate the matter and report to the Administrator the results of the

investigation and the action, if any, taken or proposed.

(b) If corrective action by the user of the product is necessary for safety

because of any noncompliance or defect specified in paragraph (a) of this

section, the DAS shall submit the information necessary for the issue of an

Airworthiness Directive under Part 39.

Sec. 21.493  Current records.

(a) The DAS shall maintain, at its facility, current records containing--

(1) For each product for which it has issued an STC under this subpart, a

technical data file that includes any data and amendments thereto (including

drawings, photographs, specifications, instructions, and reports) necessary

for the STC;

(2) A list of products by make, model, manufacturer's serial number and, if

applicable, any FAA identification, that have been altered under the DAS

authorization; and

(3) A file of information from all available sources on alteration

difficulties of products altered under the DAS authorization.

(b) The records prescribed in paragraph (a) of this section shall be--

(1) Made available by the DAS, upon the Administrator's request, for

examination by the Administrator at any time; and

(2) In the case of the data file prescribed in paragraph (a)(1) of this

section, identified by the DAS and sent to the Administrator as soon as the

DAS no longer operates under this subpart.

Subpart N--Approval of Engines, Propellers, Materials, Parts, and Appliances:

Import

Sec. 21.500  Approval of engines and propellers.

Each holder or licensee of a U.S. type certificate for an aircraft engine

or propeller manufactured in a foreign country with which the United States

has an agreement for the acceptance of those products for export and import,

shall furnish with each such aircraft engine or propeller imported into this

country, a certificate of airworthiness for export issued by the country of

manufacture certifying that the individual aircraft engine or propeller--

(a) Conforms to its U.S. type certificate and is in condition for safe

operation; and

(b) Has been subjected by the manufacturer to a final operational check.

[Amdt. 21-25, 34 FR 14068, Sept. 5, 1969]

Sec. 21.502  Approval of materials, parts, and appliances.

(a) A material, part, or appliance, manufactured in a foreign country with

which the United States has an agreement for the acceptance of those

materials, parts, or appliances for export and import, is considered to meet

the requirements for approval in the Federal Aviation Regulations when the

country of manufacture issues a certificate of airworthiness for export

certifying that the individual material, part, or appliance meets those

requirements, unless the Administrator finds, based on the technical data

submitted under paragraph (b) of this section, that the material, part, or

appliance is otherwise not consistent with the intent of the Federal Aviation

Regulations.

(b) An applicant for approval of a material, part, or appliance must, upon

request, submit to the Administrator any technical data respecting that

material, part, or appliance.

[Amdt. 21-25, 34 FR 14068, Sept. 5, 1969]

Subpart O--Technical Standard Order Authorizations

Source: Docket No. 19589, 45 FR 38346, June 9, 1980, unless otherwise

noted.

Sec. 21.601  Applicability.

(a) This subpart prescribes--

(1) Procedural requirements for the issue of Technical Standard Order

authorizations;

(2) Rules governing the holders of Technical Standard Order authorizations;

and

(3) Procedural requirements for the issuance of a letter of Technical

Standard Order design approval.

(b) For the purpose of this subpart--

(1) A Technical Standard Order (referred to in this subpart as "TSO") is

issued by the Administrator and is a minimum performance standard for

specified articles (for the purpose of this subpart, articles means

materials, parts, processes, or appliances) used on civil aircraft.

(2) A TSO authorization is an FAA design and production approval issued to

the manufacturer of an article which has been found to meet a specific TSO.

(3) A letter of TSO design approval is an FAA design approval for a

foreign-manufactured article which has been found to meet a specific TSO in

accordance with the procedures of Sec. 21.617.

(4) An article manufactured under a TSO authorization, an FAA letter of

acceptance as described in Sec. 21.603(b), or an appliance manufactured under

a letter of TSO design approval described in Sec. 21.617 is an approved

article or appliance for the purpose of meeting the regulations of this

chapter that require the article to be approved.

(5) An article manufacturer is the person who controls the design and

quality of the article produced (or to be produced, in the case of an

application), including the parts of them and any processes or services

related to them that are procured from an outside source.

(c) The Administrator does not issue a TSO authorization if the

manufacturing facilities for the product are located outside of the United

States, unless the Administrator finds that the location of the

manufacturer's facilities places no undue burden on the FAA in administering

applicable airworthiness requirements.

Sec. 21.603   TSO marking and privileges.

(a) Except as provided in paragraph (b) of this section and Sec. 21.617(c),

no person may identify an article with a TSO marking unless that person holds

a TSO authorization and the article meets applicable TSO performance

standards.

(b) The holder of an FAA letter of acceptance of a statement of conformance

issued for an article before July 1, 1962, or any TSO authorization issued

after July 1, 1962, may continue to manufacture that article without

obtaining a new TSO authorization but shall comply with the requirements of

Secs. 21.3, 21.607 through 21.615, 21.619, and 21.621.

(c) Notwithstanding paragraphs (a) and (b) of this section, after August 6,

1976, no person may identify or mark an article with any of the following TSO

numbers:

(1) TSO-C18, -C18a, -C18b, -C18c.

(2) TSO-C24.

(3) TSO-C33.

(4) TSO-C61 or C61a.

Sec. 21.605  Application and issue.

(a) The manufacturer (or an authorized agent) shall submit an application

for a TSO authorization, together with the following documents, to the

Manager of the Aircraft Certification Office for the geographic area in which

the applicant is located:

(1) A statement of conformance certifying that the applicant has met the

requirements of this subpart and that the article concerned meets the

applicable TSO that is effective on the date of application for that article.

(2) One copy of the technical data required in the applicable TSO.

(3) A description of its quality control system in the detail specified in

Sec. 21.143. In complying with this section, the applicant may refer to

current quality control data filed with the FAA as part of a previous TSO

authorization application.

(b) When a series of minor changes in accordance with Sec. 21.611 is

anticipated, the applicant may set forth in its application the basic model

number of the article and the part number of the components with open

brackets after it to denote that suffix change letters or numbers (or

combinations of them) will be added from time to time.

(c) After receiving the application and other documents required by

paragraph (a) of this section to substantiate compliance with this part, and

after a determination has been made of its ability to produce duplicate

articles under this part, the Administrator issues a TSO authorization

(including all TSO deviations granted to the applicant) to the applicant to

identify the article with the applicable TSO marking.

(d) If the application is deficient, the applicant must, when requested by

the Administrator, submit any additional information necessary to show

compliance with this part. If the applicant fails to submit the additional

information within 30 days after the Administrator's request, the application

is denied and the applicant is so notified.

(e) The Administrator issues or denies the application within 30 days after

its receipt or, if additional information has been requested, within 30 days

after receiving that information.

[Doc. No. 19589, 45 FR 38346, June 9, 1989, as amended by 54 FR 39291, Sept.

25, 1989]

Sec. 21.607   General rules governing holders of TSO authorizations.

Each manufacturer of an article for which a TSO authorization has been

issued under this part shall--

(a) Manufacture the article in accordance with this part and the applicable

TSO;

(b) Conduct all required tests and inspections and establish and maintain a

quality control system adequate to ensure that the article meets the

requirements of paragraph (a) of this section and is in condition for safe

operation;

(c) Prepare and maintain, for each model of each article for which a TSO

authorization has been issued, a current file of complete technical data and

records in accordance with Sec. 21.613; and

(d) Permanently and legibly mark each article to which this section applies

with the following information:

(1) The name and address of the manufacturer.

(2) The name, type, part number, or model designation of the article.

(3) The serial number or the date of manufacture of the article or both.

(4) The applicable TSO number.

Sec. 21.609  Approval for deviation.

(a) Each manufacturer who requests approval to deviate from any performance

standard of a TSO shall show that the standards from which a deviation is

requested are compensated for by factors or design features providing an

equivalent level of safety.

(b) The request for approval to deviate, together with all pertinent data,

must be submitted to the Manager of the Aircraft Certification Office for the

geographic area in which the manufacturer is located. If the article is

manufactured in another country, the request for approval to deviate,

together with all pertinent data, must be submitted through the civil

aviation authority in that country to the FAA.

[Doc. No. 19589, 45 FR 38346, June 9, 1980, as amended by Amdt. 21-67, 54 FR

39291, Sept. 25, 1989]

Sec. 21.611  Design changes.

(a) Minor changes by the manufacturer holding a TSO authorization. The

manufacturer of an article under an authorization issued under this part may

make minor design changes (any change other than a major change) without

further approval by the Administrator. In this case, the changed article

keeps the original model number (part numbers may be used to identify minor

changes) and the manufacturer shall forward to the appropriate Aircraft

Certification Office for the geographic area any revised data that are

necessary for compliance with Sec. 21.605(b).

(b) Major changes by manufacturer holding a TSO authorization. Any design

change by the manufacturer that is extensive enough to require a

substantially complete investigation to determine compliance with a TSO is a

major change. Before making such a change, the manufacturer shall assign a

new type or model designation to the article and apply for an authorization

under Sec. 21.605.

(c) Changes by person other than manufacturer. No design change by any

person (other than the manufacturer who submitted the statement of

conformance for the article) is eligible for approval under this part unless

the person seeking the approval is a manufacturer and applies under Sec.

21.605(a) for a separate TSO authorization. Persons other than a manufacturer

may obtain approval for design changes under Part 43 or under the applicable

airworthiness regulations.

[Doc. No. 19589, 45 FR 38346, June 9, 1989, as amended by Amdt. 21-67, 54 FR

39291, Sept. 25, 1989]

Sec. 21.613  Recordkeeping requirements.

(a) Keeping the records. Each manufacturer holding a TSO authorization

under this part shall, for each article manufactured under that

authorization, keep the following records at its factory:

(1) A complete and current technical data file for each type or model

article, including design drawings and specifications.

(2) Complete and current inspection records showing that all inspections

and tests required to ensure compliance with this part have been properly

completed and documented.

(b) Retention of records. The manufacturer shall retain the records

described in paragraph (a)(1) of this section until it no longer manufactures

the article. At that time, copies of these records shall be sent to the

Administrator. The manufacturer shall retain the records described in

paragraph (a)(2) of this section for a period of at least 2 years.

Sec. 21.615  FAA inspection.

Upon the request of the Administrator, each manufacturer of an article

under a TSO authorization shall allow the Administrator to--

(a) Inspect any article manufactured under that authorization;

(b) Inspect the manufacturer's quality control system;

(c) Witness any tests;

(d) Inspect the manufacturing facilities; and

(e) Inspect the technical data files on that article.

Sec. 21.617  Issue of letters of TSO design approval: import appliances.

(a) A letter of TSO design approval may be issued for an appliance that is

manufactured in a foreign country with which the United States has an

agreement for the acceptance of these appliances for export and import and

that is to be imported into the United States if--

(1) The country in which the appliance was manufactured certifies that the

appliance has been examined, tested, and found to meet the applicable TSO

designated in Sec. 21.305(b) or the applicable performance standards of the

country in which the appliance was manufactured and any other performance

standards the Administrator may prescribe to provide a level of safety

equivalent to that provided by the TSO designated in Sec. 21.305(b); and

(2) The manufacturer has submitted one copy of the technical data required

in the applicable performance standard through its civil aviation authority.

(b) The letter of TSO design approval will be issued by the Administrator

and must list any deviation granted to the manufacturer under Sec. 21.609.

(c) After the Administrator has issued a letter of TSO design approval and

the country of manufacture issues a Certificate of Airworthiness for Export

as specified in Sec. 21.502(a), the manufacturer shall be authorized to

identify the appliance with the TSO marking requirements described in Sec.

21.607(d) and in the applicable TSO. Each appliance must be accompanied by a

Certificate of Ai@orthiness for Export as specified in Sec. 21.502(a) issued

by the country of manufacture.

Sec. 21.619  Noncompliance.

The Administrator may, upon notice, withdraw the TSO authorization or

letter of TSO design approval of any manufacturer who identifies with a TSO

marking an article not meeting the performance standards of the applicable

TSO.

Sec. 21.621  Transferability and duration.

A TSO authorization or letter of TSO design approval issued under this part

is not transferable and is effective until surrendered, withdrawn, or

otherwise terminated by the Administrator.