PART 71--DESIGNATION OF CLASS A, CLASS B, CLASS C, CLASS D, AND

CLASS E AIRSPACE AREAS; AIRWAYS; ROUTES; AND REPORTING POINTS

Subpart A--General; Class A Airspace

Sec.

71.1  Applicability.

71.3  [Reserved]

71.5  Reporting points.

71.7  Bearings, radials, amd mileages.

71.9  Overlapping airspace designations.

71.31  Class A airspace.

71.33  Class A airspace areas.

Subpart B--Class B Airspace

71.41  Class B airspace.

Subpart C--Class C Airspace

71.51  Class C airspace.

Subpart D--Class D Airspace

71.61  Class D airspace.

Subpart E--Class E Airspace

71.71  Class E airspace.

71.73  Classification of Federal airways.

71.75  Extent of Federal airways.

71.77  [Reserved]

71.79  Designation of VOR Federal airways.

Subpart F--[Reserved]

Subpart G--[Reserved]

Subpart H--Reporting Points

71.901  Applicability.

Authority: 49 U.S.C app. 1348(a), 1354(a), 1510, E.O. 10854; 49 U.S.C.

106(g); 14 CFR 11.69.

EFFECTIVE DATE NOTE: Amdt. 71-14, 56 FR 65654, Dec. 17, 1991, revised

part 71 effective September 16, 1993. The text of the table of contents of

part 71 in effect until September 16, 1993 reads as follows:

PART 71--DESIGNATION OF FEDERAL AIRWAYS, AREA LOW ROUTES, CONTROLLED

AIRSPACE, REPORTING POINTS, JET ROUTES, AND AREA HIGH ROUTES

Special Federal Aviation Regulation No. 45-1

Subpart A--General

Sec.

71.1  Applicability.

71.3  Classification of Federal airways.

71.5  Extent of Federal airways.

71.6  [Reserved]

71.7  Control areas.

71.9  Continental Control Area.

71.11  Control zones.

71.12  Terminal control areas.

71.13  Transition areas.

71.14  Airport radar service areas.

71.15  Positive control areas.

71.17  Reporting points.

71.19  Bearings; radials; miles.

Subparts B-K [Note]

Subpart L--Airport Radar Service

71.501  Designation.

Subpart M--Jet Routes and Area High Routes

71.601  Applicability.

71.603  Jet routes.

71.605  Area routes above 18,000 feet MSL.

71.607  Jet route descriptions.

71.609  Area high route descriptions.

Authority: 49 U.S.C app. 1348(a), 1354(a), 1510, E.O. 10854, 24 FR 9565, 3

CFR, 1959-1963 Comp., p. 389; 49 U.S.C. 106(g); 14 CFR 11.69.

Special Federal Aviation Regulation No. 45-1

Contrary provisions of the Federal Aviation Regulations notwithstanding,

1. The Robert Mueller Municipal Airport, Austin, TX, and the Port Columbus

International Airport, Columbus, OH, are designated Airport Radar Service

Areas (ARSA) airports at which, unless otherwise provided for in a Notice to

Airmen issued by the ARSA facility, radar services will be provided

continuously.

2. For purposes of this Special Federal Aviation Regulation:

(a) The Austin, TX, ARSA is that airspace extending upward from the surface

to and including 4,600 feet MSL within a 5 nautical mile radius of the Robert

Mueller Municipal Airport and that airspace extending upward from 2,000 feet

MSL to 4,600 feet MSL, within a 10 nautical mile radius of Robert Mueller

Municipal Airport from the 027 deg. true bearing from the airport clockwise

to the 207 deg. true bearing from the airport and that airspace extending

upward from 2,300 feet MSL to 4,600 feet MSL within a 10 nautical mile radius

of the airport from the 207 deg. true bearing from the airport clockwise to

the 027 deg. true bearing from the airport.

(b) The Columbus, OH, ARSA is that airspace extending upward from the

surface to and including 4,800 feet MSL within a 5 nautical mile radius of

the Port Columbus International Airport and that airspace extending upward

from 2,500 feet MSL to 4,800 feet MSL within a 10 nautical mile radius of

Port Columbus International Airport from the 008 deg. true bearing from the

airport clockwise to the 127 deg. true bearing from the airport and that

airspace extending upward from 2,200 feet MSL to 4,800 feet MSL within a 10

nautical mile radius of the airport from the 127 deg. true bearing from the

airport clockwise to the 008 deg. true bearing from the airport.

(c) The primary airport is the airport for which the ARSA is designated; a

satellite airport is any other airport, heliport, helipad, etc., within that

ARSA.

3. Unless otherwise authorized or required by ATC--

(a) Arrivals and Overflights--no person may operate an aircraft in an ARSA

unless two-way radio communication is established with ATC prior to entering

that ARSA and is thereafter maintained with ATC while within that ARSA;

(b) Departures--no person may operate an aircraft within an ARSA unless

two-way radio communication is maintained with ATC while within that ARSA,

except that for aircraft departing a satellite airport, two-way radio

communication is established as soon as practicable and thereafter maintained

with ATC while within that ARSA;

(c) ATC Instructions and Clearances--except in an emergency, no person may,

while within an ARSA, operate an aircraft contrary to an ATC clearance or

instruction;

(d) Traffic Patterns--no person may takeoff or land an aircraft within an

ARSA except in compliance with FAA arrival and departure traffic patterns;

and

(e) Ultralight Vehicle and Parachute Jump Operations--no person may operate

an ultralight vehicle within, or make a parachute jump within or into, an

ARSA except under the terms of an ATC authorization issued by the ATC

facility having jurisdiction over that ARSA.

4. In addition to the required preflight actions of FAR Section 91.103,

each pilot in command shall review NOTAM's pertaining to flight within an

ARSA prior to conducting flight in an ARSA.

This SFAR terminates at 12:01 a.m. local time on June 21, 1985, unless

sooner superseded or rescinded.

(Secs. 307 and 313(a), Federal Aviation Act of 1985, as amended (49 U.S.C.

1348, 1354(a)); 49 U.S.C. 106(g) (Revised, Pub. L. 97-449, January 12, 1983);

14 CFR 11.45; and 14 CFR 11.65)

[Doc. No. 23708, 49 FR 47177, Nov. 30, 1984; 49 FR 49089, Dec. 18, 1984, as

amended by Amdt. 71-13, 54 FR 34331, Aug. 18, 1989]

Effective Date Note: At 54 FR 34331, August 18, 1989, SFAR 45-1 was amended

by changing the cross reference "Section 91.5" to read "Section 91.103",

effective August 18, 1990.

Subpart A--General; Class A Airspace

Effective Date Note: Amdt. No. 71-14, 56 FR 65655, Dec. 17, 1991, revised

Subpart A effective September 16, 1993. Subpart A in effect until September

16, 1993 reads as follows:

Subpart A--General

Authority: 49 U.S.C. 1348(a) and 1354(a); 49 U.S.C. 106(g) (Revised, Pub.

L. 97-449, January 12, 1983); 14 CFR 11.69.

Source: 46 FR 403, Jan. 2, 1981, unless otherwise noted.

Sec. 71.1 Applicability.

The complete listing for all Class A, Class B, Class C, Class D, and Class

E airspace areas and for all reporting points can be found in FAA Order

7400.9B, Airspace Designations and Reporting Points, dated July 18, 1994.

This incorporation by reference was approved by the Director of the Federal

Register in accordance with 5 U.S.C. 552(a) and 1 CFR part 51. The approval

to incorporate by reference FAA Order 7400.9B is effective September 16,

1994, through September 15, 1995. During the incorporation by reference

period, proposed changes to the listings of Class A, Class B, Class C, Class

D, and Class E airspace areas and to reporting points will be published in

full text as proposed rule documents in the Federal Register. Amendments to

the listings of Class A, Class B, Class C, Class D, and Class E airspace

areas and to reporting points will be published in full text as final rules

in the Federal Register. Periodically, the final rule amendments will be

integrated into a revised edition of the order and submitted to the Director

of the Federal Register for approval for incorporation by reference in this

section. Copies of FAA Order 7400.9B may be obtained from the Document

Inspection Facility, APA-220, Federal Aviation Administration, 800

Independence Avenue, SW., Washington, DC 20591, (202) 267-3485. Copies of FAA

Order 7400.9B may be inspected in Docket No. 27855 at the Federal Aviation

Administration, Office of the Chief Counsel, AGC-200, Room 915G, 800

Independence Avenue, SW., Washington, DC weekdays between 8:30 a.m. and 5:00

p.m., or at the Office of the Federal Register, 800 North Capitol Street,

NW., Suite 700, Washington, DC. This section is effective September 16, 1994,

through September 15, 1995.

[Amdt. 71-23, 59 FR 43035, Aug. 22, 1994]

Effective date note: Amdt. No. 71-20, 58 FR 36298, July 6, 1993, revised

Sec. 71.1 effective from September 16, 1993, through September 15, 1994. See

text set out below.

Sec. 71.1   Applicability.

The complete listing for all Class A, Class B, Class C, Class D, and Class

E airspace areas and for all reporting points can be found in FAA Order

7400.9A, Airspace Designations and Reporting Points, dated June 17, 1993.

This incorporation by reference was approved by the Director of the Federal

Register in accordance with 5 U.S.C. 552(a) and 1 CFR part 51. The approval

to incorporate by reference FAA Order 7400.9A is effective September 16,

1993, through September 15, 1994. During the incorporation by reference

period, proposed changes to the listings of Class A, Class B, CLass C, Class

D, and Class E airspace areas and to reporting points will be published in

full text as proposed rule documents in the Federal Register. Amendments to

the listings of Class A, Class B, Class C, Class D, and Class E airspace

areas and to reporting points will be published in full text as final rules

in the Federal Register. Periodically, the final rule amendments will be

integrated into a revised edition of the order and submitted to the Director

of the Federal Register for approval for incorporation by reference in this

section. Copies of FAA Order 7400.9A may be obtained from the Document

Inspection Facility, APA-220, Federal Aviation Administration, 800

Independence Avenue, SW., Washington, DC 20591 (202) 267-3485. Copies of FAA

Order 7400.9A may be inspected in Docket No. 27342 at the Federal Aviation

Administration, Office of the Chief Counsel, AGC-200, room 915G, 800

Independence Avenue, SW., Washington, DC weekdays between 8:30 a.m. and 5

p.m., or at the Office of the Federal Register, 800 North Capitol Street,

NW., suite 700, Washington, D.C. This section is effective September 16,

1993, through September 15, 1994.

[Amdt. 71-20, 58 FR 36298, July 6, 1993]

*****************************************************************************

59 FR 43034, No. 161, Aug. 22, 1994

SUMMARY: This action amends the Federal Aviation Regulations relating to

airspace designations to reflect the approval by the Director of the Federal

Register of the incorporation by reference of FAA Order 7400.9B, Airspace

Designations and Reporting Points. This action also explains the procedures

FAA will use to amend the listings of Class A, Class B, Class C, Class D, and

Class E airspace areas and reporting points incorporated by reference.

EFFECTIVE DATE: These regulations are effective September 16, 1994, through

September 15, 1995. The incorporation by reference of FAA Order 7400.9B is

approved by the Director of the Federal Register as of September 16, 1994,

through September 15, 1995.

*****************************************************************************

Sec. 71.3   [Reserved]

Effective Date Note: Amdt. No. 71-14, 56 FR 65655, Dec. 17, 1991, reserved

this section effective September 16, 1993. The text of Sec. 71.3 in effect

until September 16, 1993 reads as follows:

Sec. 71.3   Classification of Federal airways.

Federal airways are classified as follows:

(a) Colored Federal airways:

(1) Green Federal airways.

(2) Amber Federal airways.

(3) Red Federal airways.

(4) Blue Federal airways.

(b) VOR Federal airways.

*****************************************************************************

56 FR 65638, No. 242, Dec. 17, 1991

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) to

adopt certain recommendations of the National Airspace Review (NAR)

concerning changes to regulations and procedures in regard to airspace

classifications. These changes are intended to: (1) Simplify airspace

designations; (2) achieve international commonality of airspace designations;

(3) increase standardization of equipment requirements for operations in

various classifications of airspace; (4) describe appropriate pilot

certificate requirements, visual flight rules (VFR) visibility and distance

from cloud rules, and air traffic services offered in each class of airspace;

and (5) satisfy the responsibilities of the United States as a member of the

International Civil Aviation Organization (ICAO). The final rule also amends

the requirement for minimum distance from clouds in certain airspace areas

and the requirements for communications with air traffic control (ATC) in

certain airspace areas; eliminates airport radar service areas (ARSAs),

control zones, and terminal control areas (TCAs) as airspace classifications;

and eliminates the term "airport traffic area." The FAA believes simplified

airspace classifications will reduce existing airspace complexity and thereby

enhance safety.

EFFECTIVE DATE: These regulations become effective September 16, 1993,

except that Secs. 11.61(c), 91.215(d), 71.601, 71.603, 71.605, 71.607, and

71.609 and Part 75 become effective December 12, 1991, and except that

amendatory instruction number 20, Sec. 71.1 is effective as of September 15,

1993, and that Secs. 71.11 and 71.19 become effective October 15, 1992. The

incorporation by reference of FAA Order 7400.7 in Sec. 71.1 (amendatory

instruction number 20) is approved by the Director of the Federal Register

as of December 17, 1991, through September 15, 1993. The incorporation by

reference of FAA Order 7400.9 in Sec. 71.1 (amendatory instruction number 24)

is approved by the Director of the Federal Register as of September 16,

1993 through September 15, 1994.

*****************************************************************************

Sec. 71.5   Reporting points.

The reporting points listed in subpart H of FAA Order 7400.9B (incorporated

by reference, see Sec. 71.1) consist of geographic locations at which the

position of an aircraft must be reported in accordance with part 91 of this

chapter.

[Amdt. 71-14, 56 FR 65655, Dec. 17, 1991, as amended by Amdt. 71-20, 58 FR

36299, July 6, 1993; Amdt. 71-23, 59 FR 43035, Aug. 22, 1994]

*****************************************************************************

59 FR 43034, No. 161, Aug. 22, 1994

SUMMARY: This action amends the Federal Aviation Regulations relating to

airspace designations to reflect the approval by the Director of the Federal

Register of the incorporation by reference of FAA Order 7400.9B, Airspace

Designations and Reporting Points. This action also explains the procedures

FAA will use to amend the listings of Class A, Class B, Class C, Class D, and

Class E airspace areas and reporting points incorporated by reference.

EFFECTIVE DATE: These regulations are effective September 16, 1994, through

September 15, 1995. The incorporation by reference of FAA Order 7400.9B is

approved by the Director of the Federal Register as of September 16, 1994,

through September 15, 1995.

*****************************************************************************

Sec. 71.7   Bearings, radials, and mileages.

All bearings and radials in this part are true and are applied from point

of origin and all mileages in this part are stated as nautical miles.

Effective Date Note: Amdt. No. 71-14, 56 FR 65655, Dec. 17, 1991, revised

this section effective September 16, 1993. The text of Sec. 71.7 in effect

until September 16, 1993 reads as follows:

Sec. 71.7   Control areas.

Control areas consist of the airspace designated in Subparts B, C, E, and

J, but do not include the continental control area. Unless otherwise

designated, control areas include the airspace between a segment of a main

VOR Federal airway and its associated alternate segments with the vertical

extent of the area corresponding to the vertical extent of the related

segment of the main airway.

*****************************************************************************

56 FR 65638, No. 242, Dec. 17, 1991

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) to

adopt certain recommendations of the National Airspace Review (NAR)

concerning changes to regulations and procedures in regard to airspace

classifications. These changes are intended to: (1) Simplify airspace

designations; (2) achieve international commonality of airspace designations;

(3) increase standardization of equipment requirements for operations in

various classifications of airspace; (4) describe appropriate pilot

certificate requirements, visual flight rules (VFR) visibility and distance

from cloud rules, and air traffic services offered in each class of airspace;

and (5) satisfy the responsibilities of the United States as a member of the

International Civil Aviation Organization (ICAO). The final rule also amends

the requirement for minimum distance from clouds in certain airspace areas

and the requirements for communications with air traffic control (ATC) in

certain airspace areas; eliminates airport radar service areas (ARSAs),

control zones, and terminal control areas (TCAs) as airspace classifications;

and eliminates the term "airport traffic area." The FAA believes simplified

airspace classifications will reduce existing airspace complexity and thereby

enhance safety.

EFFECTIVE DATE: These regulations become effective September 16, 1993,

except that Secs. 11.61(c), 91.215(d), 71.601, 71.603, 71.605, 71.607, and

71.609 and Part 75 become effective December 12, 1991, and except that

amendatory instruction number 20, Sec. 71.1 is effective as of September 15,

1993, and that Secs. 71.11 and 71.19 become effective October 15, 1992. The

incorporation by reference of FAA Order 7400.7 in Sec. 71.1 (amendatory

instruction number 20) is approved by the Director of the Federal Register

as of December 17, 1991, through September 15, 1993. The incorporation by

reference of FAA Order 7400.9 in Sec. 71.1 (amendatory instruction number 24)

is approved by the Director of the Federal Register as of September 16,

1993 through September 15, 1994.

*****************************************************************************

Sec. 71.9   Overlapping airspace designations.

(a) When overlapping airspace designations apply to the same airspace, the

operating rules associated with the more restrictive airspace designation

apply.

(b) For the purpose of this section--

(1) Class A airspace is more restrictive than Class B, Class C, Class D,

Class E, or Class G airspace;

(2) Class B airspace is more restrictive than Class C, Class D, Class E, or

Class G airspace;

(3) Class C airspace is more restrictive than Class D, Class E, or Class G

airspace;

(4) Class D airspace is more restrictive than Class E or Class G airspace;

and

(5) Class E is more restrictive than Class G airspace.

Effective Date Note: Amdt. No. 71-14, 56 FR 65655, Dec. 17, 1991, revised

this section effective September 16, 1993. The text of Sec. 71.9 in effect

until September 16, 1993 reads as follows:

Sec. 71.9  Continental control area.

The Continental Control Area consists of the airspace at and above 14,500

feet MSL overlying the 48 contiguous States, including the waters within 12

nautical miles of the coast of the 48 contiguous States; the District of

Columbia; Alaska, including the waters within 12 nautical miles of the coast

of Alaska; excluding the Alaska peninsula west of longitude 160 deg.00'00"

W. The Continental Control Area does not include the airspace less than

1,500 feet above the surface of the earth.

[Doc. No. 25768, 54 FR 265, Jan. 4, 1989, as amended by Amdt. 71-19, 58 FR

12136, Mar. 2, 1993]

*****************************************************************************

58 FR 12128, No. 39, Mar. 2, 1993

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) by

designating additional control areas as offshore airspace areas or en route

domestic airspace areas, as appropriate; revising certain additional control

areas; including restricted and prohibited areas in the Continental Control

Area; eliminating domestic area low routes; eliminating control areas

associated with jet routes outside the Continental Control Area; eliminating

domestic high altitude reporting points; eliminating certain domestic low

altitude reporting points; eliminating the special air traffic rules for

Flushing, New York; and replacing the Valparaiso, Florida terminal area and

special air traffic rules with the Elgin, Florida Class D airspace areas.

These amendments respond to recommendations from the National Airspace Review

(NAR) and meet a goal of the Airspace Reclassification final rule--to

simplify airspace assignment and use.

EFFECTIVE DATES: The amendments to Secs. 71.1 and 71.9 which are currently

in effect, become effective April 1, 1993, through September 15, 1993; the

removal of Sec. 71.6 becomes effective April 1, 1993; the removal of subpart

P of part 93 becomes effective June 20, 1993; the amendments to Secs. 71.1,

71.33 and 71.71 which are effective September 16, 1993, and the removal of

Sec. 71.77, become effective September 16, 1993; and the removal of subpart F

of part 93 becomes effective December 9, 1993.

*****************************************************************************

Sec. 71.31  Class A airspace.

The airspace descriptions contained in Sec. 71.33 and the routes contained

in subpart A of FAA Order 7400.9B (incorporated by reference, see Sec. 71.1)

are designated as Class A airspace within which all pilots and aircraft are

subject to the rating requirements, operating rules, and equipment

requirements of Part 91 of this chapter.

[Amdt. 71-14, 56 FR 65655, Dec. 17, 1991, as amended by Amdt. 71-20, 58 FR

36299, July 6, 1993; Amdt. 71-23, 59 FR 43035, Aug. 22, 1994]

*****************************************************************************

59 FR 43034, No. 161, Aug. 22, 1994

SUMMARY: This action amends the Federal Aviation Regulations relating to

airspace designations to reflect the approval by the Director of the Federal

Register of the incorporation by reference of FAA Order 7400.9B, Airspace

Designations and Reporting Points. This action also explains the procedures

FAA will use to amend the listings of Class A, Class B, Class C, Class D, and

Class E airspace areas and reporting points incorporated by reference.

EFFECTIVE DATE: These regulations are effective September 16, 1994, through

September 15, 1995. The incorporation by reference of FAA Order 7400.9B is

approved by the Director of the Federal Register as of September 16, 1994,

through September 15, 1995.

*****************************************************************************

Sec. 71.33   Class A airspace areas.

(a) That airspace of the United States, including that airspace overlying

the waters within 12 nautical miles of the coast of the 48 contiguous States,

from 18,000 feet MSL to and including FL600 excluding the states of Alaska

and Hawaii, Santa Barbara Island, Farallon Island, and the airspace south of

latitude 25 deg.04'00" North.

(b) That airspace of the State of Alaska, including that airspace overlying

the waters within 12 nautical miles of the coast, from 18,000 feet MSL to and

including FL600 but not including the airspace less than 1,500 feet above the

surface of the earth and the Alaska Peninsula west of longitude 160

deg.00'00" West.

(c) The airspace areas listed as offshore airspace areas in subpart A of

FAA Order 7400.9B (incorporated by reference, see Sec. 71.1) that are

designated in international airspace within areas of domestic radio

navigational signal or ATC radar coverage, and within which domestic ATC

procedures are applied.

[Amdt. 71-14, 56 FR 65655, Dec. 17, 1991, as amended by Amdt. 71-19, 58 FR

12137, Mar. 2, 1993; Amdt. 71-23, 59 FR 43035, Aug. 22, 1994]

*****************************************************************************

59 FR 43034, No. 161, Aug. 22, 1994

SUMMARY: This action amends the Federal Aviation Regulations relating to

airspace designations to reflect the approval by the Director of the Federal

Register of the incorporation by reference of FAA Order 7400.9B, Airspace

Designations and Reporting Points. This action also explains the procedures

FAA will use to amend the listings of Class A, Class B, Class C, Class D, and

Class E airspace areas and reporting points incorporated by reference.

EFFECTIVE DATE: These regulations are effective September 16, 1994, through

September 15, 1995. The incorporation by reference of FAA Order 7400.9B is

approved by the Director of the Federal Register as of September 16, 1994,

through September 15, 1995.

*****************************************************************************

Subpart B--Class B Airspace

Effective date note: Amdt. 71-14, 56 FR 65655, Dec. 17, 1991, added Subpart

B to Part 71 effective September 16, 1993.

Sec. 71.41  Class B airspace.

The Class B airspace areas listed in subpart B of FAA Order 7400.9B

(incorporated by reference, see Sec. 71.1) consist of specified airspace

within which all aircraft operators are subject to the minimum pilot

qualification requirements, operating rules, and aircraft equipment

requirements of part 91 of this chapter. Each Class B airspace area

designated for an airport in subpart B of FAA Order 7400.9B (incorporated by

reference, see Sec. 71.1) contains at least one primary airport around which

the airspace is designated.

[Amdt. 71-14, 56 FR 65655, Dec. 17, 1991, as amended by Amdt. 71-20, 58 FR

36299, July 6, 1993; Amdt. 71-23, 59 FR 43035, Aug. 22, 1994]

*****************************************************************************

59 FR 43034, No. 161, Aug. 22, 1994

SUMMARY: This action amends the Federal Aviation Regulations relating to

airspace designations to reflect the approval by the Director of the Federal

Register of the incorporation by reference of FAA Order 7400.9B, Airspace

Designations and Reporting Points. This action also explains the procedures

FAA will use to amend the listings of Class A, Class B, Class C, Class D, and

Class E airspace areas and reporting points incorporated by reference.

EFFECTIVE DATE: These regulations are effective September 16, 1994, through

September 15, 1995. The incorporation by reference of FAA Order 7400.9B is

approved by the Director of the Federal Register as of September 16, 1994,

through September 15, 1995.

*****************************************************************************

Subpart C--Class C Airspace

Effective Date Note: Amdt. 71-41, 56 FR 65655, Dec. 17, 1991, added Subpart

C to Part 71 effective September 16, 1993.

*****************************************************************************

56 FR 65638, No. 242, Dec. 17, 1991

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) to

adopt certain recommendations of the National Airspace Review (NAR)

concerning changes to regulations and procedures in regard to airspace

classifications. These changes are intended to: (1) Simplify airspace

designations; (2) achieve international commonality of airspace designations;

(3) increase standardization of equipment requirements for operations in

various classifications of airspace; (4) describe appropriate pilot

certificate requirements, visual flight rules (VFR) visibility and distance

from cloud rules, and air traffic services offered in each class of airspace;

and (5) satisfy the responsibilities of the United States as a member of the

International Civil Aviation Organization (ICAO). The final rule also amends

the requirement for minimum distance from clouds in certain airspace areas

and the requirements for communications with air traffic control (ATC) in

certain airspace areas; eliminates airport radar service areas (ARSAs),

control zones, and terminal control areas (TCAs) as airspace classifications;

and eliminates the term "airport traffic area." The FAA believes simplified

airspace classifications will reduce existing airspace complexity and thereby

enhance safety.

EFFECTIVE DATE: These regulations become effective September 16, 1993,

except that Secs. 11.61(c), 91.215(d), 71.601, 71.603, 71.605, 71.607, and

71.609 and Part 75 become effective December 12, 1991, and except that

amendatory instruction number 20, Sec. 71.1 is effective as of September 15,

1993, and that Secs. 71.11 and 71.19 become effective October 15, 1992. The

incorporation by reference of FAA Order 7400.7 in Sec. 71.1 (amendatory

instruction number 20) is approved by the Director of the Federal Register

as of December 17, 1991, through September 15, 1993. The incorporation by

reference of FAA Order 7400.9 in Sec. 71.1 (amendatory instruction number 24)

is approved by the Director of the Federal Register as of September 16,

1993 through September 15, 1994.

*****************************************************************************

Sec. 71.51  Class C airspace.

The Class C airspace areas listed in subpart C of FAA Order 7400.9B

(incorporated by reference, see Sec. 71.1) consist of specified airspace

within which all aircraft operators are subject to operating rules and

equipment requirements specified in part 91 of this chapter. Each Class C

airspace area designated for an airport in subpart C of FAA Order 7400.9B

(incorporated by reference, see Sec. 71.1) contains at least one primary

airport around which the airspace is designated

[Amdt. 71-14, 56 FR 65655, Dec. 17, 1991, as amended by Amdt. 71-20, 58 FR

36299, July 6, 1993; Amdt. 71-23, 59 FR 43035, Aug. 22, 1994]

*****************************************************************************

59 FR 43034, No. 161, Aug. 22, 1994

SUMMARY: This action amends the Federal Aviation Regulations relating to

airspace designations to reflect the approval by the Director of the Federal

Register of the incorporation by reference of FAA Order 7400.9B, Airspace

Designations and Reporting Points. This action also explains the procedures

FAA will use to amend the listings of Class A, Class B, Class C, Class D, and

Class E airspace areas and reporting points incorporated by reference.

EFFECTIVE DATE: These regulations are effective September 16, 1994, through

September 15, 1995. The incorporation by reference of FAA Order 7400.9B is

approved by the Director of the Federal Register as of September 16, 1994,

through September 15, 1995.

*****************************************************************************

Subpart D--Class D Airspace

Effective Date Note: Amdt. No. 71-14, 56 FR 65655, Dec. 17, 1991, added

Subpart D to Part 71 effective September 16, 1993.

Sec. 71.61  Class D airspace.

The Class D airspace areas listed in subpart D of FAA Order 7400.9B

(incorporated by reference, see Sec. 71.1) consist of specified airspace

within which all aircraft operators are subject to operating rules and

equipment requirements specified in part 91 of this chapter. Each Class D

airspace area designated for an airport in subpart D of FAA Order 7400.9B

(incorporated by reference, see Sec. 71.1) contains at least one primary

airport around which the airspace is designated.

[Amdt. 71-14, 56 FR 65655, Dec. 17, 1991, as amended by Amdt. 71-20, 58 FR

36299, July 6, 1993; Amdt. 71-23, 59 FR 43035, Aug. 22, 1994]

*****************************************************************************

59 FR 43034, No. 161, Aug. 22, 1994

SUMMARY: This action amends the Federal Aviation Regulations relating to

airspace designations to reflect the approval by the Director of the Federal

Register of the incorporation by reference of FAA Order 7400.9B, Airspace

Designations and Reporting Points. This action also explains the procedures

FAA will use to amend the listings of Class A, Class B, Class C, Class D, and

Class E airspace areas and reporting points incorporated by reference.

EFFECTIVE DATE: These regulations are effective September 16, 1994, through

September 15, 1995. The incorporation by reference of FAA Order 7400.9B is

approved by the Director of the Federal Register as of September 16, 1994,

through September 15, 1995.

*****************************************************************************

Subpart E--Class E Airspace

Effective Date Note: Amdt. No. 71-14, 56 FR 65655, Dec. 17, 1991, added

Subpart E to Part 71 effective September 16, 1993.

*****************************************************************************

56 FR 65638, No. 242, Dec. 17, 1991

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) to

adopt certain recommendations of the National Airspace Review (NAR)

concerning changes to regulations and procedures in regard to airspace

classifications. These changes are intended to: (1) Simplify airspace

designations; (2) achieve international commonality of airspace designations;

(3) increase standardization of equipment requirements for operations in

various classifications of airspace; (4) describe appropriate pilot

certificate requirements, visual flight rules (VFR) visibility and distance

from cloud rules, and air traffic services offered in each class of airspace;

and (5) satisfy the responsibilities of the United States as a member of the

International Civil Aviation Organization (ICAO). The final rule also amends

the requirement for minimum distance from clouds in certain airspace areas

and the requirements for communications with air traffic control (ATC) in

certain airspace areas; eliminates airport radar service areas (ARSAs),

control zones, and terminal control areas (TCAs) as airspace classifications;

and eliminates the term "airport traffic area." The FAA believes simplified

airspace classifications will reduce existing airspace complexity and thereby

enhance safety.

EFFECTIVE DATE: These regulations become effective September 16, 1993,

except that Secs. 11.61(c), 91.215(d), 71.601, 71.603, 71.605, 71.607, and

71.609 and Part 75 become effective December 12, 1991, and except that

amendatory instruction number 20, Sec. 71.1 is effective as of September 15,

1993, and that Secs. 71.11 and 71.19 become effective October 15, 1992. The

incorporation by reference of FAA Order 7400.7 in Sec. 71.1 (amendatory

instruction number 20) is approved by the Director of the Federal Register

as of December 17, 1991, through September 15, 1993. The incorporation by

reference of FAA Order 7400.9 in Sec. 71.1 (amendatory instruction number 24)

is approved by the Director of the Federal Register as of September 16,

1993 through September 15, 1994.

*****************************************************************************

Sec. 71.71  Class E airspace.

Class E Airspace consists of:

(a) The airspace of the United States, including that airspace overlying

the waters within 12 nautical miles of the coast of the 48 contiguous states

and Alaska, extending upward from 14,500 feet MSL up to, but not including

18,000 feet MSL, and the airspace above FL600, excluding--

(1) The Alaska peninsula west of longitude 160 deg.00'00"W.; and

(2) The airspace below 1,500 feet above the surface of the earth.

(b) The airspace areas designated for an airport in subpart E of FAA Order

7400.9B (incorporated by reference, see Sec. 71.1) within which all aircraft

operators are subject to the operating rules specified in part 91 of this

chapter.

(c) The airspace areas listed as domestic airspace areas in subpart E of

FAA Order 7400.9B (incorporated by reference, see Sec. 71.1) which extend

upward from 700 feet or more above the surface of the earth when designated

in conjunction with an airport for which an approved instrument approach

procedure has been prescribed, or from 1,200 feet or more above the surface

of the earth for the purpose of transitioning to or from the terminal or en

route environment. When such areas are designated in conjunction with airways

or routes, the extent of such designation has the lateral extent identical to

that of a Federal airway and extends upward from 1,200 feet or higher. Unless

otherwise specified, the airspace areas in the paragraph extend upward from

1,200 feet or higher above the surface to, but not including, 14,500 feet

MSL.

(d) The Federal airways described in subpart E of FAA Order 7400.9B

(incorporated by reference, see Sec. 71.1).

(e) The airspace areas listed as en route domestic airspace areas in

subpart E of FAA Order 7400.9B (incorporated by reference, see Sec. 71.1).

Unless otherwise specified, each airspace area has a lateral extent

identical to that of a Federal airway and extends upward from 1,200 feet

above the surface of the earth to the overlying or adjacent controlled

airspace.

(f) The airspace area listed as offshore airspace areas in subpart E of

FAA Order 7400.9B (incorporated by reference, see Sec. 71.1) that are

designated in international airspace within areas of domestic radio

navigational signal or ATC radar coverage, and within which domestic ATC

procedures are applied. Unless otherwise specified, each airspace area

extends upward from a specified altitude up to, but not including, 18,000

feet MSL.

[Amdt. 71-14, 56 FR 65655, Dec. 17, 1991, as amended by Amdt. 71-19, 58 FR

12137, Mar. 2, 1993; Amdt. 71-16, 58 FR 15259, Mar. 19, 1993; Amdt. 71-20, 58

FR 36299, July 6, 1993; Amdt. 71-21, 58 FR 44127, Aug. 19, 1993; Amdt. 71-23,

59 FR 43035, Aug. 22, 1994]

*****************************************************************************

59 FR 43034, No. 161, Aug. 22, 1994

SUMMARY: This action amends the Federal Aviation Regulations relating to

airspace designations to reflect the approval by the Director of the Federal

Register of the incorporation by reference of FAA Order 7400.9B, Airspace

Designations and Reporting Points. This action also explains the procedures

FAA will use to amend the listings of Class A, Class B, Class C, Class D, and

Class E airspace areas and reporting points incorporated by reference.

EFFECTIVE DATE: These regulations are effective September 16, 1994, through

September 15, 1995. The incorporation by reference of FAA Order 7400.9B is

approved by the Director of the Federal Register as of September 16, 1994,

through September 15, 1995.

*****************************************************************************

Sec. 71.73  Classification of Federal airways.

Federal airways are classified as follows:

(a) Colored Federal airways:

(1) Green Federal airways.

(2) Amber Federal airways.

(3) Red Federal airways.

(4) Blue Federal airways.

(b) VOR Federal airways.

Effective Date Note: Amdt. No. 71-14, 56 FR 65656, Dec. 17, 1991, added

this section to Part 71 effective September 16, 1993.

*****************************************************************************

56 FR 65638, No. 242, Dec. 17, 1991

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) to

adopt certain recommendations of the National Airspace Review (NAR)

concerning changes to regulations and procedures in regard to airspace

classifications. These changes are intended to: (1) Simplify airspace

designations; (2) achieve international commonality of airspace designations;

(3) increase standardization of equipment requirements for operations in

various classifications of airspace; (4) describe appropriate pilot

certificate requirements, visual flight rules (VFR) visibility and distance

from cloud rules, and air traffic services offered in each class of airspace;

and (5) satisfy the responsibilities of the United States as a member of the

International Civil Aviation Organization (ICAO). The final rule also amends

the requirement for minimum distance from clouds in certain airspace areas

and the requirements for communications with air traffic control (ATC) in

certain airspace areas; eliminates airport radar service areas (ARSAs),

control zones, and terminal control areas (TCAs) as airspace classifications;

and eliminates the term "airport traffic area." The FAA believes simplified

airspace classifications will reduce existing airspace complexity and thereby

enhance safety.

EFFECTIVE DATE: These regulations become effective September 16, 1993,

except that Secs. 11.61(c), 91.215(d), 71.601, 71.603, 71.605, 71.607, and

71.609 and Part 75 become effective December 12, 1991, and except that

amendatory instruction number 20, Sec. 71.1 is effective as of September 15,

1993, and that Secs. 71.11 and 71.19 become effective October 15, 1992. The

incorporation by reference of FAA Order 7400.7 in Sec. 71.1 (amendatory

instruction number 20) is approved by the Director of the Federal Register

as of December 17, 1991, through September 15, 1993. The incorporation by

reference of FAA Order 7400.9 in Sec. 71.1 (amendatory instruction number 24)

is approved by the Director of the Federal Register as of September 16,

1993 through September 15, 1994.

*****************************************************************************

Sec. 71.75  Extent of Federal airways.

(a) Each Federal airway is based on a center line that extends from one

navigational aid or intersection to another navigational aid (or through

several navigational aids or intersections) specified for that airway.

(b) Unless otherwise specified:

(1) Each Federal airway includes the airspace within parallel boundary

lines 4 miles each side of the center line. Where an airway changes

direction, it includes that airspace enclosed by extending the boundary lines

of the airway segments until they meet.

(2) Where the changeover point for an airway segment is more than 51 miles

from either of the navigational aids defining that segment, and--

(i) The changeover point is midway between the navigational aids, the

airway includes the airspace between lines diverging at angles of 4.5 deg.

from the center line at each navigational aid and extending until they

intersect opposite the changeover point; or

(ii) The changeover point is not midway between the navigational aids, the

airway includes the airspace between lines diverging at angles of 4.5 deg.

from the center line at the navigational aid more distant from the changeover

point, and extending until they intersect with the bisector of the angle of

the center lines at the changeover point; and between lines connecting these

points of intersection and the navigational aid nearer to the changeover

point.

(3) Where an airway terminates at a point or intersection more than 51

miles from the closest associated navigational aid, it includes the

additional airspace within lines diverging at angles of 4.5 deg. from the

center line extending from the associated navigational aid to a line

perpendicular to the center line at the termination point.

(4) Where an airway terminates, it includes the airspace within a circle

centered at the specified navigational aid or intersection having a diameter

equal to the airway width at that point. However, an airway does not extend

into an oceanic control area.

(c) Unless otherwise specified--

(1) Each Federal airway includes that airspace extending upward from 1,200

feet above the surface of the earth to, but not including, 18,000 feet MSL,

except that Federal airways for Hawaii have no upper limits. Variations of

the lower limits of an airway are expressed in digits representing hundreds

of feet above the surface or MSL and, unless otherwise specified, apply to

the segment of an airway between adjoining navigational aids or

intersections; and

(2) The airspace of a Federal airway, within the lateral limits of a Class

E airspace area with a lower floor, has a floor coincident with the floor of

that area.

(d) A Federal airway does not include the airspace of a prohibited area.

Effective Date Note: Amdt. No. 71-14, 56 FR 65656, Dec. 17, 1991, added

this section to Part 71 effective September 16, 1993.

*****************************************************************************

56 FR 65638, No. 242, Dec. 17, 1991

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) to

adopt certain recommendations of the National Airspace Review (NAR)

concerning changes to regulations and procedures in regard to airspace

classifications. These changes are intended to: (1) Simplify airspace

designations; (2) achieve international commonality of airspace designations;

(3) increase standardization of equipment requirements for operations in

various classifications of airspace; (4) describe appropriate pilot

certificate requirements, visual flight rules (VFR) visibility and distance

from cloud rules, and air traffic services offered in each class of airspace;

and (5) satisfy the responsibilities of the United States as a member of the

International Civil Aviation Organization (ICAO). The final rule also amends

the requirement for minimum distance from clouds in certain airspace areas

and the requirements for communications with air traffic control (ATC) in

certain airspace areas; eliminates airport radar service areas (ARSAs),

control zones, and terminal control areas (TCAs) as airspace classifications;

and eliminates the term "airport traffic area." The FAA believes simplified

airspace classifications will reduce existing airspace complexity and thereby

enhance safety.

EFFECTIVE DATE: These regulations become effective September 16, 1993,

except that Secs. 11.61(c), 91.215(d), 71.601, 71.603, 71.605, 71.607, and

71.609 and Part 75 become effective December 12, 1991, and except that

amendatory instruction number 20, Sec. 71.1 is effective as of September 15,

1993, and that Secs. 71.11 and 71.19 become effective October 15, 1992. The

incorporation by reference of FAA Order 7400.7 in Sec. 71.1 (amendatory

instruction number 20) is approved by the Director of the Federal Register

as of December 17, 1991, through September 15, 1993. The incorporation by

reference of FAA Order 7400.9 in Sec. 71.1 (amendatory instruction number 24)

is approved by the Director of the Federal Register as of September 16,

1993 through September 15, 1994.

*****************************************************************************

Sec. 71.77  [Reserved. Amdt. 71-19, 58 FR 12137, Mar. 2, 1993]

Effective date note: Amdt. No. 71-14, 56 FR 65656, Dec. 17, 1991, added

this section to Part 71 effective September 16, 1993.

*****************************************************************************

58 FR 12128, No. 39, Mar. 2, 1993

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) by

designating additional control areas as offshore airspace areas or en route

domestic airspace areas, as appropriate; revising certain additional control

areas; including restricted and prohibited areas in the Continental Control

Area; eliminating domestic area low routes; eliminating control areas

associated with jet routes outside the Continental Control Area; eliminating

domestic high altitude reporting points; eliminating certain domestic low

altitude reporting points; eliminating the special air traffic rules for

Flushing, New York; and replacing the Valparaiso, Florida terminal area and

special air traffic rules with the Elgin, Florida Class D airspace areas.

These amendments respond to recommendations from the National Airspace Review

(NAR) and meet a goal of the Airspace Reclassification final rule--to

simplify airspace assignment and use.

EFFECTIVE DATES: The amendments to Secs. 71.1 and 71.9 which are currently

in effect, become effective April 1, 1993, through September 15, 1993; the

removal of Sec. 71.6 becomes effective April 1, 1993; the removal of subpart

P of part 93 becomes effective June 20, 1993; the amendments to Secs. 71.1,

71.33 and 71.71 which are effective September 16, 1993, and the removal of

Sec. 71.77, become effective September 16, 1993; and the removal of subpart F

of part 93 becomes effective December 9, 1993.

*****************************************************************************

Sec. 71.79  Designation of VOR Federal airways.

Unless otherwise specified the place names appearing in the descriptions of

airspace areas in Subpart E of FAA Order 7400.9B (incorporated by reference,

see Sec. 71.1) designated as VOR Federal airways indicate VOR or VORTAC

navigational facilities identified by those names.

[Amdt. 71-14, 56 FR 65657, Dec. 17, 1991, as amended by Amdt. 71-20, 58 FR

36299, July 6, 1993; Amdt. 71-23, 59 FR 43035, Aug. 22, 1994]

*****************************************************************************

59 FR 43034, No. 161, Aug. 22, 1994

SUMMARY: This action amends the Federal Aviation Regulations relating to

airspace designations to reflect the approval by the Director of the Federal

Register of the incorporation by reference of FAA Order 7400.9B, Airspace

Designations and Reporting Points. This action also explains the procedures

FAA will use to amend the listings of Class A, Class B, Class C, Class D, and

Class E airspace areas and reporting points incorporated by reference.

EFFECTIVE DATE: These regulations are effective September 16, 1994, through

September 15, 1995. The incorporation by reference of FAA Order 7400.9B is

approved by the Director of the Federal Register as of September 16, 1994,

through September 15, 1995.

*****************************************************************************

Subpart F--[Reserved]

Effective Date Note: Amdt. No. 71-14, 56 FR 65657, Dec. 17, 1991, added

Subpart F to Part 71 effective September 16, 1993.

*****************************************************************************

56 FR 65638, No. 242, Dec. 17, 1991

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) to

adopt certain recommendations of the National Airspace Review (NAR)

concerning changes to regulations and procedures in regard to airspace

classifications. These changes are intended to: (1) Simplify airspace

designations; (2) achieve international commonality of airspace designations;

(3) increase standardization of equipment requirements for operations in

various classifications of airspace; (4) describe appropriate pilot

certificate requirements, visual flight rules (VFR) visibility and distance

from cloud rules, and air traffic services offered in each class of airspace;

and (5) satisfy the responsibilities of the United States as a member of the

International Civil Aviation Organization (ICAO). The final rule also amends

the requirement for minimum distance from clouds in certain airspace areas

and the requirements for communications with air traffic control (ATC) in

certain airspace areas; eliminates airport radar service areas (ARSAs),

control zones, and terminal control areas (TCAs) as airspace classifications;

and eliminates the term "airport traffic area." The FAA believes simplified

airspace classifications will reduce existing airspace complexity and thereby

enhance safety.

EFFECTIVE DATE: These regulations become effective September 16, 1993,

except that Secs. 11.61(c), 91.215(d), 71.601, 71.603, 71.605, 71.607, and

71.609 and Part 75 become effective December 12, 1991, and except that

amendatory instruction number 20, Sec. 71.1 is effective as of September 15,

1993, and that Secs. 71.11 and 71.19 become effective October 15, 1992. The

incorporation by reference of FAA Order 7400.7 in Sec. 71.1 (amendatory

instruction number 20) is approved by the Director of the Federal Register

as of December 17, 1991, through September 15, 1993. The incorporation by

reference of FAA Order 7400.9 in Sec. 71.1 (amendatory instruction number 24)

is approved by the Director of the Federal Register as of September 16,

1993 through September 15, 1994.

*****************************************************************************

Subpart G--[Reserved]

Effective Date Note: Amdt. No. 71-14, 56 FR 65657, Dec. 17, 1991, added

Subpart G to Part 71 effective September 16, 1993.

*****************************************************************************

56 FR 65638, No. 242, Dec. 17, 1991

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) to

adopt certain recommendations of the National Airspace Review (NAR)

concerning changes to regulations and procedures in regard to airspace

classifications. These changes are intended to: (1) Simplify airspace

designations; (2) achieve international commonality of airspace designations;

(3) increase standardization of equipment requirements for operations in

various classifications of airspace; (4) describe appropriate pilot

certificate requirements, visual flight rules (VFR) visibility and distance

from cloud rules, and air traffic services offered in each class of airspace;

and (5) satisfy the responsibilities of the United States as a member of the

International Civil Aviation Organization (ICAO). The final rule also amends

the requirement for minimum distance from clouds in certain airspace areas

and the requirements for communications with air traffic control (ATC) in

certain airspace areas; eliminates airport radar service areas (ARSAs),

control zones, and terminal control areas (TCAs) as airspace classifications;

and eliminates the term "airport traffic area." The FAA believes simplified

airspace classifications will reduce existing airspace complexity and thereby

enhance safety.

EFFECTIVE DATE: These regulations become effective September 16, 1993,

except that Secs. 11.61(c), 91.215(d), 71.601, 71.603, 71.605, 71.607, and

71.609 and Part 75 become effective December 12, 1991, and except that

amendatory instruction number 20, Sec. 71.1 is effective as of September 15,

1993, and that Secs. 71.11 and 71.19 become effective October 15, 1992. The

incorporation by reference of FAA Order 7400.7 in Sec. 71.1 (amendatory

instruction number 20) is approved by the Director of the Federal Register

as of December 17, 1991, through September 15, 1993. The incorporation by

reference of FAA Order 7400.9 in Sec. 71.1 (amendatory instruction number 24)

is approved by the Director of the Federal Register as of September 16,

1993 through September 15, 1994.

*****************************************************************************

Subpart H--Reporting Points

Effective Date Note: Amdt. No. 71-14, 56 FR 65657, Dec. 17, 1991, added

Subpart H to Part 71 effective September 16, 1993.

*****************************************************************************

56 FR 65638, No. 242, Dec. 17, 1991

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) to

adopt certain recommendations of the National Airspace Review (NAR)

concerning changes to regulations and procedures in regard to airspace

classifications. These changes are intended to: (1) Simplify airspace

designations; (2) achieve international commonality of airspace designations;

(3) increase standardization of equipment requirements for operations in

various classifications of airspace; (4) describe appropriate pilot

certificate requirements, visual flight rules (VFR) visibility and distance

from cloud rules, and air traffic services offered in each class of airspace;

and (5) satisfy the responsibilities of the United States as a member of the

International Civil Aviation Organization (ICAO). The final rule also amends

the requirement for minimum distance from clouds in certain airspace areas

and the requirements for communications with air traffic control (ATC) in

certain airspace areas; eliminates airport radar service areas (ARSAs),

control zones, and terminal control areas (TCAs) as airspace classifications;

and eliminates the term "airport traffic area." The FAA believes simplified

airspace classifications will reduce existing airspace complexity and thereby

enhance safety.

EFFECTIVE DATE: These regulations become effective September 16, 1993,

except that Secs. 11.61(c), 91.215(d), 71.601, 71.603, 71.605, 71.607, and

71.609 and Part 75 become effective December 12, 1991, and except that

amendatory instruction number 20, Sec. 71.1 is effective as of September 15,

1993, and that Secs. 71.11 and 71.19 become effective October 15, 1992. The

incorporation by reference of FAA Order 7400.7 in Sec. 71.1 (amendatory

instruction number 20) is approved by the Director of the Federal Register

as of December 17, 1991, through September 15, 1993. The incorporation by

reference of FAA Order 7400.9 in Sec. 71.1 (amendatory instruction number 24)

is approved by the Director of the Federal Register as of September 16,

1993 through September 15, 1994.

*****************************************************************************

Sec. 71.901  Applicability.

Unless otherwise designated:

(a) Each reporting point listed in Subpart H of FAA Order 7400.9B

(incorporated by reference, see Sec. 71.1) applies to all directions of

flight. In any case where a geographic location is designated as a reporting

point for less than all airways passing through that point, or for a

particular direction of flight along an airway only, it is so indicated by

including the airways or direction of flight in the designation of

geographical location.

(b) Place names appearing in the reporting point descriptions indicate VOR

or VORTAC facilities identified by those names.

[Amdt. 71-14, 56 FR 65657, Dec. 17, 1991, as amended by Amdt. 71-20, 58 FR

36299, July 6, 1993; Amdt. 71-23, 59 FR 43035, Aug. 22, 1994]

*****************************************************************************

59 FR 43034, No. 161, Aug. 22, 1994

SUMMARY: This action amends the Federal Aviation Regulations relating to

airspace designations to reflect the approval by the Director of the Federal

Register of the incorporation by reference of FAA Order 7400.9B, Airspace

Designations and Reporting Points. This action also explains the procedures

FAA will use to amend the listings of Class A, Class B, Class C, Class D, and

Class E airspace areas and reporting points incorporated by reference.

EFFECTIVE DATE: These regulations are effective September 16, 1994, through

September 15, 1995. The incorporation by reference of FAA Order 7400.9B is

approved by the Director of the Federal Register as of September 16, 1994,

through September 15, 1995.

*****************************************************************************